3GPP TSG-RAN WG3 #127                                       R3-250640

Athens, Greece, 17 - 21 February, 2025

Agenda item:	13.2

Source:	Samsung

Title:	Discussion on Inter-gNB-CU LTM

Document for:	Discussion & Decision

# Introduction

RAN3 had some discussions on inter-CU LTM in previous meetings, and below agreements are achieved:

RAN3#125: Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting: Option 1: Reuse CHO Cancel Option 2: Rename CHO Cancel Option 3: Introduce new procedure The following documents are agreed R3-244836 (TP for LTM BLCR for TS38.300): Inter-CU LTM basic procedure RAN3#125bis: Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered. The LTM Configuration IDs are allocated by the source CU. WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS. Continue to work on the solutions comparison and details… The following documents are endorsed R3-245032 (BL CR to 38.300) Inter-CU LTM basic procedure The following documents are agreed R3-245841 (TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure R3-245842 (TP for TS 38.423) Xn support for inter-CU LTM RAN3#126: RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message. Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation. Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation? Continue to work on the solutions comparison and details…

In this paper, we will further discuss the basic procedure for inter-gNB-CU LTM based on agreed BLCR in [1], [2], and discuss the potential issues on Xn/F1 interface.

# Discussion

To achieve the analysis for network interface impact for the scenario of inter-gNB-CU LTM, we draw the potential message flows of the whole inter-gNB-CU LTM process as shown in the below Fig.1 to provide the basic description for each involved message flows and our view for the issues raised in stage 2 BLCR in [1] and [2]. After that, we finally summarize our proposals.

Fig.1 Message Flows for Inter-gNB-CU LTM

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the LTM lower layer RRC configurations.

6.  The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

10.	The candidate gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information of candidate cells for subsequent LTM. The candidate gNB-CU may include the common CSI-RS resource configuration.

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message. The candidate gNB-DU may also provide the CSI-RS report configuration.

12.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 9 and 12 are mandatory or optional.

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB-DU.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE 1: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

18.	Early synchronization to the target candidate cell(s) may be performed.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via the TA INFORMATION TRANSFER message.

Editor’s Note: Details are FFS on step 20.

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB-DU.

23.	The source gNB-DU determines to initiate inter-gNB-CU LTM.

24.	The source gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, the TCI state ID(s), and the TA values for subsequent LTM.

26.	The source gNB-CU sends the CELL SWITCH NOTIFICATION message to the target gNB-CU to indicate the initiation of Cell Switch command to the UE.

Editor’s Note: Details are FFS on step 26.

27.	The target gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The target gNB-DU detects the UE access.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

31.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

32/33. The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell. In return, the source gNB-CU sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE2:	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

34. The target gNB-CU sends a PATH SWITCH REQUEST message to 5GC to trigger 5GC to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

35.	The 5GC confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB-CU.

36.	The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

Editor’s Note: The new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs. Details are FFS.

37. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

38.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

39.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

NOTE 3: The steps 17-39 can be performed multiple times for subsequent LTM cell switch executions.

From the above overview of inter-gNB-CU LTM procedure, the impacts on Xn/F1 interface are mainly reflected in the following aspects:

## Issue 1: Early TA RACH resource allocation

In RAN2#125bis/126 meeting, the following agreement was reached on early UL Sync:

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

From the RAN2’s agreements, we note Rel-18 option is baseline for early TA acquisition. In R18, early TA acquisition by PDCCH order is supported, and the dedicated different RACH resource is allocated by candidate cell for each candidate gNB-DU. RACH resources could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different DU.

In our view, this mechanism is also supported for inter-gNB-CU case and we should allow the possibility of RACH resource shared only for among UEs belonging to same DU. Hence, if LTM gNB is a split gNB, dedicated different RACH resource still can be allocated by candidate cell for each candidate gNB-DU.

However, only gNB-DU ID information is not enough since two gNB-DUs of different gNB-CUs may have the same gNB-DU ID and candidate gNB does not know which gNB the gNB-DU belongs to. Therefore, both the gNB-DU ID IE and the belonging gNB ID IE should be included in the HANDOVER REQUEST message to candidate gNB for requesting early TA RACH resource.

Moreover, to support subsequent LTM, early TA RACH resource can be allocated previously by candidate cell for each LTM gNB-DUs. Therefore, HANDOVER REQUEST is initiated per-candidate cell by source gNB-CU which contains a list of pair <gNB-DU ID and associated belonging gNB ID>, and next, candidate gNB-CU carries this list of the pair <gNB-DU ID and associated belonging gNB ID> in the UE CONTEXT SETUP REQUEST message to request early TA RACH resource to candidate gNB-DU to which candidate cell belongs. Each candidate cell in candidate gNB-DU allocates different dedicated early TA RACH resources based on each pair of <gNB-DU ID and associated belonging gNB ID>.

Therefore, based on above discussion and Early TA RACH resource allocation mechanism, we propose as below:

Proposal 1-1: Introduce a list of the pair <gNB-DU ID and associated belonging gNB ID> in the HANDOVER REQUEST and UE CONTEXT SETUP REQUEST message for the request for early TA RACH resource to each candidate cell.

Proposal 1-2: Each candidate cell in candidate gNB-DU allocates different dedicated early TA RACH resources based on each pair of <gNB-DU ID and associated belonging gNB ID>. And therefore, introduce a list of RACH resource for each pair <gNB-DU ID and associated belonging gNB ID> in the UE CONTEXT SETUP RESPONSE and HANDOVER REQUEST ACKNOWLEDGE message.

## Issue 2: FFS the message type and detail in TA INFORMATION TRANSFER message

Regarding early TA information transfer from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU in step 20,

We already agreed to introduce a new TA INFORMATION TRANSFER message to transfer the TA information. But whether it is a UE associated message or non-UE associated message is still FFS. Since R18 F1AP DU-CU/CU-DU TA INFORMATION TRANSFER message is a non-UE associated message, to inherit the legacy principle, we suggest this XnAP message is also specified as a non-UE associated message. Furthermore, following the current detail IEs in F1AP DU-CU/CU-DU TA INFORMATION TRANSFER, we have a proposal below.

Proposal 2-1: Inherit related F1AP TA Information Transfer message type and including IEs, XnAP TA INFORMATION TRANSFER message is defined as a non-UE associated XnAP message to transfer early TA information from candidate gNB to source gNB including related LTM information below.

- XnAP TA INFORMATION TRANSFER message to include:

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Tag ID Pointer

  - Source gNB-DU ID (indicate the corresponding source gNB-DU to which source gNB-CU forwards TA information)

PDCCH ordered early RA procedure is supported for inter-CU LTM, while candidate gNB-DU calculates out the TA value according to Msg1 from UE, early TA information transfer procedure is triggered from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU.

F1AP non-UE-associated DU-CU TA INFORMATION TRANSFER message could be reused to transfer TA value and the associated RACH resource information from the candidate gNB-DU to candidate gNB-CU.

However, only indicating source gNB-DU ID and associated RACH resource information in DU-CU TA INFORMATION TRANSFER message as in current specification is not enough, since candidate gNB-CU does not know which gNB/gNB-CU via a non-UE-associated XnAP signalling to forward this information to.

Therefore, source gNB ID IE is also indicated to candidate gNB-CU in the F1AP DU-CU TA INFORMATION TRANSFER message.

Proposal 2-2: Both source gNB-DU ID and source gNB ID are included in F1AP non-UE-associated DU-CU TA INFORMATION TRANSFER message, for candidate gNB-CU to forward TA information to source gNB-CU indicated by source gNB ID using XnAP non-UE-associated signaling.

## Issue 3: FFS the LTM configuration update procedure for subsequent LTM

For supporting subsequent inter-CU LTM, RAN3 have reached the following agreement last meeting:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Confirm the name of the new procedure as “LTM Configuration Update”.

FFS on whether this procedure is mandatory or optional and FFS on details in message.

To support subsequent inter-CU LTM, the LTM information (e.g., early sync configuration, LTM configuration ID, and data forwarding addresses, etc) may need to sync up among candidate gNBs.

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

Similar as F1AP, there are possible three options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) during the initial LTM preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB (non-split gNB) sends LTM MAC CE to UE or source gNB-CU (split gNB) receives the DU-CU Cell Switch Notification message.

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

We think all the options can work and therefore this procedure can be optional. However, from the view of the preparation for CSI resource/report configuration, in last RAN3#126 meeting, below agreement is reached.

- The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. - The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Therefore, LTM preparation for CSI resource/report configuration should be ready via Handover and LTM Configuration Update procedure before RRC Reconfiguration to UE. From this sense, we think the better approach is Option 1 and prefer to make this procedure mandatory.

Proposal 3: RAN3 to agree that LTM Configuration Update procedure is a mandatory procedure which is sent to candidate gNBs during initial LTM preparation since LTM CSI resource/report configuration can be generated via Handover and LTM Configuration Update procedure.

## Issue 4: Data forwarding mechanism for subsequent LTM

This issue is raised in previous meeting, for subsequent LTM, how to transmit the collected data forwarding information to the candidate gNB(s) need to be further discussed in RAN3.

Data forwarding is a mechanism that can reduce data transmission interruption during inter-gNB (conditional) handover procedure. In current data forwarding principle, source gNB generates the data forwarding proposal via the Data Forwarding and Offloading Info from source NG-RAN node IE in HO REQ message, target gNB provides the data forwarding addresses per-PDU session or per-DRB via the Data Forwarding Info from target NG-RAN node IE in HO REQ ACK message.

From the view of RAN3, data forwarding is also supported for inter-CU LTM. However, in subsequent inter-CU LTM, the source gNB will be changed after the execution of LTM, any one candidate gNB may be changed as source gNB for the following execution of LTM. Based on previous discussion, data forwarding information may be transferred by below possible three options:

- Option 1: Send the collected data forwarding addresses information to the candidate gNB(s) during the initial LTM preparation.

- Option 2: Send the collected data forwarding addresses information to the target gNB via Cell Switch Notification message when the LTM is triggered.

- Option 3: Send the collected data forwarding addresses information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

Regarding the Option1, one potential issue is that these data forwarding addresses information are never be used if UE never does access this candidate gNB when the whole subsequent LTM phase is end.

Regarding the Option2, one potential issue is that UE may be failure to access target gNB when LTM is executed, these data forwarding addresses information is still not used.

From my view, option3 is best which avoid the drawback of option1&2, it is more reliable and not introduce information waste.

In short, based on above analysis, in order to avoid to introduce information waste and adopt a more reliable way, we have below proposals for the data forwarding mechanism for subsequent LTM.

Proposal 4: Source gNB sends all the data forwarding addresses information to target gNB when the source gNB receives the Handover Success message from the target gNB.

## Issue 5: FFS the detail in Cell Switch Notification message

Regarding Cell Switch Notification from source gNB-DU to candidate gNB-DU via source gNB-CU and candidate gNB-CU, we already agree to introduce a new XnAP Cell Switch Notification message. But the details are FFS on this message.

Following the current detail IEs in F1AP DU-CU CELL SWITCH NOTIFICATION in R18 intra-CU LTM, since TA values may be also forwarded from source gNB-DU to candidate gNB-DU for subsequent LTM use, we have a below proposal.

Proposal 5: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 6: FFS the detail for the procedure that the new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs

In last RAN3#126 meeting, below agreement is reached to support subsequent LTM, and the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

In plus, we need to consider how to identify the same UE when candidate gNB receives the LTM Configuration Update message from new source gNB. One possible solution is that initial source node UE XnAP ID or last source node UE XnAP ID information is transferred from new source gNB to other candidate gNBs.

Due to initial source node is possible to be released if no LTM candidate cells is existed in initial source node when first time LTM is executed, initial source node UE XnAP ID information may be released in candidate gNBs. So it is more reliable that last source node UE XnAP ID information is transferred to other candidate gNBs.

Proposal 6: Introduce last source node UE XnAP ID in LTM Configuration Update message from new source gNB to other candidate gNBs after inter-CU LTM Cell Switch, so that candidate gNB can identify the same UE and setup the UE associated Xn singnalling connection.

## Issue 7: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Regarding the LTM candidate cells cancel or release, based on previous agreements, source gNB or candidate gNB can initiate LTM cells cancel/release by Handover Cancel message or LTM Cancel message.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

However, after each inter-CU LTM Cell Switch, new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs via LTM Configuration Update/Acknowledge. If some LTM candidate cells are released due to some reasons, this procedure cannot be initiated by source gNB or candidate gNB until the setup for the UE associated Xn singnalling connection between new source gNB and candidate gNB. Therefore, if LTM candidate cells cancel/release can be initiated by the LTM Configuration Update/Acknowledge message, the following Handover Cancel or LTM Cancel does not need be initiated which can save signalling procedure.

Therefore, it has a benefit to introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge if there are LTM candidate cells to be released during the setup for UE associated Xn singnalling connection.

Proposal 7: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

## Issue 8: Signaling enhancement on XnAP and F1AP to support CSI-RS configuration during LTM preparation phase

In last meeting, RAN3 agreed to add information of CSI-RS configuration in addition to SSB in the XnAP messages as below to follow decision to support CSI-RS for L1 measurement.

- The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. - The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

CSI-RS configuration/report information is well captured in the HANDOVER REQUEST/ACK and LTM CONFIGURATION UPDATE/ACK message.

We have missed only one point related to the subsequent LTM. That is, when we consider subsequent LTM, CSI-RS configuration of source gNB-CU which transmits HANDOVER REQUEST message should also be included in the message, as we did for SSB (CSI Resource Configuration IE).

Therefore, we can simply update BLCR including corresponding IE as below.

Proposal 8-1. RAN3 agree to add CSI-RS Resource Configuration in the HANDOVER REQUEST message.

In addition to XnAP message, same enhancement is needed for F1AP. We can simply do the same work in the UE CONTEXT SETUP/MODIFICATION REQUEST/RESPONSE message, i.e., include CSI-RS Resource Configuration and Report IE in the corresponding message.

Proposal 8-2. RAN3 agree to enhance F1AP message to include CSI-RS measurement related information similar to XnAP.

#### 9.2.1.xx1	LTM Information Request

This IE contains a request for LTM information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Configuration | O |  | OCTET STRING |  |
| CSI Resource Configuration | O |  | OCTET STRING |  |
| CSI-RS Resource Configuration | O |  | OCTEC STRING |  |
| LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  |
| Request For CSI-RS Configuration | O |  | ENUMERATED (true, …)(FFS) |  |

As CSI-RS Resource Configuration IE is included in the LTM Information Request, we can remove duplicated information in the LTM UPDATE CONFIGURATION message. It is captured in the tabular below.

#### 9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Early Sync Information | O |  | FFS |  | YES | ignore |
| Early Sync Information for SUL | O |  | FFS |  | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| C | O |  | O |  | Y | i |

# Conclusion

This contribution firstly analyses the whole message flows of inter-CU LTM and propose some enhancements for Xn interface on below aspects. Therefore, we invite RAN3 to agree below proposals for supporting the basic inter-CU LTM procedure.

## Issue 1: Early TA RACH resource allocation

Proposal 1-1: Introduce a list of the pair <gNB-DU ID and associated belonging gNB ID> in the HANDOVER REQUEST and UE CONTEXT SETUP REQUEST message for the request for early TA RACH resource to each candidate cell.

Proposal 1-2: Each candidate cell in candidate gNB-DU allocates different dedicated early TA RACH resources based on each pair of <gNB-DU ID and associated belonging gNB ID>. And therefore, introduce a list of RACH resource for each pair <gNB-DU ID and associated belonging gNB ID> in the UE CONTEXT SETUP RESPONSE and HANDOVER REQUEST ACKNOWLEDGE message.

## Issue 2: FFS the message type and detail in TA INFORMATION TRANSFER message

Proposal 2-1: Inherit related F1AP TA Information Transfer message type and including IEs, XnAP TA INFORMATION TRANSFER message is defined as a non-UE associated XnAP message to transfer early TA information from candidate gNB to source gNB including related LTM information below.

- XnAP TA INFORMATION TRANSFER message to include:

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Tag ID Pointer

  - Source gNB-DU ID (indicate the corresponding source gNB-DU to which source gNB-CU forwards TA information)

Proposal 2-2: Both source gNB-DU ID and source gNB ID are included in F1AP non-UE-associated DU-CU TA INFORMATION TRANSFER message, for candidate gNB-CU to forward TA information to source gNB-CU indicated by source gNB ID using XnAP non-UE-associated signaling.

## Issue 3: FFS the LTM configuration update procedure for subsequent LTM

Proposal 3: RAN3 to agree that LTM Configuration Update procedure is a mandatory procedure which is sent to candidate gNBs during initial LTM preparation since LTM CSI resource/report configuration can be generated via Handover and LTM Configuration Update procedure.

## Issue 4: Data forwarding mechanism for subsequent LTM

Proposal 4: Source gNB sends all the data forwarding addresses information to target gNB when the source gNB receives the Handover Success message from the target gNB.

## Issue 5: FFS the detail in Cell Switch Notification message

Proposal 5: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 6: FFS the detail for the procedure that the new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs

Proposal 6: Introduce last source node UE XnAP ID in LTM Configuration Update message from new source gNB to other candidate gNBs after inter-CU LTM Cell Switch, so that candidate gNB can identify the same UE and setup the UE associated Xn singnalling connection.

## Issue 7: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Proposal 7: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

## Issue 8: Signaling enhancement on XnAP and F1AP to support CSI-RS configuration during LTM preparation phase

Proposal 8-1. RAN3 agree to add CSI-RS Resource Configuration in the HANDOVER REQUEST message.

Proposal 8-2. RAN3 agree to enhance F1AP message to include CSI-RS measurement related information similar to XnAP.

# Reference

[1] R3-247932 (BL CR to 38.300) Support for Inter-CU LTM

[2] R3-247933 (BL CR to 38.423) Xn support for inter-CU LTM