3GPP TSG-RAN WG3 Meeting #127	R3-250645

Athens, Greece, February 17th – 21st, 2025

Agenda item:	13.3

Source: 	LG Electronics Inc.

Title: 	Discussions on RAN3 impacts from Conditional Intra-CU LTM

Document for:	Discussion and Decision

# Introduction

One other main topic of Rel-19 mobility enhancement [1] has finally opened for discussions in RAN3:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

The first focus of Rel-19 Conditional LTM (C-LTM) is on the basics of “Intra-CU” and “non-DC” scenario, for which other WGs have already progressed a bit. Especially, the leading WG (RAN2) has discussed from October 2024 and achieved the following progress [2]:

## RAN2-127bis (Hefei, October 2024)

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

## RAN2-128 (Orlando, November 2024)

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Considering the above, we discuss potential impacts on our existing RAN3 Intra-CU LTM designs for C-LTM.

# Discussion

## General

The core design of LTM is based on a common CSI resource configuration and a cell-specific CSI report configuration. The latter configuration was central to NW-triggered mobility as it determines how a UE, while being served under a specific cell, performs L1 measurement reporting so that NW can make cell-switching decisions for LTM.

Conditional LTM (C-LTM) aims to shift this responsibility by allowing NW to pre-configure some conditions for the UE. These conditions enable the UE to autonomously trigger a cell switch without requiring explicit instructions from NW at the right moment. This concept is nothing new, and has been extensively abused from Rel-16 mobility enhancements. For C-LTM, what we need for the “cell-specific configuration” part would be the triggering (execution) conditions, instead of CSI “report” configuration.

Given this, we think it would be over-engineering to completely distinguish C-LTM from existing NW-triggered LTM from the feature level or to re-design it from scratch. Instead, C-LTM should be built upon the existing F1AP signaling framework designed for Rel-18 NW-triggered LTM, ensuring harmonization at least on a per-candidate-cell basis: NW should have the flexibility to configure a UE such that traditional LTM is applied for one candidate cell, while C-LTM operates for another candidate cell, all through a single RRC configuration upfront. Our works for C-LTM should carve this in mind from the beginning.

Proposal 1: Leverage the existing F1AP signaling framework for Rel-18 NW-triggered LTM as the foundation for Rel-19 C-LTM, rather than designing a completely separate signaling framework.

Proposal 2: Ensure that the F1AP framework supports the flexibility to configure both traditional LTM and C-LTM for different candidate cells within the same UE, allowing a single RRC reconfiguration to configure traditional LTM for some candidate cells while enabling C-LTM for others.

## Impacts from C-LTM preparation

RAN2 has already agreed on the use of both L1-based and L3-based execution conditions for LTM. The L1-based execution condition is evaluated using certain L1 measurement results, measured from the (common) CSI resources configured for LTM. Similar to CSI report configuration, it is the DU’s responsibility to generate an L1-based execution condition.

L3-based execution condition (generated by CU) is also possible. If L3-based condition is prepared for a candidate cell, L1-based condition for the same cell may no longer be necessary. Or, both can be configured and evaluated simultaneously, which appears feasible according to RAN2’s progress so far.

In Rel-18, the F1AP signaling for LTM was designed so that DU (always) generates and provides the cell-specific CSI report configuration whenever it receives the CSI resource configuration from CU. As a result, for C-LTM, CU needs to explicitly instruct a DU to generate L1-based execution conditions instead of CSI report configuration when providing the CSI resource configuration. On the other hand, if CU decides to use only L3-based execution conditions for a specific candidate cell, then L1-based execution condition from the corresponding DU is unnecessary. In this case, the CSI resource configuration (necessary for building L1-based execution condition) can be simply not provided to the DU, ensuring the DU neither generates a CSI report configuration nor an L1-based execution condition.

Proposal 3: For C-LTM preparation, CU should explicitly tell the DU to generate L1-based execution condition, not CSI report configuration, while providing CSI resource configuration. A simple “C-LTM” indicator added into LTM Information Setup IE and LTM Information Modify IE would suffice.

RAN2 also agreed the following:

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

The first bullet needs a special attention as it requires the C-LTM configuration of each candidate cell include something generated by the other cell, i.e. initial source cell.

If C-LTM is configured for a UE, an initial cell switch from the UE’s current serving cell can be based on C-LTM. In this case, the first bullet means that the execution conditions evaluated at the current serving cell are NOT configured via the current serving cell’s configuration (who generated those initial execution conditions). Instead, they are configured via each candidate cell’s configuration (i.e. LTM-Candidate-r18) separately [3]:

We provide the Figure 2 as an illustration: In Figure 2, there are three CLTM configurations. Each CLTM configuration includes the initial execution condition (blue font) and a list of subsequent execution conditions (green font). Taking CLTM configuration 1 as an example, when the candidate cell 1 fulfils the Condition 1, UE performs CLTM and cell 1 becomes the serving cell. Then UE evaluates the Condition 1-2 for the candidate cell 2 and the Condition 1-3 for the candidate cell 3 for subsequent conditional LTM. Figure 2. RRC structure for the configuration of execution conditions A potential ASN.1 structure is provided in Annex.

This means that, if L1-based C-LTM is decided at the initial source cell, those L1-based execution conditions generated by S-DU needs to be provided to the CU separately for each candidate cell, so that CU can compile each initial execution condition into each candidate cell’s LTM configuration. F1AP impact is foreseen.

In case L3-based C-LTM is decided at the initial source cell, we see no F1 impact as it is the CU who generates initial L3 execution conditions and compiles final LTM configurations of candidate cells.

Proposal 4: L1-based C-LTM from the initial source cell (i.e. current serving cell) requires the S-DU to supply the generated L1-based execution conditions for each candidate cell separately to the CU, so that CU can compile each condition into each candidate cell’s LTM configuration accordingly.

Proposal 5: For that, enhance the F1AP UE CONTEXT MODIFICATION RESPONSE message > LTM Configuration IE to include a list of execution conditions generated from the current serving cell for each candidate cell.

The second bullet does not require special attentions, as it is typical for a cell’s LTM configuration to include conditions that it generates for the UE. However, if RAN2's RRC structure places the execution conditions for subsequent C-LTM (i.e., those that must be evaluated when a candidate cell becomes the serving cell) separately under the LTM-Candidate-r18 of that candidate cell, a special attention is required for L1-based execution conditions, because the candidate cell’s DU must also provide these conditions explicitly to the CU (not hidden under the candidate cell’s CellGroupConfig).

Currently, there is no requirement for a candidate cell’s DU to supply those subsequent C-LTM L1-based execution conditions for other candidate cells individually. The DU just needs to provide the whole for inclusion into its LTM-Candidate-r18. However, to maintain consistency in the F1AP signaling structure, we think it is better to harmonize the stage-3 designs of delivering initial and subsequent C-LTM L1-based execution conditions to CU. In fact, Proposal 5, which propose to enhance the F1AP UE CONTEXT MODIFICATION RESPONSE message to carry L1-based execution conditions from the initial source cell, can also be applied for carrying subsequent C-LTM L1-based execution conditions from a candidate cell. Then, the gap is on the F1AP UE CONTEXT SETUP RESPONSE message for which we can enhance similarly as in Proposal 5.

Proposal 6: To ensure F1AP signalling consistency in delivering L1-based execution conditions for subsequent C-LTM, also enhance the F1AP UE CONTEXT SETUP RESPONSE message > LTM Configuration IE to include a list of execution conditions generated by a candidate cell for other candidate cells.

## Impacts from C-LTM early synchronization

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

From the above RAN2 agreements, we can see that the traditional LTM mechanisms, where early UL synchronization is triggered by PDCCH and early DL synchronization by MAC CE, are re-used for C-LTM. As in traditional LTM, these triggers normally come from a DU serving the UE. However, one key difference in C-LTM is that CU can also trigger early UL synchronization based on L3 measurement reports. This requires new F1AP signaling, as CU needs to instruct the serving DU to initiate PDCCH-ordered early RACH for the UE. In fact, this aligns with the Rel-18 FR1 L3-based LTM discussions, which also allows the CU to manage LTM operations using L3 measurement reports. There, a new class-2 F1AP signaling from CU to command the serving DU to trigger early UL/DL synchronizations and/or cell switching is under discussions. This signaling agreed there can be applied for C-LTM, for the CU to trigger early TA acquisition based on L3 measurement reports.

Moreover, in C-LTM, enabling RACH-less LTM requires that a TA value obtained by a candidate cell and forwarded to the serving cell be configured in the UE before it executes the cell switch toward that candidate cell. This is different from the traditional NW-triggered LTM, where a TA value is configured in the UE only when NW instructs a cell switch (i.e., via the MAC CE Cell Switch Command). In contrast, C-LTM requires that only a TA value be pre-configured in the UE before the UE executes cell switching. To support this, RAN2 has agreed on a new MAC CE, and for this, no further F1AP impact is foreseen.

It is still unclear whether early DL synchronization (TCI state activation/deactivation) can be triggered by CU based on L3 measurement reports for C-LTM. But, RAN3 can address once RAN2 makes more progress. In the meantime, our focus can remain on the CU to DU F1AP signaling for triggering early TA acquisition.

Proposal 7: For C-LTM, define a new class-2 F1AP signaling from CU to command the serving DU to trigger early TA acquisition. If the similar F1AP signalling is agreed for Rel-18 FR1 L3-based LTM, it can be re-used for C-LTM only for triggering early TA acquisition.

Proposal 8: For C-LTM, FFS whether early DL synchronization can also be triggered from CU, pending RAN2 progress.

## Impacts from C-LTM execution

The main impact of UE’s autonomous execution is when a C-LTM execution happens across DUs.

In C-LTM, the existing Cell Switch Notification mechanism cannot be applicable because the serving DU does not know when the UE will execute a cell switch. Unless RAN2 defines a completely new mechanism that allows the UE to “notify” the serving DU of its cell switch and which target cell right after execution (which seems unlikely), the serving DU remains oblivious of the UE’s executions.

This is expected, as Cell Switch Notification is entirely designed for NW-triggered mobility (C-LTM follows a fundamentally different philosophy).

Figure 1: Inter-DU C-LTM execution procedures

Therefore, as illustrated in Figure 1, the previous serving DU, which is unaware that an Inter-DU C-LTM has been executed by the UE, requires additional notification from the CU to properly stop data transmission for the UE, revert the previous serving cell to a prepared state (if it remains one of the configured LTM candidate cells), and inform any DL packets not successfully delivered to the UE for retransmissions by the CU.

Such additional Step 5-7 handling (instead of the existing Cell Switch Notification) is not new, which has been well-documented in our RAN3 specifications for conditional mobilities of previous releases [4]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 8.2.1.3	Inter-gNB-DU Conditional Handover or Conditional PSCell Change or subsequent CPAC

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for conditional handover or conditional PSCell change or subsequent CPAC. Figure 8.2.1.3-1 shows the inter-gNB-DU conditional mobility procedure for intra-NR.

Figure 8.2.1.3-1: Inter-gNB-DU Conditional Handover or Conditional PSCell Change or subsequent CPAC for intra-NR

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

13.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU and indicates to stop the data transmission for the UE. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE 1:	The step 13 may happen before step 12, as soon as the gNB-CU knows which cell the UE has successfully accessed.

NOTE 2:	The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate target gNB-DUs, if any, to cancel conditional handover or conditional PSCell change for the UE.

14.	The source gNB-DU responds to the gNB-CU with the UE CONTEXT MODIFICATION RESPONSE message.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Proposal 9: For C-LTM, do not consider the existing Cell Switch Notification mechanism (designed entirely for NW-triggered mobility) unless RAN2 defines a completely new mechanism that allows the UE to “notify” the serving DU of its cell switch and which target cell right after C-LTM execution.

Proposal 10: Instead, for C-LTM, after CU becomes aware of the UE’s Inter-DU cell switch execution through ACCESS SUCCESS, informs the previous serving DU, similarly following TS 38.401 Figure 8.2.1.3-1 Steps 13-14. No stage-3 F1AP impact is foreseen.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## General

Proposal 1: Leverage the existing F1AP signaling framework for Rel-18 NW-triggered LTM as the foundation for Rel-19 C-LTM, rather than designing a completely separate signaling framework.

Proposal 2: Ensure that the F1AP framework supports the flexibility to configure both traditional LTM and C-LTM for different candidate cells within the same UE, allowing a single RRC reconfiguration to configure traditional LTM for some candidate cells while enabling C-LTM for others.

## Impacts from C-LTM preparation

Proposal 3: For C-LTM preparation, CU should explicitly tell the DU to generate L1-based execution condition, not CSI report configuration, while providing CSI resource configuration. A simple “C-LTM” indicator added into LTM Information Setup IE and LTM Information Modify IE would suffice.

Proposal 4: L1-based C-LTM from the initial source cell (i.e. current serving cell) requires the S-DU to supply the generated L1-based execution conditions for each candidate cell separately to the CU, so that CU can compile each condition into each candidate cell’s LTM configuration accordingly.

Proposal 5: For that, enhance the F1AP UE CONTEXT MODIFICATION RESPONSE message > LTM Configuration IE to include the list of execution conditions generated from the current serving cell for each candidate cell.

Proposal 6: To ensure F1AP signalling consistency in delivering L1-based execution conditions for subsequent C-LTM, also enhance the F1AP UE CONTEXT SETUP RESPONSE message > LTM Configuration IE to include a list of execution conditions generated by a candidate cell for other candidate cells.

## Impacts from C-LTM early synchronization

Proposal 7: For C-LTM, define a new class-2 F1AP signaling from CU to command the serving DU to trigger early TA acquisition. If the similar F1AP signalling is agreed for Rel-18 FR1 L3-based LTM, it can be re-used for C-LTM only for triggering early TA acquisition.

Proposal 8: For C-LTM, FFS whether early DL synchronization can also be triggered from CU, pending RAN2 progress.

## Impacts from C-LTM execution

Proposal 9: For C-LTM, do not consider the existing Cell Switch Notification mechanism (designed entirely for NW-triggered mobility) unless RAN2 defines a completely new mechanism that allows the UE to “notify” the serving DU of its cell switch and which target cell right after C-LTM execution.

Proposal 10: Instead, for C-LTM, after CU becomes aware of the UE’s Inter-DU cell switch execution through ACCESS SUCCESS, informs the previous serving DU, similarly following TS 38.401 Figure 8.2.1.3-1 Steps 13-14. No stage-3 F1AP impact is foreseen.

The corresponding 38.401 and F1AP TPs are also proposed in [5].

# Reference

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom

[2] R3-250008/R2-2411134, RAN2 agreements on Inter-CU LTM and Conditional LTM, RAN2

[3] R2-2410022, “Discussion on conditional LTM”, Xiaomi

[4] TS 38.401, NG-RAN; Architecture description (Release 18)

[5] R3-250646, “(TP for NR_Mob_Ph4 TS 38.401 and TS 38.473) Conditional LTM”, LG Electronics Inc.