3GPP TSG-RAN WG3 Meeting #127	R3-250660

Athens, Greece, 17 – 21 February, 2025

Agenda item:	8.1

Source: 	Huawei

Title: 	(TP for TS 38.413) XR available data rate report

Document for:	Approval

# 1. Introduction

This contribution is to show the potential change needed for TS 38.413 to support the XR available data rate report, according to the discussion in R3-250294.

# Annex - TP for TS 38.413

Start of Change

#### 9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.19 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [9]. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| Available Data Rate Report Thresholds |  | 0..1 |  |  | YES | ignore |
| >DL Available Data Rate Report Thresholds | O |  | Available Data Rate Report Threshold List 9.3.1.x |  | - |  |
| >UL Available Data Rate Report Thresholds | O |  | Available Data Rate Report Threshold List 9.3.1.x |  | - |  |

Next Change

9.3.1.x 	Available Data Rate Report Threshold List

This IE contains a list of available data rate report thresholds. It is used for available data rate report for UL and DL as specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available Data Rate Report Threshold List |  | 1..<maxnoofThresholds> |  |  |
| >Identifier | M |  | INTEGER (0..FFS) |  |
| >Bit Rate Threshold | M |  | Bit Rate 9.3.1.4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of thresholds allowed to be provided by the CN. Value is FFS. |

End of Change