3GPP TSG-RAN WG3 Meeting #127	R3-250680

Athens, Greece, 17th – 21th Feb 2025

Title: 	Discussion on Support of IoT NTN

Source: 	CMCC

Agenda item: 	15.2

Document Type:	Discussion

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 has been agreed in RP-240776:

The aim of this WI is enhancement of IoT-NTN with the following objectives:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN3 #125 meeting, the following is captured in chairlady’s notes:

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

In RAN #106, the WID has been revised to include the regenerative payload in IoT NTN in RP-243278:

  - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

2	Discussion

Since in #125 meeting, RAN3 has agreed that: RAN3 decides to work on Split MME architecture and Full CN on board architecture. But in RAN#106, the WID has been revised to support full eNB as regenerative payload.

Observation 2: For Split MME and Full CN on board architecture, the current S1AP can be reused to support S&F satellite operation.

The core network needs to use NAS signal to inform UE on the support of S&F. However it is RAN2 scope to discuss how to inform UE the satellite in S&F operation, and RAN2 had already reach the agreements as below:

- S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

Observation 3: No need to discuss in RAN3 how to support S&F indication to UE.

Single Satellite and multiple Satellite deployment are discussed and compared. If only one satellite provides service for the UE, it may take a long time for the satellite complete one round. And since the orbit is an oval, after one round, it is most possible that the satellite can’t return to cover the same UE. It may need to take several rounds for the satellite to return and cover the same UE. So it is obvious that single satellite deployment takes too long to store the message. It is hard to finish the authentication between UE and CN in time and provide the service.

Multiple satellite solution is also discussed in SA2, but with MME/C-SGN on board. Up to now there is no discussion on whether S&F Satellite operation with only eNB on board and multiple satellite (with ISL or without ISL) deployment shall be supported.

Proposal 1: Both single and multiple satellites deployment on support of S&F operation serving a UE need to be discussed in RAN3. RAN3 can start from single satellite deployment first.

For Split MME architecture, no matter single satellite or multiple satellites deployment, both the eNB and MME(CP) and S1AP are in the same satellite. So the current UE suspend/resume procedures for CIoT can be reused.

Proposal 2: For Split MME architecture, the current control plan procedures via S1 can be reused, including both Current MO/MT for Control Plane CIOT EPS Optimisation.

But since the S-GW(UP) may be on the other satellite or ground, so when feeder link shut down, or there is ISL and thus no X2 HO can be executed between satellites, the current UP CIOT solution can’t work anymore. In these cases,

Proposal 3: It is beneficial for RAN3 to discuss how to enhance the current UP CIOT EPS Optimisation.

For Full CN on board architecture, both eNB, MME(CP) and S-GW(UP) are on the same satellite. So no S1AP enhancements is needed.

Proposal 4: For Full CN architecture, the current S1 can be reused and no more enhancement needed.

3	Summary

In this contribution, we provide considerations on the IoT NTN in RAN3, following observations and proposals are made:

Observation 1: From SA2 TR, Most S&F solution for different deployment scenarios suggest to have both eNB and MME/C-SGN on board, which needs no enhancement on S1AP.

Observation 2: For Split MME and Full CN on board architecture, the current S1AP can be reused to support S&F satellite operation.

Observation 3: No need to discuss in RAN3 how to support S&F indication to UE.

Proposal 1: Both single and multiple satellites deployment on support of S&F operation serving a UE need to be discussed in RAN3. RAN3 can start from single satellite deployment first.

Proposal 2: For Split MME architecture, the current control plan procedures via S1 can be reused, including both Current MO/MT for Control Plane CIOT EPS Optimisation.

Proposal 3: It is beneficial for RAN3 to discuss how to enhance the current UP CIOT EPS Optimisation.

Proposal 4: For Full CN architecture, the current S1 can be reused and no more enhancement needed.

4	Reference

[1] 	RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]       TR 23.700 -29, Study on integration of satellite components in the 5G architecture

[3]       TS 36.413, S1 application protocol (S1AP)

[4]       RAN3_125_agenda_20240823EOM, chairlady’s note