3GPP TSG-RAN WG3 Meeting #R3-127		           R3-250718

Athens, Greece, Feb 17-21, 2024

Agenda Item:	11.2

Source: 	Rakuten, CATT

Title:  	Enhancement in Data Collection Request procedure for UE Performance Data

Document for:	Discussion and decision

# Introduction

Related to UE performance feedback of the handed over UEs from the target node to the source node, we list below the relevant references:

From 38.743:

Following are the notes from RAN3#125-bis meeting:

Slice UE performance:

Observation based on current specs: Average packet delay UL/DL, average UE throughput (UL/DL), and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

The granularity of UE performance:

Op1: per S-NSSAI per PDU Session: ZTE, CMCC, Jio, TI, E///, LGE

Opt2: per S-NSSAI: Lenovo, SS, HW, NEC

Opt3: per-QoS flow groups: CATT, Nok

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

Following are the notes from RAN3#126 meeting:

Companies have different understandings on if per slice per UE performance (e.g., throughput) is really supported by SA5 specification.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

In this paper, we will discuss a proposal which eliminates the need for supporting only one granularity of UE performance feedback by adding an IE in Data Collection Request procedure which serves as a filter. This will enable the setting of the desired granularity

# Discussion

In the previous meetings, it has been discussed and agreed that after successful HO completion from source to target gNB, the target gNB will send UE performance metrics to the source gNB. This will assist the source gNB to evaluate the merits of its AIML-based mobility decisions and act as feedback to validate the accuracy of its AI/ML training and prediction.

In RAN3#125-bis meeting, we concluded the discussion of UE performance with a working assumption that a UE performance per PDU session associated to a requested S-NSSAI is reported from target node to source node. And it was left for further investigation if other finer granularity of UE performance is feasible. However, even by the end of the previous meeting, RAN3#126, there was no consensus among the companies on what is supported by SA5 in terms of the definitions of some of these metrics.

## Options of granularity of UE performance metric

Based on the contributions received in the previous meeting, these are the various levels at which UE performance feedback is desired by different companies:

- Per S-NSSAI level

- Per PDU level

- Per DRB level

- Per QoS-flow level

The companies have explained why some of these granularities are not suitable for this use case. And many papers have highlighted the lack of clarity in the standard from SA5 on how the measurements mentioned in TS 28.558 are computed, especially for Average DL/UL UE Throughput. This problem is very well exemplified in [4].

Observation 1: There is no consensus amongst the companies on the most suitable granularity of UE performance feedback from the target gNB to the source gNB.

Observation 2: There is lack of clarity in the standard from SA5 on how some of the measurements mentioned in TS 28.558 are computed.

Given this, we propose adding a new IE in the Data Collection Request which gives the flexibility of setting the desired granularity for the requested UE performance data.

Proposal 1: Adding a new IE in the Data Collection Request which gives the flexibility of setting the desired granularity for the requested UE performance data.

## Data Collection Request

In Data Collection Request (Section 9.1.3.26 in 38.423), the Report Characteristics for Data Collection IE has been defined as a 32-bit string, and the bitmap, set by the source NG RAN node, indicates to the target NG RAN node which measurements are required [3]. In the current standard, these metrics are defined at the per cell granularity by default. As of now, nine bits have been identified as shown in the figure below:

It is assumed that further bits will be identified to represent the new metrics or pre-existing metrics of different granularities to represent the requirements of the use cases like Network Slicing and Capacity and Coverage Optimization and future use cases. However, we feel that using a bitmap (based on legacy procedures) can become limiting when many more AIML-based use cases might be supported, and the number of metrics to be exchanged between target and source nodes might become very large.

The problem statement can therefore be summarized as:

- There is no consensus among companies on the granularity of UE performance feedback

- The Report Characteristic for Data Collection is limited to only 32 measurements that can be sent from target node to the source node.

To address the first issue, we propose the introduction of a filter IE, which will provide the flexibility in the reporting procedure. This filter will identify the granularity at which the performance metrics need to be reported by NG-RAN node2 when it receives the Data Collection Request from NG-RAN node1. The filter values can be what has already been defined and supported in 28.558. This is an optional field, and if not set, the default granularity of per-cell level measurement will continue to be supported.

- We propose a new IE to be added to the Data Collection Procedure for the purpose of selecting different filter/sub-counters for the UE measurements mentioned in Report Characteristics for Data Collection.

To address the second issue, there are several options possible. We’ve discussed some of them in our earlier contributions. The first step is that RAN3 recognizes the limitations of the current Report Characteristics for Data Collection and invites the companies to propose methods to address this issue. We propose that RAN3 acknowledges the problem statement, and requests for proposals for solution from participating companies.

- We propose that RAN3 acknowledges the problem of limited number of measurements that can be supported by using Report Characteristic for Data Collection and invites proposals for potential solution.

## Report Filter for Data Collection

As mentioned in Proposal 1 above, a filter IE associated with the UE measurements mentioned in the Report Characteristic for Data Collection can address the requirements of several AI/ML based use cases. Using the filter, the source gNB may indicate to the target gNB that the UE measurements may be reported in the specified granularity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Other bits are ignored by the NG-RAN node2. | YES | reject |
| Report Filter for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(16)) | Measurements may be reported in the required granularity by use of applicable filters defined in 28.552 First bit = default Second Bit = QoS Third bit = DRB Fourth Bit = PDU Fifth Bit = SNSSAI Sixth Bit = Cell Other bits are ignored by the NG-RAN node2 | YES |  |

For example, for the current use case of network slicing, if the report of the UE performance feedback is required at per PDU Session level then the bitmap for the filter would be 0001000000000000.

The selected filter can also be a combination of multiple filters to get the required sub counters at the source NG RAN node. Therefore, if the UE performance feedback is required at per DRB per PDU Session level, then the bitmap for the filter would be 0011000000000000.

The selected filter is applied to the UE performance metrics identified in Report Characteristics for Data Collection and reported to the source NG-RAN node1. The filter based proposed solution is applicable not only to the AIML based Network Slicing, but it may also be leveraged for other use cases.

# Conclusion

We have the following observations and proposals:

Observation 1: There is no consensus amongst the companies on the most suitable granularity of UE performance feedback from the target gNB to the source gNB.

Observation 2: There is lack of clarity in the standard from SA5 on how some of the measurements mentioned in 28.558 are computed.

Proposal 1	We propose a new IE to be added to the Data Collection Procedure for the purpose of selecting different filter/sub-counters for the UE measurements mentioned in Report Characteristics for Data Collection.

Proposal 2	We propose that RAN3 acknowledges the problem of limited number of measurements that can be supported by using Report Characteristic for Data Collection and invites proposals for potential solution.

# Reference

[1] TS28.552 5G performance measurements

[2] TS25.558 UE level measurements for 5G system

[3] TS38.423 Xn application protocol (XnAP)

[4] R3-247412