3GPP TSG-RAN WG3 Meeting #127			                               R3-250724

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:       10.2

Source:     ZTE Corporation

Title: 	(TP for 38.401 38.473 38.300) Discussion on MRO

Document for:	Discussion and Approval

1	Introduction

This contribution provides  a further discussion on enhancements to MRO for LTM, CHO-CPAC, and subsequent CPAC.

2	MRO for LTM

2.1 failure due to wrong beam

During the LTM procedure, the UE switches to the cell and beam indicated by MAC CE, but If the UE is switched to the right cell but wrong beam , and the connection in the target cell would be failed within a short period if beam recovery is also failed. Therefore, LTM failure caused by switching to the correct cell but the wrong beam should be considered for MRO optimization.

Proposal 1：LTM failure caused by switching to the correct cell but the wrong beam should be considered for MRO optimization.

In this case, to optimize the selection of TCI state(s) and target beams during LTM preparation and execution, both UE-based and network-based solutions are potential approaches. However, since the relevant target beam are UE-associated and determined during the cell switch,  the network cannot reliably record all details in real time from the MAC CE. Therefore, the UE-based solution is preferred.

Proposal 2：For MRO optimization of failure caused by selecting the wrong beam,  RAN3 considers a UE-based solution that enhances the RLF report with the beam in the LTM cell switch command, and the beam selected during re-establishment afterwards.

2.2 Near failure due to beam failure recovery

If LTM is successful but the UE experiences a beam failure shortly afterward, triggering the BFR procedure with a beam different from the one configured in the cell-switch command, this case should also be considered a near-failure case for MRO optimization.

Proposal 3：Introduces a new near failure case where BFR happens soon after a successful LTM cell switch, and UE re-selects to another beam in the same target cell.

During the previous RAN3 meeting, there are different optimization methods, such as network-based (e.g. signaling of Beam Failure Recovery over F1) or UE-based (e.g. Beam Failure Recovery in SHR), as listed in the SOD in R3-247837, as shown below:

Already proposed during RAN3#125-bis but not discussed due to lack of time, some companies (in [1], [8] and [11]) propose to optimize the selection of TCI state(s) and therefore target beams at LTM preparation and execution. Can RAN3 agree to study solutions to optimize target beam selection? Different solutions, network-based (e.g. signaling of Beam Failure Recovery over F1) or UE-based (e.g. Beam Failure Recovery in SHR) are presented. Network-based vs UE-based solution?

For failure cases related to the wrong beam as discussed above, we think that the network cannot reliably record all details in real time from the MAC CE. Therefore, the UE-based solution is preferred. To ensure a consistent approach to handling near-failure cases involving BFR, we propose adopting the UE-based solution as well, specifically by introducing a new SHR trigger: a beam failure together with a BFR happens soon after an LTM cell switch is successfully executed.

Proposal 4：For MRO optimization of near failure involving BFR,  RAN3 considers a UE-based solution that introduces a new SHR trigger: a beam failure together with a BFR happens soon after an LTM cell switch is successfully executed.

2.3 SHR handling

The existing T310, T312, and T304 timers can be reused as SHR triggers in LTM near-failure scenarios. Following current MRO principles, upon receiving the SHR, the CU would first analyze the cause of the near-failure and then forward the SHR to the source DU if triggered by T310 or T312 (Configured at source DU), or to the target DU if triggered by T304 (Configured at target DU). We suggest to capture this SHR handling approach  in TS 38.401 stage 2. Additionally, if a trigger cause for beam failure recovery is introduced in the SHR report, this stage 2 would need to be updated accordingly.

Proposal 5: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex A.)

2.4 RACH-less LTM

In the last meeting, the choice between a UE-based or a network-based solution for TA acquisition and the indication of LTM candidate cells was discussed. See chair notes below:

Discuss RAN2 agreements on UE-based solutions for TA acquisition and indication of LTM candidate cells in RLF Report. RAN3 to discuss the following network-based solution: 1.Source gNB can store the Early UL sync configuration and UE-based TA configuration and can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in CSC by "retrieving" the UE context upon receiving the RLF Report? 2.There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE's LTM candidate cells at the time of RLF.

If the network configures LTM for a UE, the UE associated LTM configuration,e.g., candidate cells and UL sync configurations may be reconfigured multiple times during the UE's session. A network-based solution would require storing all historical configuration information. Since an RLF can be reported within 48 hours, the proper functioning of the network-based solution relies on the network's ability to retrieve the UE's historical context and past configurations, and corresponding timestamps. However, this approach is highly dependent on the network's implementation. For this reason, we propose adopting a UE-based solution.

Proposal 6:  The UE-based solution for TA acquisition and the indication of LTM candidate cells in the RLF report is sufficient.

2.5 Subsequent LTM

If subsequent LTM is configured for UE, various report might be generated, in a higher frequency. However, the failure information or the sub-optimal success report (if there are any), might be over-written very soon. If subsequent LTM goes on and on, network might fail to fetch the report in time, and won't be able to spot the issues in a subsequent LTM (and therefore to optimize it).  (Note: a similar issue also exists in SCPAC)

Proposal 7: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.  (Note: a similar issue also exists in SCPAC).

In the last meeting, the following issue of ping-pong optimization was discussed, but no consensus was reached.

Which node is responsible for avoiding LTM cell switch ping-pong (CU or DU)?

CU sends UHI to DUs?

If the CU sends the LTM related UHI to the target DU during cell switch (e.g., via CU-DU CELL SWITCH NOTIFICATION), the target DU can immediately use this information to make better decisions for future handovers , and can also use the UHI for long-term parameter optimization. Additionally, this method avoids the need for immediate reconfiguration of the LTM for UE, which aligns better with the principles of MRO, as MRO focuses on long-term optimization rather than short-term adjustments.

Proposal 8: The CU sends the UHI to the target DU during cell switch (e.g., via the CU-DU CELL SWITCH NOTIFICATION).  (The corresponding TP for TS38.473 is provided in Annex B).

3	MRO for CHO-CPAC

3.1 Failure case :

3.1.1 Case 1c, 2c, 2c, 3c, 7c, 8c:

During past meetings, no agreement was reached on cases where PCell failure and PScell failure occur simultaneously, such as Cases 1c, 2c, 3c, 7c, and 8c. However, we believe these cases are common, therefore, we propose considering Cases 1c, 2c, 3c, 7c, and 8c in Rel-19.

Proposal 9: Consider Cases 1c, 2c, 3c, 7c, and 8c in Rel-19.

3.1.2 Case 9:

Following case 9 has been agreed to address in past meetings. It means CHO execution condition and the associated CPAC execution condition cannot be met simultaneously.

| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |
| --- | --- | --- |

During the RAN3#125bis meeting, the following enhancement of RLF were agreed:

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Regarding the above enhancement, if applied to Case 9, this would seems that before an RLF encountered, the UE might record a list of candidate cells that individually met their conditions, while their paired candidate cells failed to meet. Clearly, some cells in this list would likely appear multiple times. However, this approach seems unnecessary. the UE only needs to record the number of times each specific candidate cell met the conditions individually before an RLF encountered. This way, MRO can identify which candidate cell had a higher number, helping to analyze why its configured paired cell was consistently unable to meet the conditions at the same time.

Proposal 10: In order to support MRO for case 9, enhance RLF report to include:

- Identifiers of candidate PCell(s) or PSCell(s) that met execution conditions before RLF, while their paired cells did not;

- The number of times each candidate cell met the execution conditions.

3.1.3 Case 10:

During past meeting, Companies propose to consider case 10: UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure, which may be different from case 9.

In Case 10, the UE needs to record the identifiers of these non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred.Similar to Case 9, each pair only needs to be recorded once, along with the number of times it met the conditions.

Proposal 11: In order to support MRO for case 10, enhance RLF report to include:

- Identifiers of non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred;

- The number of times each non-associated cell pair met the execution conditions.

3.1.4 Too later CHO failure:

During the RAN3#126 meeting, the following agreements were reached, while the method for the last serving MN to inform the (candidate) target MN of the failure in case of too late CHO remains FFS.

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

- Which message is used from the last serving MN to the candidate MN(s):

- Handover Report

- via new message

For case 3a),  the problem may be brought by the (candidate) target MN  due to unmet CPC/CPA execution conditions configured by the (candidate) target MN.

In case 9a), the problem may also be brought by the source MN or  the (candidate) target MN or both,  as the CHO execution condition and the associated CPAC execution condition cannot be met simultaneously before an RLF occurs. As discussed in Section 3.1.2, if the UE records the number of times each specific candidate cell (PCell or PSCell) individually met the conditions in the RLF report, MRO can analyze which candidate cells were recorded more frequently and identify the nodes most likely responsible for the failure. This approach not only prevents excessive simultaneous optimization on both the source and target nodes but also minimizes unnecessary optimization across an excessive number of candidate MNs.

Proposal 12: In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization. However, this handling method should prevent unnecessary optimization on both the source and target nodes while also minimizing unnecessary optimization across an excessive number of candidate MNs.

In the current XnAP handover report message, there is no definition for a "too late handover" trigger type, as shown below. This is because, in the case of legacy HO and CHO, the last serving gNB is the node responsible for too later handove failure.

9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …) |  | YES | ignore |

Proposal 13: Reuse XnAP handover report message for the last serving MN to inform too later handover failure to the candidate MN(s), and enhance the message accordingly.

Based on Proposal 12 and 13, we suggest revising Stage 2 as highlighted below, building on the agreed CR (R3-247905) from the latest meeting.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the candidate NG-RAN node(s) controlling the candidate PSCell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn.

Editor’s note:  the NG-RAN node receiving the failure indication shall prevent unnecessary optimization across an excessive number of candidateNG-RAN node(s).

Proposal 14: RAN3 agrees the TP for TS38.300 for Too Late Handover when UE is configured with CHO with candidate SCG(s).  (The TP  is provided in Annex C).

3.2 Near failure case:

Based on current specification, in addition to CHO-CPAC configuration, the network can provide a complementary CHO-only configuration /execution condition to the same UE.  And in case the CHO-CPAC execution conditions are not meet, instead the CHO-only configuration can be used when the execution condition is meet. In this case, no failure report will create while the  CHO-CPAC configuration seems useless. From RAN3 point view, the useless of CHO-CPAC with CHO-only can be taken as one type of near failure case.

Proposal 15: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

4	MRO for SCPAC

In past RAN3 meetings, the issue was raised of whether to support forwarding the SCG UHI to the target node during the S-CPAC procedure to avoid ping-pong immediately. However, we believe this type of real-time optimization introduces significant complexity of SCPAC reconfiguration at network side and  is not aligned with the principles of MRO, which focuses on long-term optimization. Additionally, it would require modifications to the UE's SCPAC configuration whenever a ping-pong is detected..

Proposal 16:  The real-time reconfiguration of SCPAC to avoid ping-pong, e.g., forwarding the SCG UHI to the target node during the S-CPAC procedure, is not aligned with the principles of MRO, which focus on long-term optimization. As RAN2 has lowered the priority of SCPAC, we propose that RAN3 close the discussion on this optimization.

At the RAN3#125 meeting, it was agreed that "Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.". In Rel-18 MRO, the MN may include the CPC candidate cell list and CPC execution conditions to the SN via the SCG FAILURE INFORMATION REPORT over Xn for failure analysis.To differentiate the CPC configuration over Xn, RAN3 discuss whether to introduce  SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

Proposal 17:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

- 4 Conclusion:

The following proposals are provided for MRO for LTM:

Proposal 1：LTM failure caused by switching to the correct cell but the wrong beam should be considered for MRO optimization.

Proposal 2：For MRO optimization of failure caused by selecting the wrong beam,  RAN3 considers a UE-based solution that enhances the RLF report with the beam in the LTM cell switch command, and the beam selected during re-establishment afterwards.

Proposal 3：Introduces a new near failure case where BFR happens soon after a successful LTM cell switch, and UE re-selects to another beam in the same target cell.

Proposal 4：For MRO optimization of near failure involving BFR,  RAN3 considers a UE-based solution that introduces a new SHR trigger: a beam failure together with a BFR happens soon after an LTM cell switch is successfully executed.

Proposal 5: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex A.)

Proposal 6:  The UE-based solution for TA acquisition and the indication of LTM candidate cells in the RLF report is sufficient.

Proposal 7: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.  (Note: a similar issue also exists in SCPAC).

Proposal 8: The CU sends the UHI to the target DU during cell switch (e.g., via the CU-DU CELL SWITCH NOTIFICATION).  (The corresponding TP for TS38.473 is provided in Annex B).

The following proposals are provided for MRO for CHO-CPAC:

Proposal 9: Consider Cases 1c, 2c, 3c, 7c, and 8c in Rel-19.

Proposal 10: In order to support MRO for case 9, enhance RLF report to include:

- Identifiers of candidate PCell(s) or PSCell(s) that met execution conditions before RLF, while their paired cells did not;

- The number of times each candidate cell met the execution conditions.

Proposal 11: In order to support MRO for case 10, enhance RLF report to include:

- Identifiers of non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred;

- The number of times each non-associated cell pair met the execution conditions.

Proposal 12: In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization. However, this handling method should prevent unnecessary optimization on both the source and target nodes while also minimizing unnecessary optimization across an excessive number of candidate MNs.

Proposal 13: Reuse XnAP handover report message for the last serving MN to inform too later handover failure to the candidate MN(s), and enhance the message accordingly.

Proposal 14: RAN3 agrees the TP for TS38.300 for Too Late Handover when UE is configured with CHO with candidate SCG(s).  (The TP  is provided in Annex C).

Proposal 15: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

The following proposals are provided for MRO for SCPAC:

Proposal 16:  The real-time reconfiguration of SCPAC to avoid ping-pong, e.g., forwarding the SCG UHI to the target node during the S-CPAC procedure, is not aligned with the principles of MRO, which focus on long-term optimization. As RAN2 has lowered the priority of SCPAC, we propose that RAN3 close the discussion on this optimization.

Proposal 17:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

Annex A: 	TP to TS38.401 for LTM SHR handing

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU may forward the SHR report to the source gNB-DU  in case of the SHR triggered by the T310 and T312 timer,or to the target gNB-DU in case of the SHR triggered by the T304 timer.

Annex B: 	TP to TS38.473 for Subsequent LTM

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.3.10	CU-DU Cell Switch Notification

8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

If the UE History Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it for future LTM cell switching.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| UE History Information | O |  | OCTET STRING | UE History Information related to LTM, as same definition in TS 38.413 [3] | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

Annex C: 	TP to TS38.300 (based on R3-247905)

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO or CHO with candidate SCG(s) is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the candidate NG-RAN node(s) controlling the candidate PSCell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn.

Editor’s note:  the NG-RAN node receiving the failure indication shall prevent unnecessary optimization across an excessive number of candidateNG-RAN node(s).

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

Retrieval of information needed for problem analysis

In order to retrieve relevant information collected at the network side as part of the UE context, the UE provides C-RNTI used in the last serving cell. If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node. In the Handover Preparation procedure, the source gNB can request the target gNB to provide information on DL LBT failures at the target gNB during handover execution.

Handling multiple reports from a single failure event

In case the RRC re-establishment fails and the RRC connection setup succeeds, MRO evaluation of intra-RAT mobility connection failures may be triggered twice for the same failure event. In this case, only one failure event should be counted.

<<<<<<<<<<<<<<<<<<<< Next OF CHANGES >>>>>>>>>>>>>>>>>>>>

- 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, and if UE is configured with CHO with candidate SCG(s), it may inform the source MN where the mobility configuration may caused the failure by means of the Handover Report procedure over Xn. Otherwise, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>