3GPP TSG RAN WG3 Meeting #128	R3-253009

Malta, MT, 19 - 23 May, 2025

3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2503012

Wuhan, China, April 7th – 11th, 2025

Title:	LS on RAN2 agreements for security key handling in inter-CU LTM

Response to:

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	RAN2

To:	RAN3

Cc:

Contact person:	Jing Liu

liu.jing30@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

Regarding the security key handling in Rel-19 Inter-CU LTM, RAN2 made the following agreements at RAN2#129bis meeting:

Agreements: - For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE. - NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included. - NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG. - A list of sk-counters is linked to a Rel-19 set ID configured by the SN. - At RLF and reconfiguration with sync failure: - If RLF: - If the selected candidate cell has the same Rel-19 set ID as source (no security key change), the UE performs fast failure recovery (same as in Rel-18). - If reconfiguration failure (inter-CU LTM): - If the selected candidate cell has the same Rel-19 set ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 set IDs is allowed. - The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message.

The term “R19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different R19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update. From UE perspective, security key update is performed when the R19 set IDs are different between the serving cell and the target cell, whether it is intra-CU or inter-CU LTM can be transparent to the UE.

2	Actions

To RAN3 group

ACTION: 	RAN2 kindly asks RAN3 to take the above agreements into consideration in future work.

3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #130	19th – 23rd May 2025	Malta, MT

TSG-RAN2 Meeting #131	25th – 29th Aug 2025	India, IN