3GPP TSG RAN WG3 Meeting #128	R3-253023

Malta, MT, 19 - 23 May, 2025

3GPP TSG-SA5 Meeting #160	S5-252056

Goteborg, Sweden, 07 - 11 April 2025

Title:	Reply LS on per-UE UE performance metrics

Response to:	LS in S5-251324 / R3-247775 from RAN 3 on per-UE UE performance metrics

Release:	Rel-19

Work Item:	AIML_MGT_Ph2

Source:	SA5

To:	RAN3

Cc:	RAN2

Contact person:	Hassan Al-Kanani

Hassan.alkanani@emea.nec.com

Yizhi Yao

Yizhi.yao@intel.com

Zu Qiang

zu.qiang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA5 would like to thank RAN3 for their LS (R3-247775) regarding the interpretation of Average UE Throughput DL and Average UE Throughput UL measurements defined in TS 28.558, particularly with respect to their granularity and the "DRB." prefix in their measurement names.

SA5 confirms that the formulas specified in fields (c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 calculate average UE throughput based on per DRB level measurement for a UE. The formulas aggregate data across DRBs for each UE, ensuring that the reported throughput measurement represents the overall throughput per UE rather than being limited to individual DRBs.

The rationale for using the "DRB." prefix in the measurement names, as listed in field (e), is to indicate that the measurements originate from data transmissions occurring at the DRB level. However, these measurements are aggregated to provide a per-UE throughput metric. Specifically:

- The formulas include a summation over DRBs, ensuring that the result represents the total throughput per UE rather than individual DRB-level values.

- The "DRB." prefix in field (e) indicates that the contributing measurements originate from DRBs, but they are aggregated to reflect per-UE throughput.

- The definitions in the tables in field (c) in clauses 6.3.1.4.1 and 6.3.1.4.2 are based on DRB level measurements, but the summation ensures that the final metric is at the UE level.

In summary, although the measurement inputs are taken per DRB, they are aggregated to provide throughput metrics at the UE level.

# 2	Actions

To: RAN3, CC: RAN2

ACTION: SA5 kindly asks RAN3 to take the above clarification into their consideration and look forward to continued collaboration with RAN3 on this topic.

# 3	Dates of next TSG SA WG5 meetings

SA5#161		19 May - 23 May 2025			Fukuoka, Japan

SA5#162		25 August - 29 August 2025			Goteborg, Sweden