3GPP TSG-RAN WG3#128 	R3-253137

St. Julien’s, Malta, 19-23 May 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Inter-CU LTM with Dual Connectivity

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-250339 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This paper discusses the remaining FFS from RAN3#127bis related to inter-CU SCG LTM and some key aspects related to Inter-CU MCG LTM, which were not treated in RAN3 yet.

# 2	Inter-CU LTM with Dual Connectivity

At the last RAN3 meetings, there was progress with respect to Inter-CU SCG LTM. However, some aspects related to inter-CU SCG LTM were still marked as FFS. Furthermore, Inter-CU MCG LTM has still to be discussed in RAN3.

## 2.1 Inter-CU SCG LTM – Open issues

Following aspects were identified as FFS during the last RAN3 meeting:

Include the following information in SN Change Required (Source SN to MN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for each candidate SN (FFS) CSI Resource configuration request (explicit way) Early sync information request (explicit way) Include the following information in SN Addition Request (MN to Candidate SN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for the candidate SN (explicit way) CSI Resource configuration request (explicit way) Early sync information request (explicit way) It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message. For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN. The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN. Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message. Continue to work on the solutions comparison and details…

Remaining FFS-1 – Implicit or explicit indication of suggested PSCell list in SN Change Required, SN Addition Request

In Rel-18 S-CPAC, the suggested PSCell list is part of CG-Config which is signalled transparently in inter node Xn-AP message. In our view, a similar approach may be used for inter-CU SCG LTM. Likewise, we don’t see any strong motivation to indicate the PSCell list in an explicit way different from how it is signalled in S-CPAC.

Proposal 1: RAN3 to take approach of implicit indication to signal PSCell list and maximum no. of PSCells for each candidate SN via INM similar to Rel-18 S-CPAC.

Remaining FFS-2 - Whether MN can request SCG reference configuration from the source SN.

Rel-18 S-CPAC, supports both MN and SN initiated scenarios, and for the MN initiated S-CPAC scenario, the MN needs to obtain SCG reference configuration from source SN. However, as RAN2 has agreed to support only SN initiated Inter-CU SCG LTM in Rel-19, SN is expected to always include the SCG reference configuration while triggering SN initiated SN Change Required. In our view, a similar approach may be used for inter-CU SCG LTM.

Proposal 2: RAN3 to remove the FFS, as there is no need for MN to request SCG reference configuration from source SN for inter-CU SCG LTM in Rel-19.

## 2.1 Inter-CU MCG LTM Issues

The WI scope indicates both Inter-CU MCG LTM with SN unchanged, and Inter-CU MCG LTM with SN release as in scope of the WID. In the following sections we discuss open issues with respect to these two scenarios.

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released.

Issue 1 - Candidate MN prepares a different candidate PSCell at source SN

In Inter-CU MCG LTM with unchanged SN, it is possible that a candidate MN prepares a different PSCell even if it retains the source SN. Thus, both scenarios should be considered, and handling would vary as follows.

Scenario 1: Candidate MN retains the source PSCell: Even though the PSCell remains unchanged after Inter-CU MCG LTM execution, the UE must perform reconfiguration with sync as SN security key changes due to MN change.

Scenario 2: Candidate MN prepares a different PSCell: Intra-CU PSCell HO is triggered, as a result of Inter-CU MCG LTM PCell change.

In scenario 2 above, given that the Intra-CU PSCell change (or Intra-CU SCG LTM) may result from Inter-CU MCG LTM execution, the network must ensure it considers the latest PSCell measurements while triggering PSCell HO.

Proposal 3: For Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change is based on latest UE reported radio measurements.

Issue 2 – SCG release and add

Given that it is the decision of candidate MN during HO preparation to retain or release the SCG, some of the Inter-CU MCG LTM candidate nodes during HO preparation may prepare an MCG + SCG configuration (when SCG is retained at source SN) or a MCG only configuration (when SCG is not retained at source SN). Hence, it may result in a combination of inter-CU MCG LTM preparation for MN candidates, some with SCG retained and some with SCG released.

In the context of the above stated scenario, RAN2 has confirmed the support of SCG release and subsequently SCG add for Inter-CU MCG LTM, as well as include the SCG configuration in the mrdc-SecondaryCellGroupConfig. Hence, if the Inter-CU MCG LTM results in SCG release at the target, the SCG may be added during a subsequent PCell change if the candidate configuration included the SCG configuration as part of mrdc-SecondaryCellGroupConfig.

Proposal 4: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes SCG configuration.

Issue 3 - Avoiding blind SCG or PSCell addition

An SCG Addition or a PSCell Addition during subsequent Inter-CU MCG LTM execution (after SCG release during earlier Inter-CU MCG execution) may result into a potential blind PSCell addition if the network does not consider latest PSCell measurements to identify the suitable PSCell at the time of inter-CU MCG LTM execution. Further, if there is more than one candidate target PSCells prepared associated with the Inter-CU candidate target PCell, considering the latest UE reported radio measurements for candidate PSCells is essential to identify the suitable candidate PSCell and the associated LTM Candidate Configuration ID to avoid user experience and throughput degradation after SCG addition.

Proposal 5: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

## 2.2	Other Issues

Issue 4 - Configuration Mismatch during LTM Cell Switch

During initial preparation of candidate target node, the UE’s source cell configuration is used to prepare the target cell configuration for the prepared candidate target nodes. Given that the configuration at the UE after the initial preparation may vary, by the time of LTM cell switch or HO execution, the candidate target cell configuration prepared during the initial LTM preparation may not be aligned with the latest configuration at the UE. The handling of such a problem may require reconfiguring the UE at the time of cell switch or HO execution. However, to support subsequent LTM, the UE reconfiguration after cell change must be avoided. We propose RAN3 to discuss this issue and consider signalling enhancements for handling mismatch in the PDU session and DRB configuration between the serving and the target cells.

Proposal 6: RAN3 to derive a solution to address configuration mismatch between the serving and the target cells after initial preparation, and at the time of HO execution to support subsequent LTM cell change.

# 3	Conclusions

Inter-CU SCG LTM Issues

Proposal 1: RAN3 to take approach of implicit indication to signal PSCell list and maximum no. of PSCells for each candidate SN via INM similar to Rel-18 S-CPAC.

Proposal 2: RAN3 to remove the FFS, as there is no need for MN to request SCG reference configuration from source SN for inter-CU SCG LTM in Rel-19.

Inter-CU MCG LTM Issues

Proposal 3: For Inter-CU MCG LTM where a Candidate MN prepares a different PSCell resulting in an Intra-CU PSCell Handover, the PSCell change is based on latest UE reported radio measurements.

Proposal 4: If Inter-CU MCG LTM results in SCG release, then SCG may be added during subsequent LTM PCell change if the Inter-CU MCG LTM candidate configuration includes SCG configuration.

Proposal 5: In the case of Inter-CU MCG LTM involving SCG Addition (after SCG release), the PSCell addition is based on latest UE reported radio measurements for candidate PSCells.

Proposal 6: RAN3 to derive a solution to address configuration mismatch between the serving and the target cells after initial preparation, and at the time of HO execution to support subsequent LTM cell change.