3GPP TSG-RAN3 Meeting #128	R3-253139

Malta, MT, May 19th – 23rd, 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Interface management procedures for A-IoT

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on remaining issues related to interface management and functionalities to support Topology 1 of AIoT Topology.

# Discussion

## Area information for A-IoT

SA2 had sent a LS to RAN3 asking whether A-IoT RAN node can provide supported A-IOT RAN node serving area and BS reader serving area and requested RAN3 to define these parameters:

SA2 assumes that OAM can configure the AIOTF with needed information for this selection.

SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information to the core network. e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area

SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF.

If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters

RAN3 then agreed the following in RAN3#127bis:

Introduce a new “A-IoT Area”

FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message.

Further, SA2 further asked the following questions in a different LS:

Q1. SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area.

In, RAN3#127bis, some companies wanted to reuse the concept of Tracking Areas (TAs) so that each gNB can indicate its supported A-IoT service area(s) using the existing Supported TA List IE in NG Setup Request and in turn AIOTF can request inventory in a certain area based on the indicated TAs.

We would like to remind that there is no concept of Tracking areas at the A-IoT device and RAN2 also agreed to not support any Tracking Area Update (TAU) like procedure for A-IoT device(s).

Further, a gNB supporting only A-IoT doesn’t need to understand the concept of tracking areas or cells and in our view, it is therefore easier to specify AIoT service areas as geographical areas rather than reusing TAI and/or cells to indicate the AIoT service area.

A gNB supporting only A-IoT doesn’t need to understand the concept of Tracking areas (TAs) and cells. Also, there is no concept of TAs and cells over the A-IoT radio interface.

A gNB supporting both A-IoT and Uu can understand the concept of tracking areas or cells and potentially we can reuse TAI and/or cells to indicate the AIoT service area in this case. But the Tracking Area Codes might already be used for Uu communication and hence it might be difficult to reuse the existing tracking areas or reserve a dedicated TA for A-IoT.

A gNB supporting both A-IoT and Uu might be already using tracking areas for Uu communication and hence it might be difficult to reuse the existing tracking areas or reserve a dedicated TA for A-IoT.

Considering the deployment of gNB only supporting AIoT and to keep its implementation simple, we propose the following:

A-IoT areas will not be indicated in terms of tracking areas and/or cells.

SA2 asked another question in the LS to RAN3:

Q2. SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

Regarding Q2, we think a reader can be part of multiple supported areas as shown below.

Consider the above example,

- gNB1 manages R1, R2 and R7

  - R1 is part of Area 1

  - R2 is part of Area 1 and 2

  - R7 is part of Area 1, 2 and 3

- gNB2 manages R3 and R5

  - R3 is part of Area 2

  - R5 is part of Area 2 and 3

- gNB3 manages R4 and R6

  - R4 is part of Area 1 and 3

  - R6 is part of Area 3

Although a reader can be part of multiple areas, it is to be noted that a reader is managed by only one gNB. We don’t think we need to support a scenario where a reader needs to be managed by multiple gNBs as this may lead to more complexity e.g., in case of inter-vendor scenarios.

Considering the above illustration and explanation, the following is proposed:

A-IoT Areas are expressed in terms of geograhical areas and define an “A-IoT Area ID” to identify each A-IoT geographical area.

For each A-IoT enabled gNB, OAM indicates to AIOTF the list of readers supported by the gNB and the A-IoT Area ID(s) associated with each reader

A reader can be part of multiple A-IoT areas (e.g., a reader may be deployed in the overlap of more than one geographical area).

In Topology 1, a BS reader is managed by only one gNB.

## Reader and A-IoT Area information in NG signalling

RAN3 agreed the following in RAN3#127:

AIoTF obtain the AIoT RAN information (supported AIoT area, served reader ID list, location of reader) via OAM configuration

Besides the OAM based solution, we think that the AIOTF should be able to discover the above information dynamically via signalling with gNB. Even though we are currently considering T1, and the readers would be predominantly static and thereby the reader serving area fixed, we think we should allow for deployments where gNB readers are mobile e.g., readers mounted on a moving vehicle or change of reader’s serving area due to switch on/off or power adaptation.

Readers supported by a gNB may change “dynamically” in some scenarios e.g, if readers are mounted on a moving vehicle and/or due to Reader Tx/Rx power adaptation or Reader switch on/off

We therefore propose the following:

gNB provides the following information to AIOTF in NG Setup Request/RAN Configuration Update

  - Readers supported by the gNB

  - A-IoT Area ID(s) associated with each Reader

  - Reader location

## Reader selection

The following was agreed in RAN3#127:

AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area

Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers

WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list

In case the AIOTF indicates a list of readers in which it wants to perform inventory, there was discussion last meeting whether the gNB can perform the “final” selection of readers (e.g., can reject the readers indicated by the AIOTF or can select new readers outside the indicated list).

Some of the reasons provided by the proponents were that the gNB might be doing some coverage adaptation (e.g., due to CCO) and should therefore decide whether it can use the reader. In our view, this does not require the gNB to have the final say. The gNB can still try using that reader via repeated A-IoT paging attempts and the inventory might fail in the worst case. But there is no reason for gNB to ignore the readers indicated by the AIOTF.

Further in our view, the gNB should not override the selection of AIOTF as the gNB might not know why the AIOTF requested the gNB to use a certain reader e.g., this could be a reader in the coverage overlap of multiple gNBs which can be also used for the A-IoT devices not in the gNB’s coverage or this could be a “special” reader which the AF/AIOTF wants to use. In case the AIOTF only indicates an area, gNB can “select” a subset of readers within the indicated area as already agreed in RAN3.

An AIOTF may want to use specific readers for performing Inventory (e.g., those readers in the coverage overlap of multiple gNBs or in a certain location) and the gNB might not have such knowledge.

A gNB performing some link adapdation of Readers (e.g., Reader’s coverage adjustment or Reader’s Tx/Rx power adaptation) can still attempt to perform inventory using the list of readers indicated by the AIOTF and perform multiple rounds of inventory if it fails. There is no need for gNB to downselect or upselect from the list of readers indicated by the AIOTF.

We therefore propose to turn the WA and FFS into agreement as well to cover all possible scenarios.

Upon receiving only the reader list in Inventory Request, the gNB uses all the readers indicated by the reader list to perform inventory.

Upon receiving both the area and the reader list in Inventory Request, the gNB selects all the readers within the reader list and may select additional readers within the indicated area.

## A-IoT support indicator in NG Setup Request/Response

AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

FFS how to handle A-IoT only support case.

FFS how to handle the not applicable mandatory IEs

RAN3 agreed to introduce an AIoT indicator in the NGAP Setup Request message. The detail of this indicator will be further discussed.

In RAN3#127, RAN3 agreed to support the scenario where a gNB only supports communication with A-IoT devices over A-IoT radio and this is also captured in the BL CR to 38.300.

Besides the legacy deployment scenario (i.e., gNB supporting only Uu), RAN3 needs to discuss how to support different deployment scenarios as shown in the figure below:

- gNB supporting only A-IoT radio with direct connectivity to AIOTF (case 1)

- gNB supporting only A-IoT radio with indirect connectivity to AIOTF (case 2)

- gNB supporting both Uu and AIoT with direct connectivity to AIOTF (case 3)

- gNB supporting both Uu and AIoT with indirect connectivity to AIOTF (case 4)

For the above deployment scenarios, RAN3 further needs to discuss how to handle the A-IoT non applicable mandatory IEs.

Currently, NG Setup Request contains some mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) which are not applicable to A-IoT e.g., there is no concept of TA List (if TA is not reused as A-IoT area) or slices or Paging DRX for A-IoT.

NG Setup Response contains some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity) which are also not all relevant to A-IoT e.g., there is no GUAMI at A-IoT device or there is no requirement to support AMF load balancing for which Relative AMF Capacity was defined.

NG Setup Request contains mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) and NG Setup Response also includes some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

We think we can handle the different deployment scenarios mentioned above and the mandatory IEs by using the already agreed “A-IoT support” flag in NG Setup Request with appropriate code-points – {A-IoT only, A-IoT + Uu}. Therefore, the following is proposed.

To handle different deployment scenarios (gNB supporting only A-IoT, gNB supporting both A-IoT and Uu), include an “AIoT support flag” in NG Setup Request message with code-points {A-IoT only, A-IoT + Uu}

If this “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT only”, the AMF can ignore those IEs that are not applicable to A-IoT (e.g., TAI Slice Support List, Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request. Further, gNB can include random/junk values in those A-IoT non-applicable mandatory IEs.

If this “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT + Uu”, the AMF will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT only”, the AMF can ignore those IEs that are not applicable to A-IoT (e.g., TAI Slice Support List, Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request.

If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT + Uu”, the AMF will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

Also in response, if the AMF includes an “A-IoT support” flag with value “A-IoT only” in NGAP Setup Response, gNB can ignore the A-IoT non applicable mandatory values upon receiving this NGAP Setup Response. AMF can include random/junk values in those A-IoT non applicable mandatory IEs

If the AMF includes an “A-IoT support” flag with value “A-IoT only” in NGAP Setup Response, gNB can ignore the A-IoT non applicable mandatory values (AMF Name, GUAMI, Relative AMF capacity) upon receiving this NGAP Setup Response.

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 Area information for A-IoT

Observation 1:	A gNB supporting only A-IoT doesn’t need to understand the concept of Tracking areas (TAs) and cells. Also, there is no concept of TAs and cells over the A-IoT radio interface.

Observation 2:	A gNB supporting both A-IoT and Uu might be already using tracking areas for Uu communication and hence it might be difficult to reuse the existing tracking areas or reserve a dedicated TA for A-IoT.

Proposal 1:	A-IoT areas will not be indicated in terms of tracking areas and/or cells.

Proposal 2:	A-IoT Areas are expressed in terms of geograhical areas and define an “A-IoT Area ID” to identify each A-IoT geographical area.

Proposal 3:	For each A-IoT enabled gNB, OAM indicates to AIOTF the list of readers supported by the gNB and the A-IoT Area ID(s) associated with each reader

Proposal 4:	A reader can be part of multiple A-IoT areas (e.g., a reader may be deployed in the overlap of more than one geographical area).

Proposal 5:	In Topology 1, a BS reader is managed by only one gNB.

2.2 Reader and A-IoT Area information in NG signalling

Observation 3:	Readers supported by a gNB may change “dynamically” in some scenarios e.g, if readers are mounted on a moving vehicle and/or due to Reader Tx/Rx power adaptation or Reader switch on/off

Proposal 6:	gNB provides the following information to AIOTF in NG Setup Request/RAN Configuration Update

a. Readers supported by the gNB

b. A-IoT Area ID(s) associated with each Reader

c. Reader location

2.3 Reader selection

Observation 4:	An AIOTF may want to use specific readers for performing Inventory (e.g., those readers in the coverage overlap of multiple gNBs or in a certain location) and the gNB might not have such knowledge.

Observation 5:	A gNB performing some link adapdation of Readers (e.g., Reader’s coverage adjustment or Reader’s Tx/Rx power adaptation) can still attempt to perform inventory using the list of readers indicated by the AIOTF and perform multiple rounds of inventory if it fails. There is no need for gNB to downselect or upselect from the list of readers indicated by the AIOTF.

Proposal 7:	Upon receiving only the reader list in Inventory Request, the gNB uses all the readers indicated by the reader list to perform inventory.

Proposal 8:	Upon receiving both the area and the reader list in Inventory Request, the gNB selects all the readers within the reader list and may select additional readers within the indicated area.

2.4 A-IoT support indicator in NG Setup Request/Response

Observation 6:	NG Setup Request contains mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) and NG Setup Response also includes some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

Proposal 9:	To handle different deployment scenarios (gNB supporting only A-IoT, gNB supporting both A-IoT and Uu), include an “AIoT support flag” in NG Setup Request message with code-points {A-IoT only, A-IoT + Uu}

Proposal 10:	If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT only”, the AMF can ignore those IEs that are not applicable to A-IoT (e.g., TAI Slice Support List, Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request.

Proposal 11:	If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT + Uu”, the AMF will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

Proposal 12:	If the AMF includes an “A-IoT support” flag with value “A-IoT only” in NGAP Setup Response, gNB can ignore the A-IoT non applicable mandatory values (AMF Name, GUAMI, Relative AMF capacity) upon receiving this NGAP Setup Response.