3GPP TSG-RAN WG3 Meeting #128	R3-253148

Malta, MT, 19 – 23 May 2025

Agenda Item:	22.2

Source: 				LG Electronics Inc., FirstNet

Title:			Discussion on service continuity in Multi-hop relay

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 WID on NR sidelink multi-hop relay was agreed at RAN #104 [1]. One of the objectives in this WID is to specify intra-gNB service continuity scenarios for Multi-hop U2N Relay as follows:

… The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused. …

In RAN3#127bis meeting, it was agreed that from the RAN3 point of view, Scenarios A and B can be supported by the existing procedures, thus having no RAN3 impact. For Scenarios C and D, RAN3 decided to wait for RAN2 progress. In this contribution, we will discuss the RAN3 impacts on intra-gNB service continuity for Scenarios C and D, and then provide our views on it.

2.	Discussion

In RAN2#129bis meeting, the following was agreed in [2]:

Agreement in RAN2#129bis [2]:

RAN2 will not address cases of service continuity towards a relay path with intermediate relays in idle/inactive under the current WID.

In Rel-17 single-hop relay operation, the target U2N Relay UE can be in RRC_INACTIVE/IDLE state. Therefore, the reception of the RRCReconfigurationComplete message from the U2N Remote UE will trigger RRC setup/resume procedure for the U2N Relay UE to enter RRC_CONNECTED state. However, in Rel-19 multi-hop relay operation, the U2N Relay UE(s) in RRC_INACTIVE/IDLE is not considered as the candidate target(s) according to RAN2 agreement. Therefore, since the gNB is already aware that some U2N Relay UEs in RRC_CONNECTED are connected to the gNB via the multi-hop indirect path, it can choose this multi-hop indirect path as the target. Then, as soon as the gNB decides to switch the U2N Remote UE to the multi-hop indirect path, the reconfigurations to target U2N Relay UEs in the multi-hop indirect path are always performed among U2N Relay UEs, the target gNB-DU and gNB-CU in Scenarios C and D.

Observation 1: In Scenarios C and D, the U2N Relay UE(s) in RRC_INACTIVE/IDLE is not considered as the candidate target(s).

Observation 2: In Scenarios C and D, the gNB is already aware that some U2N Relay UEs in RRC_CONNECTED are connected to the gNB via the multi-hop indirect path.

Observation 3: In Scenarios C and D, the reconfigurations to target U2N Relay UEs in the multi-hop indirect path are always performed among U2N Relay UEs, the target gNB-DU and gNB-CU.

During the reconfiguration, the PC5 Relay RLC channel configuration for relaying the RBs of the U2N Remote UE and the bearer mapping configuration are configured to all U2N Relay UEs in target multi-hop indirect path. To this end, the gNB-CU needs to request to the gNB-DU serving the multi-hop indirect path the PC5 Relay RLC channel configurations towards the child UE and parent UE. In the U2N Remote UE’s initial access procedure, since RAN3 already has the working assumption to reuse the Peer UE ID IE as the counterpart information, this is already covered in F1AP.

Observation 4: In the U2N Remote UE’s initial access procedure, RAN3 already agreed to enhance the F1AP messages (e.g., Peer UE ID) to configure the PC5 Relay RLC channels towards the child UE and parent UE.

Observation 5: During the reconfigurations to the U2N Relay UEs in the target path, the PC5 Relay RLC channels towards the child UE and parent UE, and the bearer mapping configuration can be configured by using the F1AP signalings.

From RAN3’s point of view, when the gNB-DU serving the direct path and the gNB-DU serving the multi-hop indirect path are different, the inter-gNB-DU switch from direct to indirect path in Figure 8.19.4.1-1 of TS 38.401 can be used as a baseline to support Scenario C. For example, the inter-gNB-DU switch from direct to multi-hop indirect path can be illustrated in Figure 1.

Figure 1: Overall procedure for inter-gNB-DU switch direct to multi-hop indirect path

In Figure 1, the gNB-CU already knows that the U2N Relay UEs in multi-hop indirect path are connected via the target gNB-DU. Therefore, it repeats the reconfiguration to each U2N Relay UE in multi-hop indirect path via the target gNB-DU. Then, the gNB-CU sends the UE CONTEXT SETUP REQUEST message of the U2N Remote UE to the target gNB-DU. In Rel-17 inter-gNB-DU switch from direct to indirect path, the UE CONTEXT SETUP REQUEST message contains the following path switch configuration.

TS 38.473: … #### 9.3.1.263 Path Switch Configuration This IE provides information for switching to an indirect path from a direct path or from another indirect path. This IE is also used for only releasing the direct path during MP. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Target Relay UE ID; M; BIT STRING (SIZE(24)); Corresponds to the targetRelayUE-Identity contained in the CellGroupConfig IE, defined in TS 38.331 [8]. Remote UE Local ID; M; 9.3.1.267. T420; M; ENUMERATED (ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000); Corresponds to the t420 contained in the CellGroupConfig IE, defined in TS 38.331 [8]. …

Based on this IE, the gNB-DU generates the CellGroupConfig IE containing the sl-PathSwitchConfig-r17 IE. We think that in Rel-19 Multi-hop Relay, this IE can be used as a baseline to support Scenarios C and D. For example, the destination L2 ID of the First Relay UE in the target multi-hop indirect path can be set to the Target Relay UE ID IE. Our understanding is that there is no issue to reuse the T420 and the Remote UE Local ID. One issue to be discussed is whether the information (e.g., L2 ID) for other U2N Relay UEs in the multi-hop indirect path should be provided to the U2N Remote UE. Anyhow, whether to reuse the existing Path Switch Configuration IE is pending to RAN2’s final decision.

Proposal 1: Whether to reuse the existing Path Switch Configuration IE for Scenarios C and D is pending to RAN2 discussion on the sl-PathSwitchConfig-r17 IE.

Since the PC5 connections among the First Relay UE, the Intermediate Relay UE, and the Last Relay UE are already established, the PC5 connection between the U2N Remote UE and the target First Relay UE is only established in Step 11.

For the case where the gNB-DU serving the direct path and the gNB-DU serving the multi-hop relays are same, the existing intra-gNB-DU switch from direct to indirect path in Figure 8.19.4.2-1 of TS 38.401 can be used as a baseline. Similar to the inter-gNB-DU case, the UE CONTEXT MODIFICATION REQUEST message of the Remote UE is used to support the intra-gNB-DU case in Scenario C. Also, if RAN2 finally decides to reuse the sl-PathSwitchConfig-r17 IE for Scenario C, this message already contains this IE, thus having no RAN3 impact.

Proposal 2: For Scenario C, the existing procedures in Figures 8.19.4.1-1 and 8.19.4.2-1 of TS 38.401 can be used as a baseline.

From F1 signaling point of view, there is no big difference between the existing single-hop switch from direct to indirect path in TS 38.401 and Rel-19 direct to multi-hop indirect path switching, except the PC5 Relay RLC channel configurations towards the child UE and the Parent UE. The main difference is that in Rel-19 direct to multi-hop indirect path switching, the gNB initiates the Reconfigurations to the all U2N Relay UEs to support the U2N Remote UE on the multi-hop indirect path. However, this point can be covered by the RAN2 baseline procedure. Hence, there is no need for the RAN3 to capture the overall procedure to support Scenario C.

Proposal 3: No need to capture the procedure for Scenario C into TS 38.401.

For Scenario B, RAN2 discussed whether the target U2N Relay UE is the (First/Intermediate/Last) Relay UE which is on the source multi-hop indirect path, and concluded the following agreements [3]:

Agreement in RAN2#128 [3]:

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

In Scenario D, a similar approach could be discussed. That is, the source U2N Relay UE can be a new Relay UE which is not on the target multi-hop indirect path. It is also possible that the source U2N Relay UE can be the (First/Intermediate/Last) Relay UE which is on the target multi-hop indirect path. For now, RAN3 can consider the first case (i.e., the source U2N Relay UE is a new Relay UE which is not on the target multi-hop indirect path) as a baseline. RAN2 will determine which cases are additionally supported in Rel-19.

Proposal 4: In Scenario D, RAN3 should first consider that the source U2N Relay UE is a new Relay UE which is not on the target multi-hop indirect path.

Regarding the case that the source U2N Relay UE is a new U2N Relay UE which is not on the target relay path, there may be two sub-cases. When the gNB-DU serving the single-hop indirect path and the gNB-DU serving the multi-hop indirect path are same, the U2N Remote UE moves from one cell serving the source single-hop indirect path to another cell serving the target multi-hop indirect path within the same gNB-DU. In this case, the gNB-CU initiates the UE Context Modification procedure to the gNB-DU to inform the path switch from single-hop indirect path to the multi-hop indirect path. We think that the Path Switch Configuration IE containing the L2 ID of the target First Relay UE can be also included in the UE CONTEXT MODIFICATION REQUEST message. In addition, the gNB-CU can request to the gNB-DU the PC5 Relay RLC channel configurations for relaying the RBs of the U2N Remote UE by using the F1AP messages with the Peer UE ID IE, and then configure it to each U2N Relay UEs in target multi-hop indirect path. Therefore, it seems that no enhancement in the intra-gNB-DU case is needed for Scenario D.

For the case where the gNB-DU serving the source single-hop indirect path and the gNB-DU serving the target multi-hop indirect path are different, the inter-gNB-DU switch from single-hop indirect to multi-hop indirect path can be illustrated in Figure 2.

Figure 2: Overall procedure for inter-gNB-DU switch from single-hop indirect to multi-hop direct path

As mentioned earlier, if RAN2 agrees to reuse the sl-PathSwitchConfig-r17 IE for Scenarios C and D, the gNB-CU sends to the target gNB-DU the UE CONTEXT SETUP REQUEST message containing the existing Path Switch Configuration IE in Step 7. Upon reception of this IE, the gNB-DU then generates the ReconfigurationWithSync IE including the sl-PathSwitchConfig-r17 IE. Hence, there is no need to further enhance the existing UE Context Setup procedure to support the inter-gNB-DU switch from single-hop indirect to multi-hop indirect path.

Proposal 5: In Scenario D, there is no need to further enhance the UE Context Setup and UE Context Modification procedure.

As discussed in Proposal 3, from F1 signaling point of view, there is no big difference between the procedure for the inter-gNB-DU switch from single-hop indirect to single-hop indirect path and the inter-gNB-DU switch from single-hop indirect to multi-hop indirect path, except the PC5 Relay RLC channel configurations towards the child UE and the Parent UE. Also, in Rel-18 U2N Relay enhancement, it was already agreed in RAN3 that the overall procedure for path switch from single-hop indirect to single-hop indirect path is not introduced in TS 38.401. Hence, there is no need for the RAN3 to capture the path switch from single-hop indirect to multi-hop indirect path into TS 38.401.

Proposal 6: No need to capture the overall procedure for Scenario D into TS 38.401.

3.	Conclusion

In this contribution, we focused the RAN3 impacts on intra-gNB service continuity for Scenarios C and D, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: Whether to reuse the existing Path Switch Configuration IE for Scenarios C and D is pending to RAN2 discussion on the sl-PathSwitchConfig-r17 IE.

Proposal 2: For Scenario C, the existing procedures in Figures 8.19.4.1-1 and 8.19.4.2-1 of TS 38.401 can be used as a baseline.

Proposal 3: No need to capture the procedure for Scenario C into TS 38.401.

Proposal 4: In Scenario D, RAN3 should first consider that the source U2N Relay UE is a new Relay UE which is not on the target multi-hop indirect path.

Proposal 5: In Scenario D, there is no need to further enhance the UE Context Setup and UE Context Modification procedure.

Proposal 6: No need to capture the overall procedure for Scenario D into TS 38.401.

4.	Reference

- RP-242349, “Revised WID on NR sidelink multi-hop relay,” LG Electronics, InterDigital, FirstNet, September, 2024.

- R2-2502984, “Report from session on positioning and sidelink relay,” Session Chair (MediaTek), April, 2025.

- R2-2501334, “Report from session on positioning and sidelink relay,” Session Chair (MediaTek), November, 2024.