3GPP TSG-RAN WG3 Meeting #128	R3-253156

Malta, MT 19th – 23th May 2025

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	Discussion on split architecture

Document for:	Discussions & Approval

1	Introduction

This contribution aligns with the objectives [1] of the Rel-19 WI on 'Enhancements for AI/ML in NG-RAN.' Based on the conclusions and progress made in Rel-18 Leftovers, as captured in TR 38.743 [2], we present our analysis of the normative work for NR-DC Mobility Optimization, Support of split architecture, and Continuous MDT Collection.

The detailed objectives of the WI are listed as follows:

- Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3]

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]:

  - Mobility Optimization for NR-DC

  - Split architecture support for Rel-18 use cases

  - Continuous MDT collection targeting the same UE across RRC states

Note: Coordination with RAN2, SA5 when needed if any.

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

Regarding the support of split architecture, we have following agreements and left issues:

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level. Send the LS to SA5 to seek the clarification on UE throughput UL/DL at PDCP level and UL/DL Packet Loss Rate at PDCP layer. WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19. Defer the discussion on packet loss rate in the split architecture in the next meeting. For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity? Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request. Agree the TP to 37.480 and TP to 37.483 in this meeting for the new procedure over E1 interface. Change the agreement: Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU. To the following WA: WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

In this contribution, we further provide our views on the impacts on UE performance metrics and measured EC over E1&F1 interfaces. Additionally, the corresponding TPs are provided.

2	Discussion

2.1 UE performance feedback over E1 and F1

As discussed in the last meeting, RAN3 reached consensus on considering the following UE performance metrics at the PDCP level: Average UL/DL throughput, Average UL/DL packet delay, and Average DL packet loss. An LS has been sent to SA5 to seek confirmation on the feasibility of this approach.

Moreover, we agreed to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over the E1 interface to support the transfer of UL/DL UE throughput and UL/DL packet delay as part of the UE performance metrics. In the current BLCR, only the new procedure was captured due to time constraints. Therefore, the request and report of UL/DL UE throughput and UL/DL packet delay should also be included in the BLCR.

- The request and report of UL/DL UE throughput and UL/DL packet delay should also be included in the DATA COLLECTION REQUEST message and DATA COLLECTION UPDATE message.

The DATA COLLECTION REQUEST message over the E1 interface is a non-UE-associated message. Therefore, the indication should be introduced in an E1 UE-associated message. In inter-node scenarios, the DATA COLLECTION ID is introduced in the HANDOVER REQUEST message to identify the data collection context, enabling the target node to determine which UE should be measured based on the configuration it received previously. Similarly, the CU-UP also needs to know which UE’s performance metrics it should measure at the time of bearer establishment.

- Introduce the DATA COLLECTION ID in the BEARER CONTEXT SETUP/MODIFICATION REQUEST message. DATA COLLECTION ID includes gNB-CU-CP Measurement ID and gNB-CU-UP Measurement ID.

Another issue is whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection for packet delay, e.g., start, stop, UL, DL, and periodicity?

For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

As we agreed in the previous meeting,

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

For the QoS monitoring mechanism, the CU-UP configures the QoS monitoring setup in the UE CONTEXT SETUP/MODIFICATION message by including the QoS Monitoring Request IE, which instructs the DU to perform the measurements of packet delay.

| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

From my perspective, it is better to decouple the QoS monitoring mechanisms and AI/ML-related data collection. Otherwise, when both features need to be supported simultaneously, their configurations would become identical, which is not a user-friendly or flexible design. Therefore, a new IE similar to the QoS Monitoring Request can be introduced to specifically configure AI/ML-related data collection.

- Introduce the AI/ML Data Collection Request IE in the UE CONTEXT SETUP/MODIFICATION message to decouple it from the QoS monitoring mechanisms.

In Rel-18, it was agreed that Packet Delay should be configured/reported for both UL and DL simultaneously, rather than reported separately. Therefore, in the AI/ML Data Collection Request IE, it is sufficient to indicate "Both UL/DL" only, without the need to specify either UL or DL individually. The codepoint “Stop” is needed to stop the measurement collection.

- The codepoint in the AI/ML Data Collection Request IE is {Both, Stop, …}.

With the respect to periodicity, the question is how the periodicity of Assistance Information Data transmission over F1-U aligns with the periodicity configured for AI/ML features over DATA COLLECTION REQUEST message.

In my understanding, there are two potential options to address this alignment:

- Option 1 (By implementation): Utilize the Assistance Info. Report Polling Flag in the DL USER DATA (PDU Type 0) header. This flag can be set by gNB-CU to instruct gNB-DU to report packet delay values, allowing the reporting behavior to be triggered in a manner that aligns with the periodicity configured for the AI/ML use case.

- Option 2 (By explicit configuration): Introduce explicit periodocity for AI/ML purposes in the F1AP UE CONTEXT SETUP message. This would allow the CU to indicate the desired averaging window and periodicity for AI/ML delay reporting, enabling DU to coordinate accordingly.

In the case where Option 1 (i.e., using the Assistance Info. Report Polling Flag) is leveraged, the CU-UP can implement appropriate logic to collect and process packet delay statistics aligned with each configured feature. This approach allows the CU-UP to interpret the single Delay Result IE in the Assistance Information Data based on the context of the reporting trigger, and thus associate the reported value with the intended feature (e.g., AI/ML or QoS Monitoring). Therefore, we think there is no need to indicate the periodicity configuration with the AI/ML DATA COLLECTION REQUEST IE in the UE CONTEXT SETUP/MODIFICATION message.

- There is no need to indicate the periodicity configuration with the AI/ML DATA COLLECTION REQUEST IE in the UE CONTEXT SETUP/MODIFICATION message.

2.2 Measured EC over F1 interface

Regarding the EC metrics, we previously agreed to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over the F1 interface to transfer measured EC from DU to CU. However, some companies expressed the desire to revisit this agreement after receiving feedback from SA5 and observing progress regarding UE performance feedback.

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

At this meeting, we have received the reply LS [3] from SA5, indicating an update to their specification. Therefore, RAN3 needs to continue the work to enhance the F1 interface to support the transfer of EC. Upon further consideration, we have observed that the F1 interface currently only needs to transfer the measured EC. Introducing a new dedicated procedure for this limited usage seems excessive.

Hence, we propose to revert the previous agreement on introducing a new Data Collection Reporting procedure over the F1 interface for Energy Cost. Instead, utilize existing Resource Status Reporting procedures to transfer the measured Energy Cost from DU to CU.

- Turn WA to agreement: Reuse the existing Resource Status Reporting procedures to transfer the measured Energy Cost from DU to CU.

The encoding format of measured energy cost over F1 interface shall follow the agreement of measured energy cost over Xn interface.

- Define the Energy Cost IE over F1 interface as an INTEGER (0..10000, ...).

3	Conclusion

Following is our proposals and observations:

- The request and report of UL/DL UE throughput and UL/DL packet delay should also be included in the DATA COLLECTION REQUEST message and DATA COLLECTION UPDATE message.

- Introduce the DATA COLLECTION ID in the BEARER CONTEXT SETUP REQUEST message. DATA COLLECTION ID includes gNB-CU-CP Measurement ID and gNB-CU-UP Measurement ID.

- The codepoint in the AI/ML Data Collection Request IE is {Both, Stop, …}

- There is no need to indicate the periodicity configuration with the AI/ML DATA COLLECTION REQUEST IE in the UE CONTEXT SETUP/MODIFICATION message.

- Turn WA to agreement: Reuse the existing Resource Status Reporting procedures to transfer the measured Energy Cost from DU to CU.

- Define the Energy Cost IE over F1 interface as an INTEGER (0..10000, ...).

4	Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] R3-251529, LS reply to LS on energy saving for split gNB, SA5