3GPP TSG-RAN WG3 Meeting #128                                                              R3-253172

Malta, MT, 19 - 23 May, 2025

Agenda item: 		17.3

Source: 		Qualcomm Incorporated

Title: 	(TPs to BL CR 38.401) Stage-2 procedures and remaining open issues for OD SIB1

Document for:		Discussion

# 1	Introduction

RAN3 achieved the following agreements related to OD SIB1 [1]:

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed Focus on case2 in RAN3 first. Case 2 can be supported with the below details: UL WUS configuration is decided by NES Cell DU. UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface. Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures. It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration. In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: - One Choice “Start with UL WUS Condiguration” - One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”. The Xn/F1 TP capturing the above agreements in the tabular are agreed in: R3-252385 (Xn) and R3-252386 (F1).

The following discussion aims to define the stage-2 procedures for the supports of OD SIB1. In this context, remaining stage-2 and stage-3 issues are identified.

# 2	Discussion

The Annex below includes a TP to the BL CR to 38.401 with the procedures for OD SIB1 operation. The TP includes the various editor’s notes with open stage-2 and stage-3 issues RAN3 needs to resolve. A separate contribution includes the skeleton for the BL CR to 38.401.

The stage-2 issues are the following:

Proposal 1-1: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION REQUEST to include for each UL WUS configuration the corresponding NES Cells that use this configuration.

Proposal 1-2: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION REQUEST to include for each UL WUS configuration a list of recommended Cell-A’s that should transmit this UL WUS configuration.

Proposal 2-1: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION RESPONSE to include for each UL WUS configuration the corresponding cell-As that transmit this configuration, or how this message indicates whether an UL WUS configuration is transmitted by at least some cell-A’ vs. not transmitted at all.

Proposal 2-2: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION RESPONSE to include any information in response to gNB1’s request to discontinue transmission of UL WUS configuration(s).

Proposal 3: RAN3 to decide whether it is the NES gNB-DU or the NES gNB-CU that delays the start of OD SIB1 operation for a NES cell until peer gNBs have activated UL WUS transmission.

Proposal 4: RAN3 to decide whether cell-A gNB can also determine for a cell to resume the transmission of an UL WUS configuration after it has stopped it before.

The stage-3 issues are the following:

Proposal 5-1: RAN3 to decide whether the F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE messages to include an explicit code point to indicate that a cell does not support OD SIB1 operation or whether the absence of an UL WUS configuration IE can serve as an implicit indication for this purpose.

Proposal 5-2: RAN3 to decide whether the F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE messages to include an indication for no support of OD SIB1 operation only when the cell discontinues OD SIB1 operation mode or also when it has not supported OD SIB1 operation before.

Proposal 6: RAN3 to decide how to refer to an UL WUS configuration in the UL WUS CONFIGURATION PRIVISION RESPONSE message, e.g., via explicit OCTET string or via UL WUS configuration ID included in the UL WUS CONFIGURATION PROVISIONREQUEST message.

Proposal 7: RAN3 to define the code point(s) in the served cell information of the GNB-CU CONFIGURATION UPDATE message to indicate that the cell that can activate OD SIB1 operation mode or that it should discontinue OD SIB1 operation mode.

Proposal 8: RAN3 to define stage-3 enhancements based on agreements for stage-2 issues above.

TP to BL CR to TS 38.401:

Proposal 9: RAN3 to approve TP for BL CR to TS 38.401 in the Annex.

# 3	Conclusion

The following discussion aims to define a stage-2 procedure for the support of OD SIB1. The following observations and proposals have been made:

The stage-2 issues are the following:

Proposal 1-1: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION REQUEST to include for each UL WUS configuration the corresponding NES Cells that use this configuration.

Proposal 1-2: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION REQUEST to include for each UL WUS configuration a list of recommended Cell-A’s that should transmit this UL WUS configuration.

Proposal 2-1: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION RESPONSE to include for each UL WUS configuration the corresponding cell-As that transmit this configuration, or how this message indicates whether an UL WUS configuration is transmitted by at least some cell-A’ vs. not transmitted at all.

Proposal 2-2: RAN3 to decide whether the UL WUS CONFIGURATION PROVISION RESPONSE to include any information in response to gNB1’s request to discontinue transmission of UL WUS configuration(s).

Proposal 3: RAN3 to decide whether it is the NES gNB-DU or the NES gNB-CU that delays the start of OD SIB1 operation for a NES cell until peer gNBs have activated UL WUS transmission.

Proposal 4: RAN3 to decide whether cell-A gNB can also determine for a cell to resume the transmission of an UL WUS configuration after it has stopped it before.

The stage-3 issues are the following:

Proposal 5-1: RAN3 to decide whether the F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE messages to include an explicit code point to indicate that a cell does not support OD SIB1 operation or whether the absence of an UL WUS configuration IE can serve as an implicit indication for this purpose.

Proposal 5-2: RAN3 to decide whether the F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE messages to include an indication for no support of OD SIB1 operation only when the cell discontinues OD SIB1 operation mode or also when it has not supported OD SIB1 operation before.

Proposal 6: RAN3 to decide how to refer to an UL WUS configuration in the UL WUS CONFIGURATION PRIVISION RESPONSE message, e.g., via explicit OCTET string or via UL WUS configuration ID included in the UL WUS CONFIGURATION PROVISIONREQUEST message.

Proposal 7: RAN3 to define the code point(s) in the served cell information of the GNB-CU CONFIGURATION UPDATE message to indicate that the cell that can activate OD SIB1 operation mode or that it should discontinue OD SIB1 operation mode.

Proposal 8: RAN3 to define stage-3 enhancements based on agreements for stage-2 issues above.

TP to BL CR to TS 38.401:

Proposal 9: RAN3 to approve TP for BL CR to TS 38.401 in the Annex.

# Reference

[1]  Chair notes, 3GPP TSG RAN WG3 Meeting #127bis, Wuhan, China, April 2025

# Annex: TP for BL CR to 38.401

START OF CHANGE

## 8.x	Procedures for network energy savings

### 8.x.1	Procedures for on-demand SIB1

The high-level functionality of on-demand SIB1 for network energy savings is specified in TS 38.300 [2].

The following describes the inter-gNB signaling procedures and enhancements to intra-gNB signaling for split architecture in support of on-demand SIB1.

Figure 8.x.1-1 shows an example for the procedure, where a gNB (gNB1) with cells supporting OD SIB1 operation requests a peer gNB (gNB2) to start or to stop transmitting the UL WUS configuration(s) of these cells.

Figure 8.x.1-1: Procedure for provisioning of UL WUS configuration to peer gNBs in support of OD SIB1

The procedure has the following steps:

1. gNB1-DU sends to gNB1-CU the UL WUS configuration for each cell that supports OD SIB1 operation using the F1 SETUP REQUEST or GNB-DU CONFIGURATION UPDATE messages. gNB1-DU further indicates to gNB1-CU the cells that have discontinued support of OD SIB1 operation via the GNB-DU CONFIGURATION UPDATE message.

Editor’s NOTE: For stage 3, FFS whether an explicit code point is used to indicate that a cell does not support OD SIB1 operation or whether the absence of an UL WUS configuration IE can serve as an implicit indication for this purpose.

Editor’s NOTE: For stage 3, FFS whether an indication for no support of OD SIB1 operation is only sent when the cell discontinues OD SIB1 operation mode or also when it has not supported it before or recently.

2. gNB1-CU replies with the F1 SETUP RESPONSE or GNB-DU CONFIGURATION UPDATE ACKNOWLEDGEMENT message, accordingly.

3. gNB1-CU determines the peer gNBs that are suitable to transmit the UL WUS CONFIGURATION(s) received and/or that should stop transmitting UL WUS CONFIGURATION(s) for cells that have discontinued support of OD SIB1 operation.

4. gNB1 sends the UL WUS CONFIGURATION PROVISION REQUEST message to gNB2 including UL WUS CONFIGURTION(s) of cells that support OD SIB1 mode, and it indicates whether ongoing transmissions of UL WUS CONFIGURATION can be discontinued.

Editor’s NOTE: FFS whether gNB1-CU to include for each UL WUS configuration the corresponding NES cells (e.g., using NR CGI) that use this configuration.

Editor’s NOTE: FFS whether gNB1-CU to include for each UL WUS configuration a list of recommended Cell-A’s that should transmit this UL WUS configuration.

5. gNB2-CU determines the cells that should transmit the UL WUS configuration(s) received. It further determines the cells that should discontinue transmission of UL WUS configuration(s).

6. gNB2 sends UL WUS CONFIGURATION PROVISION RESPONSE message to gNB1 including information on the UL WUS configurations transmitted.

Editor’s NOTE: FFS whether this message contains for each UL WUS configuration the corresponding cell-As that transmit this configuration, or how this message indicates whether an UL WUS configuration is transmitted by at least some cell-A’ vs. not transmitted at all.

Editor’s NOTE: FFS whether this message contains any information in response to gNB1’s request to discontinue transmission of UL WUS configuration(s).

Editor’s NOTE: For stage 3, FFS how to refer to an UL WUS configuration in the UL WUS CONFIGURATION PRIVISION RESPONSE message, e.g., via explicit OCTET string or via UL WUS configuration ID included in the UL WUS CONFIGURATION PROVISIONREQUEST message.

7. gNB2-CU provides updated SIBx information to gNB2-DU via the GNB-CU CONFIGURATION UPDATE message, which contains the new UL WUS configuration(s) to be transmitted and/or the indication to discontinue transmission of UL WUS configuration(s) for each cell affected.

Editor’s NOTE: FFS the exact term for SIBx.

Editor’s NOTE: For stage 3, FFS whether discontinuation of transmission of an UL WUS configuration will be indicated implicitly, e.g., via absence of an UL WUS configuration configured for SIBx or absence of configuration of SIBx, or explicitly, e.g., via codepoint for each UL WUS configuration.

8. gNB2-DU replies with the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGEMENT message, and it performs SIBx update for the cells, where it has changed.

9. Based on the confirmation of transmission of UL WUS configuration(s) received from peer gNBs, gNB1-CU determines the gNB1-DU cells that can enter OD SIB1 operation mode.

10. gNB1-CU informs gNB1-DU about the cells that can activate OD SIB1 operation via the GNB-CU CONFGIGURATION UPDATE message.

Editor’s NOTE: For stage 3, FFS code points for this indication.

NOTE: Before gNB1-CU indicates to gNB1-DU that a cell can activate OD SIB1 operation, it should leave sufficient time for peer gNBs to start the transmission of this cell’s UL WUS configuration, based on implementation.

11. gNB1-DU sends the GNB-CU CONFGIGURATION UPDATE ACKNOWLEDGE message to gNB1-CU and activates OD SIB1 operation for the cells indicated by gNB1-CU.

Editor’s NOTE: FFS whether it is the NES gNB-DU rather than NES gNB-CU that delays start of OD SIB1 operation until peer gNBs have activated UL WUS transmission.

Figure 8.x.1-2 shows an example for the procedure, where a gNB (gNB2) indicates to a peer gNB (gNB1) that it discontinues the transmission of an UL WUS configuration.

Figure 8.x.1-2: Procedure for update of UL WUS transmission status to peer gNBs

The procedure has the following steps:

1. gNB2-CU determines that a cell should discontinue the transmission of an UL WUS configuration.

Editor’s NOTE: FFS whether the cell-A gNB can also determine for a cell to resume the transmission of an UL WUS configuration. In case this is supported, the following procedure needs to be updated accordingly.

2. gNB2 informs gNB1 about the discontinuation of the transmission of an UL WUS configuration via the UL WUS CONFIGURATION STATUS UPDATE message.

Editor’s NOTE: FFS whether this message includes the cell-A(s) for which the transmission of this UL WUS configuration has been discontinued.

3. Based on the information received from gNB2 and potentially other peer gNBs, gNB1-CU determines the cells that should discontinue OD SIB1 operation.

4. gNB1-CU informs gNB1-DU about the cells that should discontinue OD SIB1 operation using the GNB-CU CONFGIGURATION UPDATE message.

Editor’s NOTE: For stage 3, FFS code points for this indication.

5. gNB1-DU replies with the GNB-CU CONFGIGURATION UPDATE ACKNOWLEDGE message and discontinues OD SIB1 mode for the cells indicated by gNB1-CU.

6. gNB2-CU may provide updated SIBx information to gNB2-DU via the GNB-CU CONFIGURATION UPDATE message containing the indication to discontinue transmission of UL WUS configuration(s) for each cell affected.

Editor’s NOTE: FFS the exact term for SIBx.

7. gNB2-DU replies with the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGEMENT message, and it performs SIBx update for the cells, where it has changed.

NOTE: Before gNB2-CU sends the indication to gNB2-DU to discontinue the transmission of an UL WUS configuration(s), it should leave sufficient time for gNB1 to deactivate the OD SIB1 operation for the cells affected, based on implementation.

END OF CHANGE