3GPP TSG-RAN WG3 Meeting #128	R3-253175

Malta, MT, 19 – 23 May 2025

Agenda Item:	12.2

Source: 				LG Electronics Inc.

Title:			Further consideration on support of WAB

Document for:		Discussion and Decision

1.	Introduction

In RAN3#126 meeting, there was no conclusion on how to prevent multi-hop backhauling during the UE’s initial access. Two possible options can be considered to solve this issue. In RAN3#127bis meeting, RAN3 had common understanding that Option 1 is implementation specific, but it is still FFS on whether to support Option 3. Another open issue is whether and how to setup Xn interface between the WAB-gNBs. Also, based on the SA2 reply LS [1], RAN3 agreed to support two-logical gNB solution for the UE’s AMF change. However, whether to support the single-gNB solution with two logical cells is still FFS. In this contribution, we will discuss the remaining open issues (e.g., for multi-hop backhauling, the Xn setup between WAB-gNB, and the mobility), and provide our views on it.

2.	Discussion

In RAN3#126 meeting, RAN3 agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO [2].

- For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

However, there was no conclusion on RAN based approach for preventing multi-hop backhauling at the WAB-MT’s initial access. The following options could be considered [2]:

- Option 1: For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

- Option 3: For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

In RAN3#127bis meeting, RAN3 concluded that Option 1 is implementation specific and it is not subject to specifications changes. However, it is still FFS on whether to support Option 3 in this release.

In Option 3, the WAB-gNB broadcasts a new indicator in SIB to bar WAB-MT. Since the WAB-MT does not attempt to access to the WAB-gNB cell due to a new indicator, unnecessary access trials from WAB-MT can be prevented. However, the WAB-gNB needs to be enhanced to broadcast a new indicator in SIB, thus resulting in Stage 3 specification impact. Our understanding is that since this solution is very similar to the approach in Rel-18 mobile IAB, foreseen specification impacts are already well known from Rel-18 mobile IAB.

Observation 1: Option 3 prevents unnecessary access trials from WAB-MT.

Observation 2: Option 3 has Stage 3 specification impact, which is already known from Rel-18 mobile IAB.

Based on the above observations, the following proposal is suggested to RAN3:

Proposal 1: In this release, RAN3 also supports Option 3 to avoid the multi-hop WAB for initial access.

Proposal 1a: If RAN3 decides to support Option 3, it is proposed to send a LS to consult with RAN2.

For Case b) in SA2 LS [3], it seems that the WAB-MT tries to access to the co-located WAB-gNB. During the initial access of the WAB-MT in IDLE mode, it cannot access to the co-located WAB-gNB because the co-located WAB-gNB is not yet initialized. After the WAB-gNB initialization, the WAB-MT may detect the WAB-gNB cells as the suitable cell. However, the WAB-MT can obtain the information (e.g., gNB ID, TAC, PCI, NCGI, and so on) related to the co-located WAB-gNB by using internal interface between WAB-MT and WAB-gNB. Therefore, the WAB-MT itself avoids accessing the WAB-gNB cells by implementation.

For the WAB-MT in CONNECTED mode, the BH-gNB may determine to initiate the handover of the WAB-MT towards the co-located WAB-gNB. This can be avoided by e.g. WAB implementation. For example, the WAB-MT may not report to the BH-gNB the measurement results for the co-located WAB-gNB. As mentioned earlier, this is because the WAB-MT can obtain the information related to the co-located WAB-gNB by using internal interface between WAB-MT and WAB-gNB. In this case, the BH-gNB does not consider the WAB-gNB as a candidate target node. In addition, RAN3 already agreed to introduce the WAB-MT Identifier in Xn Setup and NG-RAN node Configuration Update procedure. Therefore, the BH-gNB can explicitly discover the co-location of the WAB-MT and the WAB-gNB.

Proposal 2: The WAB implementation enables the WAB-MT to prevent accessing to the WAB-gNB belonging to same WAB-node.

Based on the above proposals, RAN3 should provide the answers for the SA2 questions on how to prevent the multi-hop WAB during initial access. The above proposals are captured into a draft reply LS in Appendix. Therefore, the following proposal is suggested to RAN3:

Proposal 3: It is proposed to send a reply LS to SA2 in Appendix#1.

Based on the RAN3 agreement in [2], SA2 has agreed the following CR [4] to support the prevention of the multi-hop backhauling.

S2-2501321 [4]: … #### 5.49.2.2 Configuration of the MWAB-gNB The MWAB-gNB's configuration is managed by the OAM of the MWAB Broadcasted PLMN/SNPN and the address (or FQDN) of the OAM server per MWAB Broadcasted PLMN/SNPN may be configured at the MWAB-gNB. The OAM server of the MWAB Broadcasted PLMN/SNPN provides (re-)configuration parameters considering location of MWAB as specified in TS 38.401 [42], e.g. for the purpose including the access stratum operation of the MWAB-gNB, the N2, N3 and Xn interface management, activating/deactivating the MWAB-gNB operation and to assist the MWAB-gNB providing information used by MWAB-UE for the BH PDU Session(s) management via URSP processing. NOTE 1: It is expected the OAM of the MWAB Broadcasted PLMN/SNPN is aware of location in which the MWAB operation can be authorized and also can obtain MWAB's current location. Only when the MWAB is authorized successfully, the configuration information can be provisioned to the MWAB-gNB. Alternatively, the OAM can also provision configuration parameters related to different areas, as defined in TS 38.401 [42]. The MWAB-gNB shall be configured with the supported S-NSSAI(s) in the MWAB Broadcasted PLMN/SNPN. NOTE 2: A MWAB-gNB is not expected to be configured to support any dedicated S-NSSAI for BH PDU Session(s) used by MWAB-UEs for MWAB operation in the MWAB Broadcasted PLMN/SNPN as specified in TS 38.401 [42]. The support at a MWAB-gNB of S-NSSAIs dedicated to MWAB-UE BH PDU Session(s) in the MWAB Broadcasted PLMN/SNPN would mean that the MWAB-gNB could support serving the MWAB-UEs BH-PDU Session(s) in the broadcasted PLMN/SNPN. … #### 5.49.5.3 Impact of MWAB mobility The mobility of MWAB includes the MWAB-UE and the MWAB-gNB aspect. - The mobility of the MWAB-UE reuses the exiting UE mobility procedures specified for normal UE. - During Mobility, a MWAB-gNB can obtain or apply a new configuration as specified in TS 38.401 [42] from the OAM server of MWAB Broadcasted PLMN and, if any new AMF was provided as part of the new configuration, the MWAB-gNB sets up the N2 connection with the new AMF and it releases the N2 connection with any old AMF removed from the MWAB-gNB configuration. - The MWAB-UE may request new or modify existing BH PDU Sessions for the corresponding N2, N3 and Xn interfaces, upon request from the MWAB-gNB as it moves in the network and the MWAB-gNB updates the TNL association as specified in clause 5.21.1 and TS 38.401 [42] to use the new TNL information associated with the new BH PDU Session(s) as needed. - SSC mode 3 may be applied to the BH PDU Sessions to minimize traffic interruption when UPF re-allocation for these BH PDU Sessions is required during the MWAB mobility. - When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change. - During handover of a MWAB-UE BH-PDU sessions towards a target MWAB-gNB, the MWAB-gNB fails the handover as specified in TS 38.401 [42] as the dedicated slices for BH PDU sessions of the MWAB-UE are not supported by the MWAB-gNB.

In this CR, it seems that the target WAB-gNB may accept the handover for the WAB-MT if it is configured by OAM to support the dedicated slices for WAB operation. However, we think that this is not aligned with the previous RAN3 agreement. As discussed earlier, it was agreed in RAN3 that the target WAB-gNB always rejects the handover procedure for the WAB-MT. For prevention of multi-hop backhauling during the HO, therefore, there is the mismatch to be solved between RAN3 and SA2.

Proposal 4: It is proposed to discuss how to solve the mismatch between the RAN3 and SA2 for prevention of multi-hop backhauling during the HO.

The RAN3 agreement on prevention of multi-hop backhauling during the HO is not yet reflected in the BL CR to TS 38.401. Also, if the solutions for initial access case (including the WAB-MT accessing to co-located WAB-gNB) are agreed, it is better to have a new sub-clause to capture the agreements on the prevention of multi-hop backhauling in TS 38.401.

Proposal 5: It is proposed to introduce a new sub-clause to capture the agreements on the prevention of multi-hop backhauling in TS 38.401.

In RAN3#127 meeting, it was agreed to include the Additional ULI IE (which contains a CGI and a TAI) into the User Location Information IE in the BL CR to TS 38.413 [5]. Therefore, this IE is provided to the UE’s AMF by using the non UE-associated signalling. In RAN3#127bis meeting, however, it was also agreed that the WAB-gNB reports the Additional ULI to the network within the NG Setup and RAN Configuration Update procedures in order to provide the Additional ULI for the case where there are no UEs in CONNECTED state. As a result, the Additional ULI to indicate the location information of WAB-node is currently provided to the UE’s AMF by using the non UE-associated signalling and UE-associated signalling.

We think that the Additional ULI needs to be removed from the UE-associated signalling. As soon as the WAB-gNB is initialized (e.g., OAM configuration), it establishes the NG connection(s) toward the AMF(s). Since the Additional ULI is already included in the NG SETUP REQUEST message, the AMF can determine the current location of the WAB-node. When the additional ULI has changed (e.g. due to WAB-node movement), the WAB-gNB derives the new Additional ULI and reports it to the AMF by the RAN CONFIGURATION UPDATE message. Therefore, although the Additional ULI is removed from the UE-associated signallings, there is no issue to determine the current location of the WAB-node in the AMF.

Proposal 6: The Additional ULI is removed from the UE-associated signallings.

In TS 23.501, SA2 already captured that the Additional ULI IE is provided to the AMF by the UE-associated signallings. To reflect the RAN3 agreements to the SA2 specification, therefore, it is needed to inform the SA2 of the removal of the Additional ULI IE in the UE-associated signallings.

Proposal 6a: It is proposed to inform the SA2 of the removal of the Additional ULI in the UE-associated signallings (a draft Reply LS in Appendix#2).

It is still FFS whether to introduce a “WAB-gNB” indication in the NG SETUP REQUEST message. Since the Additional ULI in the NG SETUP REQUEST message enables the AMF to discover that the transmitting NG-RAN node is a WAB-gNB, there is no need to introduce another new indication in this NGAP message.

Proposal 6b: Upon the reception of the Additional ULI in the NG SETUP REQUEST message, the AMF discovers that the transmitting NG-RAN node is a WAB-gNB.

One of the objectives in this WID is to support the Xn interface by the WAB-gNB with the BH RAN node and with other neighbouring gNBs, including how to avoid setting up Xn between WAB-gNBs. However, we think that in some cases, the Xn connection between two WAB-gNBs is beneficial to support the UE mobility. For example, if one WAB-gNB is operated as a stationary gNB for a long time, the Xn interface can be established between this stationary WAB-gNB and the other moving WAB-gNB. Since the operator has additional information for WAB (e.g., the expected mobility (i.e., stationary/mobile) and the expected moving trajectory of the WAB-node), it can decide whether to allow the establishment of Xn interface between the WAB-gNBs. Then, this information needs to be (pre-)configured to the WAB-gNB and/or the UE’s AMF by OAM.

Proposal 7: The Xn interface between WAB-gNBs can be established.

As shown in Figure X.1-1 of [6], after the WAB-gNB setup, the WAB-gNB can initiate the establishment of the Xn connection(s) towards the BH-gNB and/or other NG-RAN node(s) by sending the Xn SETUP REQUEST message. According to previous RAN3 agreement, the WAB-gNB includes the information (i.e., the WAB-MT Identifier IE) related to the WAB-MT to the BH-gNB to indicate the co-location of the WAB-MT and the WAB-gNB. Based on Solution 3, the WAB-gNB cell information may be also provided to other NG node(s). In order to support Proposal 7, we think that the WAB-gNB can contain additional assistance information (e.g., the expected mobility (i.e., stationary/mobile) and the expected moving trajectory of the WAB-node, and so on) related to the WAB mobility in the Xn SETUP REQUEST message. If the receiving NG-RAN node is another WAB-gNB, it decides whether to establish the Xn interface with the WAB-gNB based on this assistance information. For example, if both WAB-gNBs have a similar moving characteristics/route, or either or both of WAB-gNBs are stationary, the receiving WAB-gNB can respond with the Xn SETUP RESPONSE message. In addition, it is possible for the receiving WAB-gNB to include additional assistance information related to the WAB mobility in the Xn SETUP RESPONSE message. The initiating WAB-gNB can take this information into account to manage the Xn interface between WAB-gNBs.

If the Xn setup between WAB-gNBs is not allowed or deemed not appropriate, the receiving WAB-gNB should reject the Xn setup with the initiating WAB-gNB and reply with the Xn SETUP FAILURE message. When the Xn SETUP FAILURE message contains appropriate information (e.g., WAB-gNB indication, new cause value “Target not allowed due to WAB”), the initiating node does not reinitiate the Xn Setup procedure towards the same receiving WAB-gNB. In addition, the receiving WAB-gNB may provide additional assistance information related to the WAB mobility to enable the initiating WAB-gNB to determine whether and when to reinitiate the Xn Setup procedure towards the same receiving node.

Proposal 8: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 9: The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent re-initiation of the Xn Setup procedure towards the same receiving WAB-gNB.

In addition, the details of the WAB-MT Identifier IE are FFS. The purpose of this ID is to help the BH-gNB discover the co-location of the WAB-MT and the WAB-gNB. We think that the WAB-MT’s C-RNTI allocated by the BH-gNB can be used as this IE. After the WAB-MT setup phase, the information related to the BH-gNB (e.g., the C-RNTI and the Global gNB ID) is provided to the WAB-gNB by the internal implementation. Therefore, the WAB-gNB becomes aware of the BH-gNB serving the WAB-MT, and then includes the WAB-MT’s C-RNTI allocated by the BH-gNB. As a result, the BH-gNB can discover the transmitting NG-RAN node is the WAB-gNB co-located with the WAB-MT.

Proposal 10: The WAB-MT’s C-RNTI allocated by the BH-gNB is used for the WAB-MT Identifier IE.

According to SA2 reply LS in [1], two CRs [7][8] are endorsed to support the two logical gNB solutions. However, there is no SA2 consensus on single gNB solutions suggested by the RAN3. Based on the SA2, RAN3 in [9] agreed to support two-logical gNB solution for the UE’s AMF change. However, whether to support the single-gNB solution with two logical cells is still FFS. Considering the limited time for this WI, it is proposed to only support the two logical gNB solution in this release.

Proposal 11: In this release, the two logical gNB solution is only supported for UE’s AMF change during WAB-gNB mobility.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 12: It is proposed to agree the corresponding TPs in [10].

3.	Conclusion

In this contribution, we focused on the remaining open issues (e.g., for multi-hop backhauling, Xn setup between WAB-gNB, and the mobility), and provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: In this release, RAN3 also supports Option 3 to avoid the multi-hop WAB for initial access.

Proposal 1a: If RAN3 decides to support Option 3, it is proposed to send a LS to consult with RAN2.

Proposal 2: The WAB implementation enables the WAB-MT to prevent accessing to the WAB-gNB belonging to same WAB-node.

Proposal 3: It is proposed to send a reply LS to SA2 in Appendix#1.

Proposal 4: It is proposed to discuss how to solve the mismatch between the RAN3 and SA2 for prevention of multi-hop backhauling during the HO.

Proposal 5: It is proposed to introduce a new sub-clause to capture the agreements on the prevention of multi-hop backhauling in TS 38.401.

Proposal 6: The Additional ULI is removed from the UE-associated signallings.

Proposal 6a: It is proposed to inform the SA2 of the removal of the Additional ULI in the UE-associated signallings (a draft Reply LS in Appendix#2).

Proposal 6b: Upon the reception of the Additional ULI in the NG SETUP REQUEST message, the AMF discovers that the transmitting NG-RAN node is a WAB-gNB.

Proposal 7: The Xn interface between WAB-gNBs can be established.

Proposal 8: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 9: The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent re-initiation of the Xn Setup procedure towards the same receiving WAB-gNB.

Proposal 10: The WAB-MT’s C-RNTI allocated by the BH-gNB is used for the WAB-MT Identifier IE.

Proposal 11: In this release, the two logical gNB solution is only supported for UE’s AMF change during WAB-gNB mobility.

Proposal 12: It is proposed to agree the corresponding TPs in [10].

4.	Reference

- R3-250073/S2-2501336, “Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility),” SA2, January, 2025.

- R3-247828, “Summary of Offline Discussion on WAB,” NTTDOCOMO (moderator), November, 2024.

- R3-244019/S2-2407345, “LS on questions regarding FS_VMR_Ph2,” Samsung, August, 2024.

- S2-2501321, “MWAB configuration and Additional ULI Clarification,” Ericsson, Samsung, January, 2025.

- R3-252531, “(BL CR to 38.413 for WAB) Support for Wireless Access Backhaul”, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm, Samsung, CATT, Jio Platforms (JPL), Lenovo, April, 2025.

- R3-253078, “(BL CR to 38.401 for WAB) Support for Wireless Access Backhaul,” Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms, May, 2025.

- S2-2501319, “On MWAB mobility,” Nokia, January, 2025.

- S2-2501320, “MWAB Mobility handling clarification,” Ericsson, January, 2025.

- R3-250898, “(TP for WAB BL CR for TS 38.401) WAB mobility”, CATT, China Telecom, Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, Huawei, Samsung, Qualcomm, NTTDOCOMO, LGE, Jio, February, 2025.

- R3-253176, “(TP to TS 38.401, 38.413 and 38.423) TP for WAB support,” LG Electronics, May, 2025.

- R3-250071/S2-2501324, “Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI),” SA2, January, 2025.

- R3-253013/S2-2504110, “Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI),” SA2, May, 2025.

5.	Appendix#1: Reply LS for S2-2501324 [11]

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #128 R3-25xxxx Malta, MT, 19th – 23rd May 2025 # Title: [Draft] Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control) # Response to: R3-250071/S2-2501324 # Release: Rel-19 # Work Item: FS_VMR_Ph2, NR_WAB_5GFemto-Core Source: RAN3 To: SA2, RAN2 Cc: - Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: R3-253176 # 1 Overall description RAN3 thanks SA2 for the LS on questions regarding FS_VMR_Ph2. RAN3 would like to provide the following feedback to SA2 regarding the remaining open issues. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. RAN3’s Answer for MWAB-gNB in IDLE in Case a): RAN3 has discussed the aspect on how to prevent multi-hop backhauling and then concluded that the MWAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the MWAB-UEs of other WAB-nodes avoid (re)selecting the MWAB-gNB cells. This approach is implementation specific and it is not subject to specifications changes. In order to prevent unnecessary access trials of the MWAB-UE towards the MWAB-gNB, RAN3 also considers that it is beneficial for the MWAB-gNB to broadcast a new indication that this is a MWAB cell in SIB. Hence, RAN3 would like to request RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. RAN3’s Answer for Case b): RAN3’s view is that this can be managed by e.g. MWAB implementation. That is, the MWAB-UE does not consider the cell of the co-located MWAB-gNB as the suitable cell because it can obtain the information (e.g., gNB ID, TAC, PCI, NCGI, and so on) related to the co-located MWAB-gNB by the internal interface. For the handover case, the MWAB-UE does not report the measurement results for the co-located MWAB-gNB to the BH-gNB. In addition, the MWAB-gNB provides the information about co-location of MWAB-gNB and MWAB-UE to BH-gNB during Xn Setup procedure. The TP to the 38.401 BL CR (R3-253176) capturing the above RAN3 agreements is provided in the attachment. 2. Actions: To SA2: ACTION: RAN3 kindly asks SA2 to take the above into account. To RAN2: ACTION: RAN3 kindly asks RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129 25 – 29 August 2025 Bangalore, India 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ

-----------------End of the First Change---------------

6.	Appendix#2: Reply LS for S2-2504110 [12]

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #128 R3-25xxxx Malta, MT, 19th – 23rd May 2025 # Title: [Draft] Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI) # Response to: R3-253013/S2-2504110 # Release: Rel-19 # Work Item: VMR_Ph2, NR_WAB_5GFemto-Core Source: RAN3 To: SA2 Cc: RAN2 Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: R3-253176 # 1 Overall description RAN3 thanks SA2 for the Reply LS on FS_VMR_Ph2 solution impacts to RAN (Additional ULI). RAN3 would like to further provide the following feedback to SA2. With respect to Additional ULI, RAN3 agreed that the MWAB-gNB reports the additional ULI to the network within the non UE-associated procedures (i.e., NG Setup and RAN Configuration Update procedures) instead of the NGAP UE-associated procedures. Therefore, the Additional ULI is removed from the NGAP UE-associated procedures. Meanwhile, RAN3 notices that clause 5.49.4 of TS 23.501 states the usage of Additional ULI in the NGAP UE-associated procedures. The TP to the 38.413 BL CR (R3-253176) capturing the above RAN3 agreements is provided in the attachment. 2. Actions: To SA2: ACTION: RAN3 would like to call for SA2 to take the above RAN3 agreements on additional ULI into account and update their specification(s). 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129 25 – 29 August 2025 Bangalore, India 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ

-----------------End of the First Change---------------