3GPP TSG-RAN WG3 Meeting #128	R3-253182

Malta, MT, 19th – 24th May, 2025

Agenda Item:	23.2

Source:	Huawei

Title:	Consideration on LTE-based 5G Broadcast Phase 2

Document for: Discussion

# 1	Introduction

In RAN#106, a work item on introduction of time and frequency interleaving for LTE- based 5G Terrestrial Broadcast was approved [1], and the RAN3 involved objective of the WI are listed as follows:

For MBMS-dedicated cells, specify time-frequency interleavers [RAN1] and corresponding signaling [RAN2, RAN3] - For the time-interleaving part of 5G Broadcast TFI, one transport block is mapped to multiple non-consecutive subframes, following the principles of NR TBoMS. The following steps are followed: - Scale the TBS by a factor of N, where N is the number of subframes over which the TB is mapped. - RAN1 to discuss the details of the scaling, the value(s) for N and the set of subframes over which the TB is mapped, including how to schedule the TBs. - Within one subframe, the transmitted rate matched bits are read consecutively from the circular buffer. - In a given subframe, the procedure in TS 36.212 Section 5.1.4.1.2 applies (except for the parameter k0k0 which needs to be modified according to point 3 below) - Before frequency interleaving (if applied), the mapping from coded bits to RE follows the current specification. - Across subframes belonging to the same TB, the starting point for reading from the circular buffer (k0(k0 in TS 36.212 Section 5.1.4.1.2) is to be decided by RAN1 - NOTE 1: The maximum TBS after scaling and soft buffer size should follow limitations imposed by current LTE categories. RAN1 to discuss how to capture this limitation. - NOTE 2: Time interleaving is not applied for PMCH carrying MSI and/or MCCH. - For the frequency-interleaving part of 5G Broadcast TFI, interleaving is performed via a row-column interleaving with the following steps: - Data symbols in an OFDM symbol are written column-wise. - RAN1 to decide what is the unit over which data symbols are written (e.g. a single RE, four REs, etc.) - The rows and elements in each row may be permuted, with details to be decided by RAN1. - Data symbols in an OFDM symbol are read row-wise. - RAN1 to decide the number of {rows, columns} of the interleaver. NOTE: Time-frequency interleaving is not applicable to 0.37kHz SCS.

In this contribution, we will analyse the RAN3 impacts to support time and frequency interleavers for MBMS service.

# 2	Discussion

According to the WID, the time-frequency interleaving (TFI) is introduced for MBMS-dedicated cells to improve the performance in the presence of multipath and fading channels. The eNB should broadcast the MBMS session using TFI, which mainly disperses the transmission resources in time domain and/or frequency domain for a MBMS data. Considering the scenario of interleaving method, the interleaving enabled UE will receive and combine the transmission from multiple dedicated cells, it would require that the cells in an MBSFN area adopt the synchronized transmission resource. Thus, we should assume that the interleaving capabilities of all eNBs in an MBSFN area are aligned, i.e., all eNBs in the MBSFN area support or not support time/frequency interleaving simultaneously. The network should provide or not provide time/frequency interleaver transmission for one MBMS session in an MBSFN area synchronously.

Proposal 1: RAN3 assumes that the interleaving capabilities of all eNBs in an MBSFN area are aligned, i.e., all eNBs in the MBSFN area support or not support time/frequency interleaving simultaneously.

Based on the above analysis, the synchronized transmission should be assured in an MBSFN area. Considering that the MCE is currently responsible for deciding on whether to use MBSFN and the allocation of the radio resources used by all eNBs in the MBSFN area for multi-cell MBMS transmissions using MBSFN operation, it should be the MCE to decide whether to enable the time/frequency interleaving and the configuration of time/frequency interleaving,

Currently, the MCE indicates the configuration of radio resources using M2 SETUP RESPONSE message and MBMS SCHEDULING INFORMATION message to the eNBs in the MBSFN area. Therefore, the MCE should also indicate the configuration of time/frequency interleaving to eNBs in the MBSFN area.

Proposal 2: The MCE makes decision on whether to enable the time/frequency interleaving and the configuration of time/frequency interleaving, and indicates the configuration of time/frequency interleaving to the eNBs in the MBSFN area.

  - M2AP impact for Time Interleaving

RAN1 has achieved some agreements about time interleaving, as following shown:

Agreement Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH. Agreement The basic transmission pattern of PMCH with time interleaving consists of a set of (M x N) consecutive MBSFN subframes, excluding MCCH and MSI, where two transmissions of the same TB are separated by (M-1) MBSFN subframes, excluding MCCH and MSI. Agreement Support at least the following values of time interleaver depth: - Values M={4,8,16,32} and N={1,2,4,8,16} are supported. - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI. FFS: larger values, including M=64, N={32,64}. Agreement The agreement in the previous meeting is updated as follows: Regarding configuration for time interleaving transmission, the following options can be considered: - Option 1: Per PMCH configuration, e.g., via pmch-Config - Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

From RAN3 point of view, the MCE should indicate the time interleaving parameters M and N to eNBs in MBMS SCHEDULING INFORMATION message, where parameter N indicates whether time interleaving is enabled and the TBS scaling factor N, and parameter M indicates the separation between two transmissions of the same TB.

Considering that the granularity of time interleaving configuration is still under discussion by RAN1, i.e., per PMCH or per MBS session, RAN3 can discuss the potential M2AP impacts based on these two RAN1 options.

- If the parameters are indicated per PMCH configuration, the MCE should indicate the time interleaving configuration in PMCH Configuration IE in the MBMS SCHEDULING INFORMATION message;

- If the parameters are indicated per MBMS session, the MCE should indicate the time interleaving configuration in MBMS Session List per PMCH IE in the MBMS SCHEDULING INFORMATION message.

RAN3 can wait for RAN1 conclusion on the granularity of time interleaving configuration.

Proposal 3: The MCE should indicate the time interleaving parameters M and N to eNBs in M2AP MBMS SCHEDULING INFORMATION message, the parameter N indicates whether time interleaving is enabled and the TBS scaling factor N, and the parameter M indicates the separation between two transmissions of the same TB.

Proposal 4: It is FFS whether to include the parameter N and M in the PMCH Configuration IE or the MBMS Session List per PMCH IE, which is pending to RAN1 decision on the granularity of time interleaving configuration.

In the RAN1#120 meeting and RAN1#120-bis meeting, there were some agreements about the the issue of mismatch between MSI periodicity and the interleaving pattern:

Agreement RAN1 to further study the potential issue of mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes. Agreement In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern: - Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations. - May include the case of scheduling different M/N within the same MSI periodicity - FFS the values of the new periodicities. - RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity - Option 2: Drop the time-interleaved TBs which could not be successfully decoded. - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour) - Other Options not precluded

As can be seen, RAN1 is discussing the issue of mismatch between MSI periodicity and the interleaving pattern which will cause resource wastage. To address that issue, some new periodicities for MCH scheduling period may need to be introduced as an extended set including the intermediate values among the periodicities defined for the legacy configurations. From RAN3 point of view, the PMCH Configuration IE need to extended to include the new periodicities for MCH scheduling period, and the detail values of the new MCH scheduling period should be pending to RAN1 further conclusion.

Proposal 5: RAN3 extend the PMCH Configuration IE to include the new periodicities for MCH scheduling period, and FFS for the detail values which are pending to RAN1 further conclusion.

  - M2AP impact for frequency Interleaving

There is no RAN1 conclusion on whether the frequency interleaving applies to MCCH and MSI MAC CE or not, but from RAN3 point of view, we need to wait for RAN1 conclusion on the supporting granularity of frequency interleaving, i.e., per MBSFN area, per PMCH, or per MBS Session. There are several alternatives on where to include the indicator (frequency interleaving enable or not):

- If the granularity of frequency interleaving indicator is per MBSFN area level, the MCE shall provide Frequency Interleaving Indicator IE in MBSFN Area Configuration Item Ies IE of MBMS SCHEDULING INFORMATION message.

- If the granularity of frequency interleaving indicator is per PMCH config level, the MCE shall provide Frequency Interleaving Indicator IE in PMCH Configuration IE of MBMS SCHEDULING INFORMATION message.

- If the granularity of frequency interleaving indicator is per MBMS session level, the MCE shall provide Frequency Interleaving Indicator IE in MBMS Session List per PMCH IE of MBMS SCHEDULING INFORMATION message.

RAN3 can wait for RAN1 final conclusion on the supporting granularity of frequency interleaving.

Note that in case the frequency interleaving applies to MCCH and MSI MAC CE, only the per MBSFN area level can be used.

Proposal 6: The MCE should indicate whether the frequency interleaving is enabled or not in M2AP MBMS SCHEDULING INFORMATION message.

Proposal 7: It is FFS in which IE the frequency interleaving indicator should be included, i.e., in the MBSFN Area Configuration Item Ies IE, or PMCH Configuration IE or MBMS Session List per PMCH IE, subject to RAN1 decision on whether the supporting granularity of frequency interleaving.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

General

- Proposal 1: RAN3 assumes that the interleaving capabilities of all eNBs in an MBSFN area are aligned, i.e., all eNBs in the MBSFN area support or not support time/frequency interleaving simultaneously.

- Proposal 2: The MCE makes decision on whether to enable the time/frequency interleaving and the configuration of time/frequency interleaving, and indicates the configuration of time/frequency interleaving to the eNBs in the MBSFN area.

M2AP impacts for time interleaving

- Proposal 3: The MCE should indicate the time interleaving parameters M and N to eNBs in M2AP MBMS SCHEDULING INFORMATION message, the parameter N indicates whether time interleaving is enabled and the TBS scaling factor N, and the parameter M indicates the separation between two transmissions of the same TB.

- Proposal 4: It is FFS whether to include the parameter N and M in the PMCH Configuration IE or the MBMS Session List per PMCH IE, which is pending to RAN1 decision on the granularity of time interleaving configuration.

- Proposal 5: RAN3 extend the PMCH Configuration IE to include the new periodicities for MCH scheduling period, and FFS for the detail values which are pending to RAN1 further conclusion.

M2AP impacts for frequency interleaving

- Proposal 6: The MCE should indicate whether the frequency interleaving is enabled or not in M2AP MBMS SCHEDULING INFORMATION message.

- Proposal 7: It is FFS in which IE the frequency interleaving indicator should be included, i.e., in the MBSFN Area Configuration Item Ies IE, or PMCH Configuration IE or MBMS Session List per PMCH IE, subject to RAN1 decision on whether the supporting granularity of frequency interleaving.

The corresponding M2AP CR is provided in [2], in which we also show the RAN3 alternatives subject to RAN1 final decision.

# 4	Reference

[1] RP-250794, “Revised WID on LTE-based 5G Broadcast Phase 2”, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 2025

[2] R3-253182 Introduction of LTE-based 5G Broadcast Phase 2, CR to TS 36.443, Rel-19, Huawei

# Annex. RAN1 agreements

  - RAN1#120 meeting

Agreement

For TBS scaling by N, down-select one of the two options below:

- Option 1: Calculate the total number of PRBs NRBtotal as NRBtotal=NRBDL×N, wherein, NRBDL is system bandwidth;

  - If NRBtotal≤110, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBtotal, obtain the corresponding TBS;

  - Else, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBDL, obtain the corresponding TBS’; and calculate the final TBS as TBS=round{TBS'×N}

  - The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

- Option 2: Obtain the initial TBS (TBS') as per current specifications, and calculate the final TBS as TBS=round{TBS'×N}

  - The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

Agreement

At least values of N={1,4,8} are supported, where N=1 indicates time interleaving is disabled.

  - FFS: other values

Agreement

LBRM is applied for MCH (excluding MCCH and MSI) with time interleaving.

  - FFS spec impact

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is derived based on DL-SCH as per TS 36.306 set by UE category.

  - FFS when the UE category has multiple maximum TBS

Agreement

The maximum number of transport block bits received within a TTI defined by the UE category should include the number of bits for unicast and broadcast, where the number of bits for broadcast is the one after being scaled up by N.

Agreement

The basic transmission pattern of PMCH with time interleaving consists of a set of (M x N) consecutive MBSFN subframes, excluding MCCH and MSI, where two transmissions of the same TB are separated by (M-1) MBSFN subframes, excluding MCCH and MSI.

Agreement

Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Agreement

RAN1 to further study the potential issue of mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes.

Agreement

The equation of k0 for time interleaving is modified to avoid puncturing of systematic bits.

- Further study the exact equation for k0 for the n-th subframe of a transmission belonging to a same TB k0n, n∈{0, 1, 2, …, N-1}), including

  - Option 1: for each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe.

  - Option 2: For each CB of the TB, follow the principle of NR TBoMS to determine k0n

  - Option 1 and Option 2 assumes all CBs of the TB have the same RV index for all CBs of the TB, indexed by k0n

- FFS whether different CBs of the TB mapped to each subframe may have different RV indices (e.g., indexed by different k0n per CB)

Agreement

The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows

- Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol

  - FFS: How to determine X

- Append y with <NULL> elements, as required, to obtain yapp

- Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0.

- FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations

- Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s.

- The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211.

FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

Agreement

The granularity of the frequency interleaver is 1 RE.

Agreement

For TBS scaling by N, the following is clarified for the rounding operation in option 1 and option 2 in previous agreement:

- In case two TBSs are at the same distance of TBS'×N, the larger TBS is chosen.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Agreement

Values of M = 16 and M = 8 are supported.

- FFS other values

- Note: some UE categories and depending on factors such as (value of N, bandwidth, MCS) may not be able to receive an interleaved MTCH with M=16.

Agreement

The previous agreement is modified as follows:

The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows

- Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol

  - FFS: How to determine X

    - Option 1: X is equal to the number of codeblocks that are mapped to the OFDM symbol

    - Option 2: X=32

    - Other options are not precluded.

- Append y with <NULL> elements, as required, to obtain yapp

- Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0.

- FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations

- Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s.

- The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211.

FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all

OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

  - RAN1#120-bis meeting

Agreement

Support at least the following values of time interleaver depth:

- Values M={4,8,16,32} and N={1,2,4,8,16} are supported.

  - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI.

- FFS: larger values, including M=64, N={32,64}.

Agreement

In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

  - May include the case of scheduling different M/N within the same MSI periodicity

  - FFS the values of the new periodicities.

- RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity

  - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity

  - Option 2: Drop the time-interleaved TBs which could not be successfully decoded.

    - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour)

  - Other Options not precluded

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

- Working assumption:

  - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Agreement

The following steps are applied to the rows of the frequency interleaver:

- Perform inter-row permutation with a reorder of the row indexes Ri', which is calculated as:

Ri'=R'i-1+d########################,          R'i-1≤R-d-1 #i×dR#,                 else.

With, 1≤i≤R-1, R'0=0, and d=R/R/RC-C+1.

- Perform a cyclic shift to the elements in the j-th row with an offset S(j) defined as:

Sj=modj+12×-1j,C, j=0,1,2,…,R-1

Agreement

LBRM parameters (e.g. NIR) are consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

- These multiple UEs may belong to different UE categories, thus having different values of Nsoft, NIR.

- FFS: how the consistent NIR and NCB is calculated.

Agreement

The agreement in the previous meeting is updated as follows:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

  - Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

  - Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

Agreement

Obtain the initial TBS (𝑇𝐵𝑆′) as per current specifications, and calculate the final TBS as 𝑇𝐵𝑆=𝑟𝑜𝑢𝑛𝑑{𝑇𝐵𝑆′×𝑁}.

- The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.