3GPP TSG-RAN WG3 Meeting #128	R3-253187

St. Julian's, Malta, EU, 19-23 May 2025

Agenda item:	10.3.2

Source:	Nokia

Title:	(MDT TP to BL CR to TS 38.300) Retrieval of slice information after SHR and other slice-related enhancements

Document for:	Discussion

# 1	Introduction

At RAN3#127-bis, the problem of identifying slices that the UE was configured with before executing HO was considered: if the HO results in SHR generation, the source may not be able to retrieve this information. It was discussed that it is just an example of a wider problem of identifying UE config when SHR is received. In this paper we discuss the options to retrieve information about UE config (e.g. slice used).

Also, at RAN3 #127, it was agreed to ask RAN2 to verify if optimisation of the slice-based reselection mechanism requires enhancements to the logged MDT. Later, at RAN3 #127-bit, a brief discussion concerning UE config retrieval for SHR took place. We remind the already proposed problem and solution [1].

# 2	Discussion

## 2.1	Retrieval of the UE slice configuration

As it was observed in various discussions, SHR does not offer any way to help the source node to retrieve the UE configuration used when setting up the SHR. This means, the slices used by the UE when configuring the HO are not known, either. At RAN3 #127-bis, following options were discussed:

- To add on-going slice ID in SHR.

- To add source C-RNTI in SHR.

- To add Mobility information in SHR to solve slice issue.

- To add source C-RNTI and Mobility information over XnAP.

Option 1 is very limited: it helps identify the slice used, but does not resolve the underlying issue of identifying needed aspects of UE configuration. Thus, it shall not be considered.

Proposal 1-1: The problem of identification of the UE configuration shall be addressed completely. Thus, option 1 (adding slice information to the SHR) shall not be further considered.

Options 2 and 3 are similar: they assume the UE records the identification of the configuration, either in the form of the “mobility information”, or straightforwardly as the source C-RNTI, are includes it in the SHR. This way, it will always reach the source node and enables identifying UE config – even if the SHR is fetched by a third node, the content of the SHR will be provided to the source.

Selection between options 2 and 3 shall be made in RAN3. Both were already discussed and both are known to be feasible. Option 2 is more straightforward and requires less signalling, but forces the source node to “block” the C-RNTI until possible SHR arrives. In case of fetching the report after a while in 3rd node, this may pose a serious limitation. Option 3 is much more flexible (the Mobility Information may also contain C-RNTI, if the implementation of the source is made so), but since the Information must be delivered to the UE, it may be more signalling-intensive. However, these two options are not exclusive: similar like in XnAP, both, the source C-RNTI and the Mobility Index may be included (the latter optionally, if provided from the source node) thus leaving the selection up to the source’s implementation.

Observation 1-1: For the problem of UE config identification, options 2 and 3 have drawbacks: either blocking of C-RNTI for a possibly long while or increasing Uu signalling. However, these options are not mutually exclusive, rather complement each other.

The biggest challenge is the limited time left in Rel.19: it may not be feasible for RAN2 to enable full support for combined options 2/3. Therefore, RAN3 may need to decide to leave resolving the problem for Rel.20.

If RAN3 decides to solve the issue in Rel.19, option 4 is a feasible solution: it does not require involving RAN2, because it assumes the information helping identify the UE configuration is delivered outside of the SHR container. In case when the reported SHR is fetched at the target, where both, the source C-RNTI and possibly the Mobility Information are known, this is likely the best option. It fails, however, if the SHR is fetched from the UE at a 3rd node, which was not involved in the HO.

Observation 1-2: For the problem of UE config identification, option 4 will not work in the margin of cases, where the SHR is not fetched right after the HO. It may be acceptable compromise, if the solution is to be enabled yet in Rel.19.

Typically, solutions enabled in 3GPP try to be complete, i.e. not to leave known scenarios when it does not work. Therefore, combined option 2/3 may be better.

Proposal 1-2: Combined option 2/3 may be the best, as it covers all the scenarios and leaves good flexibility for the source’s implementation. RAN3 shall select it while understanding it will be enabled only in Rel.20 (e.g. as SON leftovers).

## 2.2	Feedback for CHO optimisation

The problem and solution were already proposed [1], and even briefly discussed. The conclusion of the last meeting may hint it is off the table:

Only focus on Slice-based Cell Reselection and the above open issue in R19 SONMDT Slicing.

We understand, however, that the guidance does not preclude continuing discussing already proposed issues.

Rel-18 network slicing enhancements introduced support of network slice deployments with service areas that are smaller than the Tracking Area (TA) [2]. This Rel-18 feature is important to deploy highly customized and localized network slices without the need for TA re-design. In such deployments, one potential mobility issue at network slice border is, for example, how to select the neighbour cell for handover (HO). For example, based on UE measurements, the source Cell 1 may have two HO candidates, Cell 2 and Cell 3, but only Cell 3 supports all the slices the UE needs. However, as the source cell does not know in advance the UE trajectory, it has to also prepare Cell 2. This is especially relevant for Conditional HO (CHO) type of mobility as there is a delay between HO preparation and execution. Therefore, Cell 2 cannot be excluded from prepared cells to avoid Radio Link Failure (RLF) and total loss of service even if a HO to Cell 2 would mean loss of service for some of the required slices. Also, currently, in the CHO execution phase, the UE cannot make any prioritization of potential target cells based on the required slices of the UE. In Rel-18 networks, this becomes critical as the slices may only be supported in a small number of cells and above situation of mobility across the slice border could appear more often.

Observation 2-1: In case of CHO, the source may need to prepare also cells that do not have all the necessary slices available, and it does not know if the selected non-supporting target was indeed the only option for the UE.

Once the UE successfully completes the HO, the source cell would know which target was selected by the UE and therefore would also know which slices of the UE will continue in the selected target cell and which slices will not continue, as the source cell knows the slice support of all its neighbor cells. However, based on current information, the source cell cannot detect how far the UE was from the candidates that do have all the slices available. For proper root cause analysis, it is crucial to differentiate the cases of UE1, UE2 and UE3 in Figure 1.

- For UEs with execution location similar to UE1, it might help to adjust the mobility parameters like CIO to prioritize HO from Cell 1 to Cell 3 instead of Cell 2 to ensure service continuity for all the slices of the UE.

- However, for UEs with mobility similar to UE2 or UE3, it might not be sufficient to change the mobility parameters only, as these UEs are far away from Cell 3 at the point of HO. However, if majority of the UEs that are connected to Slice B follow this pattern, this could also indicate a coverage problem in the Slice B planning.

Figure 1: UE Mobility at Network Slice Border

Observation 2-2: Service degradation due to handovers to cells that do not support necessary slices can be avoided if the source cell can analyze the condition at the execution of CHO of the majority of the UEs associated with a certain network slice.

One potential way to record above mentioned measurements is to enhance the Successful HO report (SHR) as the HO was successful from radio perspective. The enhancements could include:

- Adding a new trigger condition to record an SHR i.e. if the UE observes after decoding the HO command at the point of HO execution that one or more of its connected slices will not be supported by the selected target cell, the UE triggers logging of SHR.

- The SHR cause value would also need to be enhanced to let the network know that it was due to non-support of the slice at target cell and service discontinuity.

Once the selected target cell detects that the SHR was generated for network slice service continuity problem, the target cell also forwards the SHR to the original source cell for proper root cause analysis and corrective action, as defined for the SHR handling.

Proposal 2-1: The SHR mechanism is enhanced to enable recording the SHR in case when some of the used slices are not supported in the target cell and that a slice-related cause value is added.

# 3	Conclusions

In this paper, we discuss the problem of retrieving the slice configuration in case of SHR. First, we propose that:

Proposal 1-1: The problem of identification of the UE configuration shall be addressed completely. Thus, option 1 (adding slice information to the SHR) shall not be further considered.

Then, we discuss the other options and we observe:

Observation 1-1: For the problem of UE config identification, options 2 and 3 have drawbacks: either blocking of C-RNTI for a possibly long while or increasing Uu signalling. However, these options are not mutually exclusive, rather complement each other.

Observation 1-2: For the problem of UE config identification, option 4 will not work in the margin of cases, where the SHR is not fetched right after the HO. It may be acceptable compromise, if the solution is to be enabled yet in Rel.19.

That leads us to the following proposal:

Proposal 1-2: Combined option 2/3 may be the best, as it covers all the scenarios and leaves good flexibility for the source’s implementation. RAN3 shall select it while understanding it will be enabled only in Rel.20 (e.g. as SON leftovers).

We also analyse options to avoid unnecessary handovers to cells not supporting all needed slices:

Observation 2-1: In case of CHO, the source may need to prepare also cells that do not have all the necessary slices available, and it does not know if the selected non-supporting target was indeed the only option for the UE.

Observation 2-2: Service degradation due to handovers to cells that do not support necessary slices can be avoided if the source cell can analyze the condition at the execution of CHO of the majority of the UEs associated with a certain network slice.

Based on the above, we make following proposal:

Proposal 2-1: the SHR mechanism is enhanced to enable recording the SHR in case when some of the used slices are not supported in the target cell and that a slice-related cause value is added.

The above proposal is implemented in the two annexes below: Annex 1 proposes a TP related to mobility enhancements, while Annex 2 proposes an LS to RAN2 to request needed support.

# References

- R3-250078, RAN3 #127

- 3GPP TS 23.501

- 3GPP TS 38.300

- 3GPP TS 38.331

# Annex 1: Text proposal to TS 38.300 for SHR enhancement

The below text is proposed to be added to the TS 38.300.

First change, ommited text not changed

#### 15.5.2.7	Successful HO

One of the functions of Mobility Robustness Optimization is to detect a sub-optimal successful handover event. The aim is to identify underlying conditions during successful ordinary handovers, successful DAPS handovers, or successful Conditional handovers.

For analysis of successful handover, the UE may collect Successful Handover Report (SHR) based on configuration by network, if stored, and makes the SHR available to the network as specified in TS 38.331 [12]. The UE stores the SHR until it is fetched by the network or for 48 hours after the SHR is recorded.

For SHR collected during intra-NR handover, if the target NR node fetches the SHR from the UE and the trigger of SHR is T310/T312 or slice unavailability, it may forward the information to the source NR node, i.e. the node handling the cell reported as source cell in this SHR, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

If the NG-RAN node that fetches the SHR from the UE is neither the source node nor the target node of the handover, it forwards the information to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

In case of failure shortly after successful Handover, the same mobility event may generate both a SHR and a RLF report. In this case, the node(s), which configured the SHR trigger causing the SHR, may take the duplication into account e.g. ignore the SHR.

Upon retrieval of an SHR, the receiving node may analyse whether its mobility configuration needs adjustment.

The SHR report can be used to detect one case of Intra-system Too Late Handover, namely when DAPS HO is configured but an RLF is detected in the source cell during a successful DAPS HO.

The SHR report can be collected for intra-NR handover and for handover from NR to E-UTRA.

Remaining text not changed

# Annex 2: Draft LS to RAN2

3GPP TSG-RAN WG3 #128		R3-25xxxx

St. Julian's, Malta, EU, 19-23 May 2025

Title:	[DRAFT] LS on SON for Network Slicing

Response to:

Release:	Release 19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Nokia [to be RAN3]

To:	RAN2

Cc:

Contact Person:	Krzysztof Kordybach (krzysztof.kordybach@nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	R3-25xxxx

1. Overall Description:

RAN3 would like to inform RAN2 that RAN3 agreed to work on network slice mobility optimization utilizing Successful HO report.

RAN3 identified following enhancements to SHR mechanism:

- Adding a trigger to record SHR in case of successful execution of a HO to a cell that does not support all the needed slices;

- Adding a new SHR cause to enable identification of slice-related reports;

RAN3 has agreed a TP to the BL CR to TS 38.300 related to the SHR enhancement, as attached.

2. Actions:

To RAN2

RAN3 kindly asks RAN2 to enable the needed signalling.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #129			25-29 August 2025						Bangalore, India

TSG-RAN3 Meeting #129-bis			13-17 October 2025					Prague, Czechia, EU