3GPP TSG-RAN WG3 Meeting #128	R3-253205

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		11.4

Source:		NEC

Title:			(TP for BLCR for TS38.300) Continuous MDT collection

Document for:		Discussion and decision

# Introduction

Two problems for continuous MDT collection, as below, were identified in SI and captured in the TR38.743 [1].

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In RAN3#127 meeting, one LS to SA5 [1] was agreed to ask SA5 to provide guidance on possible solutions to address the above two problems. SA5 has already replied the LS in [2].

In this contribution, we further discusses the questions from SA5 one by one and prepares a draft reply LS to SA5.

# Discussion

## 2.1	Measurement Continuity Problem

Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

In trace control and configuration parameters, a job type is a mandatory one, that defines the MDT mode. The Job Type parameter is an enumerated type with the following values:

- Immediate MDT only (0);

- Logged MDT only (1);

- ...

One possible way is that OAM introduces a new Job Type value for combined Immediate MDT and Logged MDT as below:

- Immediate MDT and Logged MDT (X)

Proposal 1: For continuous MDT configuration, OAM can introduce a new Job Type value for combined Immediate MDT and Logged MDT.

Answer to SA5: One possible solution identified by RAN3 is that OAM can introduce a new Job Type value for combined Immediate MDT and Logged MDT. With respect to the detailed configuration, RAN3 believes this is in SA5 scope.

Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

RAN3 already agreed that “Continuous collection of MDT traces is beneficial only for AI/ML training in OAM”. Training data is the initial dataset used to train machine learning algorithms on how to make predictions. In order to increase the precision of a ML model, the process of training an ML model needs large volume of input data and a rather long training time. The provision of training data is not time critical.

As in legacy, immediate MDT can be configured to UE for measurements in RRC Connected, while logged MDT in – RRC Idle and RRC Inactive. Measurement continuity in RRC Connected or RRC Idle/RRC Inactive can be achieved through existing MDT mechanism.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Regarding to the gap during RRC state transition, it is a extremely short period of time comparing to the long training time and the measurements during the transition can be negligible.

Besides, the collected raw data may still need to be processed and translated into usable information for training by data processing technology, which can also handle the sparse missing data issue, due to e.g. RRC state transition or other reasons.

In addition, three type of information, i.e. UE location information, UE measurement report and UE Mobility History Information, can be collected from UE for AI/ML. In order to limit the impact on UE power consumption and processing, continuous collection and logging of UE measurements should as much as possible rely on the measurements that are already supported in UE and so we only consider to continuous collect the above data. In this case, only UE location information may need to be considered for measurement continuity across RRC states, but this information is most likely keep unchanged in a extremely short time.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

Based on the above analysis, measurement continuity during the same RRC state can be guaranteed in legacy way and measurement discontinuity during RRC state transition can be negligible, therefore we do not think this issue needs a dedicated solution.

Proposal 2: Measurement continuity issue is negligible for continuous MDT collection and a dedicated solution is not needed.

Answer to SA5: After further analysis, RAN3 agreed that “the Measurement Continuity Problem across different RRC states” problem is negligible and thus does not need a specific solution. This issue can be closed.

## 2.1	Trace Correlation Problem

Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

In previous LS from SA5 [3], SA5 mentioned that

“SA5 agree that there is a benefit to have a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session. So, some data analytics function (e.g., AIML related function) can have a better understanding that the collected data sets are measured from the same source. Therefore, it would be good to provide some kind of identifier to indicate the measurement was done in the same or different UE.”

As confirmed by SA5, an identifier that is unique within the lifetime of the measurement session is needed to indicate the measurement was done in the same or different UE. Therefore, RAN3 also agreed to use a temporary identifier, that can be available at the TCE and is not revealing the UE identity, to enable correlation of the management-based MDT reports.

One solution about using Trace Reference (TR) and Trace Recording Session Reference (TRSR) for trace correction, the one mentioned in SA5’s reply LS, was proposed. However, TR and TRSR are configured only for Logged MDT, and so gNB needs to recover these information from Logged MDT reports if available and attach it in the Immediate MDT records from the same UE. Therefore, this solution brings complexity to gNB and may not be feasible if no Logged MDT report available.

Observation 3: Using TR and TRSR for trace correction brings complexity to gNB and may not be feasible if no Logged MDT report available.

To address this issue, we prefer OAM to allocate a new temporary UE identifier for continuous MDT collection. For example, OAM can allocate to gNB a temporary UE identifier for Continuous MDT collection in Trace Session Activation message, which can be included in MDT records and reports to TCE by gNB, and then TCE can use it to combine MDT records from a same UE. In addition, this UE temporary identifier can also be included in logged MDT measurement configuration, which to implicitly request UE to perform continuous MDT, and then UE will continue the measurements across RRC states and even during RRC states transition.

Proposal 3: OAM allocates a new temporary UE identifier for continuous MDT collection, and gNB includes it in MDT records and reports to TCE for trace correlation.

Answer to SA5: Using TR and TRSR for trace correction brings complexity to gNB and may not be feasible if no Logged MDT report available. Therefore, besides reusing TR and TRSR, RAN3 suggests OAM to allocate a new temporary UE identifier for continuous MDT collection, which can be included in MDT records and reports to TCE by gNB.

A TP for BLCR TS38.300 to add the OAM requirement for continuous MDT collection reflecting Proposal 1 and Proposal 3 is provided in Annex.

Proposal 4: Agree TP for BLCR TS38.300 on OAM requirement for continuous MDT collection in Annex.

A draft reply LS including the above Answer to SA5 is provided in [4].

Proposal 5: Agree the reply LS to SA5 in [4].

# Proposals

In this contribution, we provide our further analysis on continuous MDT collection for AI/ML based on questions from SA5 and the following observation and proposals were drawn from the analysis.

Proposal 1: For continuous MDT configuration, OAM can introduce a new Job Type value for combined Immediate MDT and Logged MDT.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

Proposal 2: Measurement continuity issue is negligible for continuous MDT collection and a dedicated solution is not needed.

Observation 3: Using TR and TRSR for trace correction brings complexity to gNB and may not be feasible if no Logged MDT report available.

Proposal 3: OAM allocates a new temporary UE identifier for continuous MDT collection, and gNB includes it in MDT records and reports to TCE for trace correlation.

Proposal 4: Agree TP for BLCR TS38.300 on OAM requirement for continuous MDT collection in Annex.

Proposal 5: Agree the reply LS to SA5 in [4].

# Reference

[1] R3-250891, LS on Continuous MDT

[2] S5-252114, Reply LS to LS on Continuous MDT

[3] R3-245021, Reply LS on improved KPIs involving end-to-end data volume transfer time analytics

[4] R3-253206, [draft] Reply LS to Reply LS to LS on Continuous MDT

# Annex	TP for BLCR TS38.300

<Start of change>

### 16.20.4	OAM Requirements

OAM configures the following:

-	The minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area; and

-	The recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

-	The Job Type with value “Immediate MDT and Logged MDT (X)” (FFS) for continuous MDT collection.

-	The temporary UE identifier for trace correction in TCE.

<End of change>