3GPP TSG-RAN WG3 Meeting #128	R3-253209

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		17.2

Source:		NEC

Title:			(TP to BLCR TS38.473) Support on-demand SSB

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, we continuously discussed whether NES gNB-CU can indicate NES gNB-DU the allowed cell list for on-demand SSB. Although majorities prefer the following but no agreement can be achieved:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

Besides, there is still one open issue on Xn impact due to on-demand SSB:

- Normal cell to request NES cell to provide on-demand SSB over Xn for layer3 measurement?

In this contribution, we share our views on the above issues and analyse potential RAN3 impacts.

# Discussion

## RAN1 agreements update

Below are the new agreements made in RAN1#120bis meeting.

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}

  - N2= [8]

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement

Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

- Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

  - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

    - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - (Working assumption) T=T_min

  - RAN4 to confirm the above two working assumptions

  - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

- od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap.

- od-ssb-PositionsInBurst can be absent for Case #2

  - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- A CSI report configuration can be associated with

  - both of AO-SSB and OD-SSB

    - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met

    - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst

    - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

  - OD-SSB only

    - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

## RAN2 agreements update

Below are the new agreements made in RAN2#129bis meeting.

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

## Cells allowed to be OD-SSB list from gNB-CU to gNB-DU

Considering gNB-CU is more knowledgeable on the SSB operation status (e.g. no always-on SSB, always-on SSB with a long periodicity or always-on SSB as in legacy) of the cells with all DUs connected to it and neighbour cells, gNB-CU can provide some guidance to gNB-DU for OD-SSB SCell operation, taking both energy saving and UE performance into account. For example, gNB-CU can suggest to gNB-DU e.g. OD-SSB allowed cells and, for each SCell, SSB operation mode, OD-SSB transmission periodicity, OD-SSB transmission duration, etc.

In legacy, gNB-CU can provide gNB-DU a Cells Allowed to be Deactivated List IE, and upon receiving this IE, gNB-DU shall consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy optimization purpose.

For OD-SSB, as defined in RAN1, there are two cases:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

A cell operating in Case #1 is similar as a cell with SSB beam deactivated, therefore the same logic as the existing cell deactivation in gNB-DU can be followed, i.e. gNB-CU can provide gNB-DU a Cells Allowed to be OD-SSB List. This newly proposed Cells Allowed to be OD-SSB List is an optional IE and it can be provided by gNB-CU to gNB-DU as assistance information. Once receiving this, gNB-DU can take it into account and still decide by itself whether to perform OD-SSB.

Proposal 1: gNB-CU can provide gNB-DU a Cells Allowed to be OD-SSB List to assist the OD-SSB operation in gNB-DU.

Regarding how to provide the assistance information from gNB-CU to gNB-DU, there are two options:

Option 1: Reusing Cells Allowed to be Deactivated List IE

In Rel-18 NES, a Cells Allowed to be Deactivated List IE is added in GNB-CU CONFIGURATION UPDATE message, and “If Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy optimization purpose”[1]. Some companies proposed that this IE can be reused for OD-SSB and, after receiving this, gNB-DU can decide whether to perform cell deactivation or OD-SSB activation. The procedure description of this IE needs to be updated to support OD-DDB case. Below is the detailed changes in TS38.473.

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells or it is allowed to activate OD-SSB within the indicated SCells for network energy saving purpose.

Option 2: To add a new IE e.g. Cells Allowed to be OD-SSB List IE

The existing Cells Allowed to be Deactivated List IE is added as an assistance information for beam deactivation in a cell owned by gNB-DU, which is used for a different purpose than OD-SSB operation. In order not to impact the Rel-18 beam deactivation operation, it is more advantageous to use a separate IE, e.g. Cells Allowed to be OD-SSB List IE, to carry the assistance information for OD-SSB operation. This new IE can be added in the existing GNB-CU CONFIGURATION UPDATE message. Below is the detailed changes in TS38.473.

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the SCells Allowed to be OD-SSB List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to activate OD-SSB within the indicated scells for network energy saving purpose.

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| <<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| SCells Allowed to be OD-SSB List |  | 0..1 |  |  | YES | ignore |
| >SCells Allowed to be OD-SSB List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

Comparing the two options, option 2 has no impact on legacy SSB beam deactivation and provide more accurate guidance to gNB-DU on whether the Cells Allowed List is for deactivation or OD-SSB. For example, some cells in a gNB-DU may allowed to be deactivated, while some other cells in the same gNB-DU may be better to only activate OD-SSB

However considering the standard impact, as a compromise, option 1 is also acceptable as a simplified solution.

Proposal 2: Reuse the existing Cells Allowed to be Deactivated List IE but update the relevant procedure texts as OD-SSB assistance information from gNB-CU to gNB-DU.

TP to BLCR TS38.473, mirroring proposal 2, is available in Annex.

Proposal 3: To agree the TP to BLCR TS38.473 in Annex.

## Xn impacts

Considering the scenario needs to be analyzed is for UEs in connected mode configured with CA, for both intra-/inter-band CA [1], in which a PCell is always associated to a CD-SSB located on the synchronization raster, we do not see the need to use a neighbour gNB to tiger OD-SSB SCell operation and we think this should be an intra-gNB issue.

One issue was brought up about how to address L3 measurement of an OD-SSB Cell, whether NES cell can be triggered by a neighbor cell over Xn for OD-SSB activation? According to the existing RAN1 agreement, On-demand SSB on the cell is not located on synchronization raster; On-demand SSB on the cell is non-cell-defining SSB. And RAN2’s assumption is that UE does not expect to measure SSB when OD-SSB transmission is deactivated if no always-on SSB on the cell. Besides, RAN2 reached a new WA that “When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB”.

Therefore, from the analysis above, no Xn impact is foreseen for OD-SSB SCell operation, unless RAN1 or RAN2 reaching any new agreements that will change the above working assumptions.

Proposal 4: No Xn impact is foreseen for on-demand SSB SCell operation, unless current working assumptions changed in RAN1/RAN2.

# Proposals

In this contribution, we summarized the latest RAN1 and RAN2 progress on OD-SSB and analysed the potential RAN3 impacts, and the following proposals were drawn from the analysis.

Proposal 1: gNB-CU can provide gNB-DU a Cells Allowed to be OD-SSB List to assist the OD-SSB operation in gNB-DU.

Proposal 2: Reuse the existing Cells Allowed to be Deactivated List IE but update the relevant procedure texts as OD-SSB assistance information from gNB-CU to gNB-DU.

Proposal 3: To agree the TP to BLCR TS38.473 in Annex.

Proposal 4: No Xn impact is foreseen for on-demand SSB SCell operation, unless current working assumptions changed in RAN1/RAN2.

# Reference

[1] TS38.473, NG-RAN; F1 application protocol (F1AP)

# Annex TP to BLCR TS38.473

<Start of change>

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells or it is allowed to activate OD-SSB within the indicated SCells for network energy saving purpose.

<End of change>