3GPP TSG-RAN WG3 Meeting #128	R3-253210

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		17.3

Source:		NEC

Title:			Support on-demand SIB1

Document for:		Discussion and decision

# Introduction

In RAN3#126 meeting, one new class 1 procedure for providing UL WUS Configuration from NES Cell gNB to Cell A gNB and one new class 2 procedure for informing UL WUS configuration broadcast stop from Cell A gNB to NES Cell gNB were agreed.

In RAN3#127bis meeting, we further agreed the following and all the agreements are reflected in the BLCRs in [1] [2].

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

One Choice “Start with UL WUS Configuration”

One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

This contribution further discusses the open issues and the corresponding RAN3 impacts of OD-SIB1.

# Discussion

## Naming FFS

Regarding the new defined class 1 procedure and the new class 2 procedure, one FFS is the names of the procedures and messages.

According to the purpose of the class 1 procedure, i.e. providing UL WUS Configuration from NES Cell gNB to Cell A gNB, it is suitable to name it as “UL WUS Configuration Provision” procedure. The three corresponding messages should be named as

- UL WUS Configuration Provision Request

- UL WUS Configuration Provision Response

- UL WUS Configuration Provision Failure

Proposal 1: To name the class 1 procedure as “UL WUS Configuration Provision” procedure and the three corresponding messages as “UL WUS Configuration Provision Request”, “UL WUS Configuration Provision Response” and “UL WUS Configuration Provision Failure” (to remove corresponding Naming FFS in [1]).

Regarding the new class 2 procedure, according to the agreement made in last meeting, “A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB”, therefore, we suggest to directly name the procedure as “UL WUS Configuration Broadcast Stop Indication”.

Proposal 2: To rename the class 2 procedure and the corresponding message as “UL WUS Configuration Broadcast Stop Indication”.

Another open issue in last meeting is how to address the role of the NG-RAN node1 and NG-RAN node2 in the procedure description in [1]. Since each newly defined UL WUS Configuration related message is with a specific direction, it will cause confusion if a general node name, e.g. NG-RAN node1 and NG-RAN node2, is used in the procedure description. We think it should clearly identify the NG-RAN node1 that serves NES cell as NES-NG-RAN node and the NG-RAN node2 that serves Cell A as Cell A-NG-RAN node.

Proposal 3: To use “NES-NG-RAN node” and “Cell A-NG-RAN node” to represent NG-RAN node1 and NG-RAN node2 respectively in [1].

## UL WUS Configuration Provision Request message

For the new Class 1 message (UL WUS Configuration Provision Request), RAN3 agreed to use a CHOICE structure  and include two codepoints i.e. “Start” and “Stop”. However, there is another one “pause/resume”, which means the Cell A gNB remembers the UL WUS Configuration when it receive the “Pause” and NES uses “resume”, was proposed, but still FFS.

In our view, “pause/resume” is not needed. If the UL WUS configuration broadcasting in Cell A just needs to be stopped for a short time, then it is not an issue if there is no any indication from NES cell and Cell A just keeps providing UL WUS configuration. RAN2 already agreed that “If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”. If the NES Cell goes back to SIB1 broadcasting, UE can receive SIB1 in legacy way and thus there should be no impact to UE. As more than one UL WUS configurations from several NES cells can be incorporated into one new SIBx, in this case, there should be no additional signalling waste even if Cell A still provides the UL WUS configuration for that NES cell. While, if the UL WUS configuration broadcasting in Cell A needs to be stopped for a long time, then it is not reasonable to let Cell A keep the configuration and the Choice “Stop” can be used to inform Cell A to release the configuration and stop the broadcasting.

If NES Cell needs to update its UL WUS configuration, UL WUS Configuration Provision procedure can be reused with a new UL WUS configuration included.

Therefore, we think “pause/resume” is not needed in UL WUS Configuration Provision Request message.

Proposal 4: One Choice “pause/resume” is not needed in the UL WUS Configuration Provision Request message.

Considering there could be more than one NES cells in one gNB to request a same Cell A for UL WUS configuration broadcasting at the same time, therefore a NES cell list needs to be added in the UL WUS Configuration Provision procedure e.g. as below.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >> UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331 [10]. |  |  |
| >> NES Cell List |  |  |  |  |  |  |
| >>> NR CGI | O | 1 .. <maxnoofCellsinNGRANnode> | 9.2.2.7 | Indicates the NES cells of which the UL WUS Configuration applies. |  |  |
| >Stop |  |  |  |  |  |  |
| >> NES Cell List |  |  |  |  |  |  |
| >>> NR CGI | M | 1 .. <maxnoofCellsinNGRANnode> | 9.2.2.7 | Indicates the NES cells of which the UL WUS Configuration applies. |  |  |

Proposal 5: To add NES Cell List in UL WUS Configuration Provision Request message to indicate the NES cells of which the UL WUS Configuration applies.

Proposal 6: To agree the message structure for UL WUS Configuration Provision Request message as above.

## UL WUS Configuration Provision Response message

Regarding the UL WUS Configuration Provision Response message, a NES cell list can also be included and the message is shown as below.

### 9.x.x.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by the NG-RAN node2 to the peer NG-RAN node1 to indicate that the UL WUS Configuration provision for one or more NES cell(s) has (have) been accepted.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NES Cell List |  |  |  |  |  |  |
| >> NR CGI | M | 1 .. <maxnoofCellsinNGRANnode> | 9.2.2.7 | Indicates the NES cells of which the UL WUS Configuration are accepted. |  |  |

Proposal 7: To agree the message structure for UL WUS Configuration Provision Response message as above.

## UL WUS Configuration Provision Failure message

The UL WUS Configuration Provision Failure message is shown as below.

### 9.x.x.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by the NG-RAN node2 to inform the peer NG-RAN node1 that UL WUS configuration provisioning for all requested NES cells failed.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |

Proposal 8: To agree the message structure for UL WUS Configuration Provision Failure message as above.

## UL WUS configuration broadcasting capability transfer

If a cell, based on its capability, can work as a Cell A, which means it can support UL WUS configuration broadcasting for NEC cell(s), it can exchange its UL WUS configuration broadcasting capability with neighbouring gNB in advance, e.g. during Xn setup procedure. This can greatly reduce the UL WUS Configuration Provision request failure rate.

Proposal 9: Cell A can exchange its UL WUS configuration broadcasting capability with neighbouring gNB during Xn setup procedure.

## Multiple Cell A scenario

As agreed in RAN#105 meeting, for Rel-19 NES, support Case-2 on OD-SIB1:

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

There is one possible scenario for Case 2 that, for one NES Cell, multiple Cell A(s) are applicable. In this case, the NES Cell can select which Cell A(s) to broadcast UL WUS configuration for it, and it can select only one or several or all Cell A(s) from the applicable Cell A(s). Then the UL WUS Configuration Provision Request form NES cell gNB to different Cell A gNBs can be sent one by one or at the same time. Afterwards, the NES cell can decide how many WUS Configuration Provision Acknowledge message(s) it needs to receive before activating OD-SIB1. This can all be up to implementation.

Proposal 10: In case of multiple Cell A(s) appliable for one NES Cell, the NES Cell can select which Cell A(s) for UL WUS configuration provision.

Proposal 11: UL WUS Configuration Provision Request form NES cell gNB to different Cell A gNBs can be sent one by one or at the same time, which can be up to implementation.

Proposal 12: NES cell can decide how many WUS Configuration Provision Acknowledge message(s) it needs to receive before activating OD-SIB1, which can be up to implementation.

## Assistance information for OD-SIB1 activation/deactivation

Besides, there can be a scenario i.e. multiple NES Cells under a single Cell A, it is beneficial to perform some coordination by Cell A on e.g. which NES Cells under that Cell A can activate OD-SIB1 transmission, taking both energy saving and UE performance into account.

Proposal 13: Cell A can provide guidance to assist NES Cell on assistance information.

In split architecture, considering gNB-CU is more knowledgeable on the SIB1 transmission status and load status of all NES Cells and neighbor cells, we prefer gNB-CU to decide OD-SIB1 activation/deactivation of a NES Cell, or at least gNB-CU should provide guidance to assist gNB-DU on OD-SIB1 activation/deactivation of a NES Cell.

Proposal 14: gNB-CU decides or assists gNB-DU on the OD-SIB1 activation/deactivation of a NES Cell, and sends the request or assistance information to gNB-DU.

# Proposals

In this contribution, we addressed the open issues for OD-SIB1 of a NES Cell based on the latest agreements and the following proposals were drawn from the analysis.

Proposal 1: To name the class 1 procedure as “UL WUS Configuration Provision” procedure and the three corresponding messages as “UL WUS Configuration Provision Request”, “UL WUS Configuration Provision Response” and “UL WUS Configuration Provision Failure” (to remove corresponding Naming FFS in [1]).

Proposal 2: To rename the class 2 procedure and the corresponding message as “UL WUS Configuration Broadcast Stop Indication”.

Proposal 3: To use “NES-NG-RAN node” and “Cell A-NG-RAN node” to represent NG-RAN node1 and NG-RAN node2 respectively in [1].

Proposal 4: One Choice “pause/resume” is not needed in the UL WUS Configuration Provision Request message.

Proposal 5: To add NES Cell List in UL WUS Configuration Provision Request message to indicate the NES cells of which the UL WUS Configuration applies.

Proposal 6: To agree the message structure for UL WUS Configuration Provision Request message as above.

Proposal 7: To agree the message structure for UL WUS Configuration Provision Response message as above.

Proposal 8: To agree the message structure for UL WUS Configuration Provision Failure message as above.

Proposal 9: Cell A can exchange its UL WUS configuration broadcasting capability with neighbouring gNB during Xn setup procedure.

Proposal 10: In case of multiple Cell A(s) appliable for one NES Cell, the NES Cell can select which Cell A(s) for UL WUS configuration provision.

Proposal 11: UL WUS Configuration Provision Request form NES cell gNB to different Cell A gNBs can be sent one by one or at the same time, which can be up to implementation.

Proposal 12: NES cell can decide how many WUS Configuration Provision Acknowledge message(s) it needs to receive before activating OD-SIB1, which can be up to implementation.

Proposal 13: Cell A can provide guidance to assist NES Cell on assistance information.

Proposal 14: gNB-CU decides or assists gNB-DU on the OD-SIB1 activation/deactivation of a NES Cell, and sends the request or assistance information to gNB-DU.

# Reference

[1] R3-252498, Introduction of Network Energy Saving Enhancement

[2] R3-252499, (BL CR to 38.473) Introduction of Network Energy Saving Enhancement