3GPP TSG-RAN WG3 Meeting #128	R3-253225

Malta, MT, 19 – 23 May 2025

Agenda item:	12.3

Source:	Nokia

Title:	Completion of other open points of NR Femto

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

At the last RAN3#126, baseline CRs was endorsed for TS 38.413 which is almost complete.

At the last RAN3#127, two text proposals for functional aspects of stage 2 were agreed and the baseline CR was endorsed for TS 38.300 which is almost complete as well.

This paper investigates remaining open points.

The stage 2 security aspects from SA3 incoming LS are addressed in tdoc [2].

This tdoc addresses other remaining open issues.

# 2	Discussion

- Remaining open points other than security

Terminology: NR Femto or NR Femto Node?

In 4G, HeNB has only one cell. Therefore, the term HeNB was enough to identify both the node and the cell.

In 5G, an NR femto can have multiple cells. Therefore, we need two different terms: one to identify the node, one to identify the cell. We need the term “NR Femto cell” to identify a cell. Therefore, it seems more straightforward to use the term “NR Femto node” to designate the node. Also, SA2 and SA3 use the term “NR Femto node”. Therefore, we have a slight preference to use the term “NR femto node” but of course both can work.

Proposal 1: use the terms “NR Femto node” and “NR Femto cell” to designate respectively the node and the cell.

Slicing information to NR Femto node?

In tdoc [2], it is analysed that NR Femto node connected via GW doesn’t need to receive the list of slices supported by the GW in the NG Setup Response because it doesn’t perform NG flex function. In contrast, the same NR femto node connected directly to AMF would need to receive this list. As a consequence, tdoc [3] proposes to change the list of slices into conditional presence in the NG setup response.

Even though it is true that NR femto node connected via GW doesn’t need the list of slices we think that the list can be simply ignored when received and not needed. It is assumed that the NR Femto when connected via GW is aware through O&M and can ignore the list. We propose to just add some clarification to 38.413 to avoid issues for this case.

Proposal 2: clarify the (non)usage of the list of slices when received by the NR Femto connected to GW in TS 38.413.

Mobility issues

SA2 TS 23.501 has specified the provisioning of CAG ID by the femto owner which can be for temporary usage. This means that allowed CAG ID which was typically long lasting before can become very temporary. This temporary CAG ID needs some special handling. For example, when CAG ID expires the N2 connection is currently abruptly removed if it is in use according to TS 23.501:

-	The AMF shall update the Allowed CAG list in the Mobility Restrictions towards NG-RAN if the evaluation of the time validity information of an entry in the Allowed CAG list changes between true and false, unless the AMF releases the NAS signalling connection to the UE based on operator's policy if the evaluation of the validity condition changes from true to false.

Because with NR Femtos the occurrence of CAG ID expiry becomes “normal” a graceful approach is more suitable than releasing N2. For example, the NR Femto should be notified in order to enable performing a handover on time and avoid the complete release.

Proposal 3: enhance TS 38.413 to inform gNB of CAG time validity before it expires in order to enable handovers before CAG expiry.

- Remaining Security Aspects

RAN3 has received the LS from SA3 in [5] in response to [4] saying:

SA3 has agreed that all the verifications listed in the LS need to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed). SA3 agreed the attached CRs to update TS 33.545 to incorporate relevant verification requirements and mechanisms as well

The impact of security aspect on stage 2 has been addressed in tdoc [2]. The SA3 CR attached to this SA3 LS is in [6] and states:

The NR Femto GW shall verify that all UE associated messages from Femtos operating in closed access mode can be mapped to a specific CAG ID and that this CAG ID is allowed for the identity of the originating Femto. In the absence of a NR Femto GW, the AMF shall perform this CAG ID verification.

The above statement means that SA3 considers that an NR Femto may not be trusted depending on the environment, similar to 4g femto. In 4g, because 4g femto may not be trusted the MME does not send the Handover Restriction List to the HeNB but instead the MME performs the membership verification for closed cells itself.

Similarly, for 5g the question arises that depending whether the NR Femto is in trusted environment or not (e.g. at home) it may not be sound to send the Allowed PNI NPN List to the NR Femto. To make it flexible and because it is related to the deployment use case, the AMF could be configured whether to send or not the Allowed PNI NPN restriction List within the Initial Context Setup Request message to NR Femtos. However, this assumes that AMF could discriminate if a received Initial UE Message originates from an NR Femto. This requires a specific femto indication in the Initial UE message.

Proposal 4: send a specific Femto indication in the Initial UE message of TS 38.413 from NR Femto to enable control of sending Allowed PNI NPN List or not.

# 3	Conclusion

This paper has explored the remaining open points and made following proposals:

Proposal 1: use the terms “NR Femto node” and “NR Femto cell” to designate respectively the node and the cell.

Proposal 2: clarify the (non)usage of the list of slices when received by the NR Femto connected to GW in TS 38.413.

Proposal 3: enhance TS 38.413 to inform gNB of CAG time validity before it expires in order to enable handovers before CAG expiry.

Proposal 4: send a specific Femto indication in the Initial UE message of TS 38.413 from NR Femto to enable control of sending Allowed PNI NPN List or not.

The above proposals are implemented in the BL CR TS 38.413 in tdoc [7].

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-253224, [TP for BL CR NR Femto 38.300] Completion of Security Aspects of NR Femto

- R3-251786, Discussion on slice feature and access control for NR Femtos

- R3-250822, LS on Security Verification related to NR Femtos

- R3-253021, Reply LS on Security Verification related to NR Femtos

- S3-251699, Update NR Femto Gateway Functionality

- R3-253226, [TP for BL CR NR Femto 38.413] Completion of other open points of NR Femto