3GPP TSG-RAN WG3 Meeting #128	R3-253251

# Malta, MT, 19th – 23th May 2025

Title: 	(TP for TS38.473, 38.401) Conditional intra-gNB-CU LTM

Source: 	ZTE Corporation

# Agenda item:	13.3

Document for:	Discussion and Decision

- Introduction

In the RAN3 #127bis meeting, we have achieved the following progress.

For CLTM focus firstly on scenarios without CU-UP change. The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages. Details FFS to be continued. Reuse the ACCESS SUCCESS message from Candidate DU to CU to inform the cell change. For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission. For CLTM, the existing LTM network signaling for LTM Cell Switch Completion phase is reused. FFS whether a cell can be simultaneously configured for LTM and CLTM. FFS whether its needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition. Whether needs to indicate from CU to source DU and candidate DU when the cell is configured with L3 CLTM, separately from L1 CLTM.

In the RAN3 #127 meeting, we have achieved the following progress.

Conditional LTM The CU decides to configure C-LTM. The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM. Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition. UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition. UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition. Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail. For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

RAN2 has achieved the following progress in RAN2 #129bis meeting.

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

RAN2 has achieved the following progress in RAN2 #129 meeting.

Agreements on RRC issues:

- For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID.

- Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

- The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

- UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG.

- FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

- The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

- The release of the LTM configuration (MCG or SCG) follows the Rel-18 principles.

- A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

- The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

- Re-use existing L1 events for CLTM.

- For field ltm3-Offset-r19, the granularity is 0.5 dB.

- Add extension mark “…” to LTM-ConfigSCG-r19, adopt the principle that fields only applicable to inter-CU SCG change should be defined in the root of LTM-ConfigSCG-r19, not LTM-Config-r18.

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

- RAN2 assumes the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE and in the candidate cell TCI state Activation/Deactivation MAC CE, pending RAN1 confirmation.

- The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

- The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

- The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

- There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

This paper further discusses the Conditional LTM based on the above progress.

- Discussion

  - How to indicate candidate gNB-DU to configure Conditional LTM

In current F1AP spec, an indicator (i.e., LTM Indicator) is included in UE context Setup/modification request message. If CU decides to configure LTM, it sends this indicator to source DU (for intra-DU LTM) and candidate DU (for inter-DU LTM) for LTM configuration.

Observation 1: In Rel-18, if CU decides to initiate LTM configuration, it sends LTM Indicator to DU.

If taking the R18 L3 based LTM (still under discussion) into account, R19 will include following four types of LTM:

- L1 based LTM;

- L3 based LTM (still under discussion);

- L1 based C-LTM ;

- L3 based C-LTM.

For above cases of LTM, we compare the gNB-DU behavior from four aspects separately and list them in the following table:

| LTM type/gNB-DU behavior | Whether provide L1 measurement report configuration based on CSI resource | Whether provide L1 LTM execution conditions | Whether source DU triggers LTM cell switch | Whether provide Lower Layer Configuration for LTM | Receive IE from gNB-CU |
| --- | --- | --- | --- | --- | --- |
| L1 based LTM | Y | N | Y (based on UE report) | Y | LTM Indicator; CSI Resource Configuration; |
| L1 based C-LTM | Y | Y | N | Y | LTM Indicator; CSI Resource Configuration; L1 LTM execution conditions request indicator |
| L3 based LTM (still under discussion） | N | N | Y (based on message from CU) | Y | LTM Indicator; |
| L3 based C-LTM | N | N | N | Y | LTM Indicator; |

In case that CU decides to initiate L3 based Conditional LTM configuration, it is the CU to generate the L3 execution conditions. It seems that in this case, the DU is not involved to generate execution conditions.

Observation 2: If CU decides to initiate L3 based Conditional LTM configuration, the DU does not need to differentiate either LTM configuration or conditional LTM configuration.

When gNB-CU determine to configure L3 based LTM or L3 based C-LTM or both for the UE, the gNB-CU may not send  CSI Resource Configuration IE for candidate cells to the gNB-DU, so that the gNB-DU does not have to generate the LTM CSI reporting configuration, and subsequently it will not receive the L1 measurement result from the UE. However, as a candidate LTM cell, gNB-DU need to provide low layer configuration for the LTM candidate cell. LTM Indicator IE is enough for this case. No addition IE is needed.

Proposal 1: If CU decides to initiate L3 based (Conditional) LTM configuration, CU sends the existing LTM indicator to (source or candidate) DU via UE context setup/modification request message, then the DU knows that (conditional) LTM is configured and provide lower Layer Configuration for LTM. No addition IE is introduced.

In case that CU decides to initiate L1 based Conditional LTM configuration, it is the DU to generate the L1 execution conditions. It seems that in this case, the DU shall be notified of L1 based Conditional LTM configuration.

Proposal 2: If CU decides to initiate L1 based Conditional LTM configuration, the DU shall be notified of L1 based Conditional LTM configuration.

As we know, during legacy CHO, both normal L3 handover and CHO can be configured for the UE simultaneously, so it is reasonable to support simultaneously configure for LTM and C-LTM for the same UE.

Proposal 3:  RAN3 supports to configure either one or both of LTM and C-LTM for the same UE.

In addition, according to RAN2’s agreement, the network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. In other words, the network won’t configure both L1 execution condition and L3 execution condition for a CLTM candidate cell. So it is natural that the L1 LTM and L3 C-LTM won’t be configured simultaneously.

Agreements on C-LTM in RAN2#129:

From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

Observation 3: L1 based C-LTM and L3 based C-LTM will not be configured simultaneously for a UE.

But for other type of LTM, there is no restriction for configure more than one type of LTM. In summary, the UE can be configured with following cases of LTM simultaneously:

- L1 based LTM and L3 based LTM (still under discussion）

- L1 based LTM and L1 based C-LTM

- L3 based LTM (still under discussion）and L3 based C-LTM

- L1 based LTM and L3 based C-LTM

- L3 based LTM (still under discussion）and  L1 based C-LTM

Proposal 4: RAN3 agrees to configure the following type of LTM simultaneously.

- L1 based LTM and L3 based LTM

- L1 based LTM and L1 based C-LTM

- L3 based LTM and L3 based C-LTM

- L1 based LTM and L3 based C-LTM

- L3 based LTM and  L1 based C-LTM

When gNB-CU determine to configure L1 based LTM or L1 based C-LTM for the UE or both, the gNB-CU will send  CSI Resource Configuration IE to the gNB-DU, then the gNB-DU shall use it to generate the LTM CSI reporting configuration. Furthermore, if L1 based C-LTM is configured for the UE, the gNB-DU need to further provide L1 LTM execution conditions. However, based on current IE, it is hard to distinguish whether L1 based LTM is configured or not if L1 LTM execution conditions request is included.

Proposal 5: introduce a new IE to indicate whether only L1 C-LTM is configured or both L1 LTM and L1 C-LTM is configured in the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST.

| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >L1 C-LTM Indicator | O |  | ENUMERATED (L1 C-LTM, both L1 LTM and L1 C-LTM, …) |  |  |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |

In addition, if a UE is configured with both LTM and C-LTM, the candidate cells configured for LTM and C-LTM may be different. However, for the same candidate cell, if it is not only the candidate LTM cell but also the candidate C-LTM for a UE, the gNB-DU can provide the same low layer configuration for a UE, in other word, there is no need to use different low layer configurations for LTM and C-LTM. The only difference is that execution condition configuration is added for C-LTM.

Proposal 6:  For a candidate cell configured for both LTM and C-LTM, the gNB-DU provide the same low layer configuration for a UE.

During offline discussion in last RAN3 meeting, one FFS issue is whether the (Source/Candidate) DUs need to be aware of which other candidate LTM cells are configured with L1 CLTM?  According to our understanding,  the (Source/Candidate) DUs need to be aware of which other candidate LTM cells are configured with L1 CLTM, the serving DU can trigger early TA acquisition for the candidate cell configured with L1 CLTM, and can trigger both early TA acquisition and cell switch execution for the candidate cell configured with L1 LTM.

Proposal 7:  The (Source/Candidate) DUs need to be aware of which other candidate LTM cells are configured with L1 CLTM.

## 2.2	Early Synchronization

2.2.1 TCI States Configurations

In legacy LTM, UE Context Setup and Modification procedures is used to collect the TCI state configuration from candidate cells and provide it to the Source DU. Similarly, the same procedure can be reused to collect the TCI state configuration from candidate cells and provide it to the Source DU for C-LTM.

Proposal 8:  For conditional LTM, reuse UE Context Setup and Modification procedures to collect the TCI state configuration from candidate cells and provide it to the Source DU.

2.2.2 Early UL synchronization (early TA acquisition)

As we know, in L1 based LTM, gNB-DU can trigger the early TA acquisition based on L1 measurement report.  The same way can be used for L1 based C-LTM. But for L3 based C-LTM, it is hard for gNB-DU to trigger the early TA acquisition without L1 measurement report. The reasonable way is to let gNB-CU trigger the early TA acquisition based on L3 measurement report.

Proposal 9:  For conditional LTM, gNB-CU triggers the early sync by sending a F1AP message to (source) gNB-DU including the candidate cell information.

2.2.3 TAT transfer

According to RAN2#129bis agreements:

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

Since the UE maintain the TA value and Keep the early TA timer running for other candidate cells upon C-LTM cell switch execution to the target cell, it is necessary for the target cell (i.e., new source) know the TAT information such as the remaining duration of the TAT upon C-LTM execution, otherwise, the target cell does not know when to trigger early TA acquisition for other candidate cells again since it does not know when the TA value will be expired.

Observation 4: The target cell (i.e., new source) does not know when to trigger early TA acquisition for other candidate cells again since it does not know when the TA value will be expired after UE perform C-LTM cell switch execution to the target cell.

Proposal 10: For inter-DU C-LTM cell switch, source gNB-DU transfers the TAT information such as the remaining duration of the TAT after UE per for C-LTM cell switch execution to the target gNB-DU via gNB-CU.

In current procedure, after the target gNB-DU detects the UE access, it sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed. Then the gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to source gNB-DU to inform that the UE executed an inter-DU Conditional LTM. After that, the source gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message. Therefore, the source gNB-DU can include the TAT information such as the remaining duration of the TAT in the UE CONTEXT MODIFICATION RESPONSE. Then the gNB-CU forwards the received TAT information to the target gNB-DU by UE CONTEXT MODIFICATION REQUEST message.

Proposal 11: Source gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message including TAT information such as the remaining duration of the TAT. Then the gNB-CU forwards the received TAT information to the target gNB-DU by UE CONTEXT MODIFICATION REQUEST message.

- Conclusion

Observation 1: In Rel-18, if CU decides to initiate LTM configuration, it sends LTM Indicator to DU.

Observation 2: If CU decides to initiate L3 based Conditional LTM configuration, the DU does not need to differentiate either LTM configuration or conditional LTM configuration.

Observation 3: L1 based C-LTM and L3 based C-LTM will not be configured simultaneously for a UE.

Observation 4: The target cell (i.e., new source) does not know when to trigger early TA acquisition for other candidate cells again since it does not know when the TA value will be expired after UE perform C-LTM cell switch execution to the target cell.

Proposal 1: If CU decides to initiate L3 based (Conditional) LTM configuration, CU sends the existing LTM indicator to (source or candidate) DU via UE context setup/modification request message, then the DU knows that (conditional) LTM is configured and provide lower Layer Configuration for LTM. No addition IE is introduced.

Proposal 2: If CU decides to initiate L1 based Conditional LTM configuration, the DU shall be notified of L1 based Conditional LTM configuration.

Proposal 3:  RAN3 supports to configure either one or both of LTM and C-LTM for the same UE.

Proposal 4: RAN3 agrees to configure the following type of LTM simultaneously.

- L1 based LTM and L3 based LTM

- L1 based LTM and L1 based C-LTM

- L3 based LTM and L3 based C-LTM

- L1 based LTM and L3 based C-LTM

- L3 based LTM and  L1 based C-LTM

Proposal 5: Introduce a new IE to indicate whether only L1 C-LTM is configured or both L1 LTM and L1 C-LTM is configured in the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST.

Proposal 6:  For a candidate cell configured for both LTM and C-LTM, the gNB-DU provide the same low layer configuration.

Proposal 7:  The (Source/Candidate) DUs need to be aware of which other candidate LTM cells are configured with L1 CLTM.

Proposal 8:  For conditional LTM, reuse UE Context Setup and Modification procedures to collect the TCI state configuration from candidate cells and provide it to the Source DU.

Proposal 9:  For conditional LTM, gNB-CU triggers the early sync by sending a F1AP message to (source) gNB-DU including the candidate cell information.

Proposal 10: For inter-DU C-LTM cell switch, source gNB-DU transfers the TAT information such as the remaining duration of the TAT after UE per for C-LTM cell switch execution to the target gNB-DU via gNB-CU.

Proposal 11: Source gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message including TAT information such as the remaining duration of the TAT. Then the gNB-CU forwards the received TAT information to the target gNB-DU by UE CONTEXT MODIFICATION REQUEST message.

- TP for LTM BLCR for TS38.401

=======================<Start of change>=================================

#### 8.2.1.x Conditional intra-CU LTM (Intra-gNB-DU)

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for conditional LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU conditional LTM procedure for intra-NR.

Figure 8.2.1.x-1: Conditional intra-CU LTM (Intra-gNB-DU)

1.	The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for each candidate cell, containing one candidate cell ID. The gNB-CU may provide the CSI resource configuration for subsequent L1 event-triggered LTM and L1 event-triggered conditional LTM. The gNB-CU may provide the LTM configuration ID mapping list to the gNB-DU. The gNB-CU may request PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the gNB-DU. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration. If the gNB-CU determines to initiate the L1 event-triggered conditional LTM, it requests the gNB-DU to generate the conditional LTM L1 execution condition(s).

4.	If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted candidate cell. If L1 C-LTM Indicator is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU also provides a list of execution conditions generated for other candidate cells (detail FFS).

NOTE 1:	Steps 3 and 4 may be initiated multiple times for conditional LTM candidate cell preparation of multiple cells including the source cell.

Editor’s Note: Details are FFS on step 3 and 4.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU which may include the LTM configuration ID mapping list and/or the updated CSI resource configuration. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration.

6.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell  if  CSI resource configuration is included in the UE CONTEXT MODIFICATION REQUEST message. If the L1 C-LTM Indicator is included in the UE CONTEXT MODIFICATION REQUEST message, the source gNB-DU also provides a list of execution conditions generated for the candidate cells.

NOTE 2:	In case of subsequent conditional LTM, the CU-initiated UE Context Modification procedure may be invoked per each candidate cell to transfer to the gNB-DU the updated CSI resource configuration.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the conditional LTM configuration.

8.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

10.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

11.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

12.	The gNB-DU sends the MAC CE to the UE.

13.	The execution condition(s) to trigger initiation of conditional LTM is fulfilled in the UE.

14. The gNB-DU detects the UE access as specified in TS 38.300 [2].

15. The gNB-DU sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed. The gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source cell, are sent from the gNB-CU to the gNB-DU.

16.	The UE sends an RRCReconfigurationComplete message to the gNB-DU.

17.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

18.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

19.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

#### 8.2.1.y Conditional intra-CU LTM (Inter-gNB-DU)

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for Conditional LTM. Figure 8.2.1.y-1 shows the inter-gNB-DU Conditional LTM procedure for intra-NR.

Figure 8.2.1.y-1: Conditional intra-CU LTM (Inter-gNB-DU)

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID. The gNB-CU may provide the CSI resource configuration for subsequent L1 event-triggered LTM and L1 event-triggered conditional LTM.  The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s). If the gNB-CU determines to initiate the L1 event-triggered conditional LTM, it request the candidate gNB-DU(s) to generate the conditional L1 execution condition(s).

Editor’s Note: Details are FFS on step 3.

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell. If the L1 C-LTM Indicator is included in the UE CONTEXT MODIFICATION REQUEST message, the candidate gNB-DU also provides a list of execution conditions generated for other candidate cells.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU conditional LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell if  CSI resource configuration is included in the  UE CONTEXT MODIFICATION REQUEST message. If the L1 C-LTM Indicator is included in the UE CONTEXT MODIFICATION REQUEST message, the source gNB-DU also provides a list of execution conditions generated for the candidate cells.

Editor’s Note: Details are FFS on step 6.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent conditional LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the requested candidate cell  if  CSI resource configuration is included in the  UE CONTEXT MODIFICATION REQUEST message. If the L1 C-LTM Indicator is included in the UE CONTEXT MODIFICATION REQUEST message, the candidate gNB-DU also provides a list of execution conditions generated for other candidate cells.

NOTE 2: Step 7 may also be triggered after step 18 for subsequent conditional LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the conditional LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

14. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

15.	The gNB-CU forwards the TA value and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

Editor’s Note: Details are FFS on step 14 and 15 on how to handle the TAT.

16.	The source gNB-DU sends the MAC CE to the UE.

17. The execution condition(s) to trigger initiation of conditional LTM is fulfilled in the UE.

18. The target gNB-DU detects the UE access as specified in TS 38.300 [2].

19. The target gNB-DU sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed. The target gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU.

20.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

21.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

22. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to source gNB-DU to inform that the UE executed an inter-DU Conditional LTM and indicate to stop the data transmission for the UE. The source gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE 3: The step 22 may happen before step 21, as soon as the gNB-CU knows which cell the UE has successfully accessed.

23. The source gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message which may include TAT information for candidate cell(s), e.g., remaining duration of TAT.

24.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to target gNB-DU including the received TAT information for candidate cell(s) in step 23.

25.	The target gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message.

26.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

27.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

5	TP for LTM BLCR for TS38.473

/*********************Start of changes***********************/

8.3	UE Context Management procedures

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

<skip unchanged part>

If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the L1 C-LTM Indicator IE set to "L1 C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns L1 based conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message.

If the L1 C-LTM Indicator IE set to "both L1 LTM and L1 C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM and L1 based conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the LTM ConfigurationIE in the UE CONTEXT SETUP RESPONSE message for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation), in the UE CONTEXT SETUP RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the LTM Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the candidate gNB-DU.

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

<skip unchanged part>

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

/*********************Next change***********************/

8.3.4	UE Context Modification (gNB-CU initiated)

8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message and neither the LTM Information Modify IE nor the Conditional Intra-DU Mobility Information IE is present, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly. If the servingCellMO List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, configure servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT MODIFICATION RESPONSE message.

<skip unchanged part>

If the LTM Indicator IE set to "true" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the L1 C-LTM Indicator IE set to "L1 C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns L1 based conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the L1 C-LTM Indicator IE set to "both L1 LTM and L1 C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM and L1 based conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

<skip unchanged part>

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

/*********************Next change***********************/

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ---------------------------------------------<Skip unchanged part>------------------------------------------ |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >L1 C-LTM Indicator | O |  | ENUMERATED (L1 C-LTM, both L1 LTM and L1 C-LTM, …) |  |  |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |

--------------------------<Next of the change>---------------------------------

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ---------------------------------------------<Skip unchanged part>------------------------------------------ |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| > L1 C-LTM Indicator | O |  | ENUMERATED (L1 C-LTM, both L1 LTM and L1 C-LTM, …) |  |  |  |
| > Remaining duration of TAT | O |  | FFS | The detailed definition of this IE is FFS. It is sent to the Target gNB-DU. |  |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |
| LTM Security Information | O |  | 9.3.1.XX |  | YES | reject |

======================<Next of change>=================================

9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition List | O |  | 9.3.1.XXX | The detailed definition of this IE is FFS. | - |  |
| > Remaining duration of TAT | O |  | FFS | The detailed definition of this IE is FFS. It is sent by Source gNB-DU |  |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/*********************Next change***********************/

9.3.1.294	LTM Configuration ID Mapping List

This IE indicates the list of LTM cells associated with its configuration IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Configuration ID Mapping Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >L1 C-LTM Indicator | O |  | ENUMERATED (L1 C-LTM, both L1 LTM and L1 C-LTM, …) |  | YES | ignore |

/*********************Next changes***********************/

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Coverage-Modification-Cause,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-areaScope,

id-IntendedTDD-DL-ULConfig,

id-QosMonitoringRequest,

id-BHInfo,

id-IAB-Info-IAB-DU,

id-IAB-Info-IAB-donor-CU,

id-IAB-Barred,

id-SIB12-message,

id-SIB13-message,

id-SIB14-message,

id-UEAssistanceInformationEUTRA,

id-SL-PHY-MAC-RLC-Config,

id-SL-ConfigDedicatedEUTRA-Info,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRPRACHConfig,

id-TDD-UL-DLConfigCommonNR,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-TSCTrafficCharacteristics,

id-AdditionalPDCPDuplicationTNL-List,

id-RLCDuplicationInformation,

id-AdditionalDuplicationIndication,

id-mdtConfiguration,

id-TraceCollectionEntityURI,

id-NID,

id-NPNSupportInfo,

id-NPNBroadcastInformation,

id-AvailableSNPN-ID-List,

id-SIB10-message,

id-RequestedP-MaxFR2,

id-DLCarrierList,

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-Supported-MBS-FSA-ID-List,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-MBS-Broadcast-NeighbourCellList,

id-PDCPTerminatingNodeDLTNLAddrInfo,

id-ENBDLTNLAddress,

id-PRS-Resource-ID,

id-LocationMeasurementInformation,

id-SliceRadioResourceStatus,

id-CompositeAvailableCapacity-SUL,

id-NR-U,

id-NR-U-Channel-List,

id-MIMOPRBusageInformation,

id-IngressNonF1terminatingTopologyIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-rBSetConfiguration,

id-frequency-Domain-HSNA-Configuration-List,

id-child-IAB-Nodes-NA-Resource-List,

id-Parent-IAB-Nodes-NA-Resource-Configuration-List,

id-uL-FreqInfo,

id-uL-Transmission-Bandwidth,

id-dL-FreqInfo,

id-dL-Transmission-Bandwidth,

id-uL-NR-Carrier-List,

id-dL-NR-Carrier-List,

id-nRFreqInfo,

id-transmission-Bandwidth,

id-nR-Carrier-List,

id-permutation,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-SurvivalTime,

id-PDCMeasurementQuantities-Item,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoAInformation,

id-ARPLocationInfo,

id-ARP-ID,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRPRx-TEGInformation,

id-TRPBeamAntennaInformation,

id-Redcap-Bcast-Information,

id-NR-TADV,

id-SDT-MAC-PHY-CG-Config,

id-CG-SDTindicatorSetup,

id-CG-SDTindicatorMod,

id-SDTRLCBearerConfiguration,

id-SRBMappingInfo,

id-DRBMappingInfo,

id-LastUsedCellIndication,

id-Recommended-SSBs-List,

id-SSBs-withinTheCell-tobe-Activated-List,

id-SIB17-message,

id-MUSIM-GapConfig,

id-SIB20-message,

id-pathPower,

id-DU-RX-MT-RX-Extend,

id-DU-TX-MT-TX-Extend,

id-DU-RX-MT-TX-Extend,

id-DU-TX-MT-RX-Extend,

id-TAINSAGSupportList,

id-SL-RLC-ChannelToAddModList,

id-SIB15-message,

id-InterFrequencyConfig-NoGap,

id-MBSInterestIndication,

id-L571Info,

id-L1151Info,

id-SCS-480,

id-SCS-960,

id-SRSPortIndex,

id-PEISubgroupingSupportIndication,

id-NeedForGapsInfoNR,

id-NeedForGapNCSGInfoNR,

id-NeedForGapNCSGInfoEUTRA,

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-Preconfigured-measurement-GAP-Request,

id-BWP-Id,

id-ExtendedResourceSymbolOffset,

id-MusimCapabilityRestrictionIndication,

id-duplicationIndication,

id-dRB-List,

id-ChannelOccupancyTimePercentageUL,

id-RadioResourceStatusNR-U,

id-FiveG-ProSeLayer2Multipath,

id-FiveG-ProSeLayer2UEtoUERelay,

id-FiveG-ProSeLayer2UEtoUERemote,

id-TSCTrafficCharacteristicsFeedback,

id-RANfeedbacktype,

id-Mobile-TRP-LocationInformation,

id-Mobile-IAB-MT-UE-ID,

id-MobileAccessPointLocation,

id-SIB24-message,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-ERedcap-Bcast-Information,

id-NeedForInterruptionInfoNR,

id-SCPAC-Request,

id-MobileIAB-Barred,

id-F1UTunnelNotEstablished,

id-S-CPACLowerLayerReferenceConfigRequest,

id-MusimCandidateBandList,

id-PSIbasedSDUdiscardUL,

id-SIB22-message,

id-U2URLCChannelQoS,

id-SL-PHY-MAC-RLC-ConfigExt,

id-UL-RSCP,

id-BW-Aggregation-Request-Indication,

id-ReportingGranularitykminus1,

id-ReportingGranularitykminus1additionalpath,

id-ReportingGranularitykminus2,

id-ReportingGranularitykminus2additionalpath,

id-ReportingGranularitykminus3,

id-ReportingGranularitykminus3additionalpath,

id-ReportingGranularitykminus4,

id-ReportingGranularitykminus4additionalpath,

id-ReportingGranularitykminus5,

id-ReportingGranularitykminus5additionalpath,

id-ReportingGranularitykminus6,

id-ReportingGranularitykminus6additionalpath,

id-TimingReportingGranularityFactorExtended,

id-PosValidityAreaCellList,

id-SymbolIndex,

id-AggregatedPosSRSResourceIDList,

id-PhaseQuality,

id-PRSBWAggregationRequestInfoList,

id-AggregatedPRSResourceSetList,

id-MeasuredFrequencyHops,

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

id-PeerUE-ID,

id-MeasBasedOnAggregatedResources,

id-SIB23-message,

id-PointA,

id-SCS-SpecificCarrier,

id-NR-PCI,

id-E-CID-MeasuredResultsAssociatedInfoList,

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-LTMgNB-ID,

id-L1ExecutionConditionList,

id-L1C-LTMIndicator,

id-RemainingdurationofTAT,

#### -- L

LTMInformation-Setup ::= SEQUENCE {

lTMIndicator						LTMIndicator,

referenceConfiguration				ReferenceConfiguration								OPTIONAL,

cSIResourceConfiguration			CSIResourceConfiguration							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-Setup-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-Setup-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-L1C-LTMIndicator		CRITICALITY ignore	EXTENSION L1C-LTMIndicator	PRESENCE optional },

...

}

LTMConfigurationIDMappingList	::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfigurationIDMapping-Item

LTMConfigurationIDMapping-Item::= SEQUENCE{

lTMCellID			NRCGI,

lTMConfigurationID	LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer {{ LTMConfigurationIDMapping-Item-ExtIEs}}		OPTIONAL

}

LTMConfigurationIDMapping-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-L1C-LTMIndicator		CRITICALITY ignore	EXTENSION L1C-LTMIndicator	PRESENCE optional },

...

}

LTMInformation-Modify	::= SEQUENCE {

lTMIndicator						LTMIndicator,

referenceConfiguration				ReferenceConfiguration									OPTIONAL,

cSIResourceConfiguration 			CSIResourceConfiguration								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-Modify-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-Modify-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-L1C-LTMIndicator		CRITICALITY ignore	EXTENSION L1C-LTMIndicator	PRESENCE optional },

{ ID id-RemainingdurationofTAT		CRITICALITY ignore	EXTENSION RemainingdurationofTAT	PRESENCE optional },

...

}

LTMIndicator ::= ENUMERATED {true, ...}

CompleteCandidateConfigurationIndicator	::= ENUMERATED {complete, ...}

LTMConfigurationID ::=  INTEGER (1..8)

ReferenceConfigurationInformation ::= OCTET STRING

LTMConfiguration	::= SEQUENCE {

sSBInformation					SSBInformation,

referenceConfigurationInformation 	ReferenceConfigurationInformation			OPTIONAL,

completeCandidateConfigurationIndicator		CompleteCandidateConfigurationIndicator 				OPTIONAL,

lTMCFRAResourceConfig				LTMCFRAResourceConfig						OPTIONAL,

lTMCFRAResourceConfigSUL			LTMCFRAResourceConfig						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { LTMConfiguration-ExtIEs } }	OPTIONAL,

...

}

LTMConfiguration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-L1ExecutionConditionList		CRITICALITY ignore	EXTENSION L1ExecutionConditionList	PRESENCE optional },

{ ID id-RemainingdurationofTAT		CRITICALITY ignore	EXTENSION RemainingdurationofTAT	PRESENCE optional },

...

}

L1C-LTMIndicator ::= ENUMERATED {

L1C-LTM,

L1LTMandL1C-LTM,

...

}

RemainingdurationofTAT::=FFS

/*********************End of changes***********************/