3GPP TSG-RAN WG3 Meeting #128	R3-253259

Malta, MT, 19 – 23 May, 2025

Title: 	(TP to BLCR for TS 38.473 and TS 38.423) Discussion on on-demand SIB1 for UEs in idle or inactive mode

Source: 	Huawei

Agenda item:	17.3

Document Type:	Other

# Introduction

In the previous RAN3 meetings, the following agreements were made:

Agreement at RAN3#127 meeting It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration. Agreement at RAN3#127bis meeting Step4 is not needed. In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: - One Choice “Start with UL WUS Condiguration” - One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

This contribution continues the discussion and provides our views.

# Discussion

The overall flowchart is given in Fig.1 illustrating what we have achieved and the remaining issues that are needed to be discussed.

- Figure 1. The overall On-demand SIB1 procedure

## 2.1 Xn impact

### 2.1.1 The class 1 procedure

In last RAN2-129bis meeting, the general guidelines were agreed to keep the specification clear and readable as follows.

- RAN2#129bis meeting Define features, not devices. That is, definition of OD-SIB1, OD-SSB, paging adaptation etc in 38.300 and in other specifications refer to 38.300: - Avoid NES, Cell A, UL-WUS - “UE supporting OD-SIB1” where/if needed, and to replace with the UE capability name - UL-WUS is replaced with SIB1 request in e.g. field descriptions - ul-WUS-Config renamed as od-sib1-Config

- It is reasonable for RAN3 to take the same naming rule to align with RAN2 for the new defined Class 1 and Class 2 procedures including avoiding NES, Cell A, UL-WUS terminologies etc. Note that hereafter the NES node, Cell A node are used for illustration purpose.

- The exact class1 name is still to be determined, where it is named as UL WUS Configuration Provision procedure in stage 3 BLCR in [1]. Following the above discussion, the class 1 message can be renamed as On-demand SIB1 Configuration Provision procedure (including request, response, and failure message).

RAN3 to take the above RAN2 guidance for stage 2/stage 3 details, including avoiding NES, Cell A, UL-WUS terminologies etc. Naming the class 1 procedure as On-demand SIB1 Configuration Provision procedure (including request, response, and failure message).

- In the last RAN3-127bis meeting, a choice structure was agreed in the class 1 procedure. The next setup is to consider the exact IE name and references. At previous RAN2-129bis meeting, the SIBxx was captured in the current RAN2 running CR as follows in [2] with a lot of editor’s notes. It can be observed that the WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- For the Class 1 procedure, we consider that are two possible options for the reference to the RAN2 running CR.

- Option 1: referring to the od-SIB1-CellConfigList (i.e., a list of NES cells)

- Option 2: referring to the od-SIB1-Config (i.e., a single NES cell).

- For option 1, the NES node could contain many OD-SIB1 configurations after merging all the NES cells containing the same UL WUS config. But when designing the “stop” indication and even in the response message, it requires to include some NES Cells information, e.g., the PCI or NR CGI.While for opiton2, multiple XnAP procedures are needed in case there are multiple NES Cells to initiate the procedure.

- RAN3 should first discuss to take which option. Below illustrates the option 1 against the BLCR in [1]. Since there is no cell information in the od-SIB1-Config IE, it is proposed to add the cell ID to indicate towards which the NES cell the On-demand SIB1 Configuration Provision Request message is initiated. The same is true for the response message, and the failure message.

IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. NR CGI; M; 9.2.2.7; YES; reject. CHOICE Request Type; M; YES; reject. >Start. >>od-SIB1-Config; M; OCTET STRING; Includes the od-SIB1-Config contained in the SIBxx message as defined in TS 38.331 [10].. >Stop.

RAN3 to discuss which option is taken. If the option 2 (referring to the od-SIB1-CellConfig) is taken, the NR CGI and od-SIB1-Config are included in the On-demand SIB1 Configuration Provision Request message, and in the response message and failure message, a single NR CGI is included.

- In addition, in previous meetings, some companies proposed to add a list of Cell A in the class 1 procedure. In our view, the NES Cell node does not care much which Cell As could help broadcast the UL WUS configuration, but care whether the provision request is accepted or not by the Cell A node. Also, the Cell A should be aware of the neighbour cell related information based on the legacy mechanisms, e.g., the UE ANR, network interface management messages, OAM configuration, SON etc. Hence in the request and response message, the Cell A list is not needed (the details can also be found in Section 2.3).

There is no need to include the Cell A list in the class 1 procedure (including the request message and response message).

### 2.1.2 The class 2 procedure

At previous RAN3 meetings, it was agreed that the Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. A class 2 procedure is agreed by using non-UE-associated signaling. There is a remaining issue whether there is a need for the Cell A gNB itself to send the “resume” or “restart” indication to NES Cell to assist the WUS configuration broadcast again.

Whether CellA can stop and resume the broadcasting of UL WUS configuration?

- We consider this is not much needed. First, Cell A possibly will not know if the NES Cell is currently in OD-SIB1 mode, hence it becomes useless to send “start” or “restart” to the NES Cell. Second, the Cell A may not obtain the latest WUS config in time when the WUS config was updated.

- Hence the class 2 procedure can be named as On-demand SIB1 Configuration Provision Stop Indication message. Alternately it be renamed as On-demand SIB1 Configuration Provision Status Update message, including only the “stop” codepoint.

For the class 2 procedure:

- No need for Cell A gNB itself to indicate the “start” or “restart” indication to NES Cell to assist the UL WUS configuration broadcast again.

- Naming the class 2 procedure as On-demand SIB1 Configuration Provision Stop Indication or On-demand SIB1 Configuration Provision Status Update message, including only the “stop” codepoint

- include a set of NES Cell IDs.

## 2.2 F1 impact

### 2.2.1 Supporting the Xn class 1 procedure

At NES cell Node

- At previous meeting, the choice structure is included in the Served Cell Information IE captured in the F1AP BLCR as follows.

CHOICE on-demand SIB1; O; YES; ignore. >Provision [FFS]. >>UL WUS Configuration Information; M; Octet String; Includes the [FFS] IE, as defined in TS 38.331 [8].. >Stop provision [FFS]. Editor’s Note: these choice names above may need to be refined in future.

- First, regarding the codepoint, we consider the “provision” and “stop provision” are sufficient for the NES Cell DU to indicate when to stop or begin the SIB1 broadcast.

- when the NES Cell DU receives the UL WUS signal (i.e., RACH) from the UE and decides to broadcast SIB1, it can send “stop provision” indication to the NES Cell CU, which propagates to the Cell A.

- In the DU triggered interface messages, the NES Cell DU can send the “provision” indication to the NES Cell CU, which also propagates to the Cell A.

- Second, when referring to the RAN2 running CR in [1], the od-SIB1-config can be referred as per Cell level information. Third, when the NES Cell CU received the UL WUS configuration broadcast response from the Cell A, it should deliver results (i.e., a set of accepted NES cells) to the NES Cell DU in the gNB-CU configuration update request message.

Regarding the codepoint in the Choice structure, take the “provision” and “stop provision” and refer to the od-SIB1-config defined in the RAN2 running CR.

When the NES Cell CU received the On-demand SIB1 Configuration Provision response message from the Cell A, it should deliver results (i.e., a set of accepted NES cells) to the NES Cell DU in the gNB-CU configuration update request message.

Last, considering that the CU has general system level information, e.g., the neighbour Cell A deployment, system overload etc, it would be beneficial for the CU to provide some assistance information e.g., a cell list to indicate which cells are allowed to enable on-demand SIB1 operation. After that, the DU could determine itself whether to enter the on-demand SIB1 operation to save the network energy based on the load etc in time.

RAN3 to discuss whether the NES Cell CU can provide the NES Cell DU a list of allowed cells for on-demand SIB1 in the gNB-CU Configuration Update message.

At cell A Node

- At the Cell A node, the UL WUS configuration of its neighbour NES cells can be signalled from the NES Cells. According to the RAN2 running CR in [1], the Cell A CU needs to merge the same WUS configuration to associate a list of cells if their WUS configuration is same. Then the Cell A CU can include the new SIB in the gNB-CU system information to the Cell A DU. In case of any update/stop of the UL WUS configuration broadcast, the Cell A CU could update/delete the new SIB and send the gNB-CU system information to the Cell A DU.

- In addition, in the previous RAN2#127 bis meeting, it was agreed that the UL WUS configuration is included in a new SIB. It can also be observed that the not only the Cell A can broadcast the UL WUS config for NES cell, the NES Cell itself can also broadcast its neighbour cell’s WUS configuration, including its own.  Hence the above proposals also apply.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

At Cell A node, the UL WUS configuration can be generated and encoded in a new SIB by the Cell A CU, and included in the gNB-CU System Information to the Cell A DU. The same is true for the NES cell when it broadcast its neighbor cell’s WUS configuration.

### 2.2.2 Supporting the Xn class 2 procedure

For the Xn class 2 procedure, after the NES Cell CU received the “stop” message from Cell A, the “stop” WUS configuration broadcast indication should also be sent to the NES Cell gNB-DU. With the information, the DU could decide to deactivate the on-demand SIB1 state (i.e., beginning SIB1 broadcast).

In this case, it should deliver a set of not accepted NES cells) to the NES Cell DU in the gNB-CU configuration update message.

Over the F1 for the NES node, when the NES Cell CU received the “stop” UL WUS configuration broadcast indication from the Cell A, it should deliver a set of not accepted NES cells to the NES Cell DU in the gNB-CU configuration update message.

As discussed above, there are two types of information should be delivered from the NES Cell CU to the NES Cell DU. One is for the Class 1 procedure response (a set of accepted NES cells) and another is for the Class 2 procedure (a set of not accepted NES cells), hence a general IE in F1AP can be designed covering both cases with ENUMERATED values (i.e., accepted and not accepted). With the different enumerated information, the NES Cell DU could know whether it can enter the OD-SIB1 operation or return to the normal cell state. Below provides the detailed structure.

| Cells to be On-Demand SIB1 List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cells to be On-Demand SIB1 List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>On-Demand SIB1 Indicator | M |  | ENUMERATED (accepted, not accepted, ...) |  |  |  |

For F1AP, a general cell list with ENUMERATED format (accepted, not accepted) for each indicated cell is signalled from the gNB-CU to the gNB-DU.

## 2.3 Other aspects

In the SOD R3-245737, the coverage cases of the NES Cell and Cell A were listed as follows.

- NES cell is partially covered by different Cells, not all cells are Cell A; - NES Cell is covered by multiple Cell As; - gNBs are not aware their neighboring gNB NES Cell?

For the coverage information, it is up to the OAM to determine the Cell A and NES cell coverage status, e.g., whether the Cell A is overlapped or partially overlapped with NES Cell etc. Even for the CCO issue, in current RAN spec, it does not specify the coverage scenarios among neighbouring cells. It is up to the OAM/implementation when the receiving node decides to adjust its cell/beam coverage when it receives the cell coverage state information. Following the same logic, the current mechanisms can ensure how the OD-SIB1 works, e.g., the role of Cell A and NES cell, or to avoid coverage hole when the NES Cell enters on-demand SIB1 mode etc, without further signalling design.

It is up to OAM/implementations to ensure how the OD-SIB1 works, e.g., the role of Cell A and NES cell, or coverage hole avoidance etc.

The Annex provides the TPs for TS 38.473 and TS 38.423.

# 3. Conclusion

In this contribution, we discussed the on-demand SIB1 operation related issues and made the following proposals:

The class 1 procedure

- RAN3 to take the above RAN2 guidance for stage 2/stage 3 details, including avoiding NES, Cell A, UL-WUS terminologies etc. Naming the class 1 procedure as On-demand SIB1 Configuration Provision procedure (including request, response, and failure message).

RAN3 to discuss which option is taken. If the option 2 (referring to the od-SIB1-CellConfig) is taken, the NR CGI and od-SIB1-Config are included in the On-demand SIB1 Configuration Provision Request message, and in the response message and failure message, a single NR CGI is included.

There is no need to include the Cell A list in the class 1 procedure (including the request message and response message).

The class 2 procedure

For the class 2 procedure:

- No need for Cell A gNB itself to indicate the “start” or “restart” indication to NES Cell to assist the UL WUS configuration broadcast again.

- Naming the class 2 procedure as On-demand SIB1 Configuration Provision Stop Indication or On-demand SIB1 Configuration Provision Status Update message, including only the “stop” codepoint

- include a set of NES Cell IDs.

F1 impact for the class 1 procedure

Regarding the codepoint in the Choice structure, take the “provision” and “stop provision” and refer to the od-SIB1-config defined in the RAN2 running CR.

When the NES Cell CU received the On-demand SIB1 Configuration Provision response message from the Cell A, it should deliver results (i.e., a set of accepted NES cells) to the NES Cell DU in the gNB-CU configuration update request message.

RAN3 to discuss whether the NES Cell CU can provide the NES Cell DU a list of allowed cells for on-demand SIB1 in the gNB-CU Configuration Update message.

At Cell A node, the UL WUS configuration can be generated and encoded in a new SIB by the Cell A CU, and included in the gNB-CU System Information to the Cell A DU. The same is true for the NES cell when it broadcast its neighbor cell’s WUS configuration.

F1 impact for the class 2 procedure

Over the F1 for the NES node, when the NES Cell CU received the “stop” UL WUS configuration broadcast indication from the Cell A, it should deliver a set of not accepted NES cells to the NES Cell DU in the gNB-CU configuration update message.

For F1AP, a general cell list with ENUMERATED format (accepted, not accepted) for each indicated cell is signalled from the gNB-CU to the gNB-DU.

Other aspects

It is up to OAM/implementations to ensure how the OD-SIB1 works, e.g., the role of Cell A and NES cell, or coverage hole avoidance etc.

# 4. Reference

- RP-242354 Revised WID: Enhancements of network energy savings for NR

- RAN3_126_agenda_20241122_EOM1.doc

- R3-245737 Summary of offline discussions: Rel-19 Network Energy Saving, Ericsson

- R3-250055 (BL CR to 38.423) Introduction of Network Energy Saving Enhancement	Ericsson etc

- R3-250056 (BL CR to 38.473) Introduction of Network Energy Saving Enhancement	Huawei etc

# 5. TP for TS 38.473 – on top of R3-252499

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the on-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the on-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the on-demand SIB1 IE is included and set to “Provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information indicated in the od-SIB1-Config IE for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the on-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If the Cells to be On-demand SIB1 List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider the cell indicated by the NR CGI IE is accepted or not accepted as indicated by the On-Demand SIB1 indicator IE for On-Demand SIB1.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells to be On-Demand SIB1 List |  | 0..1 |  |  | YES | ignore |
| >Cells to be On-Demand SIB1 List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>On-Demand SIB1 Indicator | M |  | ENUMERATED (accepted, not accepted, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| CHOICE on-demand SIB1 | O |  |  |  | YES | ignore |
| >Provision |  |  |  |  |  |  |
| >>On-demand SIB1 Config | M |  | Octet String | Includes the od-SIB1-Config contained in the SIBxx message as defined in TS 38.331 [8]. |  |  |
| >Stop provision |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMResetInformation,

Cells-to-be-On-Demand-SIB1-List-Item

FROM F1AP-IEs

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-Cells-to-be-On-Demand-SIB1-List,

id-Cells-to-be-On-Demand-SIB1-List-Item,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	}|

{ ID id-Cells-to-be-On-Demand-SIB1-List			CRITICALITY ignore	TYPE	Cells-to-be-On-Demand-SIB1-List		PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

GNB-CU-TNL-Association-To-Add-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Add-ItemIEs } }

GNB-CU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Remove-ItemIEs } }

GNB-CU-TNL-Association-To-Update-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Update-ItemIEs } }

Cells-to-be-Barred-List			::= SEQUENCE(SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-to-be-Barred-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Allowed-to-be-Deactivated-List-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Allowed-to-be-Deactivated-List-Item	CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List-Item	PRESENCE mandatory },

...

}

Cells-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Deactivated-List-Item						CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List-Item					PRESENCE mandatory	},

...

}

Cells-to-be-On-Demand-SIB1-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-On-Demand-SIB1-List-ItemIEs } }

Cells-to-be-On-Demand-SIB1-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-On-Demand-SIB1-List-Item	CRITICALITY ignore	TYPE	Cells-to-be-On-Demand-SIB1-List-Item PRESENCE mandatory },

...

}

GNB-CU-TNL-Association-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Add-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Add-Item			PRESENCE mandatory	},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-OnDemandSIB1,

id-NESPagingAdaptationIndication,

maxNRARFCN,

maxnoofErrors,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Cells-Allowed-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Cells-Allowed-to-be-Deactivated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-Allowed-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Deactivated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Barred-Item::= SEQUENCE {

nRCGI			NRCGI	,

cellBarred		CellBarred,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Barred-Item-ExtIEs } }	OPTIONAL

}

Cells-to-be-Barred-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-IAB-Barred	CRITICALITY ignore	EXTENSION IAB-Barred		PRESENCE optional }|

{ ID id-MobileIAB-Barred	CRITICALITY ignore	EXTENSION MobileIAB-Barred		PRESENCE optional },

...

}

Cells-to-be-On-Demand-SIB1-List-Item ::= SEQUENCE {

nRCGI						NRCGI,

on-DemandSIB1Indicator		ENUMERATED {accepted, not-accepted, ...},

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-On-Demand-SIB1-List-Item-ItemExtIEs} }	OPTIONAL,

...

}

Cells-to-be-On-Demand-SIB1-List-Item-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellBarred	::=	ENUMERATED {barred, not-barred, ...}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

NeighbourNR-CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

nR-ModeInfoRel16								NR-ModeInfoRel16												OPTIONAL,

sSB-PositionsInBurst							SSB-PositionsInBurst											OPTIONAL,

nRPRACHConfig									NRPRACHConfig													OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { NeighbourNR-CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourNR-CellsForSON-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NESPagingAdaptationIndication ::= ENUMERATED {true, ...}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- O

OffsetToPointA	::= INTEGER (0..2199,...)

OnDemandPRS-Info ::= SEQUENCE {

onDemandPRSRequestAllowed				BIT STRING (SIZE (16)),

allowedResourceSetPeriodicityValues		BIT STRING (SIZE (24))	OPTIONAL,

allowedPRSBandwidthValues				BIT STRING (SIZE (64))	OPTIONAL,

allowedResourceRepetitionFactorValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedResourceNumberOfSymbolsValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedCombSizeValues					BIT STRING (SIZE (8))	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { OnDemandPRS-Info-ExtIEs} } OPTIONAL,

...

}

OnDemandPRS-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

OnDemandSIB1 ::= CHOICE {

provision						On-demandSIB1Config,

stopProvison					NULL,

choice-extension				ProtocolIE-SingleContainer { { OnDemandSIB1-ExtIEs} }

}

OnDemandSIB1-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional }|

{	ID id-ERedcap-Bcast-Information		CRITICALITY ignore	EXTENSION ERedcap-Bcast-Information	PRESENCE optional }|

{	ID id-XR-Bcast-Information			CRITICALITY ignore	EXTENSION XR-Bcast-Information			PRESENCE optional }|

{	ID id-BarringExemptionforEmerCallInfo				CRITICALITY ignore	EXTENSION BarringExemptionforEmerCallInfo				PRESENCE optional }|

{	ID id-OnDemandSIB1					CRITICALITY ignore	EXTENSION	OnDemandSIB1				PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UEPagingCapability ::= SEQUENCE {

iNACTIVEStatePODetermination						ENUMERATED {supported, ...} 	OPTIONAL,

iE-Extension				ProtocolExtensionContainer { { UEPagingCapability-ExtIEs} } 		OPTIONAL,

...

}

UEPagingCapability-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RedCapIndication		CRITICALITY ignore 	EXTENSION RedCapIndication		PRESENCE optional }|

{ 	ID id-NESPagingAdaptationIndication	CRITICALITY ignore	EXTENSION NESPagingAdaptationIndication	PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

UuRLCChannelRequiredToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelRequiredToBeReleasedItem

UuRLCChannelRequiredToBeReleasedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelRequiredToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelRequiredToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

On-demandSIB1Config  ::= OCTET STRING

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-OnDemandSIB1										ProtocolIE-ID ::= aaa

id-NESPagingAdaptationIndication					ProtocolIE-ID ::= bbb

id-Cells-to-be-On-Demand-SIB1-List					ProtocolIE-ID ::= ccc

id-Cells-to-be-On-Demand-SIB1-List-Item				ProtocolIE-ID ::= ddd

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP for TS 38.423 on top of R3-252498

### 8.x.x	On-demand SIB1 Configuration Provision

#### 8.x.x.1	General

The purpose of the On-demand SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide On-demand SIB1 configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit On-demand SIB1 configuration information.

The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop transmission of On-demand SIB1 configuration.

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: On-demand SIB1 Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the On-demand SIB1 CONFIGURATION PROVISION REQUEST message to the peer NG-RAN node2.

If the On-demand SIB1 Config is included in the On-demand SIB1 CONFIGURATION PROVISION REQUEST (Naming FFS) message, the NG-RAN node2 shall, if supported, broadcast UL WUS Configuration in SIBx and reply with the UL WUS CONFIGURATION PROVISION RESPONSE. The NG-RAN node2 stores the On-demand SIB1 configuration.

If the stop is included in the UL WUS CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, stop broadcasting UL WUS Configuration in SIBx (SIB naming FFS). Cell A gNB (Naming FFS) removes the received UL WUS Configuration.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the UL WUS CONFIGURATION PROVISION REQUEST message and the UL WUS CONFIGURATION PROVISION RESPONSE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: On-demand SIB1 Provision, unsuccessful operation

If the NG-RAN node2 cannot accept the indicated UL WUS config or the SIB1 Broadcast Status Indicator in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall respond with the UL WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the CELL ACTIVATION REQUEST message and the CELL ACTIVATION FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	On-demand SIB1 Configuration Provision Stop Indication

#### 8.x.y.1	General

The purpose of the UL WUS Configuration Provision Stop Indication procedure is to stop the provision of the On-demand SIB1 configuration accepted by the NG-RAN node following a successful On-demand SIB1 Configuration Provision procedure.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: On-demand SIB1 Configuration Transmission Status Update, successful operation

NG-RAN node1 initiates the procedure by sending the On-demand SIB1 CONFIGURATION PROVISION STOP INDICATION message to NG-RAN node2.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.x.x.x	On-demand SIB1 CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an On-demand SIB1 configuration in one or more cells.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR CGI | M |  | 9.2.2.7 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>On-demand SIB1 Config | M |  | OCTET STRING | Includes the od-SIB1-Config contained in the SIBxx message as defined in TS 38.331 [10]. |  |  |
| >Stop |  |  |  |  |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.x2	On-demand SIB1 CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) has (have) been successfully provisioned with the On-demand SIB1 configuration.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR CGI | M |  | 9.2.2.7 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.1.3.x3	On-demand SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate On-demand SIB1 configuration provision failure.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR CGI | M |  | 9.2.2.7 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.y1	On-demand SIB1 CONFIGURATION PROVISION STOP INDICATION

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to indicate stop the provision of the On-demand SIB1 configuration.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cell To Stop On-demand SIB1 Provision List |  | 1 |  | Cell ID list to which the indication applies. | YES | ignore |
| >Cell To Stop On-demand SIB1 Provision Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>