3GPP TSG-RAN WG3 Meeting #128	R3-253274

# St. Julien, Malta, 19th – 23rd May 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.300, 38.401] on stage 2 related topics

Document for:	Discussions & Approval

# 1	Introduction

This document discusses open items on stage-2 related topics and proposes changes to the current stage-2 BL CRs for TS 38.300 and TS 38.401.

# 2	Discussion

### 2.1	On stage-2 topics in BL CR 38.300

### 2.1.1	Main topics

#### 2.1.1.1	On the term “A-IoT CN node”

Section 3.2 contains an Editor’s Note regarding the term “A-IoT CN node”, i.e. whether there is a need to introduce it and whether it is appropriate.

Now, that we have used that term for quite a while and no issues have been raised, it seems to be appropriate and that term can  be kept.

Proposal 1:	Remove ENs regarding the introduction and usage of the term “A-IoT CN node”.

#### 2.1.1.2	On introducing the (newly agreed) A-IoT Area representation

Last meeting we have introduced the “A-IoT Area” in order to provide a means for indicating the service area within which an A-IoT operation shall be executed (the “Requested Service Area”, which may also contain a list of readers).

Given the fact that we have agreed to integrate A-IoT RAN functions within existing NG-RAN architecture while trying to re-use/observe exisiting NG-C interface and protocol principles, it should be quite straight forward to also follow NG-C principles where “areas” used within UE related NG-C signalling abstract from the actual geographical representation by means of an identiy (e.g. the Tracking Area Identitiy which is represented by a PLMN ID and a Tracking Area Code).

Proposal 2:	Introduce the A-IoT Area as an identity, which geographical representation is configured in the AIOTF by means of OAM.

#### 2.1.1.3	On the number of A-IoT device IDs an Inventory procedure may be concerned with

Looking into details of TPs endorsed last meeting, we observed that, according to the latest (draft) version of TS 23.369, the AF may request an A-IoT operation for multiple A-IoT device IDs, whereas on NG, it seems that the current BL CRs do not allow this (whereas this is deduced mostly from the use of singular in descriptive text in stage 2 or IE name in stage 3).

We believe that NG protocol functions should not be limited to allow just a single A-IoT device ID to be included in the Inventory Request message to allow more efficient handling of an AF request – given that e.g. in case of individual devices, the gNB could process multiple inventories in parallel (utilising e.g. the last know reader information to inter-leave the usage of the respective radio resources/readers).

If this approach is taken, inventory related assistance information should be provided on a per device ID basis.

Proposal 3:	Allow the Inventory Request message to carry multiple A-IoT device IDs. In case of “individual” device IDs, NGAP should foresee to provide per device inventory related assistance information.

#### 2.1.1.4	On Assistance information for Inventory

While the actual coding of various assistance information is still FFS, we should be able to conclude on the following items to be included:

-	approximate number of Target A-IoT devices
this information is only applicable in case the A-IoT device ID is a group ID

-	expected D2R message size
with an estimated maximum message size of 1000bits, the current ASN.1 encoding in the BL CR for NGAP (INTEGER (1..256)) seems to point into the right direction

-	time interval inventory aggregation reporting
this is part of the draft TS 23.369, see §5.4.

-	the last known reader that served the A-IoT device. 
this is expected to be provided for inventorising individual A-IoT devices.

From a stage-2 point of view, this information can be already included, and we would need to observe further findings by SA2 or RAN3 meetings.

Proposal 4:	Include, while removing any related FFSs, in stage-2, assistance information for inventory consisting of “approximate number of Target A-IoT devices”, “expected D2R message size”, “time interval inventory aggregation reporting”, “the last known reader that served the A-IoT device” and keep the FFS regarding further assistance information pending SA2 discussions.

#### 2.1.1.5	On Assistance information for Command

On assistance information for Command, the BL CR for stage 2 contains an FFS for “command type”, for which discussions at the last meeting revealed the which to indicate to NG-RAN the kind of operation that is expected to be requested from the device for allocating respective A-IoT radio resources.

Aspects of device behaviour to be regarded by NG-RAN in order to e.g. communicate with the device having a certain energy status is a pure radio interface aspect, which would not change the network behaviour given information on the “cost” of certain service operation (e.g. to write information into the non-volatile memory), as a minimum capability to perform a respective action (read/write) of the device would need to be assumed in any case.

We propose to finally remove the “command type” from the list of assistance information but include a new Editor’s Note stating that further command related assistance information needs to be finally checked with other WGs.

Proposal 5:	Remove the “command type” from the list of assistance information for command but include a new Editor’s Note stating that further command related assistance information needs to be finally checked with other WGs

### 2.1.2	Further wording/editorial updates

#### 2.1.2.1	NOTE 1 in 16.xx.x2 on deployment with direct/indirect connectivity.

We propose to re-word NOTE 1 as follows:

NOTE 1:	A deployment with both direct connectivity and indirect connectivity is not expected.

#### 2.1.2.2	others

The TP below tries to introduce a certain consistancy, e.g. lower/upper case details when using defined terms, avoiding “and/or” phrases, etc.

### 2.2	On stage-2 topics in BL CR 38.401

### 2.2.1	Cardinality aspects in case of indirect connectivity via AMF(s)

In case of indirect communication, it should be possible to apply the same connectivity cardinality as for NG-RAN node/AMF/SMF cardinality, i.e. to assume that an AIOTF is reachable by an A-IoT enabled gNB via all A-IoT enabled AMF the gNB is connected to. Such approach would have implications on TS 38.401.

Proposal 6:	Specify in TS 38.401 that each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

The cardinality aspect in case of indirect connectivity is proposed to be marked as “FFS” for now. It seems appropriate to inform SA2 on these assumptions.

Proposal 7:	Inform SA2 about the cardinality assumptions in proposal 4 (see draft LS out in R3-250446).

### 2.2.2	Introduction of AIOTF A-IoT Device NGAP ID

The BL CR for 38.401 contains an Editor’s Note regarding the introduction of an AIOTF A-IoT NGAP ID, in order to complement the RAN A-IoT Device NGAP ID, and hence aligning with UE-associated signalling.

The reason for introducing RAN and AMF UE NGAP IDs for UE-associated signalling was to cater for long-lived signalling connections, where either RAN or CN side might want to organise UE context handling in a different way and the number of CN side UE contexts is allowed to exceed the RAN side UE context by far.

In A-IoT however, the RAN A-IoT NGAP ID is significance/scope is on a per A-IoT service operation/session level, with these IDs being released once the service operation denoted by a Correlation ID is terminated.

We conclude from that, that introducing an AIOTF A-IoT Device NGAP ID would not be justified.

Proposal 8:	Remove the Editor’s Note in the BL CR for 38.401 in §6.2.1 regaring the introduction of the AIOTF A-IoT Device NGAP ID.

### 2.2.3	FFSs on the session/device contexts in new §6.X

There are Editor’s Notes for the NG-RAN node A-IoT session/device context definitions in the new section §6.X. While it is easy to ask for correction/adaptation of current definitions, it is rather hard to proof that there is something missing (the FFS is on “further details”).

So, if no further details are revealed during RAN3#128 we propose to remove these FFSs.

Proposal 9:	Remove the Editor’s Note in the BL CR for 38.401 in §6.X regarding the session/device contexts.

# 3	Conclusion

We have discussed a couple of open items, mainly related to already identified open issues contained in the BL CRs for 38.300 and 38.401.

We propose the following:

#### For BL CR 38.300 related topics:

Proposal 1:	Remove ENs regarding the introduction and usage of the term “A-IoT CN node”.

Proposal 2:	Introduce the A-IoT Area as an identity, which geographical representation is configured in the AIOTF by means of OAM.

Proposal 3:	Allow the Inventory Request message to carry multiple A-IoT device IDs. In case of “individual” device IDs, NGAP should foresee to provide per device inventory related assistance information.

Proposal 4:	Include, while removing any related FFSs, in stage-2, assistance information for inventory consisting of “approximate number of Target A-IoT devices”, “expected D2R message size”, “time interval inventory aggregation reporting”, “the last known reader that served the A-IoT device” and keep the FFS regarding further assistance information pending SA2 discussions.

Proposal 5:	Remove the “command type” from the list of assistance information for command but include a new Editor’s Note stating that further command related assistance information needs to be finally checked with other WGs

#### For BL CR 38.401 related topics:

Proposal 6:	Specify in TS 38.401 that each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

Proposal 7:	Inform SA2 about the cardinality assumptions in proposal 4 (see draft LS out in R3-250446).

Proposal 8:	Remove the Editor’s Note in the BL CR for 38.401 in §6.2.1 regaring the introduction of the AIOTF A-IoT Device NGAP ID.

Proposal 9:	Remove the Editor’s Note in the BL CR for 38.401 in §6.X regarding the session/device contexts.

#### Final Proposal

Final Proposal:	Agree on the updates of the BL CRs for 38.300 and 38.401 as shown in Annexes A and B

# Annex A:	Text Proposal against BL CR 38.300 (latest version in R3-253096, proposed changes highlight in yellow)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

2Rx XR UE: two antenna port XR UE as specified in TS 38.101-1 [18].

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, i.e. BRID or DAA.

Aerial UE communication: functionality enabling Aerial UE function, as defined in 16.18.

A-IoT CN node: The core network node which the gNB connects to 5GC for A-IoT operation. More specifically, it is the AIOTF in case the gNB directly connects with the AIOTF, and it includes both the AMF and AIOTF in case the gNB connects with the AIOTF via AMF.

Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m.

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

Boundary IAB-node: as defined in TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	A deployment with both direct connectivity and indirect connectivity is not expected.

-	In this version of the specification A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (Requested Service Area). The Requested Service Area may be indicated by means of either a list of A-IoT Areas or a list of readers or both. An A-IoT Area is indicated by the AIOTF to the gNB by means of an identity, the A-IoT Area Identity.

The AIOTF is aware of

-	the area within which the gNB supports provision of A-IoT services

-	the area represented by an A-IoT Area Identity and

-	a reader’s location

by means of OAM configuration.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and an AIOTF Identifier to identify the inventory session. The Inventory Request message also includes one or several A-IoT Device Identifications corresponding to the A-IoT device(s) which are targeted for the inventory.

The Inventory Request message also includes the Requested A-IoT Service Area and may include Inventory Assistance Information. The Inventory Assistance Information may provide an approximate number of Target A-IoT devices, an estimate of the expected D2R message size, a time interval inventory aggregation reporting, and in case individual A-IoT devices are target, the last known reader that served the A-IoT device..

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 4:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU targeting the A-IoT device and may include some command assistance information. This command assistance information may contain an estimate of the expected D2R message size, command type .

Editor’s Note x:	Specification of further parameters of the Command Request message (e.g., other assistance information from 5GC) needs further work.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message including the AIOTF Identifier and the Correlation ID received in the Command Request and the RAN A-IoT device NGAP ID.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x5 Locating A-IoT devices

In this version of specification, the A-IoT device location is reported by the gNB at Reader ID granularity to the AIOTF via the Inventory Report procedure. The AIOTF may use the A-IoT device location information in performing further A-IoT operations.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex B:	Text Proposal against BL CR 38.401 (latest version in R3-253097, proposed changes highlight in yellow)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.2.1	Principle of handling Application Protocol Identities

For UE-associated signalling, an Application Protocol Identity (AP ID) is allocated when a new UE-associated logical connection is created in either an NG-RAN node or an AMF. An AP ID shall uniquely identify a logical connection associated to a UE or A-IoT device over the NG interface or Xn interface within a node (NG-RAN node or AMF) or over the F1 interface or over the E1 interface or over the W1 interface. Upon receipt of a message that has a new AP ID from the sending node, the receiving node shall store the AP ID of the sending node for the duration of the logical connection. The receiving node shall assign the AP ID to be used to identify the logical connection associated to the UE and include it as well as the previously received new AP ID from the sending node, in the first returned message to the sending node. In all subsequent messages to and from sending node, both AP IDs of sending node and receiving node shall be included.

For MBS-associated logical connections of the E1 interface and the F1 interface the same principles for AP IDs apply as for UE-associated logical connections.

For A-IoT command operation related signalling, per A-IoT session (corresponding to an AIOTF identifier and a Correlation ID) and per A-IoT device, a RAN A-IoT Device NGAP ID to be used via the NG-C interface are introduced, as specified below.

The definitions of AP IDs as used on NG interface or Xn interface or F1 interface or E1 interface are shown below:

RAN UE NGAP ID:

A RAN UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within an gNB. When an AMF receives an RAN UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to an AMF this is included in all UE associated NGAP signalling.

The RAN UE NGAP ID shall be unique within the logical NG-RAN node.

AMF UE NGAP ID:

An AMF UE NGAP ID shall be allocated so as to uniquely identify the UE over the NG interface within the AMF. When a NG-RAN node receives an AMF UE NGAP ID it shall store it for the duration of the UE-associated logical NG-connection for this UE. Once known to a NG-RAN node this ID is included in all UE associated NGAP signalling.

The AMF UE NGAP ID shall be unique within an AMF Set as specified in TS 23.501 [3].

Old NG-RAN node UE XnAP ID:

An Old NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an old NG-RAN node. When a new NG-RAN node receives an Old NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a new NG-RAN node this ID is included in all UE associated XnAP signalling. The Old NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

New NG-RAN node UE XnAP ID:

A New NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a new NG-RAN node. When an old NG-RAN node receives a New NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an old NG-RAN node this ID is included in all UE associated XnAP signalling. The New NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Source NG-RAN node UE XnAP ID:

A Source NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a source NG-RAN node. When a target NG-RAN node receives a Source NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a target NG-RAN node this ID is included in all UE associated XnAP signalling. The Source NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

Target NG-RAN node UE XnAP ID:

A Target NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within a target NG-RAN node. When a source NG-RAN node receives a Target NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to a source NG-RAN node this ID is included in all UE associated XnAP signalling. The Target NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

M-NG-RAN node UE XnAP ID:

An M-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an M-NG-RAN node for dual connectivity. When an S-NG-RAN node receives an M-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an S-NG-RAN node this ID is included in all UE associated XnAP signalling. The M-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

S-NG-RAN node UE XnAP ID:

A S-NG-RAN node UE XnAP ID shall be allocated so as to uniquely identify the UE over the Xn interface within an S-NG-RAN node for dual connectivity. When an M-NG-RAN node receives a S-NG-RAN node UE XnAP ID it shall store it for the duration of the UE-associated logical Xn-connection for this UE. Once known to an M-NG-RAN node this ID is included in all UE associated XnAP signalling. The S-NG-RAN node UE XnAP ID shall be unique within the logical NG-RAN node.

gNB-CU UE F1AP ID:

A gNB-CU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-CU UE F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU UE F1AP ID:

A gNB-DU UE F1AP ID shall be allocated so as to uniquely identify the UE over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU UE F1AP ID it shall store it for the duration of the UE-associated logical F1-connection for this UE. The gNB-DU UE F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP UE E1AP ID:

A gNB-CU-CP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]). When a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]) receives a gNB-CU-CP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-CP UE E1AP ID shall be unique within the gNB-CU-CP (respectively the ng-eNB-CU-CP, or the eNB-CP as defined in TS 36.401[28]) logical node.

gNB-CU-UP UE E1AP ID:

A gNB-CU-UP UE E1AP ID shall be allocated so as to uniquely identify the UE over the E1 interface within a gNB-CU-UP (respectively an ng-eNB-CU-UP, or an eNB-UP as defined in TS 36.401[28]). When a gNB-CU-CP (respectively an ng-eNB-CU-CP, or an eNB-CP as defined in TS 36.401[28]) receives a gNB-CU-UP UE E1AP ID it shall store it for the duration of the UE-associated logical E1-connection for this UE. The gNB-CU-UP UE E1AP ID shall be unique within the gNB-CU-UP (respectively the ng-eNB-CU-UP, or the eNB-UP as defined in TS 36.401[28]) logical node.

ng-eNB-CU UE W1AP ID:

An ng-eNB-CU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-CU. When an ng-eNB-DU receives an ng-eNB-CU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-CU UE W1AP ID shall be unique within the ng-eNB-CU logical node.

ng-eNB-DU UE W1AP ID:

An ng-eNB-DU UE W1AP ID shall be allocated so as to uniquely identify the UE over the W1 interface within an ng-eNB-DU. When an ng-eNB-CU receives an ng-eNB-DU UE W1AP ID it shall store it for the duration of the UE-associated logical W1-connection for this UE. The ng-eNB-DU UE W1AP ID shall be unique within the ng-eNB-DU logical node.

gNB-CU MBS F1AP ID:

A gNB-CU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-CU. When a gNB-DU receives a gNB-CU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for that MBS Session. The gNB-CU MBS F1AP ID shall be unique within the gNB-CU logical node.

gNB-DU MBS F1AP ID:

A gNB-DU MBS F1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the F1 interface within a gNB-DU. When a gNB-CU receives a gNB-DU MBS F1AP ID it shall store it for the duration of the MBS-associated logical F1-connection for this MBS Session. The gNB-DU MBS F1AP ID shall be unique within the gNB-DU logical node.

gNB-CU-CP MBS E1AP ID:

A gNB-CU-CP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-CP. When a gNB-CU-UP receives a gNB-CU-CP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for that MBS Session. The gNB-CU-CP MBS E1AP ID shall be unique within the gNB-CU-CP logical node.

gNB-CU-UP MBS E1AP ID:

A gNB-CU-UP MBS E1AP ID shall be allocated so as to uniquely identify the MBS Session Context over the E1 interface within a gNB-CU-UP. When a gNB-CU-CP receives a gNB-CU-UP MBS E1AP ID it shall store it for the duration of the MBS-associated logical E1-connection for this MBS Session. The gNB-CU-UP MBS E1AP ID shall be unique within the gNB-CU-UP logical node.

A-IoT Correlation ID:

An A-IoT Correlation ID shall be allocated so as to uniquely identify an A-IoT session context. When a gNB receives an A-IoT Correlation ID it shall store it for the duration of the A-IoT service operation. The A-IoT Correlation ID shall be unique within an AIOTF as specified in TS 23.369 [z].

RAN A-IoT Device NGAP ID

A RAN A-IoT Device NGAP ID shall be allocated so as to uniquely identify an A-IoT device context over the NG interface within a NG-RAN node for a certain A-IoT session. When an AIOTF receives an RAN A-IoT Device NGAP ID, it shall store it for the duration of the A-IoT session.

The RAN A-IoT Device NGAP ID shall be unique per A-IoT session within the logical NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.1.1	Overall Architecture of NG-RAN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

In NG-Flex configuration, each NG-RAN node is connected to all AMFs of AMF Sets within an AMF Region supporting at least one slice also supported by the NG-RAN node. The AMF Set and the AMF Region are defined in TS 23.501 [3]. Each A-IoT enabled NG-RAN node is connected to all A-IoT enabled AMFs of AMF Sets within an AMF Region.

Editor’s Note:	This cardinality related aspect may need further discussions, for now it follows non-A-IoT architecture principles.

If security protection for control plane and user plane data on TNL of NG-RAN interfaces has to be supported, NDS/IP TS 33.501 [13] shall be applied.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

## 6.X	A-IoT related contexts in an NG-RAN Node

NG-RAN node A-IoT session context:

An NG-RAN node A-IoT session context is a block of information in an NG-RAN node and encompasses CP and UP, transport and radio resources to support an A-IoT Session. An NG-RAN node A-IoT session context may be associated with one or multiple NG-RAN node A-IoT device contexts.

NG-RAN node A-IoT device context:

An NG-RAN node A-IoT device context is a block of information in an NG-RAN node and encompasses CP and UP, transport and radio resources to support an A-IoT service to one A-IoT device within an NG-RAN node A-IoT session context.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>