3GPP TSG-RAN WG3 Meeting #128	R3-253275

# St. Julien, Malta, 19th – 23rd May 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.412 and 38.413] Discussions on NG-C signalling transport for A-IoT

Document for:	Discussions & Approval

# 1	Introduction

This document discusses NG-C signalling transport for A-IoT along the open topic mentioned in the BL CR for 38.412 in R3-253099.

# 2	Discussion

NGAP has been defined according to requirements on the signalling transport layer as specified in TS 38.413 section 6:

The signalling connection shall provide in sequence delivery of NGAP messages. NGAP shall be notified if the signalling connection breaks.

Further, the NG-C interface features the possibility to operate on a multitude of “SCTP endpoints per NG-RAN node/AMF pair”, with the possibility to configure the usage of each SCTP association, i.e. whether it used only for UE associated signalling, or only for non-UE associated signalling or for both.

Non-UE associated signalling is not only related to interface management protocol functions. Since Rel-15 the following protocol functions are defined to use non-UE associated signalling, e.g.:

-	Paging (see TS 23.501 §5.21.1.3)

-	Retrieve UE Information

-	UE Information Transfer

-	Configuration Transfer Procedures

-	Warning Message Transmission Procedures

-	Downlink/Uplink non UE-associated NRPPA transport

-	RIM Information Transfer Procedures

-	Multicast and Broadcast Related Signalling

-	Timing Synchronisation Status Reporting Procedures

As can be seen, there exists, apart from NG-C interface management procedure a multitude of NGAP protocol functions that uses non-UE associated signalling. Even Paging uses non-UE associated signalling.

It is expected that the discussion is not relevant for the direct connectivity option, where it is expected that in case multiple TNL associations are necessary to be setup due to high signalling load, apart from rarely triggered NG-C management procedures only A-IoT procedures are executed across the NG-C interface.

For the indirect connectivity option, we assume that the addition of yet another group of NGAP protocol functions can be managed as it was possible to manage signalling for a multitude or other NGAP functions by means of non-UE associated signalling and no new category of signalling association would need to be introduced. Therefore, the Editor’s Note in the BL CR for for 38.412 in R3-253099 can be removed.

# 3	Conclusions

The following is proposed:

Proposal 1:	A-IoT signalling traffic on NG-C is performed by means of non-UE associated signalling, irrespective the connectivity option deployed. The Editor’s Note in the BL CR for 38.412 is removed.

Proposal 2:	The agreement that A-IoT signalling traffic on NG-C is performed by means of non-UE associated signalling shall be captured in the BL CR for 38.413 accordingly within the procedural description.

Final Proposal:	Agree on the text proposals as shown in Annexes A and B below.

# Annex A	Text Proposal against BL CR 38.412 (latest version in R3-253099, proposed changes highlight in yellow)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 7	Transport layer

SCTP (IETF RFC 4960 [2]) shall be supported as the transport layer of NG-C signalling bearer. The Payload Protocol Identifier (ppid) to be used by SCTP for the application layer protocol NGAP and for DTLS over SCTP (IETF RFC 6083 [8]) is assigned by IANA in [10]. The byte order of the ppid shall be big-endian.

SCTP refers to the Stream Control Transmission Protocol developed by the Sigtran working group of the IETF for the purpose of transporting various signalling protocols over IP network.

NG-RAN node and AMF shall support a configuration with a single SCTP association per NG-RAN node/AMF pair. Configurations with multiple SCTP endpoints per NG-RAN node/AMF pair should be supported. When configurations with multiple SCTP associations are supported, the AMF may request to dynamically add/remove SCTP associations between the NG-RAN node/AMF pair. Within the set of SCTP associations established between one AMF and NG-RAN node pair, the AMF may request the NG-RAN node to restrict the usage of SCTP association for certain types of NG-C signalling. If no restriction information is provided for an SCTP association, any type of NG-C signalling is allowed via the SCTP association. Selection of the SCTP association by the NG-RAN node and the AMF is specified in TS 23.501 [3] and TS 23.502 [4]. The NG-RAN node shall establish the SCTP association. The SCTP Destination Port number value to be used by NGAP is assigned by IANA in [11]. When the AMF requests to dynamically add additional SCTP associations between the NG-RAN node/AMF pair, the SCTP Destination Port number value may be the one assigned by IANA in [11], or any dynamic port value (IETF RFC 6335 [9]). When the configuration with multiple SCTP endpoints per NG-RAN node is supported and the NG-RAN node wants to add additional SCTP endpoints, the RAN configuration update procedure shall be the first NGAP procedure triggered on an additional TNLA of an already setup NG-C interface instance after the TNL association has become operational, and the AMF shall associate the TNLA to the NG-C interface instance using the included Global RAN node ID.

Between one AMF and NG-RAN node pair:

-	A single pair of stream identifiers shall be reserved over at least one SCTP association for the sole use of NGAP elementary procedures that utilize non UE-associated signalling.

-	At least one pair of stream identifiers over one or several SCTP associations shall be reserved for the sole use of NGAP elementary procedures that utilize UE-associated signallings. However, a few pairs (i.e. more than one) should be reserved.

-	For a single UE-associated signalling, the NG-RAN node shall use one SCTP association and one SCTP stream, and the SCTP association/stream should not be changed during the communication of the UE-associated signalling until after current SCTP association is failed or removed, or TNL binding update is performed as described in TS 23.502 [3].

Transport network redundancy can be achieved by SCTP multi-homing between two end-points, of which one or both is assigned with multiple IP addresses. SCTP end-points shall support a multi-homed remote SCTP end-point. For SCTP endpoint redundancy, an SCTP endpoint (in the NG-RAN node or AMF) may send an INIT, at any time for an already established SCTP association, which the other SCTP endpoint shall handle as defined in IETF RFC 4960 [2] in subclause 5.2.

The SCTP congestion control may, using an implementation specific mechanism, initiate higher layer protocols to reduce the signalling traffic at the source and prioritise certain messages.

All principles and requirements specified for non UE-associated signalling across the NG interface apply for A-IoT related signalling. If NG-C is established between an A-IoT enabled NG-RAN node and an AIOTF, as specified in TS 23.369 [x], all principles and requirements specified for the AMF for support of non UE-associated signalling apply for the AIOTF.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# Annex B	Text Proposal against BL CR 38.413 (latest version in R3-253100, proposed changes highlight in yellow)

-----------------Start of the Next Change-----------------

### 8.xx.1	Inventory Request

#### 8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

-----------------Start of the Next Change-----------------

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node following a successful Inventory Request procedure. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

-----------------Start of the Next Change-----------------

### 8.xx.3	Command Request

#### 8.xx.3.1	General

The purpose of the Command Request procedure is to request the NG-RAN node to transfer the command to the A-IoT device over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

-----------------Start of the Next Change-----------------