3GPP TSG-RAN WG3 Meeting #128	R3-253286

Malta, MT, 19-23, May, 2025

Agenda item:	16.2

Source: 	Samsung

Title: 	(TP to BL CR for 38.413) Discussion on general issues for AIoT

Document for:	Discussion and decision

Introduction

Last meeting discussed the general aspect, and the following agreements and open issues are captured,

Turn the WA to agreement, i.e., Including AIOTF information containers in NGAP.

Enhancing the existing NGAP interface management procedures.

Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed.

Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers.

General:

- Turn previous “WA: Introduce a NGAP Class 1 Inventory Request procedure.” and “WA: Introduce a NGAP Class 1 Command Request procedure” to agreements.

- Define the Reader Index as INTEGER (1..65536, …).

- Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

- It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

In this contribution, we further discuss these open issues.

Discussion

Last meeting continued to discuss the general aspects for AIoT, and a bunch of open issues are captured.

The first open issue is about the AIoT indicator in the NG SETUP REQUEST message. Last meeting agreed to introduce an AIoT indicator, but FFS on the details because companies had different understandings,

- Option 1 is that such AIoT indicator is an AIoT-only indicator: when this indicator is present, it means that the NG-RAN node only supports AIoT radio, and all the other mandatory IEs contained in the NG SETUP REQUEST message shall be ignored by CN.

- Option 2 is that such AIoT indicator only indicates that the NG-RAN node provides AIoT radio, regardless whether the NG-RAN node also provides Uu radio.

When we consider indirect deployment via AMF,

- If we follow Option 1, it is not possible for the AMF to understand correctly when a NG-RAN node support both AIoT radio and Uu.

- If we follow Option 2, when the mandatory IEs are present, it is not possible for the AMF to understand correctly whether a NG-RAN node only supports AIoT radio and the mandatory IEs should be ignored, or a NG-RAN node supports both AIoT radio and Uu.

As a result, following either Option 1 or Option 2 may cause misunderstanding by AMF in case of indirect deployment. In order to solve the above issue, it is more suitable to add two codepoints (i.e. AIoT-only, Both) in the AIoT indicator,

- When the AIoT indicator IE is set to ‘AIoT-only’, the AMF shall ignore those unnecessary mandatory IEs contained in NG SETUP REQUEST message.

- When the AIoT indicator IE is set to ‘Both’, the AMF shall consider the NG-RAN node supports both AIoT Radio and Uu.

Proposal 1: Add two codepoints (i.e. AIoT-only, Both AIoT Radio and Uu) in the AIoT indicator IE in NG SETUP REQUEST message.

The next open issue is,

It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

And this question is actually related to how to encode the A-IoT Area contained in the Requested Service Area Information IE in INVENTORY REQUEST message.

Firstly, it is apparent that both encoding method can work with extra enhancement.

The proponent of encoding A-IoT Area as GNSS info has concern that the introduction of new A-IoT Area ID may bring extra spec impact to SA WGs. However, we do not think it is a big problem, because SA2 is also working on AIoT topic, and what matters more is we need to make a decision as soon as possible, and inform SA2 our decision.

Recall that originally in many features (such as MBS, PWS, etc.), the service area is encoded as a list of cells/TAIs over NGAP. In R19, NR NTN WI agreed to introduce GNSS info on top of cell/TAI list as service area information. The main motivation of NR NTN WI to introduce GNSS info is the area covered by Tracking Area cannot match perfectly with the MBS broadcast area provided by a satellite. And we believe the same motivation may not apply to AIoT case if we introduce a new A-IoT Area ID specifically for A-IoT services, i.e. by careful plan, deployment and configuration, the coverage area provided by an A-IoT Area ID can match perfectly with the area of a specific AIoT service. So we prefer to define a new A-IoT Area ID during Inventory Request procedure.

If the new A-IoT Area ID is adopted, similar to the design for the Tracking Area, an Area Code with 3 Octets and PLMN Identity is used for identifying an A-IoT Area.

Proposal 2: Use new A-IoT Area ID as the Requested Service Area information in INVENTORY REQUEST message.

Proposal 3: Area Code with 3 Octets and PLMN Identity is used for identifying an A-IoT Area.

In addition, RAN3 receives one reply LS from SA2 in R3-253015, with the following information,

- SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area.

- SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

The first question has been discussed above, a new type of area is preferred.

Regarding the second question, we think it is more suitable that one reader is only part of one supported Area, analogous to one TRP is only part of one cell. We think such logic is clearer and simpler.

Proposal 4: Reply to SA2 that,

- The supported Area is a new type of area

- One reader is only part of one supported Area.

The last open issue is,

FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

Firstly, it should be noted that we had the following agreement,

The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

According to the agreement above, we can obtain that the supported A-IoT Areas as well as the Reader list has already been configured to AIOTF by OAM. So we do not expect the Supported A-IoT Areas and Reader list will be provided by NGAP signaling as well; otherwise SA2 has to further discuss which one to follow (OAM or NG signaling) in every possible cases, which brings unnecessary discussion in SA2.

While for the Reader location, our understanding is that we can reuse the mechanism for requesting TRP information in NRPPa as follows,

9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type List |  | 1 |  |  | YES | reject |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info, mobile trp location info, common ta) |  | - |  |

As can be seen from NRPPa, the LMF is able to request the NG-RAN node to provide TRP geo-coordinates in NRPPa TRP Information Exchange procedure.

Similar to the TRP Information Exchange procedure in NRPPa, in order to obtain the Reader location, the AIOTF can send the request to the NG-RAN node, and the NG-RAN node provides the Reader location to AIOTF as a response.

Proposal 5: Follow the previous agreement that the Supported A-IoT Area and Reader list are configured to AIOTF by OAM. Send reply LS to SA5 on the detailed information.

Proposal 6: Introduce a new Class-1 procedure initiated by AIOTF to exchange the Reader location information (Similar to NRPPa TRP Information Exchange procedure).

Conclusion

In this paper, we provide further discussion on general aspects for Ambient IoT. The following proposals are provided,

Proposal 1: Add two codepoints (i.e. AIoT-only, Both AIoT Radio and Uu) in the AIoT indicator IE in NG SETUP REQUEST message.

Proposal 2: Use new A-IoT Area ID as the Requested Service Area information in INVENTORY REQUEST message.

Proposal 3: Area Code with 3 Octets and PLMN Identity is used for identifying an A-IoT Area.

Proposal 4: Reply to SA2 that,

- The supported Area is a new type of area

- One reader is only part of one supported Area.

Proposal 5: Follow the previous agreement that the Supported A-IoT Area and Reader list are configured to AIOTF by OAM. Send reply LS to SA5 on the detailed information.

Proposal 6: Introduce a new Class-1 procedure initiated by AIOTF to exchange the Reader location information (Similar to NRPPa TRP Information Exchange procedure).

TP to BL CR for TS 38.413

8.7	Interface Management Procedures

8.7.1	NG Setup

8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

If the NG-RAN node supports A-IoT and is communicating directly with the AIOTF, as specified in TS 23.369 [z], the NG Setup procedure, as depicted in Figures 8.7.1.2-x and 8.7.1.3-x and specified in the respective sections, is executed between the NG-RAN node and the AIOTF.

8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation with the AMF

Figure 8.7.1.2-x: NG setup: successful operation with the AIOTF

If the NG Setup procedure is executed between the NG-RAN node and the AMF:

- The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

- If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

- If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

- If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message. If the IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and use it for further AMF selection for the IAB-MT.

- The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

-	If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

-	If the RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node. If the Extended RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

-	If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE.

-	If the NB-IoT Default Paging DRX IE is included in the NG SETUP REQUEST message, the AMF shall take it into account for paging.

-	If the RAT Information IE is included in the NG SETUP REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

-	If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

-	If the NID IE within the NPN Support IE is included within a PLMN Support Item IE in the NG SETUP RESPONSE message, the NG-RAN node shall consider that the AMF supports the SNPN identified by the PLMN Identity IE and the NID IE.

-	If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

-	If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

-	If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

- If the A-IoT Indicator IE set to “a-iot only“ is included in the NG SETUP REQUEST message, the AMF shall ignore the Supported TA List IE, the Default Paging DRX IE, the UE Retention Information IE and the NB-IoT Default Paging DRX IE if also included. If the A-IoT Indicator IE set to “both“ is included in the NG SETUP REQUEST message, the AMF shall consider the NG-RAN node supports both Uu and A-IoT radio.

If the NG Setup procedure is executed between the NG-RAN node and the AIOTF:

-	The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AIOTF. The AIOTF responds with an NG SETUP RESPONSE message including the appropriate data.

If the NG Setup procedure is triggered by an NG-RAN node supporting A-IoT:

Editor’s Note:	content of NG Setup related messages for A-IoT is FFS.

--<< Skip unchanged>>--

9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| A-IoT Indicator | O |  | ENUMERATED (a-iot only, both, …) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

--<< Skip unchanged>>--

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

A2X-PC5-QoS-Parameters,

AerialUEsubscriptionInformation,

AllowedNSSAI,

AMFName,

AMFSetID,

--<< Skip unchanged>>--

WarningType,

WUS-Assistance-Information,

XrDeviceWith2Rx,

AIoTIndicator

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-A2X-PC5-QoS-Parameters,

id-AerialUEsubscriptionInformation,

id-AllowedNSSAI,

id-AMFName,

--<< Skip unchanged>>--

id-WUS-Assistance-Information,

id-XrDeviceWith2Rx,

id-SLPositioningRangingServiceInfo,

id-AIoTIndicator

--<< Skip unchanged>>--

-- **************************************************************

--

###### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	}|

{ ID id-RANNodeName					CRITICALITY ignore	TYPE RANNodeName					PRESENCE optional	}|

{ ID id-SupportedTAList				CRITICALITY reject	TYPE SupportedTAList				PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX						PRESENCE mandatory	}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX		PRESENCE optional	}|

{ ID id-Extended-RANNodeName		CRITICALITY ignore	TYPE Extended-RANNodeName			PRESENCE optional	}|

{ ID id-AIoTIndicator				CRITICALITY reject	TYPE AIoTIndicator				PRESENCE optional	},

...

}

--<< Skip unchanged>>--

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

--<< Skip unchanged>>--

AIoTIndicator ::= ENUMERATED {

a-iot only,

both,

...

}

--<< Skip unchanged>>--

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

--<< Skip unchanged>>--

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-AIoTIndicator										ProtocolIE-ID ::= xxx

END

-- ASN1STOP