3GPP TSG-RAN WG3 Meeting #128                                                                  R3-253307

Malta, MT, May 19th – 23rd, 2025

Source:	CATT

Title:	(TP for BLCR to F1AP) On support on-demand SIB1 for idle UE

Agenda Item:	17.3

Document for:	Discussion and Approval

1 Introduction

During RAN3#126 meetings[1], following agreements were achieved for on-demand SIB1:

RAN3#126 The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In the RAN3#127 meeting[2], agreements were made as below:

RAN3#127 It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

In the last meeting[3], following agreements were achieved:

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: - One Choice “Start with UL WUS Condiguration” - One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

In this contribution, we will continue to discuss the on-demand SIB1 topic.

2 Discussion

2.1 XnAP signalling

Code point of request message

Per agreements in the last meeting[3], the code points “Start with UL WUS Configuration” and “Stop” were introduced to the request message of the new Class 1 Xn procedure, while the left issue is whether the choice “pause/resume” need to be introduced. “Stop” means the Cell A gNB will stop the broadcast and remove the UL WUS configuration. “pause” means the Cell A gNB will stop the broadcast without removing the UL WUS configuration, it also means that the NES gNB can use “resume” code point to re-start the broadcast instead of using the “Start with UL WUS configuration” code point.

We understand the choice “pause/resume” is based on the assumption that the Cell A gNB should keep the UL WUS configuration stored once receiving the “Start with UL WUS Configuration”, and it can work theoretically. However, whether to store or remove storage is a pure implementation matter, it’s unreliable to expect peer gNBs to keep something stored, so it does not make sense for NES gNB to ask Cell A gNBs to store the UL WUS configuration using the “Start with UL WUS Configuration” request. Normally, NES gNB has no idea about whether an UL-WUS configuration is stored in Cell A gNB, that it should include the UL WUS configuration with each start instruction. So, the “pause/resume” choices are not needed.

Proposal 1-1: RAN3 to confirm that only “Start with UL WUS Configuration” (i.e. “Start”) and “Stop” code points are supported in the request message.

Since the “Start” code point can also be used to request the Cell A gNB to update UL WUS Configuration when the configuration is modified, the name of “Start” can be replaced with “Start or Update”.

Proposal 1-2: Rename the “Start” code point to “Start or Update” in the request message.

Whether the request/response message includes the identities of CellAs

Upon receiving the “Start or Update” code point, Cell A gNB shall determine the Cell A and evaluate whether to accept broadcasting. In principle, the request does not need to include a Cell A list for the UL WUS configuration, because the mapping relationship between NES cell and Cell A is configured not only to the NES gNB but also to the Cell A gNB by OAM. Cell A gNB can decide which cell to broadcast the UL WUS configuration based on OAM pre-configuration.

Nevertheless, for flexibility, the request message can optionally include a list of recommended Cell A associated with the UL WUS configuration. If the recommended Cell A list is included, the Cell A gNB can ignore the OAM configuration, which means a Cell A outside this list does not need to broadcast corresponding UL WUS configuration even if it is configured by OAM.

Proposal 2-1: Cell A gNB can determine which cell to broadcast the UL WUS configuration by OAM configuration.

Proposal 2-2: A recommended Cell A list can be optionally included for the UL WUS configuration in the request message. If the list is included, the UL WUS configuration should be broadcasted in the recommended Cell A(s).

If Cell A gNB accepts broadcasting the UL WUS configuration, it can send a response message to the NES gNB. An issue arises about whether the response message should include a list of Cell A which accepts the broadcasting. We think it’s not necessary. Because the Cell A and NES cell relationship is usually configured by OAM, cell A is expected to be capable of broadcasting the UL WUS configuration and should not refuse the request of NES cell normally. It’s a corner case where some of the Cell A can do the broadcast while others under the same gNB not. With the response message, the NES gNB should assume all Cell A(s) under the Cell A gNB accept the broadcasting.

Proposal 2-3: The response message does not include the identities of Cell A who can broadcast the UL WUS configuration.

The UL WUS Configuration identification

As more than one NES cell can be under the same gNB, the NES gNB can request a Cell A gNB to broadcast more than one UL WUS configuration, i.e., the request message can include request for more than one UL WUS configuration. Upon receiving the request message with multiple UL WUS Configurations, Cell A gNB can evaluate whether to accept the broadcast for each UL WUS configuration. If Cell A gNB cannot accept the broadcast for some of the WUS configurations, it should indicate that via the response message to the NES gNB. Therefore, each UL WUS configuration in the request message should be associated with an identity, to facilitate the response message indicating which UL WUS configuration is not accepted.

When none of the UL WUS configurations included in the request message can be broadcasted, Cell A gNB should respond with a failure message to the NES gNB.

The UL WUS configuration identity can also be used to identify a UL WUS configuration in the following up request of UL WUS configuration update or request of stop broadcast from the NES gNB. And the identity can be used in the new Class-2 Xn procedure for indicating the NES gNB that Cell A gNB has stopped the broadcast.

Proposal 3-1: A request message can include the request for more than one UL WUS configuration with each configuration associated with an identity.

Proposal 3-2: The response message can indicate which ULWUS configuration cannot be broadcasted using the UL WUS configuration’s identity.

Proposal 3-3: If none of the UL WUS configurations included in the request message can be broadcasted, Cell A gNB should respond with a failure message to the NES gNB.

Proposal 3-4: The UL WUS configuration identity can be used to identify a UL WUS configuration for request of UL WUS configuration update or stop broadcast from NES gNB to Cell A gNB, also used for indicating the stop of broadcast from Cell A gNB to NES gNB.

RRC IE included in UL WUS configuration IE

Last RAN2 meeting[4] agreed that one WUS configuration can be associated with a list of cells:

WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

According to the RRC Running CR[5], each UL WUS configuration broadcasted via SIBxx is delivered in od-SIB1-Config-r19 associated with a {ARFCN, PCI list} pair which is included in the OD-SIB1-CellConfig-r19.

SIBxx information element -- ASN1START -- TAG-SIBxx-START SIBxx-r19 ::= SEQUENCE { od-SIB1-CellConfigList-r19 SEQUENCE (SIZE(1..maxCellODSIB1-r19)) OF OD-SIB1-CellConfig-r19 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } OD-SIB1-CellConfig-r19 ::= SEQUENCE { carrierFreq-r19 ARFCN-ValueNR, physCellId-r19 SEQUENCE (SIZE(1..maxPCI-r19)) OF PhysCellId, od-SIB1-Config-r19 od-SIB1-Config-r19 OPTIONAL, -- Need R ... } … -- TAG-SIBxx-STOP -- ASN1STOP

So, each UL WUS configuration in the request message can also be associated with a {ARFCN, PCI list} pair, and it can be an octet string contain the OD-SIB1-CellConfig-r19 in SIBxx-r19 IE defined in RRC spec.

Proposal 4: The UL WUS configuration IE is an octet string containing the OD-SIB1-CellConfig-r19 in SIBxx-r19 IE defined by RRC spec.

2.2 F1AP signalling

gNB-CU -> gNB-DU

As NES gNB-CU can decide whether OD-SIB1 can be operated in the NES cell based on whether Cell A can broadcast the UL WUS configuration, e.g., via the response or failure message of the new Class 1 XnAP procedure, the NES gNB-CU should indicate its decision to the NES gNB-DU. Cells allowed for OD-SIB1 List IE can be added to existing gNB-CU Configuration Update message. If NES gNB-CU decide a cell can operate OD-SIB1, it can include the cell in the allowed list and send the gNB-CU Configuration Update message to NES gNB-DU, otherwise, it does not include the cell in the allowed list. In that way, the NES gNB-DU can lean whether activating OD-SIB1 for the time being is allowed to a cell.

Proposal 5-1: gNB-CU can indicate the allowed cell(s) for OD-SIB1 to gNB-DU. Add the Cells allowed for OD-SIB1 List IE in gNB-CU Configuration Update message.

If a NES cell previously allowed to operate OD-SIB1 is not allowed anymore, NES gNB-CU should indicate that to gNB-DU.

Since the allowed list for OD-SIB1 IE is assumed to include identities of all the cells allowed to operate OD-SIB1, if any cell cannot keep OD-SIB1 operation, gNB-CU can exclude the cell from the allowed list for OD-SIB1 and update the list by gNB-CU Configuration Update message. For example, when NES gNB is indicated by Cell A gNB over Xn that the broadcast of UL WUS configuration has been stopped, or when NES gNB receives the response from the Cell A gNB to the request of stop the UL WUS configuration broadcast from NES gNB over Xn, gNB-CU can decide OD-SIB1 should not be performed in the cell and sends gNB-DU the updated Cells allowed for OD-SIB1 List IE with the cell being excluded.

It can be observed that the behaviours of gNB-CU and gNB-DU with respect to handling the Cells allowed for OD-SIB1 List IE are tightly related to the cases where new Class 1 and Class 2 procedures are performed. We propose to capture the handling of this list and the related XnAP procedures in BLCR to TS 38.401.

Proposal 5-2: Capture the behaviours of gNB-CU and gNB-DU on handling the Cells allowed for OD-SIB1 List IE and related XnAP procedures in BLCR to TS 38.401.

gNB-DU -> gNB-CU

In RAN3#126 meeting[1], it was agreed that WUS configuration is provided from DU to CU via Serving Cell Information IE:

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In the last meeting[3], the on-demand SIB1 IE with two code points “Provision” and “Stop provision” was introduced to the existing “Served Cell Information” IE. “Provision” code point is used for sending UL WUS configuration from DU to CU, so that Cell A can broadcast the UL WUS configuration, e.g., at the request of NES gNB over Xn. “Stop provision” is used for indicating CU that broadcast the UL WUS configuration is not required, to let Cell A stop the broadcast, e.g., NES gNB can request Cell A gNB to stop broadcast over Xn. It can be understood that “Provision” is also used for indicating CU the update of UL WUS Configuration when the configuration is modified by DU.

It is also observed the meaning of these code points are totally aligned with those in the UL WUS CONFIGURATION PROVISION REQUEST message over Xn. Thus, we suggest names of these code points can reuse those used in the request message of the new Class 1 XnAP procedure, i.e., renaming “Provision” to “Start or Update” and renaming “Stop Provision” to “Stop”.

Proposal 6: Rename the “Provision” code point to “Start or Update” and rename the “Stop provision” code point to “Stop” in the on-demand SIB1 IE.

In present F1AP BLCR, which RRC IE the UL WUS configuration information IE in the Served Cell Information IE contains is still FFS. According to the RRC Running CR, each UL WUS configuration broadcasted via SIBxx is delivered in od-SIB1-Config-r19.

SIBxx information element -- ASN1START -- TAG-SIBxx-START SIBxx-r19 ::= SEQUENCE { od-SIB1-CellConfigList-r19 SEQUENCE (SIZE(1..maxCellODSIB1-r19)) OF OD-SIB1-CellConfig-r19 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } OD-SIB1-CellConfig-r19 ::= SEQUENCE { carrierFreq-r19 ARFCN-ValueNR, physCellId-r19 SEQUENCE (SIZE(1..maxPCI-r19)) OF PhysCellId, od-SIB1-Config-r19 od-SIB1-Config-r19 OPTIONAL, -- Need R ... } … -- TAG-SIBxx-STOP -- ASN1STOP

So, the UL WUS configuration information IE can be an octet string contain the od-SIB1-Config-r19 in SIBxx-r19 IE defined in RRC spec.

Proposal 7: The UL WUS configuration information IE is an octet string containing the od-SIB1-Config-r19 in SIBxx-r19 IE defined by RRC spec.

3 Conclusion

In the previous sections we made the following observations and proposals:

XnAP siganlling

Proposal 1-1: RAN3 to confirm that only “Start with UL WUS Configuration” (i.e. “Start”) and “Stop” code points are supported in the request message.

Proposal 1-2: Rename the “Start” code point to “Start or Update” in the request message.

Proposal 2-1: Cell A gNB can determine which cell to broadcast the UL WUS configuration by OAM configuration.

Proposal 2-2: A recommended Cell A list can be optionally included for the UL WUS configuration in the request message. If the list is included, the UL WUS configuration should be broadcasted in the recommended Cell A(s).

Proposal 2-3: The response message does not include the identities of Cell A who can broadcast the UL WUS configuration.

Proposal 3-1: A request message can include the request for more than one UL WUS configuration with each configuration associated with an identity.

Proposal 3-2: The response message can indicate which ULWUS configuration cannot be broadcasted using the UL WUS configuration’s identity.

Proposal 3-3: If none of the UL WUS configurations included in the request message can be broadcasted, Cell A gNB should respond with a failure message to the NES gNB.

Proposal 3-4: The UL WUS configuration identity can be used to identify a UL WUS configuration for request of UL WUS configuration update or stop broadcast from NES gNB to Cell A gNB, also used for indicating the stop of broadcast from Cell A gNB to NES gNB.

Proposal 4: The UL WUS configuration IE is an octet string containing the OD-SIB1-CellConfig-r19 in SIBxx-r19 IE defined by RRC spec.

F1AP signalling

- gNB-CU -> gNB-DU

Proposal 5-1: gNB-CU can indicate the allowed cell(s) for OD-SIB1 to gNB-DU. Add the Cells allowed for OD-SIB1 List IE in gNB-CU Configuration Update message.

Proposal 5-2: Capture the behaviours of gNB-CU and gNB-DU on handling the Cells allowed for OD-SIB1 List IE and related XnAP procedures in BLCR to TS 38.401.

- gNB-DU -> gNB-CU

Proposal 6: Rename the “Provision” code point to “Start or Update” and rename the “Stop provision” code point to “Stop” in the on-demand SIB1 IE.

Proposal 7: The UL WUS configuration information IE is an octet string containing the od-SIB1-Config-r19 in SIBxx-r19 IE defined by RRC spec.

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #126, MCC

- Draft Report of 3GPP TSG RAN3 meeting #127, MCC

- Draft Report of 3GPP TSG RAN3 meeting #127bis, MCC

- Draft Report of 3GPP TSG RAN2 meeting #129bis, MCC

- R2-250xxxx, Running RRC CR NES Post129bis, Ericsson

- R3-253103, (BL CR to 38.473) Introduction of Network Energy Saving Enhancement, Huawei, Ericsson, CMCC, Samsung, ZTE, Nokia, Deutsche Telekom, Lenovo, NEC, Jio

5 Annex (TP to BLCR of 38.473)

-------------------------------------Start of TP---------------------------------------------

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells allowed for on-demand SIB1 List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use the information indicated by this IE for on-demand SIB1 operation for network energy saving as specified in TS 38.401.

Editor’s Note: these procedure texts above may need to be refined in future.

>>>>>>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<<<<<<<<<<

-----------------------------------------------------Next Change-----------------------------------------------------

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Unchanged parts are skipped |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed for On-demand SIB1 |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed for On-demand SIB1 Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

-------------------------------------Next Change---------------------------------------------

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| CHOICE on-demand SIB1 | O |  |  |  | YES | ignore |
| >Start or Update |  |  |  |  |  |  |
| >>UL WUS Configuration Information | M |  | Octet String | Includes the od-SIB1-Config-r19 in SIBxx-r19 IE, as defined in TS 38.331 [8]. |  |  |
| >Stop |  |  |  |  |  |  |

| - Range bound | - Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

-------------------------------------End of TP---------------------------------------------