3GPP TSG-RAN WG3 #128	R3-253317

Malta, MT, 19th – 23rd May 2025

Agenda Item:	11.4

Source:	Lenovo

Title: 		Discussion on UE performance collection in AIML for NR-DC

Document for:	Discussion and Approval

# Introduction

In the last meeting, the following agreements were made:

The following list of exit conditions to stop the collection of UE performance at SN side:

- the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- the NR-DC with the S-NG-RAN node for the UE is released.

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

# Discussion

One aspect to be discussed is if the existing list of UE performance metrics can be reused for DC scenario or not. Note that after SN addition or SN change, the following types of radio bearers could exist depending on where it is anchored.

- MN terminated MCG/SCG/split bearer with QoS flow anchored at MN

- SN terminated MCG/SCG/split bearer with QoS flow anchored at SN

There are six types of radio bearers in DC scenario: MN terminated MCG bearer, MN terminated SCG bearer, MN terminated split bearer, SN terminated MCG bearer, SN terminated SCG bearer, SN terminated split bearer

In the meanwhile, the existing UE performance IE consists of average UE throughput DL, average UE throughput UL, average packet delay, and average packet loss DL as below:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11.

In the meanwhile, it has been agreed in the split architecture discussion that UL/DL UE throughput and UL/DL packet delay are measured at CU-UP.

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level. WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19. Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

In the split architecture discussion that UL/DL UE throughput and UL/DL packet delay are measured at CU-UP.

In the case of DC, the UL/DL packet delay and UL/DL throughput would be measured by MN in case of MN terminated bearer or measured by SN in case of SN terminated bearer.

| DC radio bearer type | Packet delay and throughput measured by |
| --- | --- |
| MN terminated MCG bearer | MN |
| MN terminated SCG bearer | MN |
| MN terminated split bearer | MN |
| SN terminated MCG bearer | SN |
| SN terminated SCG bearer | SN |
| SN terminated split bearer | SN |

In order to accurately reflect the delay caused by SN and be understood by MN, RAN3 discusses the following two options:

- Support SN to measure and report UL/DL delay/throughput per DRB level for SN terminated bearers. In this case, according to the configuration MN can aggregate and determine the overall UE performance.

- Support SN to measure and report averaged UL/DL delay/throughput for SN terminated bearers.

To reflect the delay/throughput and be understood by MN correctly, RAN3 discusses the following two options:

  - Support SN to measure and report UL/DL delay/throughput per DRB level for SN terminated bearers.

  - Support SN to measure and report averaged UL/DL delay/throughput (per UE level) for SN terminated bearers.

Another aspect raised by some companies is that if the start/stop of UE performance collection after SN addition/change should consider SCG activation/deactivation, i.e., UE performance can only be collected if SCG is activated.

In our view, it can be handled by network implementation considering:

- MN is aware of the SCG activation/deactivation state during or after the SN addition/change.

- SCG can be activated or deactivated many times within the data collection duration.

- It is possible that MN can add a SN with SCG deactivated, in this case a reasonable MN implementation shall not initiate the UE performance collection after SN addition/change.

In other word, it seems nothing breaks if SCG deactivation is not specified as an exit condition that terminates the UE performance collection after SN addition/change.

SCG deactivation is not specified as an exit condition that terminates the UE performance collection after SN addition/change

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	There are six types of radio bearers in DC scenario: MN terminated MCG bearer, MN terminated SCG bearer, MN terminated split bearer, SN terminated MCG bearer, SN terminated SCG bearer, SN terminated split bearer

Observation 2	In the split architecture discussion that UL/DL UE throughput and UL/DL packet delay are measured at CU-UP.

Based on the discussion above, we propose:

Proposal 1	To reflect the delay/throughput and be understood by MN correctly, RAN3 discusses the following two options:

a.	Support SN to measure and report UL/DL delay/throughput per DRB level for SN terminated bearers.

b.	Support SN to measure and report averaged UL/DL delay/throughput (per UE level) for SN terminated bearers.

Proposal 2	SCG deactivation is not specified as an exit condition that terminates the UE performance collection after SN addition/change