3GPP TSG-RAN WG3 #128	R3-253331

St. Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	20.2

Source:	Lenovo

Title: 		AIML for gNB assisted positioning

Document for:	Discussion and Approval

# Introduction

In this paper, we further discuss issues related to AIML for gNB assisted positioning case 3a.

# Discussion

## Case 3a: AIML for NG-RAN node assisted positioning with gNB-side model

In the previous RAN3 meeting, the following solution was agreed by RAN3 to support gNB to fetch the part B information of a UE under an on-going positioning session.

1. The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures. 2. The gNB(s) determines that data collection is needed for the UE being positioned. 3. The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35]. 4. The LMF sends the (new) NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

In the meanwhile, RAN1 has made the following agreements related to the part B information needed by gNB for case 3a.

RAN1 Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

Therefore, within the (new) NRPPa POSITIONING DATA COLLECTION REPORT message, the UE location related information shall include:

- UE location coordinates (e.g., introduce LocationCoordinates in 38.455 as defined in 37.355)

- The source of the location coordinates, if available (e.g., introduce LocationSource IE in 38.455 as defined in 37.355)

- UE location error (e.g., introduce LocationError in 38.455 as defined in 37.355)

- Timing information, (e.g., Time Stamp IE in 38.455)

- Time information quality, (e.g., Timing Measurement Quality IE in 38.455, as agreed by RAN1)

- LOS/NLOS indication, (e.g., LoS/NLoS Information IE in 38.455, as concluded by RAN1)

Within the (new) NRPPa POSITIONING DATA COLLECTION REPORT message, the UE location related information shall include:

  - UE location coordinates (e.g., introduce LocationCoordinates IE in 38.455 as defined in 37.355)

  - The source of the location coordinates (e.g., introduce LocationSource IE in 38.455 as defined in 37.355)

  - UE location error (e.g., introduce LocationError IE in 38.455 as defined in 37.355)

  - Timing information (e.g., Time Stamp IE in 38.455)

  - Time information quality (e.g., Timing Measurement Quality IE in 38.455, as agreed by RAN1)

  - LOS/NLOS indication (e.g., LoS/NLoS Information  IE in 38.455, as concluded by RAN1)

RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

For activating a gNB-sided model, one needs to know in which scenario the trained AI model will be applicable. This is by nature known by gNB itself, but difficult to transfer the meta data about the gNB-sided AI model to LMF. Besides, since last meeting, RAN3 agreed to only support RAN based monitoring for case 3a, then it should be upon gNB’s own decision if an AIML model will be activated/deactivated and used to improve the positioning accuracy for case 3a.

FFS if LMF should be aware that the reported measurements are AI/ML generated.

Since there is no mean for LMF to understand if the measurements reported by gNB based on AIML is good or not, there seems no need for LMF to be aware if the measurements is generated by AIML neither.

For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

LMF does not need to explicitly know if the measurements reported by gNB are AIML generated or not.

# Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	Within the (new) NRPPa POSITIONING DATA COLLECTION REPORT message, the UE location related information shall include:

a.	UE location coordinates (e.g., introduce LocationCoordinates IE in 38.455 as defined in 37.355)

c.	UE location error (e.g., introduce LocationError IE in 38.455 as defined in 37.355)

d.	Timing information (e.g., Time Stamp IE in 38.455)

e.	Time information quality (e.g., Timing Measurement Quality IE in 38.455, as agreed by RAN1)

f.	LOS/NLOS indication (e.g., LoS/NLoS Information  IE in 38.455, as concluded by RAN1)

Proposal 2	For Case 3a, it is upon gNB’s decision if an AIML based method will be used to improve the positioning accuracy.

Proposal 3	LMF does not need to explicitly know if the measurements reported by gNB are AIML generated or not.