3GPP TSG-RAN WG3 Meeting #128	R3-253332

Malta, MT, 19th – 23rd May, 2025

Agenda item:	21.3

Source: 	Lenovo

Title: 	(TP to XR BL CR for 38.473) On RLC retransmission enhancements

Document for:	Other

- 1. Introduction

The contribution discusses the potential RAN3 impact of RLC retransmission enhancements:

-	F1 impact of timely RLC retransmission (autonomous retransmission and enhanced polling).

-	F1 impact of unnecessary RLC retransmission avoidance.

- 2. Discussion

In Rel-19 XR work item, RLC retransmission related enhancements are under discussion, that comprises of two directions:

-	Timely RLC retransmission: to avoid the late RLC retransmission, the transmitting side of RLC entity may trigger early RLC retransmission for data based on the delay status, provided the transmitting entity did not receive a status report, which is called as timely RLC retransmission. Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement.

-	Unnecessary retransmissions avoidance: to avoid the RLC unnecessary retransmission, when the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment.

Timely RLC retransmission

In RAN2#129bis meeting, it was agreed that

For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

Observation 1: The thresholds for triggering autonomous retransmission and enhanced polling are determined based on the remaining time of the PDCP discard timer.

For uplink, gNB configures thresholds for autonomous retransmission and enhanced polling to the UE. As in the current RRC running CR [1], the configuration is in the RLC-Config IE:

#### –	RLC-Config

The IE RLC-Config is used to specify the RLC configuration of SRBs, multicast MRBs and DRBs.

UL-AM-RLC-v19xy ::=                     SEQUENCE {

stopReTxObsoleteSDU-r19         ENUMERATED {enabled}                                                       OPTIONAL,   -- Need R

autonomousReTxThreshold-r19              AutonomousReTxThreshold-r19                                       OPTIONAL,   -- Need R

enhancedPollingThreshold-r19             EnhancedPollingThreshold-r19                                      OPTIONAL    -- Need R

}

autonomousReTxTreshold Remaining time threshold used by the Tx side of the RLC entity to trigger an autonomous retransmission as specified in TS 38.322 [4]. Value for the IE AutonomousReTxThreshold in number of milliseconds. Editor's NOTE: FFS whether the autonomous retransmission is also applicable for discard for PDUs with low importance, which uses a separate timer discardTimerForLowImportance

enhancedPollingTheshold Remaining time threshold that when the remaining time of an RLC SDU determined by the discardTimer in TS 38.323 [5] falls below the threshold, the Tx side of the RLC entity triggers polling as specified in TS 38.322 [4]. Value for the IE enhancedPollingThreshold in number of milliseconds. Editor's NOTE: FFS whether enhanced polling is also applicable for discard for PDUs with low importance, which requires a separate timer discardTimerForLowImportance

Observation 2: For uplink, the thresholds that trigger autonomous retransmission and enhanced polling are configured to UE in the RLC-Config IE.

In case of CU-DU split architecture, the RLC-Config IE is generated in gNB-DU as a part of function of RLC. However, the gNB-DU does not know the PDCP discard timer information so that it is not able to provide the thresholds in the RLC-Config IE.

Observation 3: The gNB-DU does not know the PDCP discard timer information so that it is not able to provide the thresholds for triggering autonomous retransmission and enhanced polling in the RLC-Config IE.

To address the issue, the gNB-CU can send the PDCP Discard Timer value of a DRB to the gNB-DU. The gNB-DU can then configure the autonomous retransmission and/or enhanced polling thresholds for the DRB based on the received PDCP Discard Timer value. In case of PSI based discard, the gNB-CU also sends the PSI based discard timer value of a DRB to the gNB-DU.

Proposal 1: For uplink, the gNB-CU sends the PDCP Discard Timer value and PSI based discard timer value (if configured) of a DRB to the gNB-DU so that gNB-DU can then configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

For downlink, the PDCP discard timer operates in the gNB-CU. When a packet is transmitted from the gNB-CU to the gNB-DU, it may be buffered in the gNB-CU for some time. Since the gNB-DU is unaware of the remaining time on the PDCP discard timer of the packet, it cannot determine the optimal timing for performing timely RLC transmission.

One of the solutions to address the issue would be:

-	the gNB-DU determines the thresholds for triggering timely RLC retransmission (including autonomous / timer-based retransmission and enhanced polling). The gNB-DU sends the thresholds to the gNB-CU.

-	when the remaining time of an PDCP SDU falls below the threshold, the gNB-CU sends a timely retransmission indication for the PDCP SDU to the gNB-DU.

-	the gNB-DU performs timely RLC retransmission of the RLC SDU or RLC SDU segment corresponding to the PDCP SDU accordingly according to the timely retransmission indication of the PDCP SDU.

The detailed steps:

(Step 1) gNB-CU sends the configured value of PDCP Discard Timer and PSI based discard timer (if configured) of a DRB to gNB-DU.

(Step 2) gNB-DU decides the threshold(s) for timely retransmission and send it to gNB-CU

When gNB-DU decides to configure autonomous retransmission and/or enhanced polling threshold of the DRB, the gNB-DU decides the threshold(s) for autonomous retransmission and/or enhanced polling threshold of the DRB respectively.

(Step 3) gNB-CU decides whether to trigger timely retransmission according to the thresholds

When the remining time till PDCP discard timer expiry of a PDCP SDU is less than the Enhanced Polling Threshold, and the PDCP SDU has not acknowledged as successfully delivered to the UE, the gNB-CU sends an enhanced polling indication the gNB-DU.

When the remining time till PDCP discard timer expiry of the PDCP SDU is less than the Autonomous Retransmission Threshold, and the PDCP SDU has not acknowledged as successfully delivered to the UE, the gNB-CU sends an enhanced Autonomous Retransmission indication of the PDCP SDU to the gNB-DU.

The enhanced polling indication and the Autonomous Retransmission indication can be provided by F1 user plane protocol, e.g. in the DL USER DATA frame.

Proposal 2: For downlink, RAN3 discusses the signalling enhancement between CU and DU to support timely retransmission.

Unnecessary RLC retransmission avoidance

In RAN2#129 meeting, RAN2 agreed to have a combined approach to avoid unnecessary RLC retransmission, as follows:

Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

For downlink, the PDCP is located in the gNB-CU while the TX RLC entity is located in gNB-DU. To support RAN2’s agreement, some coordination between the gNB-CU and the gNB-DU is needed. The gNB-CU needs to send a discard indication of a PDCP SDU to the gNB-DU so that the gNB-DU can stop the transmission and retransmission of the SDU or the segment of the SDU. When the PDCP timer expires for a PDCP SDU, the gNB-CU sends the indication to the gNB-DU to stop the transmission and retransmission of the RLC SDU or the segment of the RLC SDU which corresponds to the PDCP SDU.

In RAN3#127bis, an open issue is identified for the issue as:

FFS on whether the gNB-CU sends an indication to the gNB-DU to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

Regarding the solution to implement the discussion in F1-U, one of solution is reusing the existing discard related IEs in the DL USER DATA frame:

The existing discard related IEs were introduced for PDCP duplication to avoid unnecessary transmission in the duplicated path if the PDCP PDUs has been delivered to UE successfully in the other path. However, the exiting ‘discard’ related IEs cannot be reused directly due to the following reasons:

-	For legacy, the existing "discard" indication applies only to RLC SDUs that are not currently being transmitted or retransmitted. It does not affect RLC PDUs that have already been submitted to lower layers.

-	For the new feature introduced in Rel-19, the "discard" indication has a further impact on ongoing transmissions or retransmissions. In RAN2, the UE behavior is defined as "stopping the transmission and retransmission of the SDU or the segment of the SDU.

Observation 4: The exiting ‘discard’ related IEs the DL USER DATA frame cannot be reused directly for unnecessary RLC retransmission avoidance.

To avoid any potential backwards compatibility issue, it makes sense to introduce a new IE in the DL USER DATA frame specifically indicates the PDCP PDU associated with an RLC SDU, or a segment of the RLC SDU, for which the transmission and retransmission should be stopped.

Proposal 3: Introduce a new IE in the DL USER DATA frame specifically indicates the PDCP PDU associated with an RLC SDU, or a segment of the RLC SDU, for which the transmission and retransmission should be stopped.

- 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Timely RLC retransmission

Observation 1: The thresholds for triggering autonomous retransmission and enhanced polling are determined based on the remaining time of the PDCP discard timer.

Observation 2: For uplink, the thresholds that trigger autonomous retransmission and enhanced polling are configured to UE in the RLC-Config IE.

Observation 3: The gNB-DU does not know the PDCP discard timer information so that it is not able to provide the thresholds for triggering autonomous retransmission and enhanced polling in the RLC-Config IE.

Proposal 1: For uplink, the gNB-CU sends the PDCP Discard Timer value and PSI based discard timer value (if configured) of a DRB to the gNB-DU so that gNB-DU can then configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

Proposal 2: For downlink, RAN3 discusses the signalling enhancement between CU and DU to support timely retransmission.

Unnecessary RLC retransmission avoidance

Observation 4: The exiting ‘discard’ related IEs the DL USER DATA frame cannot be reused directly for unnecessary RLC retransmission avoidance.

Proposal 3: Introduce a new IE in the DL USER DATA frame specifically indicates the PDCP PDU associated with an RLC SDU, or a segment of the RLC SDU, for which the transmission and retransmission should be stopped.

- 4. Reference

- R2-2502089	Draft runnnig RRC CR for R19 XR	Huawei, HiSilicon

- 5. Text Proposal

Text Proposal to TS 38.473

<<<<<<<<<<<<<<<change start>>>>>>>>>>>>>

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

<<<<<<<<<unaffected text is skipped>>>>>>>>>

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PDCP Discard Timers IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to set the parameters for uplink timely retransmission configuration as specified in TS 38.331 [8].

If the Autonomous ReTx Threshold IE is included in the DRB Setup List IE, the gNB-CU shall, if supported, use it to trigger autonomous retransmission for a PDCP PDU of the DRB as specified in TS 38.401 [4].

If the Enhanced Polling Threshold IE is included in the DRB Setup List IE, the gNB-CU shall, if supported, use it to trigger enhanced polling for the DRB as specified in TS 38.401 [4].

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

<<<<<<<<<unaffected text is skipped>>>>>>>>>

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| <<<<<<<<<unaffected text is skipped>>>>>>>>> |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PDCP Discard Timers |  |  | 9.3.1.xxx |  | YES | ignore |
| <<<<<<<<<unaffected text is skipped>>>>>>>>> |  |  |  |  |  |  |

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>

9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| <<<<<<<<<unaffected text is skipped>>>>>>>>> |  |  |  |  |  |  |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| >>Autonomous ReTx Threshold | O |  | INTEGER (1..64, ...) | Corresponds to information provided in the autonomousReTxTreshold in the RLC-Config IE as defined in TS 38.331 [8]. | YES | ignore |
| >>Enhanced Polling Threshold | O |  | INTEGER (1..64, ...) | Corresponds to information provided in the enhancedPollingTheshold in the RLC-Config IE as defined in TS 38.331 [8]. | YES | ignore |
| <<<<<<<<<unaffected text is skipped>>>>>>>>> |  |  |  |  |  |  |

<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>

9.3.1.xxx PDCP Discard Timers

This IE contains PDCP discard timers that are configured for a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Discard Timer | O |  | ENUMERATED (10, 20, 30, 40, 50, 60, 75, 100, 150, 200, 250, 300, 500, 750, 1500, Infinity) | Indicates the PDCP discard timer. The values are expressed in ms. Corresponds to information provided in the discardTimer contained in the PDCP-Config IE as defined in TS 38.331 [8]. |
| Discard Timer Extended | O |  | ENUMERATED (0.5, 1, 2, 4, 6, 8, …, 2000) | Indicates the PDCP discard timer. The values are expressed in ms. Corresponds to information provided in the DiscardTimerExt-r16 or the DiscardTimerExt2-r17 contained in the PDCP-Config IE as defined in TS 38.331 [8]. |
| PSI based Discard Timer | O |  | ENUMERATED (0, 2, 4, 6, 8, 10, 12, 14, 18, 22, 26, 30, 40, 50, 75, 100, …) | Corresponds to information provided in the discardTimerForLowImportance contained in the PDCP-Config IE as defined in TS 38.331 [8]. Unit: millisecond. |