3GPP TSG-RAN WG3 Meeting #128	R3-253341

Malta, MT, 19 – 23 May, 2025

Title:	Discussion on Subcarrier Spacing for IAB

Agenda Item:	9.2

Source:	Huawei, Qualcomm, Nokia, Nokia Shanghai Bell, CATT, Lenovo

Document for:	Discussion

1	Introduction

In Rel-17 IAB enhancement WI, the IAB Resource Coordination procedure over XnAP was supported. There are several IEs contained in the IAB RESOURCE COORDINATION REQUEST message and IAB RESOURCE COORDINATION RESPONSE message. We found that the subcarrier spacing in some IEs are not correctly captured in this specification. This paper provides more detailed analysis and proposes some solutions.

2	Discussion

The following excerption shows some related IEs in the IAB RESOURCE COORDINATION REQUEST message and IAB RESOURCE COORDINATION RESPONSE message.

----------------------------------------Excerpt from TS 38.423--------------------------------------------

9.2.2.95	gNB-DU Cell Resource Configuration

This IE contains the resource configuration of the cells served by a gNB-DU, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [40], clause 11.1.1). This IE is only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. |
| --------unrelated parts are skipped -------- |  |  |  |  |

-----------Next excerption ----------

9.2.2.96	IAB STC Info

This IE contains cell SSB Transmission Configuration (STC) information of an IAB-DU or an IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB STC-Info List |  | 1 |  |  |
| >IAB STC-Info Item |  | 1 ..<maxnoofIABSTCInfo> |  |  |
| >>SSB Frequency Info | M |  | INTEGER (0.. maxNRARFCN) | The SSB central frequency. |
| >>SSB Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …) | The SSB subcarrier spacing. |
| --------------------unrelated parts are skipped --------------------------- |  |  |  |  |

-----------Next excerption ----------

9.2.2.97	RB Set Configuration

This IE contains the configuration for up to M non-overlapping RB sets for a given gNB-DU cell, used for frequency domain resource allocation. This IE is only applicable if the gNB-DU is an IAB-DU. The maximum value of M is 8.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |
| --------------------unrelated parts are skipped --------------------------- |  |  |  |  |

-----------Next excerption ----------

GNB-DU-Cell-Resource-Configuration	::= SEQUENCE {

subcarrierSpacing				SSB-subcarrierSpacing,

dUFTransmissionPeriodicity		DUFTransmissionPeriodicity		OPTIONAL,

dUF-Slot-Config-List			DUF-Slot-Config-List			OPTIONAL,

hSNATransmissionPeriodicity		HSNATransmissionPeriodicity,

hNSASlotConfigList				HSNASlotConfigList				OPTIONAL,

rBsetConfiguration						RBsetConfiguration		OPTIONAL,

freqDomainHSNAconfiguration-List		FreqDomainHSNAconfiguration-List 	OPTIONAL,

nACellResourceConfigurationList			NACellResourceConfigurationList 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-Cell-Resource-Configuration-ExtIEs } } OPTIONAL,

...

}

-----------Next excerption ----------

IAB-STC-Info-List ::= 	SEQUENCE (SIZE(1..maxnoofIABSTCInfo)) OF IAB-STC-Info-Item

IAB-STC-Info-Item::=	SEQUENCE {

sSB-freqInfo						SSB-freqInfo,

sSB-subcarrierSpacing				SSB-subcarrierSpacing,

sSB-transmissionPeriodicity			SSB-transmissionPeriodicity,

sSB-transmissionTimingOffset		SSB-transmissionTimingOffset,

sSB-transmissionBitmap				SSB-transmissionBitmap,

iE-Extensions		ProtocolExtensionContainer { { IAB-STC-Info-Item-ExtIEs } } OPTIONAL,

...

}

-----------Next excerption ----------

RBsetConfiguration ::= SEQUENCE {

subcarrierSpacing 		SSB-subcarrierSpacing,

rBsetSize				ENUMERATED {rb2, rb4, rb8, rb16, rb32, rb64},

numberofRBSets			INTEGER(1.. maxnoofRBsetsPerCell),

iE-Extensions				ProtocolExtensionContainer { { RBsetConfiguration-ExtIEs} } OPTIONAL,

...

}

-----------Next excerption ----------

SSB-subcarrierSpacing ::= ENUMERATED {kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, ...}

----------------------------------------End of excerption from TS 38.423--------------------------------------------

Both the gNB-DU Cell Resource Configuration IE and the RB Set Configuration IE contain the Subcarrier Spacing IE which includes a value 60KHz. But in ASN.1 part, the subcarrierspacing in GNB-DU-Cell-Resource-Configuration and RBsetConfiguration refer to the SSB-subcarrierSpacing which does not contain the 60kHz. Meanwhile, we also noticed that the Subcarrier Spacing IE contained in the IAB STC Info IE also use the the SSB-subcarrierSpacing in ASN.1 part, but such IE is correctly used there.

Observation 1: In ASN.1 part, the value of subcarrierspacing in GNB-DU-Cell-Resource-Configuration and RBsetConfiguration are not align with the tabular.

To solve the issue, there are some candidate solutions.

- Option 1: (BC change) Only remove 60kHz from the tabular.

In this solution, the 60kHz will be removed for the Subcarrier Spacing IE in the gNB-DU Cell Resource Configuration IE and the RB Set Configuration IE. Such change is Backward Compatible. However, the gNB-DU Cell Resource Configuration IE and the RB Set Configuration IE are also included in TS 38.473 (F1AP specification), and the value are determined by RAN1. Option 1 will cause misalignment between F1AP and XnAP.

- Option 2: (BC change) Adding 60khz in a new IE in both tabular and ASN.1, and ignore the previous Subcarrier Spacing if this new IE is present.

In this solution, the change is backward compatible. For example, the following change is needed for the RB set Configuration IE. The change to tabular is shown below, the ASN.1 change to add the new Extended Subcarrier Spacing IE is also needed.

----------------------------------------Example for the tabular change of option 2--------------------------------------------

9.2.2.97	RB Set Configuration

This IE contains the configuration for up to M non-overlapping RB sets for a given gNB-DU cell, used for frequency domain resource allocation. This IE is only applicable if the gNB-DU is an IAB-DU. The maximum value of M is 8.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. This IE is ignored if the Extended Subcarrier Spacing IE is present. |
| RB Set Size | M |  | ENUMERATED (rb2, rb4, rb8, rb16, rb32, rb64) | Number of PRBs in each RB set. If the RB sets of IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth, the remaining RBs not part of an RB set configuration are considered as included in the last RB set. |
| Number of RB Sets | M |  | INTEGER(1.. maxnoofRBsetsPerCell) | Number of configured RB sets. The RB sets are contiguous and non-overlapping. If the NR Carrier List IE(9.2.2.63) is provided, the start RB index of the first RB set is the RB index of the lowest common RB with the SCS provided by RB Set Configuration IE, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for the IAB-DU cell. Otherwise, the lowest subcarrier of the start RB of the first RB set is aligned with point A for the IAB-DU cell. |
| Extended Subcarrier Spacing | O |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, …) |  |

----------------------------------------End of Example for the tabular change of option 2--------------------------------------------

- Option 3: (BC change) Update both the tabular and the ASN.1 part to replace the SSB-subcarrierSpacing in the two IEs with a new defined IE which includes the 60kHz.

9.2.2.95	gNB-DU Cell Resource Configuration

This IE contains the resource configuration of the cells served by a gNB-DU, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [40], clause 11.1.1). This IE is only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …, kHz60) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. |
| --------unrelated parts are skipped -------- |  |  |  |  |

-----------Next Change ----------

9.2.2.97	RB Set Configuration

This IE contains the configuration for up to M non-overlapping RB sets for a given gNB-DU cell, used for frequency domain resource allocation. This IE is only applicable if the gNB-DU is an IAB-DU. The maximum value of M is 8.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …, kHz60) | Subcarrier spacing used as reference for the RB set configuration. |
| --------------------unrelated parts are skipped --------------------------- |  |  |  |  |

-----------Next Change ----------

GNB-DU-Cell-Resource-Configuration	::= SEQUENCE {

subcarrierSpacing				SubcarrierSpacing,

dUFTransmissionPeriodicity		DUFTransmissionPeriodicity		OPTIONAL,

dUF-Slot-Config-List			DUF-Slot-Config-List			OPTIONAL,

hSNATransmissionPeriodicity		HSNATransmissionPeriodicity,

hNSASlotConfigList				HSNASlotConfigList				OPTIONAL,

rBsetConfiguration						RBsetConfiguration		OPTIONAL,

freqDomainHSNAconfiguration-List		FreqDomainHSNAconfiguration-List 	OPTIONAL,

nACellResourceConfigurationList			NACellResourceConfigurationList 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-Cell-Resource-Configuration-ExtIEs } } OPTIONAL,

...

}

-----------Next Change ----------

RBsetConfiguration ::= SEQUENCE {

subcarrierSpacing 		SubcarrierSpacing,

rBsetSize				ENUMERATED {rb2, rb4, rb8, rb16, rb32, rb64},

numberofRBSets			INTEGER(1.. maxnoofRBsetsPerCell),

iE-Extensions				ProtocolExtensionContainer { { RBsetConfiguration-ExtIEs} } OPTIONAL,

...

}

-----------Next Change ----------

SubcarrierSpacing ::= ENUMERATED {kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, ..., kHz60} --or

SubcarrierSpacing ::= ENUMERATED {kHz15, kHz30, kHz120, kHz240, kHz60, spare2, spare1, ... }

In ASN.1, a new IE is defined for the subcarrier spacing in the gNB-DU Cell Resource Configuration IE and the RB Set Configuration IE, but in the backward compatible way, then the tabular also needs to be updated to align with ASN.1. In this solution, the change can be considered as backward compatible from protocol point of view, because the kHz60 is added either in the extension bits or using the spare bit.

- Option 4: (NBC change) Keep the tabular unchanged and update ASN.1 to replace the SSB-subcarrierSpacing in the two IEs with a new defined IE which includes the 60kHz.

In ASN.1, a new IE is defined for the subcarrier spacing in the gNB-DU Cell Resource Configuration IE and the RB Set Configuration IE, while the tabular is kept as it is.

GNB-DU-Cell-Resource-Configuration	::= SEQUENCE {

subcarrierSpacing				SubcarrierSpacing,

dUFTransmissionPeriodicity		DUFTransmissionPeriodicity		OPTIONAL,

dUF-Slot-Config-List			DUF-Slot-Config-List			OPTIONAL,

hSNATransmissionPeriodicity		HSNATransmissionPeriodicity,

hNSASlotConfigList				HSNASlotConfigList				OPTIONAL,

rBsetConfiguration						RBsetConfiguration		OPTIONAL,

freqDomainHSNAconfiguration-List		FreqDomainHSNAconfiguration-List 	OPTIONAL,

nACellResourceConfigurationList			NACellResourceConfigurationList 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-Cell-Resource-Configuration-ExtIEs } } OPTIONAL,

...

}

-----------Next Change ----------

RBsetConfiguration ::= SEQUENCE {

subcarrierSpacing 		SubcarrierSpacing,

rBsetSize				ENUMERATED {rb2, rb4, rb8, rb16, rb32, rb64},

numberofRBSets			INTEGER(1.. maxnoofRBsetsPerCell),

iE-Extensions				ProtocolExtensionContainer { { RBsetConfiguration-ExtIEs} } OPTIONAL,

...

}

-----------Next Change ----------

SubcarrierSpacing ::= ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, ...}

In this solution, the change is non-backward compatibal, but this solve the issue in a clean way.

Observation 2: Option 1 is BC, i.e., removing 60kHz from the tabular will cause misalignment between XnAP and F1AP.

Observation 3: Both Option 2 and Option 3 are BC, and option 2 requires more change than option 3.

Observation 4: Option 4 is the cleanest way but the change is NBC.

Both option 2 and option 3 are NBC, and option 2 has slightly more supporters based on offline coordination, so we provide CR for Option 2 in R3-253342 and propose

- RAN3 to select option 2 and agree the CR R3-253342.

3	Conclusion

In this paper, we discussed how to solve the issue for the missing value of subcarrier spacing in some IAB related IEs, and provide the following observations and proposals:

Observation 1: In ASN.1 part, the value of subcarrierspacing in GNB-DU-Cell-Resource-Configuration and RBsetConfiguration are not align with the tabular.

Observation 2: Option 1 is BC, i.e., removing 60kHz from the tabular will cause misalignment between XnAP and F1AP.

Observation 3: Both Option 2 and Option 3 are BC, and option 2 requires more change than option 3.

Observation 4: Option 4 is the cleanest way but the change is NBC.

- RAN3 to select option 2 and agree the CR R3-253342.