3GPP TSG-RAN WG3 Meeting #128	R3-253344

Malta, MT, 19 - 23 May 2025

Agenda Item:	12.2

Source:	Huawei

Title:	(TPs for WAB BL CRs) Architecture, Access Control and Additional ULI for WAB

Document for:	Discussion

1	Introduction

In the last meeting, the following agreements were achieved for the user location information report, Xn connection management and multi-hop avoidance of Rel-19 WAB [1].

For User location information:

RAN3 to agree the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures.

It is FFS whether the additional ULI shall be removed from NG UE associated procedures

For Xn connection management:

Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication.

It is understood that reception of the WAB-MT Identifier indicates that the sender is a WAB node. The receiver will behave accordingly.

For multi-hop avoidance at intial access:

Solution 1 is implementation specific and it is not subject to specifications changes.

There is no consensus in RAN3 on an agreement for solution 3 and make the decision in next meeting. To be continued...

In this paper, we further analyse some remaining issues for the scenario, architecture, access control and the user location information report for WAB.

2	Discussion

2.1	Scenario and architecture of WAB

2.1.1	Architecture: Management of Xn interface for WAB

The WID objective includes “Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs” [2]. So, there are two issues to be discussed.

2.1.1.1 How to manage Xn interface efficiently for WAB-gNB

The mobility of the WAB-node results in dynamic changes to the Xn interface of the WAB-gNB. The WAB-gNB may need to establish new Xn interface to the newly appeared neighbour gNBs frequently. One issue here is how can the WAB-gNB acquire the TNL address of the neighbour gNBs. In legacy scenario, the TNL address of the neighbour gNBs is either configured by OAM or obtained via the Xn-C TNL address discovery procedure. The frequent update to the Xn interface will result in large overhead for these interactions. Moreover, the delay of such interactions will be another challenging factor especially considering all the transmission should be via MT’s PDU session.

So, we need to think about a more efficient way for the WAB-gNB to establish Xn interface towards neighboring gNBs (including BH-gNB). As an alternative, the BH-gNB can provide assistant information about its own neighboring gNBs (e.g., including its neighbour gNBs' TNL information) to the WAB-node.

Proposal 1: The BH-gNB can provide the TNL information of neighbour gNBs to the WAB node.

2.1.1.2 How to avoid setting up Xn between WAB-gNBs

Another issue is how to avoid establishing Xn between two WAB-nodes. The essential thing is the WAB-node should be aware of the node type of another WAB-gNB, for either to determine not initializing Xn setup towards a WAB-gNB, or to reject the Xn setup in case the Xn setup request is received from a WAB-gNB. Since last RAN3 meeting already agreed that the WAB-MT Identifier included in the XnAP messages can be used for WAB-node indication, it is proposed:

Proposal 2: A WAB-node can reject the Xn Setup Request from another WAB-node, if the XN SETUP REQUEST message contains the WAB-MT Identifier.

2.1.2	Scenario: in-band or out-band deployment

- How to determine the inband/outband mode for WAB-node?

During SI phase, both in-band deployment and out-band deployment were considered for WAB, and the in-band scenario was not excluded by the WI. Thus, one issue here is how to determine the in-band/out-band operational mode for the WAB node. According to TR 38.799 [3], after the WAB-MT setup, OAM provides the WAB-gNB the initial configuration including the operating frequency during the WAB-gNB initialization phase. On this stage the OAM may determine the in-band/out-band mode and configure the WAB-gNB frequency accordingly. Alternatively, the OAM may preconfigure the in-band/out-band mode and the WAB-gNB frequency before the WAB-MT setup. It is proposed:

- Proposal 3: OAM configures the inband/outband mode to WAB-node.

- How to select the proper BH-gNB considering the inband/outband mode for WAB-node?

The in-band mode and out-band mode have different requirements for the BH-RAN.

For out-band, the collocated WAB-gNB and WAB-MT use different frequency band, so there is no mutal interference and no resource coordination is needed between the collocated WAB-gNB and WAB-MT.

But for in-band, on the contrary, the resources used by the collocated WAB-gNB and WAB-MT must be coordinated. The BH-gNB must be an upgraded gNB to be aware of which WAB-gNB and which WAB-MT are collocated, and support the resource multiplexing coordination.

Normally, the WAB-MT needs to select a BH-gNB first and then connects to OAM and get configuration from OAM, there is a risk that the first selected BH-gNB may be not be suitable to meet the configuration for this WAB node, because the OAM of WAB-node know nothing on the BH-gNB. So, it is worth to discuss how can the WAB-node select a proper BH-gNB. The following two options can be considered.

- Option 1: WAB-MT (re-)selects a proper BH-gNB based on the configuration from OAM and the knowledge about BH-gNB. The configuration from OAM includes e.g., the inband/outband mode, the working frequency band for WAB-gNB’s cell. The knowledge about BH-gNB includes e.g., the frequency band of served cell, whether supports resource multiplexing for WAB or not. This solution requires that the BH-gNB to broadcast whether it supports resource multiplexing for WAB, and, it may also require some change on the WAB-node’s integration procedure because the WAB-node may need reselect the BH-gNB after getting configuration. For example, as shown in the Figure 1, the WAB-MT first accesses the network using the legacy cell selection criterion. After the OAM configures the WAB-gNB frequency and the WAB node’s operation mode, the WAB node verifies whether the BH-gNB is proper. If the BH-gNB is inproper (e.g., OAM configures “outband” mode to the WAB-node and configure F1 as the frequency band of the WAB-gNB’s cell, but the WAB-MT’s serving cell also use F1), then the WAB node should detach and reselect another proper BH-gNB.

Figure 1: WAB-MT (re-)select a proper BH-gNB

- Option 2: OAM configures the WAB-gNB considering the knowledge about BH-gNB. In this option, the OAM configure the WAB-gNB’s working frequency based on the BH-gNB’s resource multiplexing capability and the frequency band of WAB-MT’s serving cell. For example, if the BH-gNB does not support resource multiplexing, and it use frequency band F1 to serve the WAB-MT, then the OAM will configure outband mode to the WAB-node and assign frequency band other than F1 to WAB-gNB. This solution requires that the WAB-gNB to report the knowledge about the BH-gNB to OAM, and it also requires the BH-gNB to broadcast whether it supports resource multiplexing coordination for WAB.

From the above analysis, both options require that the BH-gNB to broadcast whether it supports the resource multiplexing coordination for WAB. And both options have some standard impacts.  It is proposed:

Observation 1: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing.

Proposal 4: The BH-gNB should broadcast whether it supports resource multiplexing coordination for WAB.

Proposal 5: RAN3 to discuss the following two alternatives for the proper BH-gNB selection:

- Option 1: WAB-MT (re-)selects a proper BH-gNB based on the configuration from OAM and the knowledge about BH-gNB.

- Option 2: OAM configures the WAB-gNB considering the knowledge about BH-gNB.

2.1.3	Scenario: NTN backhaul

According to the NOTE 5 in the WID, NTN backhaul is considered between the WAB-MT and the BH-gNB. According to TS 23.501 [5] the UE’s 5QI is set to a fixed value (5QI=10) as long as the UE is served by NTN, with the PDB directly set to a really large value (1100ms).

Fig 1. Special QoS parameters for NTN UE in TS 23.501

In the legacy NTN scenario, according to the TS 38.413 [4], the gNB will provide the RAT Information which indicates the NR satellite RAT Type to AMF during the NG Setup procedure or the RAN Configuration Update procedure as follows.

-------------Excerpted from TS 38.413-------------

9.3.1.125	RAT Information

This IE provides RAT related information associated with a TAC, used as described in TS 23.501 [5].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Information | M |  | ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT) |  |

In the WAB scenario, if the backhaul link uses NTN, the UE’s CN is naturally unaware of that according to the current RAT information report as shown above, because the WAB-gNB itself is not an NTN gNB. But if the WAB-MT adopts NTN backhaul, the served UE’s QoS parameters cannot be normally configured but should be configured considering the bottleneck of NTN backhaul. The UE’s CN needs to know the backhaul is via NTN and configures the NTN related QoS parameters for UE’s QoS flow. Specifically, the WAB-gNB can inform the UE’s CN the backhaul link type is NTN, otherwise the WAB-MT’s CN informs that to the UE’s CN which may need interactions between the UE AMF and MT AMF. The latter solution requires the WAB-MT’s AMF to be aware of the UE’s AMF and interact with it. However, there is no existing interface between the WAB-MT’s AMF and UE’s AMF. Thus, notifying via WAB-MT’s CN is more complex than via WAB-gNB.

Therefore, it is proposed:

Proposal 6:	If the backhaul is NTN link, the WAB-gNB informs UE’s CN that the BH link RAT type is NTN.

2.1.4	NG interface management

As illustrated in the following excerpt from SA2’s LS on LCS [6], the AMF can infer that the target UE is served by a MWAB through the additional ULI, and then notify the LMF that the serving cell is an MWAB. In this case, the Additional ULI acts as an implicit WAB-gNB indicator.

---------Excerption from SA2 LS on LCS R3-250016---------

-	Additional ULI sent by the MWAB-gNB to the AMF in TS 38.413: when AMF is aware that the target UE is served by a MWAB based on the additional ULI, then "the AMF serving the target UE may indicate to the LMF that the serving cell is an MWAB (if applicable)".

Observation 2: SA2’s conclusion indicates that the Additional ULI can be used by the AMF as an implicit indication of WAB-gNB.

Noted that the ULI is reported after the UE attachment, the AMF cannot identify the WAB-gNB type during the NG setup procedure via the additional ULI. Some companies proposed to introduce the WAB indication in the NG SETUP REQUEST, but the motivation to allow the AMF know the gNB is WAB-gNB before any UE attaches still need further clarification.

If the motivation for adding WAB-gNB indication in NG setup Request is justified, an extra explicit WAB-gNB indication is not needed, since RAN3 already agreed to add the additional ULI in the NG SETUP REQUEST message.

Therefore, it is proposed:

Proposal 7: The motivation for adding WAB-gNB indication in during NG Setup procedure needs to be further clarified. If the motivation is justified, the additional ULI in the NG SETUP REQUEST message can be used as such WAB-gNB indication.

2.2	Access control of WAB

2.2.1	Prevent multi-hop access

According to the agreements made in the RAN3 #126 meeting, RAN3 reached a conclusion “the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling for handover”. A TP for 38.423 to capture such agreement is provided in Annex C.

Regarding the multi-hop prevention at initial access, we mainly focus on the following two solutions in previous meetings:

- Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs.

- Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

In RAN3 #127bis meeting, it is minuted in the chair notes as a common understanding that “Solution 1 is implementation specific and it is not subject to specifications changes”, and whether to support solution 3 is still marked as a “to be continued” issue.  So, we provide our views on the necessity of solution 3 as below.

Solution 1 is up to implementation but it is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space. If not, this solution will impact the existing network because if the WAB dedicated frequency band/PCI has been used by existing gNBs. So, Solution 1 is unfriendly for the operators with resource constraint and we still need a standardized solution, then the Solution 1 is out of scope and RAN3 should not discuss it.

Observation 3:  The usage of Solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space, then a standardized solution is needed.

For solution 3, WAB-gNB broadcasts a WAB-barred or WAB-cell indication in SIB1. The current 38.331 defines variety of barring IEs for the access control, e.g., cellBarredNTN, cellBarredNES, etc. It is conventional to introduce a new barring bit for the AS access control. This solution can avoid multi-hop topology at early stage (i.e., when the WAB-MT try to access the network), and it allows the BH-gNB which is lack of enough resources for backhaul to bar the WAB-MT.

It is proposed:

Proposal 8: For multi-hop avoidance during WAB-MT initial access, RAN3 to agree that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and send LS to RAN2 for coordination.

2.2.2	Prevent Self-access

- In the SA2’s LS S2-2407345, they mentioned the following 2 scenarios to be further discussed by RAN:

---------Excerption from S2-2407345----------

Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell)  for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

- In this section, we will discuss the case b, which was not included our reply LS to SA2 in R3-247909 [7]. First, since the WAB-gNB and WAB-MT should be coordinated for the transmission and receiving operation (e.g., TDM, FDM, SDM, etc), it is not possible for the WAB-MT to consider the cell served by its co-located WAB-gNB as the suitable cell when performing cell selection even if full-duplex is supported since some isolation/interference cancellation needs to be conducted. Otherwise, the transmitting of WAB-gNB and WAB-MT will cause severe interference to each other.

- Second, even if the WAB-MT incidently detects the signal of its co-located WAB-gNB, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation, e.g., the implementation can set restriction that a WAB-MT never access the cell configured to this WAB-node by OAM. On the other hand, the RAN-based broadcasting “WAB-support/WAB-barred” can also avoid such issue.

- Observation 4: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 9: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved because this can be avoided by proper setting/implementation, and send reply LS to SA2.

2.3	User location information

In the RAN3-127 meeting, RAN3 agreed that the WAB-gNB derives the new additional ULI and reports it to the network, if required by the CN via legacy procedures, in case the additional ULI has changed e.g. due to WAB-node movement.

And, in RAN3-127bis meeting, RAN3 agreed that “WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures.” Then, some companies propose that the additional ULI shall be removed from NG UE associated procedures, but this issue is marked as “to be continued” in the chair notes.  Since adding the additional ULI when reporting UE’s ULI is requested by SA2, RAN3 cannot made the decision to remove additional ULI from UE’s location information if without SA2’s input. Therefore, it is suggested that

Proposal 10: RAN3 to coordinate with SA2 to check whether the additional ULI shall be removed from UE’s user location information.

3	Conclusion

In this contribution we discuss the architecture and access control issues in the WAB scenario.

Scenario and architecture

Proposal 1: The BH-gNB can provide the TNL information of neighbour gNBs to the WAB node.

Proposal 2: A WAB-node can reject the Xn Setup Request from another WAB-node, if the XN SETUP REQUEST message contains the WAB-MT Identifier.

- Proposal 3: OAM configures the inband/outband mode to WAB-node.

Observation 1: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing.

Proposal 4: The BH-gNB should broadcast whether it supports resource multiplexing coordination for WAB.

Proposal 5: RAN3 to discuss the following two alternatives for the proper BH-gNB selection:

- Option 1: WAB-MT (re-)selects a proper BH-gNB based on the configuration from OAM and the knowledge about BH-gNB.

- Option 2: OAM configures the WAB-gNB considering the knowledge about BH-gNB.

Proposal 6: If the backhaul is NTN link, the WAB-gNB informs UE’s CN that the BH link RAT type is NTN.

Observation 2: SA2’s conclusion indicates that the Additional ULI can be used by the AMF as an implicit indication of WAB-gNB.

Proposal 7: The motivation for adding WAB-gNB indication in during NG Setup procedure needs to be further clarified. If the motivation is justified, the additional ULI in the NG SETUP REQUEST message can be used as such WAB-gNB indication.

Access control

Observation 3:  The usage of Solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space, then a standardized solution is needed.

Proposal 8: For multi-hop avoidance during WAB-MT initial access, RAN3 to agree that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and send LS to RAN2 for coordination.

- Observation 4: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 9: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved because this can be avoided by proper setting/implementation, and send reply LS to SA2.

User location information

Proposal 10: RAN3 to coordinate with SA2 to check whether the additional ULI shall be removed from UE’s user location information.

The corresponding TPs accordingly are provided in Annex parts based on the analysis in this paper.

4	References

- RAN3 #127bis Chairman notes.

- RP-242359, New WID on additional topological enhancements for NR

- TR 38.799, Study on additional topological enhancements for NR, V19.0.0

- TS 38.413, NG Application Protocol (NGAP), Stage2, V18.3.0

- TS 23.501, System architecture for the 5G System (5GS), Stage 2, V18.5.0

- R3-250016, LS on Location service of UEs served by MWAB, SA2

- R3-247909, Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI), RAN3

Annex A:	TP for TS 38.401 BL CR

Start of Change

## X.2	Configuration of WAB-node

### X.2.1	General

The following configurations of the WAB-node may need to be updated as the node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB- node. Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

### X.2.2	IP address configuration for the WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The IP address(es) of the WAB-MT can be used for the backhauling of WAB-gNB’s NG, Xn and OAM traffic. In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the IP address(es) of the WAB-MT is used as outer IP address(es), and the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

### X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM. During the mobility of WAB-node, it can be reconfigured by the OAM, or it can remain unchanged in a certain geographical area. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

### X.2.4 	In-band/Out-band mode configuration

The in-band/out-band operation mode of the WAB-node is configurated by the OAM.

Next Change

### X.Y	Xn interface management

### X.Y.1 	General

A WAB-gNB can establish Xn connectivity with surrounding gNBs e.g. the BH-gNB serving the co-located WAB-MT.

A WAB-gNB may avoid to establish Xn connectivity with another WAB-gNB if it know the peer node is a WAB-gNB durting the Xn setup procedure.

### X.Y.2 	Neighboring discovery

The WAB-gNB may obtain the TNL information of neighbour gNBs from the BH-gNB.

Next Change

### X.Z	Backhaul link RAT type indication

The WAB-gNB should inform AMF that the BH link RAT type is NTN or TN during the NG setup procedure. The information of BH link RAT type can be considered by the AMF to configure the suitable QoS parameters for the served UE(s).

End of Change

Annex B:	TP for TS 38.413

Start of Change

8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message. If the IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and use it for further AMF selection for the IAB-MT.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

If the RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node. If the Extended RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF. If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the NG SETUP REQUEST message, the AMF shall take it into account for paging.

If the RAT Information IE is included in the NG SETUP REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the NID IE within the NPN Support IE is included within a PLMN Support Item IE in the NG SETUP RESPONSE message, the NG-RAN node shall consider that the AMF supports the SNPN identified by the PLMN Identity IE and the NID IE.

If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

If the Additional ULI IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, store this information, and take it into account for determining the location of the UEs served by the NG-RAN node, as specified in TS 23.501 [9].

If the BH Link RAT Type IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, consider that the NG-RAN node is a WAB-gNB, and take it into account when configure QoS parameters for UEs served by the NG-RAN node.

Next Change

8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-    if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the Additional ULI IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, store this information, and take it into account for determining the location of the UEs served by the NG-RAN node, as specified in TS 23.501 [9].

If the BH Link RAT Type IE is included in the RAN CONFIGURATION UPDATE message, the AMF may take it into account when configure QoS parameters for UEs served by the NG-RAN node.

Next Change

9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Additional ULI | O |  | 9.3.1.X |  | YES | ignore |
| BH Link RAT Type | O |  | ENUMERATED (NTN,TN, ...) | This IE applies only if the NG-RAN node is a WAB node. Indicates the RAT type of the backhaul link of the WAB node. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

Next Change

9.2.6.4	RAN CONFIGURATION UPDATE

This message is sent by the NG-RAN node to transfer updated application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 0..1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | O |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| Global RAN Node ID | O |  | 9.3.1.5 |  | YES | ignore |
| NG-RAN TNL Association to Remove List |  | 0..1 |  |  | YES | reject |
| >NG-RAN TNL Association to Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | - |  |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the NG-RAN node. | - |  |
| >>TNL Association Transport Layer Address at AMF | O |  | CP Transport Layer Information 9.3.2.6 | Transport layer address of the AMF. | - |  |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Additional ULI | O |  | 9.3.1.X |  | YES | ignore |
| BH Link RAT Type | O |  | ENUMERATED (NTN,TN, ...) | This IE applies only if the NG-RAN node is a WAB node. Indicates the RAT type of the backhaul link of the WAB node. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the NG-RAN node and the AMF. Value is 32. |

End of Change

Annex C:	TP for TS 38.423

Start of Change

8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message and the target NG-RAN node rejects the handover or a failure occurs during the Handover Preparation, the target NG-RAN node shall include the Requested Target Cell ID IE in the HANDOVER PREPARATION FAILURE message.

If the WAB specific S-NSSAI is included in the UE Context Information in the HANDOVER REQUEST message, and the target NG-RAN node does not support serving the WAB-node, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

Interactions with Handover Cancel procedure:

If there is no response from the target NG-RAN node to the HANDOVER REQUEST message before timer TXnRELOCprep expires in the source NG-RAN node, the source NG-RAN node should cancel the Handover Preparation procedure towards the target NG-RAN node by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source NG-RAN node shall ignore any HANDOVER REQUEST ACKNOWLEDGE or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure and remove any reference and release any resources related to the concerned Xn UE-associated signalling.

Next Change

8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

----- Unmodified part skipped -----

If the WAB-MT Identifier IE is included in the XN SETUP REQUEST message or in the XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, consider that the transmitting NG-RAN node is a WAB-gNB, and conclude that the WAB-MT identified by the WAB-MT Identifier IE is co-located with the transmitting NG-RAN node, if needed.

If the Neighbour NG-RAN Node TNL Address List IE is present in the XN SETUP REQUEST message, the NG-RAN node2 may use it for Neighbour discovery and Xn establishment accordingly.

If the Neighbour NG-RAN Node TNL Address List IE is present in the XN SETUP RESPONSE message, the NG-RAN node1 may use it for Neighbour discovery and Xn establishment accordingly.

Interactions with other procedures:

If the NG-RAN node1 receives a XN SETUP RESPONSE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding XN SETUP REQUEST message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

If the NG-RAN node1 receives a XN SETUP RESPONSE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding XN SETUP REQUEST message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

Next Change

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

----- Unmodified part skipped -----

If the TAI NSAG Support List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node shall, if supported, take this IE into account for slice aware cell reselection.

If the TAI Slice Unavailable Cell List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, take this IE into account to deduce slice resource allocation.

If the WAB-MT Identifier IE is included in the NG-RAN NODE CONFIGURATION UPDATE message or in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node shall, if supported, consider that the transmitting NG-RAN node is a WAB-gNB, and conclude that the WAB-MT identified by the WAB-MT Identifier IE is co-located with the transmitting NG-RAN node, if needed.

If the Neighbour NG-RAN Node TNL Address List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use it for Neighbour discovery and Xn establishment accordingly.

If the Neighbour NG-RAN Node TNL Address List IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 may use it for Neighbour discovery and Xn establishment accordingly.

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

Next Change

9.1.3.1	XN SETUP REQUEST

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| AMF Region Information | M |  | 9.2.3.83 | Contains a list of all the AMF Regions to which the NG-RAN node belongs. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the ng-eNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the ng-eNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >SFN Offset | O |  | 9.2.2.75 | Associated with the ECGI IE in the Served Cell Information E-UTRA IE | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Partial List Indicator NR | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells NR IE. | YES | ignore |
| Cell and Capacity Assistance Information NR | O |  | 9.2.2.41 | Contains NR cell related assistance information. | YES | ignore |
| Partial List Indicator E-UTRA | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells E-UTRA. | YES | ignore |
| Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| WAB-MT Identifier | O |  | FFS | Contains the identifier of the WAB-MT co-located with the WAB-gNB that sends this message, assigned by the WAB-MT’s BH-gNB. | YES | ignore |
| Neighbour NG-RAN Node TNL Address List | O | 0..<maxnoofNeighbourNG-RAN nodes> | 9.2.2.X | Contains a list of TNL address of the neighbour NG-RAN Node(s). | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

Next Change

9.1.3.2	XN SETUP RESPONSE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer application data for an Xn-C interface instance.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Global NG-RAN Node ID | M |  | 9.2.2.3 |  | YES | reject |
| TAI Support List | M |  | 9.2.3.20 | List of supported TAs and associated characteristics. | YES | reject |
| List of Served Cells NR |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the gNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 | This IE is not used in this version of the specification. | YES | ignore |
| List of Served Cells E-UTRA |  | 0 .. <maxnoofCellsinNG-RAN node> |  | Contains a list of cells served by the ng-eNB. If a partial list of cells is signalled, it contains at least one cell per carrier configured at the gNB | YES | reject |
| >Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >SFN Offset | O |  | 9.2.2.75 | Associated with the ECGI IE in the Served Cell Information E-UTRA IE | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| AMF Region Information | O |  | 9.2.3.83 | Contains a list of all the AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Partial List Indicator NR | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells NR IE. | YES | ignore |
| Cell and Capacity Assistance Information NR | O |  | 9.2.2.41 | Contains NR cell related assistance information. | YES | ignore |
| Partial List Indicator E-UTRA | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the List of Served Cells E-UTRA. | YES | ignore |
| Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| WAB-MT Identifier | O |  | FFS | Contains the identifier of the WAB-MT co-located with the WAB-gNB that sends this message, assigned by the WAB-MT’s BH-gNB. | YES | ignore |
| Neighbour NG-RAN Node TNL Address List | O | 0..<maxnoofNeighbourNG-RAN nodes> | 9.2.2.X | Contains a list of TNL address of the neighbour NG-RAN Node(s). | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

Next Change

9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |
| >>Served Cells To Update NR | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| >>Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >>TNL Association Usage | M |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >>TNL Association Usage | O |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value ‘0’ indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global Cell Identity 9.2.2.73 | Global Cell Identity of a cell that may replace all or part of the coverage of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| >>SSB Coverage Modification List |  | 0..1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value ‘0’ indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| WAB-MT Identifier | O |  | FFS | Contains the identifier of the WAB-MT co-located with theWAB-gNB that sends this message, assigned by the WAB-MT’s BH-gNB. | YES | ignore |
| Neighbour NG-RAN Node TNL Address List | O | 0..<maxnoofNeighbourNG-RAN nodes> | 9.2.2.X | Contains a list of TNL address of the neighbour NG-RAN Node(s). | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

Next Change

9.1.3.5	NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a neighbouring NG-RAN node to a peer node to acknowledge update of information for a TNL association.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Responding NodeType | M |  |  |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |
| >>Served E-UTRA Cells |  | 0 .. < maxnoofCellsinNG-RANnode> |  | Complete or limited list of cells served by an ng-eNB, if requested by NG-RAN node1. | YES | ignore |
| >>>Served Cell Information E-UTRA | M |  | 9.2.2.12 |  | – |  |
| >>>Neighbour Information NR | O |  | 9.2.2.13 | NR neighbours. | – |  |
| >>>Neighbour Information E-UTRA | O |  | 9.2.2.14 | E-UTRA neighbours | – |  |
| >>>SFN Offset | O |  | 9.2.2.75 | Associated with the ECGI IE in the Served Cell Information E-UTRA IE | YES | ignore |
| >>Partial List Indicator E-UTRA | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the Served E-UTRA Cells IE | YES | ignore |
| >>Cell and Capacity Assistance Information E-UTRA | O |  | 9.2.2.42 | Contains E-UTRA cell related assistance information. | YES | ignore |
| >gNB |  |  |  |  |  |  |
| >>Served NR Cells |  | 0 .. < maxnoofCellsinNG-RANnode> |  | Complete or limited list of cells served by a gNB, if requested by NG-RAN node1. | – |  |
| >>>Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >>>Neighbour Information NR | O |  | 9.2.2.13 | NR neighbours. | – |  |
| >>>Neighbour Information E-UTRA | O |  | 9.2.2.14 | E-UTRA neighbours | – |  |
| >>>Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| >>Partial List Indicator NR | O |  | Partial List Indicator 9.2.2.46 | Value “partial” indicates that a partial list of cells is included in the Served NR Cells IE | YES | ignore |
| >>Cell and Capacity Assistance Information NR | O |  | 9.2.2.41 | Contains NR cell related assistance information. | YES | ignore |
| TNLA Setup List |  | 0..1 |  |  | YES | ignore |
| >TNLA Setup Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information as received from NG-RAN node1 | – |  |
| TNLA Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >TNLA Failed To Setup Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information as received from NG-RAN node1 | – |  |
| >>Cause | M |  | 9.2.3.2 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal |  |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| WAB-MT Identifier | O |  | FFS | Contains the identifier of the WAB-MT co-located with the WAB-gNB that sends this message, assigned by the WAB-MT’s BH-gNB. | YES | ignore |
| Neighbour NG-RAN Node TNL Address List | O | 0..<maxnoofNeighbourNG-RAN nodes> | 9.2.2.X | Contains a list of TNL address of the neighbour NG-RAN Node(s). | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNGRANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between NG-RAN nodes. Value is 32. |
| maxnoofNeighbourNG-RAN nodes | Maximum no. of neighbour NG-RAN nodes. Value is 256. |

Next Change

9.2.2.X	Neighbour NG-RAN Node TNL Address List

This IE is defined to contain a list of transport layer address of neighbour NG-RAN Node(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour NG-RAN Node TNL Address List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  |
| > Transport Layer Address | M |  | 9.2.3.29 |  |

End of Change

9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission, SSB not Available, LTM Triggered, No Backhaul Resource, mIAB-node not authorized, IAB not Authorized, WAB not supported) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

End of Change

Annex D:	Reply LS to SA2

3GPP TSG-RAN3 Meeting #128	R3-xxxxxx

Malta, MT, 19 - 23 May 2025

Title:	[DRAFT] Reply LS on access restriction and additional ULI of WAB

Response to:	S2-2407345/ R3-244019

Release:	Rel-19

Work Item:	VMR_Ph2, NR_WAB_5GFemto-Core

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 thanks SA2 for the LS on FS_VMR_Ph2 (S2-2407345/R3-244019), and for the Issue-1, RAN3 only provide partially reply for issue 1a in R3-247909.

RAN3 further discussed the RAN based approach for preventing multi-hop backhauling and self-acessing at the WAB-MT’s initial access, and provides the following further replies to SA2.

Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3 further Reply to Issue-1:

- RAN3 agree the following RAN-based solution to prevent multi-hop backhauling at the WAB-MT’s initial access: WAB-gNB cells broadcast a new indicator in SIB to bar the WAB-MT.

- For self-access prevention, RAN3 do not think the “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is an issue to be solved, because this can be avoided by proper seting/implementation.

One more issue for additional ULI:

For additional ULI, RAN3 first agreed in RAN3#127 meeting that the additional ULI to be added in UE’s User location information, which will be reported to AMF in UE associated messages. Then, in RAN3 #127bis meeting, RAN3 agreed that the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures, which are non-UE associated procedures. RAN3 would like to ask SA2 to clarify whether the additional ULI in UE’s User location information is needed.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to take the above feedback for access control into account, and provide feedback on the issue for additional ULI.

3. Date of Next RAN3 Meetings:

RAN3#129			 25th August – 29th August, 2025		India, IN

RAN3#129-bis		13th October – 17th October, 2025			Prague, CZ

Annex E:	LS to RAN2

3GPP TSG-RAN3 Meeting #128	R3-xxxxxx

Malta, MT, 19 - 23 May 2025

Title:	[DRAFT] LS on Multi-hop Avoidance for WAB-MT Initial Access

Response to:

Release:	Rel-19

Work Item:	VMR_Ph2, NR_WAB_5GFemto-Core

Source:	Huawei [to be: RAN3]

To:	RAN2

Cc:

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 discussed the enhancement for avoiding multi-hop backhauling during WAB-MT initial access. To support this, RAN3 agreed that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT from selecting such cells.

2. Actions:

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above into account and update the spec to support this feature.

3. Date of Next RAN3 Meetings:

RAN3#129			 25th August – 29th August, 2025		India, IN

RAN3#129-bis		13th October – 17th October, 2025			Prague, CZ