3GPP TSG-RAN WG3 Meeting #128	R3-253347

Malta, MT, 19 - 23 May 2025

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for Femto BL CR for TS 38.300) Security related issues for NR Femto

Document for:	Discussion

1	Introduction

In previous meeting, RAN3 sent LS R3-250822/S3-25122 to SA3 [1] to ask SA3 on the necessity of verification for the node ID and CAG ID of NR femto node. SA3’s response, provided in LS S3-251668 [2] and received at this meeting, confirmed the verification roles of the NR Femto GW and the AMF.

This contribution discusses the stage 2 impact based on SA3’s clarification and provides in Annex a proposed TP for BL CR to TS 38.300, capturing the resulting verification responsibilities.

2	Discussion

According to the clarification provided in SA3 LS S3-251668, the verification responsibilities related to NR Femto identity, cell identity, and CAG ID shall be performed by the NR Femto Gateway (GW) if it is deployed. Otherwise, these responsibilities fall to the AMF. This clarification applies to key NGAP procedures such as:

- NGAP INITIAL UE MESSAGE (CAG ID verification),

- NGAP SETUP REQUEST (Femto identity verification),

- NGAP PWS RESTART INDICATION / PWS FAILURE INDICATION (Cell/gNB identity verification).

SA3’s position is summarized with the content as follws:

SA3 would like to thank RAN3 for their LS R3-250822/S3-251220 on security verification related to NR Femtos.

SA3 kindly asks RAN3 to consider following answers:

SA3 has agreed that all the verifications listed in the LS need to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed). SA3 agreed the attached CRs to update TS 33.545 to incorporate relevant verification requirements and mechanisms as well.

This confirms that the responsibility for security verification is architecture-dependent, and that such behaviour needs to be reflected in the stage 2 procedures of TS 38.300, to ensure consistency with the updated security mechanism in TS 33.545.

Therefore, it is proposed:

Proposal: RAN3 to capture the stage 2 verification responsibilities at the NR Femto GW and the AMF in TS 38.300.

The corresponding TP for the baseline CR to TS 38.300 is provided in Annex.

3	Conclusion

This paper provides analysis security verification for NR femto based on the LS from SA2, the following observation and proposal are obtained:

Proposal: RAN3 to capture the stage 2 verification responsibilities at the NR Femto GW and the AMF in TS 38.300.

4. Reference

- R3-250822/S3-25122. LS on security verification related to NR Femtos, RAN3.

- S3-251668. Reply LS on security verification related to NR Femtos, SA3.

Annex - TP for BL CR on TS 38.300

START OF CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[9]	3GPP TS 37.324: " E-UTRA and NR; Service Data Protocol (SDAP) specification".

[10]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[15]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[16]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[17]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[20]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer"

[21]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[22]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[23]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[24]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[25]	Void.

[26]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[27]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[28]	3GPP TS 24.501: "NR; Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[30]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[31]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[32]	3GPP TS 38.470: "NG-RAN; F1 application protocol (F1AP) ".

[33]	3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[34]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[35]	3GPP TS 38.101-2: "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[36]	3GPP TS 38.101-3: "User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[37]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[38]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[39]	3GPP TS 22.104 "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[41]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[42]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[43]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[44]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[45]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[46]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[47]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[48]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[49]	3GPP TS 28.541: "5G Network Resource Model (NRM)".

[50]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[51]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984".

[52]	3GPP TS 38.211: "NR; Physical channels and modulation".

[53]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS)".

[54]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[55]	3GPP TS 24.554: "Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) in 5G System (5GS) protocol".

[56]	3GPP TS 38.214: "Technical Specification Group Radio Access Network; NR; Physical layer procedures for data".

[57]	3GPP TR 38.835: "NR; Study on XR enhancements for NR".

[58]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[59]	3GPP TS 38.215: "NR; Physical layer measurements".

[60]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[61]	IETF RFC 9330: "Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[62]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)".

[63]	IETF RFC 9332: "Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[64]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[65]	3GPP TS 38.351: "NR; Sidelink Relay Adaptation Protocol (SRAP) Specification".

[xx]	3GPP TS 33.545: " Security aspects of NR Femto".

NEXT CHANGE

4.X.2	Functional Split

4.X.2.1	NR Femto

An NR Femto hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE: The NR Femto may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto shall only connect to a single NR Femto GW at one time:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE:	NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-	The TAC and PLMN ID(s) used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW:

-	AMF selection is based on the GUAMI or requested NSSAI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message.

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

4.X.2.2	NR Femto GW

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF and the NR Femto serving the UE, applying the following additional functions:

-	Terminating the UE Context Release request procedure if an explicit GW Context Release Indication is included.  In this case, the NR Femto GW releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by another NR Femto attached to it.

-	At UE context establishment (Initial Context Setup or NG Handover) the NR Femto GW sends to the NR Femto the serving AMF’s GUAMI as well as the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Path Switch, the NR Femto GW sends to the NR Femto the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	In case of INITIAL UE MESSAGE message, verifying, as defined in TS 33.545[xx], that the reported CAG ID list in NPN Access Information IE is valid for the indicated cell served by that NR Femto.

-	Terminating non-UE associated NGAP procedures towards the NR Femto and towards the AMF, applying the following additional functions:

-	In case of NG SETUP REQUEST message, verifying, as defined in TS 33.545[xx], that the Global gNB ID used by the NR Femto is valid.

- 	In case of NG PWS Restart Indication and PWS Failure Indication message, verifying, as defined in TS 33.545[xx], that the indicated cell identity is valid and replacing the gNB ID of the NR Femto with the NR Femto GW ID before sending the respective message to the AMF.

-	At Overload Start/Stop, the NR Femto GW should provide the NR Femto with the identities of the affected AMF node(s). The NR Femto uses the received information to identify the traffic and the AMF to which the overload indication applies. The NR Femto shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

-	Supporting TAC and PLMN ID used by the NR Femto.

-	Relaying the PATH SWITCH REQUEST message towards the AMF indicated by the GUAMI of the source AMF received from the NR Femto.

4.X.2.3	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto directly connected:

- 	verifying as TS 33.545[xx], that the Global gNB ID used by the NR Femto is valid when receiving the NG SETUP REQUEST message;

- 	verifying as TS 33.545[xx], that the reported CAG ID list in NPN Access Information IE is valid for the indicated cell served by that NR Femto;

- 	and verifying as TS 33.545[x], that the Global gNB ID used by the NR Femto is valid when receiving the PWS RESTART INDICATION message and the PWS FAILURE INDICATION message.

A TAI used in a NR Femto GW shall not be reused in another NR Femto GW.

END OF CHANGE