3GPP TSG-RAN WG3 Meeting #128	R3-253356

Malta, MT, 19 – 23 May, 2025

Title:	(TP for SON BLCR 38.401, 38.473) MRO for LTM

Agenda Item:	10.2

Source:	Huawei

Document for:	other

# 1	Introduction

The following was disussed in last RAN3 meeting

BFR shortly after successful LTM cell switch to the wrong beam

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)?

LTM failure due to outdated TA

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM.

RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell.

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

Network based or UE based solution for TA acquisition type

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3.

FFS for intra-DU case.

# 2	Discussion

## 2.1	TA and beam handling

We think that in order for network to determine the failure type, we need to know whether it was due to TA or beam selection. Therefore, we prefer to discuss these topics together. In our understanding the topic: “Network based or UE based solution for TA acquisition type” is not a separate problem, but part of this problem discussion.

The last RAN3 meeting has confirmed that the target DU should identify the BFR and then send the recovery info to source DU via CU. This is a near-failure scenario, but network based. Another similar near-failure case that was discussed was the almost TA failure case, which would also require information exchange from target gNB-DU to source gNB-DU.

Since we think that all cases above share the same general solution space – sending information from target to source – we think that it would be beneficial to discuss this as a common framework.

The first aspect to consider is where to put the analysis of the information. In the past we usually tried to make sure the node that uses the information to adjust the parameters also is in charge of the analysis. Therefore, we prefer to let the gNB-DU perform the analysis. In some cases, an intermediate node needs to do some analysis, e.g. to decide where to forward the information. And in some cases, the gNB-CU is in charge of the parameters and should to the analysis (e.g. candidate cells). The benefit of this principle is that the analysis is kept within node, thereby not requiring a standardised behaviour, which in turns give more freedom for implementation to select the best solution.

Next question is when to forward the information. The information may be related to information retrieved in the UE, i.e. in the RLF or SHR report. This is retrieved or forwarded to the gNB-CU who den forwards it to the specific gNB-DU as previously agreed. The important question is then whether the gNB-CU should wait for the UE reported information before sending it to the UE. We see no benefit for the gNB-CU to delay the information. In fact - the immediate indication may be helpful for the gNB-DU since he could perform actions needed to store required information while waiting for the RLF report to arrive.

There is also no obvious benefit of delaying the information in the gNB-CU.

RAN3 agree to send additional information from target gNB-DU to gNB-CU and further to the source gNB-DU without waiting for the RLF report.

Regarding the messages to be used, for the target gNB-DU to CU, the ACCESS SUCCESS or UL RRC MESSAGE TRANSFER messages seems reasonable to be re-used. We prefer to use the UL RRC MESSAGE TRANSFER .

Target gNB-DU sends additional information to gNB-CU in the UL RRC MESSAGE TRANSFER message.

For the information transfer from gNB-CU to source gNB-DU it may be possible to reuse the release message, but this is not always sent (e.g. in subsequent mobility cases). It may also be possible to use the ACCESS AND MOBILITY message which was designed with SON functionality in mind, but this is non-UE associated. Therefore we prefer to use DL RRC MESSAGE TRANSFER.

gNB-CU sends additional information to source gNB-DU in the DL RRC MESSAGE TRANSFER.

### 2.1.1	Failure cases

Then as expressed above, in order to determine the reason for the failure, it would be beneficial to distinguish between TA failures and failures to radio link quality. The RLF report is already including UE measurements, which gives an indication of the downlink radio quality. It may be difficult to determine whether a failure is due to TA or uplink radio quality solely based on this. Therefore, there has been proposals to include information from the recovery, namely information on the TA determined during the recovery and information on which beam was used during recovery.

Agree to send recovery information – TA and selected beam from target gNB-DU to source gNB-DU.

Following the discussion in the previous section, we propose to include this information in the UL RRC message transfer from target gNB-DU to CU and in DL RRC message transfer.

Include the recovery information in UL RRC message transfer and DL RRC message transfer.

If we try to merge this into the existing framework, the new scenario is an LTM-recovery in the target cell. But the current framework that we developed to describe the forwarding of RLF report does not include this case, since this specific scenario is excluded from the current failure cases. We previously proposed this as a separate failure case as a possible solution

- LTM –recovery in target cell: A failure occurs during LTM cell switch and the UE performs LTM cell recovery in the target cell.

But if we follow the solution above, this new information could be considered as a separate set of information and would therefore not be required to be included as one of the failure cases, but rather as a general recovery information. We may even choose to extend this further to into limit it to the case where target and source cell is the same, and include all case where recovery happens and consider this to be the solution for failure reporting without RLF report that has also been previously discussed and would enable to collect information also for Rel-18 UEs. The only remaining information not previously discussed for this to work would be to add the cell identity of the recovery cell.

Agree to include the recovery cell ID in the recovery information and apply this to all LTM recovery cases.

### 2.1.2	Near-failure cases

As agreed in last meeting, the recovery beam information is sent to gNB-CU and gNB-CU forwards it to source gNB-DU. As discussed above, we think this is can be done by UL RRC MESSAGE TRANSFER and ACCESS AND MOBILITY message.

But we would like to also discuss one additional possibility. Early UL synchronization is the key point to shorten the interruption time. To achieve the intended goal, it is highly dependent on how well the early UL synchronization performs to get the possibly correct TA for accessing the target cell. Whether and how badly the TA becomes invalid depends on the relative mobility and relative position between the UE and the target cell.

It would be possible for a clever DU to differentiate TA update for each UE based on additional information, e.g. UE history information or by analysing the rate of change of the TA used in the serving cell. In any case, the method for selecting how often to update the TA is up to implementation. Therefore, it would beneficial not only knowing when a TA failure occurs but also if the TA is almost outdated.

One simple way of doing this is to include this information from target gNB-DU to source gNB-DU immediately after cell switch. The source will analyse and calculate any required TA correction and could e.g. send the value of this TA to the source gNB.

To conclude, we think that it would be beneficial to collect near failure information consisting of BFR information and possibly also initial TA and send this to the source gNB-CU.

Collect near failure information consisting of BFR information and initial TA

Then following similar principles as in previous sections we propose:

Include the near failure information in UL RRC message transfer and DL RRC message transfer.

## 2.2 	C-RNTI inclusion

In last meeting we agreed that the CU identifies which node to send the information to. There was an agreement that the CU determine the failure type and send to the DU.

For failures shortly after successful handover, the source cell needs to identify the UE context based on the C-RNTI used in the source to perform the MRO analysis. It is assumed that the gNB-CU that supports this MRO feature will be able to store the association between the C-RNTI used in the target cell (last serving cell) and the corresponding C-RNTI used in the source cell. gNB-CU translates the C-RNTI in the target cell to the C-RNTI used in the source cell and convey it outside the RLF report when forwarding the information to the source gNB-DU.

In legacy MRO a similar solution was used for the same use case. The target gNB can store the C-RNTI coming in the HO preparation message and include it when sending the Handover report message. In order to facilitate this, the following stage2 text was added to 38.300:

If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure.

Without any specification text, there is nothing explaining when the C-RTNI should be included and therefore the intended functionality will not work, since implementations may choose to not include this information. Therefore, it is proposed to include a similar text in the stage2 for LTM. But since the behaviour is for F1AP, it makes more sense to capture this in 38.401 as previously proposed but not agreed in last meeting:

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell. In case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure, the gNB-CU shall include the C-RNTI used in the source cell, if available.

There were also other proposals to capture this in the semantics of stage3. This is also possible and may help the reader to understand the intention of the IE. But since semantics is not normative and since the description includes interaction between different interfaces, it is required to capture this in stage2 if we would like to follow the principles for standardisation.

Add stage2 text to specify when CRNTI should be included (see also the TP in the annex).

## 2.3	Near failure scenarios reported from UE

RAN2 has excluded the RACH-based access and HO interruption time as new trigger conditions for SHR. There is no doubt to reuse the timer related trigger, i.e. T310/T312/T304, similar to legacy L3 handover. Once the timer related trigger is satisfied, the network would better know whether the handover is a LTM cell switch or L3 handover, in order to adjust the parameter precisely and efficiently, e.g. whether it is the CU or DU to optimize accordingly. Therefore, we propose, an indication of LTM cell switch is introduced to SHR report.

An indicator of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

In last meeting, there is a discussion on the number of RLC re-transmission as a new trigger for SHR report. However, there is no RLC re-transmission for the first UL data transmission on the target cell when the UE is attempting to access the target cell, since RLC re-transmission at the UE side is triggered by the feedback from the network, i.e., RLC status report from the target cell. It is up to the UE implementation to retransmit in case the DCI received within T304. Therefore, it is not valuable to report the number of retransmissions, since this is not controlled by network.

Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Regarding the analysis of the SHR associated to an LTM mobility, there is also a role split between the source DU and target DU. Similar to the successful L3 handover analysis mechanism, the CU may forward to the DU which configures the timers causing the SHR to be generated. If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU (see also the TP in the annex).

# 3	Conclusion

This document proposes:

Proposal 1:	RAN3 agree to send additional information from target gNB-DU to gNB-CU and further to the source gNB-DU without waiting for the RLF report.

Proposal 2:	Target gNB-DU sends additional information to gNB-CU in the UL RRC MESSAGE TRANSFER message.

Proposal 3:	gNB-CU sends additional information to source gNB-DU in the DL RRC MESSAGE TRANSFER.

Proposal 4:	Agree to send recovery information – TA and selected beam from target gNB-DU to source gNB-DU.

Proposal 5:	Include the recovery information in UL RRC message transfer and DL RRC message transfer.

Proposal 6:	Agree to include the recovery cell ID in the recovery information and apply this to all LTM recovery cases.

Proposal 7:	Collect near failure information consisting of BFR information and initial TA

Proposal 8:	Include the near failure information in UL RRC message transfer and DL RRC message transfer.

Proposal 9:	Add stage2 text to specify when CRNTI should be included (see also the TP in the annex).

Proposal 10:	An indicator of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

Proposal 11:	Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Proposal 12:	If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU (see also the TP in the annex).

# Annex	TP for LTM for 38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell. In case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure, the gNB-CU shall include the C-RNTI used in the source cell, if available.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU may forward the SHR report to the source gNB-DU in case the SHR was triggered by source cell timer values (e.g. T310/T312) or the target gNB-DU in case the SHR was triggered by target cell timer values (e.g. T304).

In case of LTM recovery, the node where the UE recovers may send an indication to the gNB-CU in the UL RRC MESSAGE TRANSFER who will forward this information to the last serving gNB-DU in the in DL RRC MESSAGE TRANSFER.

In case of a successful LTM cell switch, the target gNB-DU may provide information to the to the gNB-CU in the UL RRC MESSAGE TRANSFER who will forward this information to the source gNB-DU in the in DL RRC MESSAGE TRANSFER.

# Annex	TP for LTM for 38.473

### 9.2.3	RRC Message Transfer messages

#### 9.2.3.2	DL RRC MESSAGE TRANSFER

This message is sent by the gNB-CU to transfer the layer 3 message to the gNB-DU over the F1 interface.

Direction: gNB-CU gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| old gNB-DU UE F1AP ID | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| Execute Duplication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| RRC-Container | M |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| UE Context not retrievable | O |  | ENUMERATED (true, ...) |  | YES | reject |
| Redirected RRC message | O |  | RRC-Container 9.3.1.6 | Includes the UL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| PLMN Assistance Info for Network Sharing | O |  | PLMN Identity 9.3.1.14 |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the Remote UE’s SRB0 or SRB1. | YES | ignore |
| LTM recovery Information Item |  | 0..1 |  |  | YES | ignore |
| >RA based TA Value | M |  | INTEGER (0..4095) | Indicates the TA resulting from RA-based LTM execution. | - |  |
| >Recovery Cell | FFS |  | NR-CGI |  |  |  |
| >Recovery Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Recovery UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| LTM near-failure Information Item |  | 0..1 |  |  | YES | ignore |
| >TA value | M |  | INTEGER (0..4095) | Indicates the TA resulting from initial UL transmission in target cell. | - |  |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |

#### 9.2.3.3	UL RRC MESSAGE TRANSFER

This message is sent by the gNB-DU to transfer the layer 3 message to the gNB-CU over the F1 interface.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| RRC-Container | M |  | 9.3.1.6 | Includes the UL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. In case of CG-SDT, may include the UL-CCCH-Message message or UL-CCCH1-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| Selected PLMN ID | O |  | PLMN Identity 9.3.1.14 |  | YES | reject |
| New gNB-DU UE F1AP ID | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |
| LTM recovery Information Item |  | 0..1 |  |  | YES | ignore |
| >RA based TA Value | M |  | INTEGER (0..4095) | Indicates the TA resulting from RA-based LTM execution. | - |  |
| >Recovery Cell | FFS |  | NR-CGI |  |  |  |
| >Recovery Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Recovery UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| LTM near-failure Information Item |  | 0..1 |  |  | YES | ignore |
| >TA value | M |  | INTEGER (0..4095) | Indicates the TA resulting from initial UL transmission in target cell. | - |  |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |

[snip]

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

[snip]

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMRecoveryInformation,

LTMnearfailureInformation

[snip]

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMrecoveryInformation,

id-LTMnearfailureInformation,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

[snip]

-- **************************************************************

--

#### -- DL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DL RRC Message Transfer

--

-- **************************************************************

DLRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DLRRCMessageTransferIEs}},

...

}

DLRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-oldgNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE optional	}|

{ ID id-SRBID										CRITICALITY reject	TYPE SRBID											PRESENCE mandatory	}|

{ ID id-ExecuteDuplication							CRITICALITY ignore	TYPE ExecuteDuplication							PRESENCE optional}|

{ ID id-RRCContainer								CRITICALITY reject	TYPE RRCContainer									PRESENCE mandatory	}|

{ ID id-RAT-FrequencyPriorityInformation			CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation		PRESENCE optional	}|

{ ID id-RRCDeliveryStatusRequest					CRITICALITY ignore	TYPE RRCDeliveryStatusRequest					PRESENCE optional }|

{ ID id-UEContextNotRetrievable						CRITICALITY reject	TYPE UEContextNotRetrievable					PRESENCE optional }|

{ ID id-RedirectedRRCmessage						CRITICALITY reject	TYPE OCTET STRING									PRESENCE optional }|

{ ID id-PLMNAssistanceInfoForNetShar				CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-new-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex					CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex				PRESENCE optional }|

{ ID id-SRBMappingInfo								CRITICALITY ignore	TYPE UuRLCChannelID					 				PRESENCE optional }|

{ ID id-LTMRecoveryInformation						CRITICALITY ignore	TYPE LTMRecoveryInformation						PRESENCE optional }|

{ ID id-LTMNearFailureInformation					CRITICALITY ignore	TYPE LTMNearFailureInformation					PRESENCE optional },

...

}

-- **************************************************************

--

##### -- UL RRC Message Transfer

--

-- **************************************************************

ULRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ULRRCMessageTransferIEs}},

...

}

ULRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-SRBID							CRITICALITY reject	TYPE SRBID							PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY reject	TYPE RRCContainer					PRESENCE mandatory	}|

{ ID id-SelectedPLMNID					CRITICALITY reject	TYPE PLMN-Identity					PRESENCE optional		}|

{ ID id-new-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE optional		}|

{ ID id-LTMRecoveryInformation			CRITICALITY ignore	TYPE LTMRecoveryInformation			PRESENCE optional }|

{ ID id-LTMNearFailureInformation		CRITICALITY ignore	TYPE LTMNearFailureInformation		PRESENCE optional },

...

}

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

LTMNearFailureInformation ::= SEQUENCE {

tAvalue					INTEGER (0..4095),

jointorDLTCIStateID		JointorDLTCIStateID,

uLTCIStateID			ULTCIStateID 	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { LTMNearFailureInformation-ExtIEs } }		OPTIONAL,

...

}

LTMNearFailureInformation-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMRecoveryInformation ::= SEQUENCE {

rAbasedTAValue			INTEGER (0..4095),

recoveryCell			NRCGI,

recoveryJointorDLTCIStateID		JointorDLTCIStateID,

recoveryULTCIStateID			ULTCIStateID 	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { LTMRecoveryInformation-ExtIEs } }		OPTIONAL,

...

}

LTMRecoveryInformation-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules

[snip]

maxnoHopsMinusOne						INTEGER ::= 5

maxnoAggCombinations					INTEGER ::= 2

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC

id-LTMrecoveryInformation							ProtocolIE-ID ::= 998 --to be assigned by MCC

id-LTMnearfailureInformation						ProtocolIE-ID ::= 997 --to be assigned by MCC