3GPP TSG RAN WG3#128	R3-25XXXX

Malta, MT, 19 – 23 May, 2025

Agenda item:	17.3

Source: 	China Telecom

Title: 	On support on-demand SIB1

Document for:	Discussion and decision

- Introduction

In RAN3#127, the good progress for OD-SIB1 support was made and following agreements were achieved:

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

- One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

As a result, four CRs were agreed, i.e. R3-252467, R3-252468, R3-252469 and R3-252466. Based on these agreements, a new Class 1 message will be introduced with two indications i.e. “Start” and “Stop”. However, some companies have proposed extending this to include additional indications i.e. “pause/resume”, no consensus has been reached on this proposal yet. Additionally, there is an open issue regarding whether the new message should include a cell list of OD-SIB1 as suggested by some companies.

In this paper, we discuss on these two remaining issues and present our proposals.

- Discussion

2.1 Is the “pause/resume” mechanism necessary

During RAN3#127b meeting, it was agreed to introduce the “UL WUS Configuration Provision” message with two options i.e.

“Start”: Requests Cell A gNB to transmit UL WUS configuration information.

“Stop”: Requests Cell A gNB to stop transmitting UL WUS configuration information.

Based on that, some companies have propose extending this to include the “Pause” and “Resume” mechanism. The “Pause” option could indicate that the Cell A gNB stops broadcasting UL WUS configuration information while remaining the stored configuration. The “Resume” option opposite to “Pause” could indicate resuming Cell A gNB broadcasting UL WUS configuration information.

This “Pause/Resume” mechanism allows the UL WUS configuration information to be retained at the Cell A gNB, avoiding frequent retransmissions of the UL WUS configuration between the NES Cell gNB and Cell A gNB. This feature facilitates quicker on/off switching of the NES Cells.

However, considering that the OD-SIB1 operations for an NES Cell are semi-static and not expected to switch frequently, and that the Xn interface operates over a wired network with sufficient wideband, the transmission delay is not a limiting factor for NES Cell activation or deactivation. Therefore, supporting the “Pause/Resume” mechanism is not deemed necessary, and excluding it could improve the efficiency of NES Cell activation/deactivation.

Proposal 1: Unnecessary to support for the “Pause/Resume” mechanism.

2.2 Need for including the cell list in the new message

According to agreements reached in RAN1 and RAN2, if an NES Cell gNB manages multiple cells that are to be activated as NES Cells, it shall send a message (such as “UL WUS Configuration Provision Request)  to the Cell A gNB. This message contains multiple UL WUS configurations, each corresponding to one of the NES cells.

Upon receiving the message e.g. UL WUS Configuration Provision Request, the Cell A gNB obtains the UL WUS configurations of all the NES Cells. It then decides which of its own cells will broadcast these configurations in SIBx. The decision logic for selecting the broadcasting cells can leave to the implementation, Therefore, there is no need for Cell A gNB to response with a list of its own cells that will transit the UL WUS configuration.

Proposal 2:  Allow Cell A gNB to independently decide which of its cells will broadcast the UL WUS configurations. There is no need for it to reply with a list of broadcasting cells.

An issue is whether the Cell A gNB can choose to broadcast only a subset of the UL WUS configurations for the NES Cells or whether it must boradcast all the UL WUS configurations if it sends an ACK response to the NES Cell gNB.

If the Cell A gNB is allowed to broadcast only a portion of UL WUS configurations, then the response message should include a list of the NES Cells for which the UL WUS configurations have been accepted. This enables the NES Cell gNB to identify which NES Cells can transition to OD-SIB1 mode and, which cannot.

Alternatively, if the Cell A gNB is required to accept and broadcast all UL WUS configuration for the NES Cells in order to send an ACK, then there is no need to include the list of accepted NES Cells in the respond.

We prefer the latter approach: when the Cell A gNB sends an ACK to the NES Cell gNB, it implies that it accepts and will broadcast all the provided UL WUS configurations. Therefore, the list of accepted NES Cells does not need to be included in the respond.

Proposal 3: When the Cell A gNB sends an ACK response to the NES Cell gNB, it confirms acceptance of all the UL WUS configuration for the NES Cells. As a result, there is no need to include a list of accepted NES Cells in the respond message.

- Conclusion

Based on the above discussion regarding support for on-demand SIB1 operation, we have the following proposals:

Proposal 1: Unnecessary to support for the “Pause/Resume” mechanism.

Proposal 2:  Allow Cell A gNB to independently decide which of its cells will broadcast the UL WUS configurations. There is no need for it to reply with a list of broadcasting cells.

Proposal 3: When the Cell A gNB sends an ACK response to the NES Cell gNB, it confirms acceptance of all the UL WUS configuration for the NES Cells. As a result, there is no need to include a list of accepted NES Cells in the respond message.

- Reference

[1] RP-242354, New WID: Enhancements of network energy savings for NR, Ericsson.