3GPP TSG-RAN WG3 Meeting #128	R3-253372

Malta, MT, 19th – 23rd May 2025

Title:	(TP for BLCR to TS38.473) Support of Al/ML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Approval

1. Introduction

In this contribution, we present a TP to 38.473 for AI/ML enabled CCO, capturing the following proposals in R3-253370.

Proposal 1: For the receiving side, gNB-CU generates a recommend future CCO state and sends to gNB-DU as assistance information to derive future CCO state.

2. TP for AI/ML BLCR to TS38.473 (based on R3-252505)

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |
| Recommended Future Coverage Modification | O |  | 9.3.1.x | Indicates the recommended future coverage configuration generated for cells and beams. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	Recommended Future Coverage Modification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding recommended future coverage configuration generated by a gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Future Coverage Modification List |  | 1 |  |  |
| >Recommended Future Coverage Modification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Recommended Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the recommended future coverage configuration of the concerned cell. |
| >>Recommended Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Recommended Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Recommended Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the recommended future coverage configuration of the concerned SSB beams. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

### 9.4.5	Information Element Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- P

Predicted-CCO-Assistance-Information ::= SEQUENCE {

predicted-CCO-issue-detection	Predicted-CCO-issue-detection	OPTIONAL,

predictedAffectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

timeforPredictedCCOIssue		TimeforPredictedCCOIssue,	OPTIONAL,

recommended-future-Coverage-Modification		Recommended-Future-Coverage-Modification OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Predicted-CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

Predicted-CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Predicted-CCO-issue-detection	::=	ENUMERATED {coverage, cell-edge-capacity, ...}

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- R

Recommended-Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF Recommended-Future-Coverage-Modification-Item

Recommended-Future-Coverage-Modification-Item ::= SEQUENCE {

globalNG-RANCell-ID										GlobalCell-ID,

recommended-future-cellCoverageState		INTEGER (0..63, ...),

recommended-future-SSB-Coverage-Modification-List		Recommended-Future-SSB-Coverage-Modification-List		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Recommended-Future-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Recommended-Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Recommended-Future-SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF Recommended-Future-SSB-Coverage-Modification-Item

Recommended-Future-SSB-Coverage-Modification-Item ::= SEQUENCE {

sSBIndex									INTEGER(0..63),

recommended-future-SSBCoverageState			INTEGER (0..15, ...),

iE-Extension					ProtocolExtensionContainer { { Recommended-Future-SSB-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Recommended-Future-SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

id-Recommended-Future-Coverage-Modification			ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>