3GPP TSG-RAN3 Meeting #128	R3-253375

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	(Draft reply LS) Discussion on the liaisons from RAN2/SA2

Document for:	Discussion

1	Introduction

In this contribution, we’d like to discuss the following liaisons and provide the draft reply LS in the Annexes

- RAN2 LS about inventory response size

- SA2 LS about A-IoT area

- SA2 LS about removal of service type

2	RAN2 LS about inventory response size(R3-253031)

The following is the LS from RAN2 on on D2R message size for inventory response[1], RAN2 agreed not to support segmentation for inventory response and ask SA2 and RAN3 to consider support that inventory response size can always be provided by the core network to the reader.

RAN2 discussed segmentation of D2R messages and made the following assumption on inventory response message size. Agreements on segmentation - For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. RAN2 would like to inform SA2 and RAN3 that RAN2 assumes that the inventory response size can always be provided by the core network to the reader.

The purpose of the RAN2 requirement is to enable the gNB to allocate the appropriate resources for inventory response message to accommodate the device's response, by being aware of the device ID length. This ensures that segmentation of the device can be prevented. Currently, the inventory request provides two types of device identification: either a single device ID or filter information. As discussed in our paper [2], it is proposed to use a choice IE structure to signal these two types of device identification.

| IE/Group Name | Presence |
| --- | --- |
| CHOICE A-IoT Device Identification Information |  |
| >A-IoT Device ID |  |
| >>Device Permanent Identifier | M |
| >Filter Information |  |
| >>Filter Information | M |

In the case of a single device, the gNB/reader can determine the size of the D2R message for the inventory response by knowing the size of the device ID in the inventory request. However, if filter information is provided, there are two potential methods to ascertain the device ID length.

- Option 1 involves the CN indicating the device ID length in the inventory request, as shown below; this approach awaits a response from SA2.

| IE/Group Name | Presence |
| --- | --- |
| CHOICE A-IoT Device Identification Information |  |
| >A-IoT Device ID |  |
| >>Device Permanent Identifier | M |
| >Filter Information |  |
| >>Filter Information | M |
| >>Inventory Response Message Size | M |

- Option 2: The gNB/Reader should read the filter information, which may include the device ID component information. Based on the pre-configuration by OAM, it can ascertain the device ID length.

We believe option 1 is simpler and more straightforward.

The draft reply LS based on the above discussion is in the Annex.

Proposal 1, RAN3 reply RAN2 and SA2 the following:

RAN3 understand that the inventory response message size is determined by the Device ID length.

In case of single device identification, the inventory response size can be determined by knowing the size of the device ID in the inventory request.

In the case of filtering information, the inventory response size can be determined either by the gNB reading the filter information with appropriate OAM configuration or by the Inventory Response Message Size provided by the CN.

3	SA2 LS about A-IoT area (S2-2504288)

The following is the reply LS from SA2 on Ambient IoT progress of RAN3 (SA2) in S2-2504288[3], SA2 ask two questions about A-IoT area.

SA2 thanks RAN3 for the LS about the RAN3 progress related to Architecture and OAM aspects. SA2 would like to reply as following: - AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information. [SA2 reply]: - SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area. - SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

### Question 1, whether the “supported Area” is Tracking Area or a new type of area.

According to RAN3 agreement below, a new type of area is introduced for A-IoT service for both the requested area and the supported area. But RAN3 is still discussing the encoding. Our understanding that the A-IoT area should be encoded as geographical area, which can be represented by circle or polygon, as we discussed in [2].

RAN3#127-bis - Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. - The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

Proposal 2, Answer to question 1: “Supported Area” is a new type of area, and the encoding of the “A-IoT” area is geographical area.

### Question 2, one reader is only part of one supported Area or can be part of multiple supported Areas.

RAN2 is currently discussing issues and potential solutions related to parallel service requests. As per the discussion in [4], the problems of overlapping service requests revolves around conflicts from parallel intra-reader procedures, challenges in coordinating multiple readers, ambiguous device behavior during ongoing sessions, risks of deadlock affecting all devices, low resource efficiency, and compatibility requirements between Rel-19/20. It emphasizes adherence to physical layer timing constraints and advises against specifying device behavior in overlapping scenarios if unresolved.

It is understood that numerous issues will arise if overlapping service requests are supported. To prevent overlaps, the supported areas for different gNBs should be distinct, thereby eliminating the need for inter-gNB coordination. Within a gNB, if multiple service requests are received simultaneously, the gNB implementation can handle reader coordination to prevent overlaps at the A-IoT radio.

RAN3, on the other hand, has not yet discussed the relationship between the reader and the supported area. In our view, to avoid overlapping cases, it is preferable that one reader is associated with only one supported area, which can be defined either per reader or per gNB level.

Observation 1, From the perspective of RAN2, it indicates that supporting interleaved service requests would introduce numerous issues, complexities, and challenges.

Proposal 3, Answer to question 2: "The supported area can be defined per reader or per gNB. It should be noted that different gNBs or different readers have different supported areas to avoid interleaved service requests."

4	SA2 LS about service type (	R3-253016/S2-2504294)

The following is the reply LS from SA2 on the removal of service type information in S2-2504294[5], SA2 ask if RAN2/3 has concerns to remove “service type”

SA2 has been discussing whether “Service Type” (i.e., Inventory or Command) information in “Assistance Information” provided by the AIOTF to the NG-RAN is still needed, considering the fact that RAN3 has agreed to use separate NGAP procedures for Inventory and Command respectively. SA2 would like to ask RAN2 and RAN3 whether there is any concern if SA2 agrees to remove “Service Type” from “Assistance Information”.

The initial intention to know the service type for a reader is because different behaviors for inventory only and inventory+command request. According to RAN3 agreement below, RAN3 agree to introduce “follow-on command” in the inventory request, which indicate this is a “inventory+command” service, if this IE is not present, it indicates that this is “inventory-only” service, thus, it can be understood that this IE already support indication of “service type”, then,from RAN3 understanding, there’s no concern to remove “service type”

Proposal 4, RAN3 reply that there’s no concerns to remove the “service type” as RAN3 agrees to introduce “follow-on command” indication in the inventory request which can be used to indicate the service type.

5	Conclusion

In this contribution, we discussed the issues and questions in the LSs from RAN2 and SA2 and provide the draft reply LS in the Annexes.

### RAN2 LS about inventory response size

RAN3 understand that the inventory response message size is determined by the Device ID length.

In case of single device identification, the inventory response size can be determined by knowing the size of the device ID in the inventory request.

In the case of filtering information, the inventory response size can be determined either by the gNB reading the filter information with appropriate OAM configuration or by the Inventory Response Message Size provided by the CN.

### SA2 LS about A-IoT area

Proposal 2, Answer to question 1: “Supported Area” is a new type of area, and the encoding of the “A-IoT” area is geographical area.

Observation 1, From the perspective of RAN2, it indicates that supporting interleaved service requests would introduce numerous issues, complexities, and challenges.

Proposal 3, Answer to question 2: "The supported area can be defined per reader or per gNB. It should be noted that different gNBs or different readers have different supported areas to avoid interleaved service requests."

### SA2 LS about service type

Proposal 4, RAN3 reply that there’s no concerns to remove the “service type” as RAN3 agrees to introduce “follow-on command” indication in the inventory request which can be used to indicate the service type.

6	Reference

[1]	R3-253031/R2-2503199, LS on D2R message size for inventory response, RAN2

[2] R3-253374, (TP for 38.413) General aspects on support of AIoT, Xiaomi

[3]R3-253015 (S2-2504288), LS Reply on Ambient IoT progress of RAN3, SA2

[4] R2-2503480, Device behavior for parallel service requests, Xiaomi, Huawei, CMCC, Spreadtrum, CATT, Apple, China Telecom, Transsion Holdings, vivo, InterDigital, Philips International B.V., Qualcomm Incorporated

[5] R3-253016/S2-2504294 , LS on the removal of service type information, SA2

7	Annex A (Reply to RAN2 )

Title:	LS Reply on LS on D2R message size for inventory response

Response to:	R3-253031/R2-2503199

Release:	Release 19

Work Item:	Ambient_IoT_solutions

Source:	RAN3

To:	RAN2, SA2

Cc:

Contact person:	Lisi Li

lilisi@xiaomi.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks RAN2 LS on D2R message size for inventory response.

RAN3 understand that the inventory response message size is determined by the Device ID length.

In case of single device identification, the inventory response size can be determined by knowing the size of the device ID in the inventory request.

In the case of filtering information, the inventory response size can be determined either by the gNB reading the filter information with appropriate OAM configuration or by the Inventory Response Message Size provided by the CN.

2	Actions

To RAN2, SA2

ACTION: 	RAN3 respectfully requests RAN2 and SA2 to take above replies into account in future discussion.

3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129	25-28 Aug 2025			Bangalore, IN

TSG-RAN3 Meeting #129-bis	13-17 Oct 2025			Prague, CZ

8	Annex B (Reply to SA2 on A-IoT area )

Title:	LS Reply on Ambient IoT progress of RAN3

Response to:	R3-253015 (S2-2504288)

Release:	Release 19

Work Item:	Ambient_IoT_solutions

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact person:	Lisi Li

lilisi@xiaomi.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks SA2 LS on Ambient IoT progress of RAN3 and provides the following answers to the questions.

Question 1, SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area.

RAN3 reply: “Supported Area” is a new type of area, and the encoding of the “A-IoT” area is geographical area.

Question 2, SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

RAN3 reply:The supported area can be defined per reader or per gNB. It should be noted that different gNBs or different readers have different supported areas to avoid interleaved service requests.

2	Actions

To SA2

ACTION: 	RAN3 respectfully requests SA2 to take above replies into account in future discussion.

3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129	25-28 Aug 2025			Bangalore, IN

TSG-RAN3 Meeting #129-bis	   13-17 Oct 2025			Prague, CZ

9	Annex C (Reply to SA2 on service type )

Title:	LS Reply on LS on the removal of service type information

Response to:	R3-253016/S2-2504294

Release:	Release 19

Work Item:	Ambient_IoT_solutions

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact person:	Lisi Li

lilisi@xiaomi.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks SA2 LS on the removal of service type information.

Based on RAN3's understanding, there are no concerns regarding the removal of the "service type" since RAN3 agrees to introduce a "follow-on command" indicator in the inventory request, which can be used to indicate the service type.

2	Actions

To SA2

ACTION: 	RAN3 respectfully requests SA2 to take above replies into account in future discussion.

3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129	25-28 Aug 2025			Bangalore, IN

TSG-RAN3 Meeting #129-bis	   13-17 Oct 2025			Prague, CZ