3GPP TSG-RAN3 Meeting #128	R3-253380

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	20.2

Source:	Xiaomi, MediaTek, Qualcomm Incorporated, Huawei, Apple, China Mobile

Title:	(Draft LS out) Reply LS on signalling feasibility of dataset and parameter sharing

Document for:	other

1	Introduction

In Rel-19, two-sided AI/ML CSI compression for AI/ML Air Interface is under study, during RAN1 discussions, the sharing of model parameters and/or datasets from the network side to the UE or UE-side via standardized signaling was considered. RAN1 requested feedback from RAN2 on the feasibility of standardized signaling (over-the-air and/or other approaches) for the network side to share model parameters and/or datasets with the UE or UE-side.

In [2], RAN2 provided some high-level principles and potential solutions to support the network side's sharing of model parameters and/or datasets with the UE or UE-side. These solutions include OTA (over-the-air) solutions and non-OTA solutions. Before providing the final answer to RAN1, RAN2 needs the confirmation from RAN3/SA2/SA5 regarding the assumptions on non-OTA solutions.

In this paper, we aim to discuss the feasibility of non-OTA solutions from the RAN3 perspective and provide a draft LS reply in the Annex.

2	Discussion

## RAN3 discussion scope

Based on RAN2’s analysis in [2] below, the third option in the non-OTA approaches has RAN3 impacts, i.e., data transfer from gNB to CN/OAM, thus RAN3 needs to focus on confirming RAN2’s assumption on whether this option can be supported within Rel-19 existing architecture framework.

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1). From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption. Furthermore, it does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below. Table 1. non-OTA candidate solutions Option; Impacted WG; Specification impact/Implementation impact. OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity; SA5, SA3; Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion). CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity; SA2, SA3; Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2). gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity; RAN3, SA2, SA5, SA3; Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5).

It should also be noted how to request gNB to transfer the data to UE-side training entity is not in the LS discussion scope. RAN3 should focus on the feasibility of transfer dataset/model parameter from gNB to OAM/CN.

Proposal 1, RAN3 only focus on the feasibility of the data transfer from gNB to OAM/CN and confirm RAN2’s assumption on whether it can be supported within Rel-19 existing architecture framework.

## Considerations for transfer of the dataset/model parameters

Based on the background information in the LS and RAN1/2 discussions, the dataset and model parameters are provided to the UE or UE-side training entity, and the data are collected by the gNB side, the data can be per UE level or per cell/gNB level, which is up to RAN1/2 decision.

AN2 discussed the candidate solutions and feasibility of standardized signaling for NW-side sharing dataset/model parameter to UE or UE-side training entity for below options: - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

According to the principles outlined by RAN2, the dataset and model parameters should be readable and controllable by the network, with relaxed latency requirements. The size may typically be around hundreds of megabytes, all of which can affect how data is transferred from the gNB to the OAM/CN.

Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles: - A1 - Size: From RAN2 point of view, RAN2 aims to define a unified solution (e.g. OTA, non-OTA, or a combined) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs); - A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported; - A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server); - A4 - Latency: Relaxed latency requirement and infrequent update; - A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server). - A6: Proprietary information of the network and UE should be respected and not disclosed.

Observation 1, The dataset and model parameters can be per UE level or per-cell and gNB level, it is up to RAN1/RAN2 decision.

Observation 2: The following characteristics for data transfer should be considered within the scope of RAN3 discussions:

- Average data size in the hundreds of megabytes

- Network-controlled data transfer

- Relaxed latency requirements

- Readable by the UE-side training entity

## Feasibility analysis on data transfer from gNB to OAM/CN

In general, from the RAN3 perspective, the requirement is to transfer the AI data to OAM/CN. Although different options may have different RAN3 impacts (e.g. signaling impact on NG interface or interaction between gNB and OAM), which are pending further discussion in the normative phase if necessary, it is feasible to support data transfer from gNB to OAM/CN based on existing architecture from the RAN3 perspective.

| Non-OTA approaches | Feasibility | RAN3 impacts |
| --- | --- | --- |
| gNB->OAM | It is feasible to transfer the dataset/model parameters from the gNB to the OAM. The trace mechanism or a similar mechanism can be reused; for instance, the OAM can indicate the server ID (similar to a TCE-like ID) for data transfer. There’s no architecture impact. | For data transfer: Stage 2 impact For data request, it can be further discussed in normative phase if any |
| gNB->CN | It is feasible to support the transfer of dataset/model parameters from the gNB to the CN. The CN can then transfer the data to the UE-side training entity, which can be understood as a server, via the NEF, which belongs to the SA2 scope[3]. There’s no architecture impact. | Stage 2 and stage 3 impacts, details pending to the normative phase if any |

Observation 3, it is feasible to support data transfer from gNB to OAM/CN within existing architecture from the RAN3 perspective.

Observation 4, signalling enhancements may be needed with Rel-19 architecture framework, but the details are pending to further discussion.

Proposal 2, RAN3 reply to RAN2 and RAN1, cc SA2, SA5 that it is feasible to support data transfer from gNB to OAM/CN within existing architecture from the RAN3 perspective.

3	Conclusion

In this contribution, we discuss the RAN2 LS on the signaling feasibility of dataset and parameter sharing and provide the RAN3 discussion scope and analysis on the data transfer from gNB to OAM/CN, and a draft reply LS is also provided in the Annex, having the following observations and proposals:

Proposal 1, RAN3 only focus on the feasibility of the data transfer from gNB to OAM/CN and confirm RAN2’s assumption on whether it can be supported within Rel-19 existing architecture framework.

Observation 1, The dataset and model parameters can be per UE level or per-cell and gNB level, it is up to RAN1/RAN2 decision.

Observation 2: The following characteristics for data transfer should be considered within the scope of RAN3 discussions:

- Average data size in the hundreds of megabytes

- Network-controlled data transfer

- Relaxed latency requirements

- Readable by the UE-side training entity

Observation 3, it is feasible to support data transfer from gNB to OAM/CN from the RAN3 perspective.

Observation 4, signalling enhancements may be needed with Rel-19 architecture framework, but the details are pending to further discussion.

Proposal 2, RAN3 reply to RAN2 and RAN1, cc SA2, SA5, SA3 that it is feasible to support data transfer from gNB to OAM/CN from the RAN3 perspective.

4	Reference

[1]  R2-2500015 (R1-2410922), LS on signalling feasibility of dataset and parameter sharing, RAN1

[2]  R3-253010(R2-2503169), Reply LS on signalling feasibility of dataset and parameter sharing, RAN2

[3]  S2-2505025, Discussion on signalling feasibility of dataset and parameter sharing, Xiaomi

5	Draft LS reply

Title:	Reply LS on signalling feasibility of dataset and parameter sharing

Response to:	R3-253010 (R2-2503169)

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	RAN2, RAN1

Cc:	SA2, SA5, SA3

Contact person:	Lisi Li

lilisi@xiaomi.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks RAN2 LS on signalling feasibility of dataset and parameter sharing.

RAN3 considers it feasible to support the transfer of dataset/model parameters from gNB to OAM/CN in the non-OTA approach, with possible signaling enhancements within the existing Rel-19 architecture framework from RAN3's perspective.

2	Actions

To RAN2, RAN1

ACTION: 	RAN3 respectfully requests RAN2 and RAN1 to take above replies into account in future discussion.

3	Dates of next TSG RAN WG 3 meetings

TSG-RAN3 Meeting #129	25-28 Aug 2025			 Bangalore, IN

TSG-RAN3 Meeting #129-bis	13-17 Oct 2025			 Prague, CZ