3GPP TSG-RAN WG3 Meeting #128       	   				    R3-253384

Malta, MT, 19th – 23th May 2025

Agenda Item:		14.3

Source:			China Telecom

Title:	Further discussion on Support of NTN Regenerative payload

Document for:	 	Discussion

- Introduction

The Rel-19 NTN enhancements WI contains the following objective[1]:

Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we will continue to discuss the potential impact of supporting NTN regeneration payloads from RAN3 perspective.

- Discussion

In case of hard feeder link switch over, a satellite/gNB can only be connected to one NTN-GW at the same time. If 5GC does not know the hard feeder link switch over, the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW. The old NTN-GW has no way to deliver it to the gNB or forward it to the new NTN-GW, which will result in packet loss. Therefore, at the last RAN3 meeting, we agreed to introduce the Downlink NG Transmission Control IE to suspend and resume data and signaling procedures in the RAN Configuration Update procedure[2].

Observation 1: In case of hard FLSO, it is beneficial for the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update.

On the other hand, some companies suggest further check the functionality relationship between Downlink NG Transmission Control IE and NG suspend/resume procedure.

In our view, the application scenarios and problem-solving approaches of these two solutions are entirely distinct. The original intention of introducing NG suspend/resume procedure is to solve the mobility of NTN Regenerative payload / gNB. Up to now, RAN3 has introduced the NG removal procedure to address this problem. However, the benefits of the NG suspend/resume procedure remain unclear. First, it does not significantly save NG signaling overhead. Second, AMF is inefficient in managing a large number of NG interfaces, while only a very small number of NG interfaces are “active” at any timing duration. Due to the earth rotation, it may take several hours for the same NGSO satellite/gNB to return to the previous AMF service area. Maintaining these interfaces will result in large storage consumption of AMF and gNB. RAN3 has agreed that no new NGAP suspend/resume procedures in R19.

Observation 2: The new indicator introduced to solve NG Connection interruption due to hard feeder link switch over is unrelated to NG suspend/resume procedure.

Proposal 1: Remove the ENs related to hard FLSO from BL CRs.

Moreover, given the extensive coverage of the NTN cell, modifying the DL TNL address for a potentially large number of UEs is necessary during the feeder link switch over.This process is time-consuming, may lead to interruptions, and may trigger signaling storms in the RAN and CN. We propose RAN3 to study if any other optimization is required for hard feeder link switch scenario, e.g. using a non-UE associated NGAP procedure to update the DL TNL address of all connected UEs.

Figure 1: Feeder link switchover without AMF change

- The gNB onboard the satellite connects to 5GC via NTN-GW1. In case of hard feeder link switch over, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure.

2~4. The AMF informs the corresponding SMF(s) about the upcoming feeder link switchover receive from gNB. Based on feeder link switchover information, the SMF(s) controls the UPF(s) to buffer the DL data of the corresponding PDU sessions. It can be existing per UE procedures or new non-UE associated procedures, depending on further SA2 study.

- The gNB onboard the satellite performs Feeder link switchover to NTN-GW2. After the Feeder link is switched, the gNB is assumed to get a new TNLA.

- The gNB connects to AMF via NTN-GW2 by using the new TNLA as described in proposal 3.

7~8. The gNB needs to inform the UPF for the change of TNL address for all connected UEs, this can be performed by using existing procedures e.g. PDU Session Resource Modify Indication procedure or define new non-UE associated procedures.

Proposal 2: It is proposed to study if any other optimization is required for hard feeder link switch scenario, e.g. using a non-UE associated NGAP procedure to update the DL TNL address of all connected UEs.

- Conclusions

In this paper, we have the following proposal:

Observation 1: In case of hard FLSO, it is beneficial for the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update.

Observation 2: The new indicator introduced to solve NG Connection interruption due to hard feeder link switch over is unrelated to NG suspend/resume procedure.

Proposal 1: Remove the ENs related to hard FLSO from BL CRs.

Proposal 2: It is proposed to study if any other optimization is required for hard feeder link switch scenario, e.g. using a non-UE associated NGAP procedure to update the DL TNL address of all connected UEs.

- References

[1]	RP-241667, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.

[2]	R3-252409, (TP to BL CR for TS 38.413) Enhancement to support hard FLSO (Nokia, Nokia Shanghai Bell, Qualcomm, Xiaomi, China Telecom)

- TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN payloads.

16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links.

16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NOTE 1:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

NOTE 2:	In regenerative NTN payload, AMF is not changed for the UEs during the feeder link switchover.

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>