3GPP TSG-RAN WG3 Meeting #128       	   				    R3-253385

Malta, MT, 19th – 23th May 2025

Agenda Item:		15.2

Source:			China Telecom

Title:		Further discussion on support of S&F in IoT NTN

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 IoT NTN enhancements WI contains the following objective[1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

  - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Last RAN3 meeting has discussed the architecture of IoT NTN regenerative payload, RAN3 decides to work on Split MME architecture and Full CN on board architecture. In this contribution, we will continue to discuss the potential standard impacts on support of regenerative payload for IoT NTN.

- Discussion

2.1 Regenerative Architecture

In previous discussions, some companies have pointed out that multiple SCTP associations are currently not supported for E-UTRAN, so eNB IP address changes due to soft feeder link switch over are not currently supported for S1. One possibility would be to specify multiple SCTP associations for S1 in a similar manner as for NG, or to address it based on implementation. Considering the service delay tolerance of Store&Forward operations, IoT NTNs may not necessarily support soft feeder link switch over. Under the hard feeder link switch over, no enhancement is required for S1. In addition, SCTP multi-homing may also solve this problem.

Observation 1: The multiple SCTP associations is not necessarily supported in IoT NTN.

Proposal 1: No enhancements on multiple SCTP associations for IoT NTN in R19.

2.1 S&F operation

Based on the conclusion of the previous meeting, RAN3 has decided to prioritize the split MME architecture and the full CN onboard architecture. To align among other WGs and avoid duplication work, we suggest RAN3 only work on split MME and full CN on board architecture during S&F satellite operation.

Split MME on board

In legacy CIoT, TS 36.300 specifies both CP solution and UP solution. For CP solution, the UE data is transferred via NAS message and handled by MME. And for UP solution, the UE data is transferred via S1-U tunnels and handled by SGW.

In split MME architecture, for the CP solution, the S&F operation can be implemented by MME onboard and MME on ground, due to the MO/MT data is buffered in MME, which may not affect eNB. For the UP solution, since the SGW is on the ground, it may cause some impact on the S1-U. Currently, considering that SA2 may not support the UP solution for IoT NTN, it is expected that it will not have an impact on S1AP.

Proposal 2: For the split MME architecture, there may be no specification impact on S1AP.

Full CN on board

For the full CN on board architecture, including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Since all CN components are onboard, the UE attach procedure can be directly finished as legacy TN or NTN, and current MO/MT procedure can be reused. If multiple satellites are used, how to share UE context between satellites belong to SA2 scope, it seems have no impact to RAN3.

Proposal 3: For the full CN on board architecture, there may be no specification impact on S1AP.

- Conclusions

In this paper, we have the following proposals:

Observation 1: The multiple SCTP associations is not necessarily supported in IoT NTN.

Proposal 1: No enhancements on multiple SCTP associations for IoT NTN in R19.

Proposal 2: For the split MME architecture, there may be no specification impact on S1AP.

Proposal 3: For the full CN on board architecture, there may be no specification impact on S1AP..

- References

[1]	RP-243278, Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc.

[2]	TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3.