3GPP TSG-RAN WG3 Meeting #128       	   				    R3-253386

Malta, MT, 19th – 23th May 2025

Agenda Item:		16.2

Source:			China Telecom

Title:		Discussion on architecture and interface management for Ambient IoT

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 Ambient IoT WID contains the following objective[1]:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this contribution, we discuss the architecture aspects and interface management procedures based on the progress achieved in last RAN3 meeting.

- Discussion

A-IoT specific data is transported between the gNB and the A-IoT CN node takes place via the NG-C interface. Last RAN3 meeting, we agreed to include AIOTF information containers in NGAP, so the protocol stack can be represented by the following figure.

- Figure 1. Protocol Stack for Topology 1

In above figure, the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication to AIOTF) or the AMF (in case of indirect communication to AIOTF).

Proposal 1: Capture the figure of protocol stack for A-IoT Topology 1.

In addition, there were still some open issues from the last meeting.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

- It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

The new A-IoT Area will be introduced, which can be similar to the legacy TAI, e.g., an A-IoT Area Identity(PLMN ID + A-IoT Area Code), OAM can be used to configure the mapping between such A-IoT Area ID and geographic area between RAN and AIOTF. The definition of A-IoT area code needs to be coordinated with SA2/SA5.

Proposal 2: The A-IoT Area is encoded as new A-IoT Area ID.

- Enhancing the existing NGAP interface management procedures.

- FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

To support inventory and command procedures, A-IoT RAN information should be provided to AIOTF, including Reader List, Reader location, Supported A-IoT Areas. We slightly prefer to transmit the Supported A-IoT Areas and/or Reader List via NG Interface Management Procedures, Reader Location information can only rely on OAM.

Proposal 3: Introduce Supported A-IoT Areas and Reader List in NG Interface Management Procedure.

- Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed.

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1. Especially in indirect scenarios, if the gNB only supports A-IoT radio, there are some mandatory IEs not applicable to A-IoT in NG Setup Request message. One possible solution is to introduce a new indicator indicating that gNB supports A-IoT, and in such case the AMF/AIOTF will ignore such not applicable mandatory IE(s).

Proposal 4: AIoT indicator is used for AMF/AIOTF to ignore mandatory IEs in the NGAP Setup Request message.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Capture the figure of protocol stack for A-IoT Topology 1.

Proposal 2: The A-IoT Area is encoded as new A-IoT Area ID.

Proposal 3: Introduce Supported A-IoT Areas and Reader List in NG Interface Management Procedure.

Proposal 4: AIoT indicator is used for AMF/AIOTF to ignore mandatory IEs in the NGAP Setup Request message.

- References

[1] 	RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.