3GPP TSG-RAN WG3 Meeting #128	R3-253398

# Malta, MT, 19th – 23rd May 2025

Agenda item:	22.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the support of multi-hop Layer-2 UE-to-Network relay

Document for:	Discussion and Decision

# 1	Introduction

RAN WID ([1]) describes following RAN3 related objective:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay… - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching …

This contribution analyses the impact to RAN3.

# 2	Terminology

RAN2 has agreed the following terminology in the TS 38.300 CR ([1])

First U2N Relay UE: an Intermediate U2N Relay UE having both PC5 connection to a parent UE and PC5 connection to a U2N Remote UE for serving the U2N Remote UE in case of multi-hop L2 U2N Relay communication.

Intermediate U2N Relay UE: a U2N Relay UE having both PC5 connection to a parent UE and PC5 connection to a child UE or a U2N Remote UE for serving the U2N Remote UE in case of multi-hop L2 U2N Relay communication.

Last U2N Relay UE: a U2N Relay UE having both Uu connection to the network and PC5 connection to a child UE for serving a U2N Remote UE in case of L2 U2N Relay communication.

Current RAN3 TS 38.401 BL CR uses “U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE”. We propose to update RAN3 BL CR to align with terminology agreed by RAN2.

Proposal 1: Update TS 38.401 BL CR to use the terminology agreed by RAN2, and delete the first EN related to terminology.

TP for TS 38.401 BL CR can be found in ([2])

# 3	Control Plane procedures

Last RAN3 meeting agreed Stage-2 TP ([1]) for Initial Access procedure, with some EN. Most of the EN pends on RAN2 progress. We prefer to address the RAN3 related EN, more specifically following EN:

15.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the U2N Last Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

Editor’s Note: FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message.

Editor’s Note: FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

…

18.	The gNB-CU configures the U2N Last Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the U2N Last Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

The gNB-CU configures the First Relay UE and the U2N Intermediate Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First Relay UE and the U2N Intermediate Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet.

Editor’s Note: FFS whether Step 18 can be performed earlier, e.g., via Steps 6-13.

…

33.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the U2N Last Relay UE, and additional PC5 Relay RLC channels for the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the U2N First Relay UE, the U2N Intermediate Relay UE, and the U2N Last Relay UE.

Editor’s Note: FFS whether this step can be performed earlier.

For the 1st EN in Step 15

Editor’s Note: FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message.

In legacy release, the gNB-DU includes the gNB-DU UE F1AP ID of Relay UE and Remote UE Local ID in the INITIAL UL RRC MESSAGE TRANSFER message, to support the gNB-CU to identify the Remote UE under a specific U2N Relay UE. We think this mechanism can still be reused for multi-hop, where the multi-hop is built step by step (e.g. the intermediate Relay UE first act as Remote IE connect with the last Relay UE, then another intermediate Relay UE connects with the previous Intermediate Relay UE, etc). During this procedure, the gNB-CU can know the Relay UE (or the first Relay UE) that the Remote UE connected with. In addition, RAN2 agreed to support legacy single-hop U2N SRAP header, and The remote UE local ID is 8 bits. So we propose to remove this EN/FFS.

For the 2nd EN in Step 15,

Editor’s Note: FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

last RAN3 meeting agreed following:

The gNB-CU provides the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU.

WA: Reuse the Peer UE ID IE as counter part information.

The Multi-hop is build step by step, i.e. the intermediate Relay UE (act as Remote UE) first connect with the Last Relay UE. The gNB-DU can know this, e.g. via the Remote UE Local ID and RB ID conveyed in the SRAP header from the Last Relay UE. Then the First Relay UE (act as Remote UE) connect with the intermediate Relay UE. Via similar information, the gNB-DU can know this. Later, when the Remote UE connect with the First Relay UE. The gNB-DU can also know this via the similar information. No enhancement is needed. So this 2nd EN can be removed.

For the EN in Step 18,

Editor’s Note: FFS whether Step 18 can be performed earlier, e.g., via Steps 6-13.”

In Rel-17/18, it is up to the gNB-CU's implementation on when to configure the U2N Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. The gNB-CU can perform it earlier. The same mechanism should be used in Rel-19. So this EN can be changed to “Note”.

For the EN in Step 33, similar as legacy, it is up to the gNB-CU’s implementation. This step can be performed earlier. So this EN can be changed to “Note”.

Proposal 2: update TS 38.401 BL CR to fix the ENs.

TP for TS 38.401 BL CR can be found in ([2])

# 4	Protocol Stacks for multi-hop

RAN2 has agreed to add the new protocol stacks for multi-hop in (7). The new protocol stacks are copied as below:

Figure 16.12.2.1-3: User plane protocol stack for multi-hop L2 UE-to-Network Relay

Figure 16.12.2.1-4: Control plane protocol stack for multi-hop L2 UE-to-Network Relay

We propose to add following protocol stacks, and clarify the existing protocol stack is for single-hop in TS 38.401 BL CR.

Figure 6.1.6-x1: User plane protocol stack for multi-hop L2 UE-to-Network Relay

Figure 6.1.6-x2: Control plane protocol stack for multi-hop L2 UE-to-Network Relay

Proposal 3: add protocol stacks for multi-hop.

# 5	intra-gNB service continuity

Last RAN3 meeting agreed

For Scenario A, both intra-DU and inter-DU cases are supported by existing procedures.

There is no need to capture the procedure for Scenario A into TS 38.401.

For Scenario B, both intra-DU and inter-DU cases are supported by existing procedures.

There is no need to capture the procedure for Scenario B into TS 38.401.

For Scenarios C and D, RAN3 waits for RAN2 progress.

For Scenario A/B, Last RAN2 meeting agreed:

Agreements:

reconfigurationWithSync without sl-PathSwitchConfig can be reused to trigger Case A (Intra-gNB multi-hop indirect to direct path switching).

reconfigurationWithSync with sl-PathSwitchConfig can be reused to trigger Case B (Intra-gNB multi-hop indirect to single-hop indirect path switching).

For Scenario C/D, Last RAN2 meeting agreed:

Agreements:

RAN2 will not address cases of service continuity towards a relay path with intermediate relays in idle/inactive under the current WID.

The Path Switch Configuration may be enhanced by RAN2 for Scenario C/D. Since RAN2 is still studying it, RAN3 can wait for RAN2 progress.

Proposal 4: For Scenario C/D, RAN3 wait for RAN2 progress.

# 6	Conclusion

In this contribution, we discussed how to support multi-hop UE-to-Network Relay. Our proposals are:

Proposal 1: Update TS 38.401 BL CR to use the terminology agreed by RAN2, and delete the first EN related to terminology.

Proposal 2: update TS 38.401 BL CR to fix the ENs.

Proposal 3: add protocol stacks for multi-hop.

TP for TS 38.401 BL CR can be found in ([2])

Proposal 4: For Scenario C/D, RAN3 wait for RAN2 progress.

# References

- R2-2503088, Multi-hop U2N Relay in TS 38.300

- R3-253399, (TP to BL CR for TS 38.401) update on ENs and protocol stack for multi-hop architecture