3GPP TSG-RAN WG3 Meeting #128	R3-253431

# St Julian, Malta, 19th – 23rd May, 2025

Title: 	On packet loss metric in UE performance

Source: 	Ericsson, Jio Platforms, Deutsche Telekom, InterDigital

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss aspects related to the Packet Loss metric, which is part of UE performance measurements. The presented discussion is applicable to split and non-split architecture deployments for Release 19. RAN3 in Release-19 is discussing split architectures as a Release-18 leftover, i.e., specify functionality to bring parity between split and non-split architecture. However, earlier discussions have shown that there exist shortcomings in deriving meaningful feedback from the Packet Loss metric based on the Release-18 specification. In the discussion below we bring forward the different shortcomings and propose a way forward for Release-19.

In the context of Release 19 discussions, the following was captured in meeting notes from R3#127-bis:

  - A Rel18 correction to the Average Packet Loss DL IE may be made when agreements on whether and how to specify Average Packet Loss in Rel19 are taken.

# 2. Discussion

Obtaining feedback to actions derived via AI/ML is an important step in bridging the gap between an AI/ML model and the real-world system. In the context of AI/ML based use-cases such as mobility optimization, RAN3 specified a mechanism to collect UE performance after an AI/ML derived mobility action. The UE performance metrics include Average UE throughput DL, Average UE throughput UL, Average Packet Delay UL, Average packet delay DL, and Average Packet loss DL, which is collected by a target gNB after handover and is signalled back to the source gNB upon fulfilment of specified exit conditions.

The source gNB, after receiving the metrics in the UE performance, may take into account the performance at the target cell and use it to tune AI/ML derived mobility actions, e.g. prioritizing actions that maximize UE performance. Such process may lead to model re-training/updating. UE performance may also be used to collect input data for model training where feedback for handed over UEs is requested without the action being derived using AI/ML models.

## Issues with the Packet Loss metric

Our companion paper to AI 9.2 in R3-253474 discusses in detail the issues with the current (Release 18) definition of the packet loss metric over the Xn interface. However, below, we summarise the central issue with the definition of the Average Packet Loss DL IE in Release 18.

•	The Average Packet Loss DL IE references to the Packet Loss Rate IE, which has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric.

•	Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

•	The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G. As a consequence, the Average Packet Loss DL IE is not capable of representing the measured packet loss rate for a UE, as the minimum loss rate it can represent is 0.1%

•	Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information. Therefore, fixing the Packet Loss Rate IE to resolve the issues with the Average Packet Loss DL IE generates impacts on well-established functions such as QoS parameters exchange

Proposal 1: RAN3 to acknowledge the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

## Solutions

As shown in Sec. 2.1.1, both the granularity and the value range of the packet loss rate metric has to be extended (in relation to the Rel.18 definition) to be able to adequately represent a measured packet loss value to a neighbour node. Based on the definition of the QoS parameters for NR, the range of packet loss rates guaranteed by the different 5QIs ranges from 10-2 to 10-8, i.e., 0.01% to 0.00000001%. In reality, one may observe measured packet loss rate values that range between 10-1 to 10-10.

Proposal 2: The new definition of Packet loss rate for UE performance shall accommodate for a range of packet loss rate values from 10-10 to 10-1.

The second aspect of the packet loss metric (in relation to the Rel.18 definition) is the granularity of the steps in the signalled Packet loss rate values. As with any measurement, a higher measurement granularity means that the sender can communicate finer details of the measurement to the receiver. A granularity of 10-8 (0.00000001%) is sufficient enough, especially to accommodate for high-bandwidth services and services with strict packet loss requirements.

Proposal 3: The new definition of packet loss rate for UE performance shall accommodate a granularity that is detailed enough to use for applications with very low packet loss rates.

Based on the above proposed requirements, we show below how such a metric, i.e., packet loss rate for UE performance in Rel.19, may be defined. This IE will indicate the measured Packet Loss Rate for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Loss Rate for UE Performance | M |  | INTEGER (0..10000000, ...) | Ratio of lost packets per number of packets sent, expressed in units of 10-8 percent. |

Proposal 4: RAN3 to agree to the new definition for the Packet loss rate for UE performance metric.

## Measuring packet loss at PDCP vs other layers

In the general context of UE performance reporting for packet losses, a value derived at a higher protocol layer, i.e., the measurements on the PDCP level packet loss rate, is considered more relevant as it better reflects the performance at service-level. This is also better because it takes into account losses over the interfaces such as F1. Indeed, the packet loss at e.g. MAC level does not necessarily carry information about the quality of the service because a dropped MAC PDU may end up being successfully retransmitted with no negative effects on the service.

Furthermore, RAN3 made the following agreement at R3#127-bis:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

This new procedure on the E1 interface may be used to exchange the UE performance metrics collected at the PDCP layer. An LS was also sent to SA5 (R3-252436) enquiring about the feasibility of measuring Packet Loss Rate in UL and DL at PDCP level as well as of UE throughput measurement in UL and DL at PDCP level.

Proposal 5: RAN3 to agree to collection of packet loss rate measurements at the PDCP level and signal the collected measurements over the newly introduced Data Collection procedures.

A TP reflecting the main proposals in this paper is presented in R3-253432.

# 3. Conclusions

This paper makes the following observations and proposals:

Proposal 1: RAN3 to acknowledge the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

Proposal 2: The new definition of Packet loss rate for UE performance shall accommodate for a range of packet loss rate values from 10-10 to 10-1.

Proposal 3: The new definition of packet loss rate for UE performance shall accommodate a granularity that is detailed enough to use for applications with very low packet loss rates.

Proposal 4: RAN3 to agree to the new definition for the Packet loss rate for UE performance metric.

Proposal 5: RAN3 to agree to collection of packet loss rate measurements at the PDCP level and signal the collected measurements over the newly introduced Data Collection procedures.

A TP reflecting the main proposals in this paper is presented in R3-253432.