3GPP TSG RAN WG3 Meeting #128	R3-253433

St Julian, Malta, 19 - 23 May 2025

Title: 	On collection of delay measurements for UE performance

Source: 	Ericsson, Jio Platforms, InterDigital

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we further present arguments in context of the discussion in the preceding RAN3 meetings. During RAN3#127bis, the following agreements and open points were captured:

- Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

- For Packet delay, whether to reusing QoS monitoring mechanism or enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, periodically?

- Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

This paper focuses on the above open point and other aspects in the collection and signalling of delay measurements as part of UE performance reporting in split architecture.

# 2. Discussion

QoS in NR enables service differentiation and QoS monitoring provides observability into the delivered service performance across the 5GS. However, QoS monitoring is an optional feature that is enabled by the Application Function towards the core network, and the core subsequently indicates to the RAN to monitor the QoS of associated QoS flows. The request for QoS monitoring is then propagated across network entities and the corresponding data is collected and reported in a QoS Report.

In the context of UE performance, the request to collect metrics comes from a neighbour gNB, where the configuration to perform the UE performance collection is done as part of a Xn: Data Collection Reporting Initiation procedure and the request to monitor the UE is done as part of the Xn: Handover procedure. The difference between the UE performance case when compared to the QoS monitoring case is that the request to monitor a certain UE’s performance can be independent of a QoS monitoring request for the same UE/QoS flow. Even in cases where both QoS monitoring and UE performance co-exist, it may happen that the reporting periodicity may be different for each procedure. In such a case, the RAN node receiving either the QoS monitoring measurements or the UE performance measurements should be able to differentiate them, such that the RAN measurements for UE performance can be configured and signalled to gNB-CU-CP and the QoS monitoring measurements can be configured and signalled to the UPF.

At the last RAN3 meeting it was commented that reusing the QoS monitoring solution framework would not work because it is not possible for the gNB-DU to indicate to the gNB-CU whether the delay measurements can be admitted or not.
This was discussed also when QoS monitoring delay reporting was introduced. The reason why the gNB-DU does not indicate to the gNB-CU whether delay measurements can be admitted or not, both for QoS Monitoring as well as for the potential solution for UE performance feedback, is that delay measurements are signalled over GTP-U, and the main priority of GTP-U is to carry UP traffic. Of course, signalling delay measurements over GTP-U is possible and it works today, but there are cases where the gNB-DU may not be able to signal delay measurements as it has to give priority to UP tasks. This is why the gNB-DU does not signal an ACK/NACK to the gNB-CU for QoS monitoring reporting and this is also the reason why the gNB-DU shall not signal an ACK/NACK to the gNB-CU for delay reporting for UE Performance Monitoring. 
If the gNB-DU can report the delay measurements for UE performance monitoring, it will, but in some cases it might not.

Fundamentally, the chosen solution for delay reporting for UE Performance monitoring should satisfy the following:

- It should be possible for UE performance collection to be enabled independently of QoS monitoring

- It should be possible for UE performance collection and QoS monitoring to co-exist, with potentially different reporting periodicities

- The gNB-CU-UP receiving the measurements should be able to generate reports corresponding to QoS reporting or UE performance.

- The agreed principles and procedures should work for UE performance metrics collection in case of NR-DC, where feedback for SN terminated bearers is signalled by the SN to the MN

- The agreed principles and procedures shall facilitate obtaining/deriving either per DRB (to cover the NR-DC UE Performance case), per slice (to cover the per slice UE Performance case), or (legacy) per-UE UE performance metrics without additional complexity

Proposal 1: RAN3 to discuss solutions for UE performance according to the following principles:

- It should be possible for UE performance collection to be enabled independently of QoS monitoring

- It should be possible for UE performance collection and QoS monitoring to co-exist, with potentially different reporting periodicities

- The gNB-CU-UP receiving the measurements should be able to generate reports corresponding to QoS reporting or UE performance.

- The agreed principles and procedures should work for UE performance metrics collection in case of NR-DC, where feedback for SN terminated bearers is signalled by the SN to the MN

- The agreed principles and procedures shall facilitate obtaining/deriving either per DRB (to cover the NR-DC UE Performance case), per slice (to cover the per slice UE Performance case), or (legacy) per-UE UE performance metrics without additional complexity

#### 2.1 Mechanism to enable RAN-based metric collection for UE performance

In order to enable a RAN-based framework for the collection of delay measurements for UE performance, a similar mechanism as used for QoS monitoring is required, with the difference that the mechanism is controlled by RAN-based entities.

Below we propose a mechanism for RAN-based collection of UE performance metrics for split architecture. The trigger to collect UE performance is always associated to an incoming handover at the gNB-CU. Provided that the UE is admitted, and the handover executes without failures, the target gNB-CU sets up resources for the UE at the gNB-DU and the gNB-CU-UP.

Towards the gNB-DU, the initial setup of the DRBs corresponding to the UE is performed over the F1: UE CONTEXT SETUP procedure. If the DRB being setup corresponds to a UE for which UE performance is being requested by the source gNB, then the target gNB-CU is aware of this (via the corresponding configuration of UE performance reporting as part of the Xn: DATA COLLECTION REPORTING INITIATION and via the presence of the corresponding Data Collection ID in the Handover Request message). In order to make the gNB-DU aware of the need to gather UE performance metrics, the gNB-CU-CP may, as part of the F1: UE CONTEXT SETUP, indicate to the gNB-DU the need to monitor delay metrics for UE performance reasons in addition to the delay metrics reporting periodicity (note that we only mention delay metrics here as this paper discusses only delay measurements in UE performance), and an indication of the maximum duration for which the measurements shall be collected. The gNB-DU shall terminate the collection when at least one of the exit conditions is fulfilled, i.e., either the maximum measurement duration, or the UE moves to RRC_IDLE or RRC_INACTIVE, or the UE is handed over to another cell.

Below, we present the tabular of the UE CONTEXT SETUP message with the suggested enhancements.

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Unmodified IEs ignored |  |  |  |  |  |  |
| … |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Delay Monitoring for UE Performance Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated DRB. | YES | ignore |
| >>>>>Delay Monitoring for UE Performance Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Indicates the periodicity of reporting Delay metrics for UE performance | YES | ignore |
| >>>>>Delay Monitoring for UE Performance Duration | O |  | INTEGER (1.. 5000,…) | Time duration starting at successful UE context setup within which the UE performance measurements are collected. Unit: millisecond | YES | ignore |
| Unmodified IEs ignored |  |  |  |  |  |  |
| … |  |  |  |  |  |  |

The presence of the highlighted additional IEs triggers the gNB-DU to report delay metrics to the gNB-CU-UP with the specified reporting periodicity over the F1-U Transfer of Assistance Information procedure (as it is the case with QoS monitoring).

Proposal 2: Extend the F1-C UE CONTEXT SETUP REQUEST message with three IEs (Delay monitoring for UE performance, Reporting frequency for UE performance and Delay Monitoring for UE Performance Duration)

Similar to the discussion above, the gNB-CU-CP triggers the BEARER CONTEXT SETUP REQUEST message towards the gNB-CU-UP to setup the DRBs associated to the PDU Session. In order to make the gNB-CU-UP aware of the need to gather UE performance metrics and the need to aggregate and report the RAN delay metrics to the gNB-CU-CP, the gNB-CU-CP shall trigger (before triggering of the E1: Bearer Context Setup procedure) an E1: Data Collection Reporting Initiation procedure, where delay metrics for UE performance monitoring will be requested, possibly together with a reporting periodicity.

Of course, to indicate to the gNB-CU-UP for which UE delay metrics for UE performance is needed, the gNB-CU-CP will include a Data Collection ID in the BEARER CONTEXT SETUP REQUEST message, as shown below:

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | 9.3.1.67 |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to "UE". | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, ResumeforSDT) | Indicates the status of the Bearer Context. NOTE: This IE is not applicable to eNB-CP/eNB-UP. | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | M |  | DRB To Setup List E-UTRAN 9.3.3.1 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | Ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | M |  | 9.3.3.2 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 | Included whenever it is known by the gNB-CU-CP or by the ng-eNB-CU-CP | YES | ignore |
| Trace Activation | O |  | 9.3.1.68 |  | YES | ignore |
| NPN Context Information | O |  | 9.3.1.84 |  | YES | reject |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.89 |  | YES | ignore |
| CHO Initiation | O |  | ENUMERATED (True, …) |  | YES | reject |
| Additional Handover Information | O |  | ENUMERATED(Discard PDCP SN, …) | If set to “Discard PDCP SN”, indicates that the forwarded PDCP SNs have to be removed | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.98 |  | YES | ignore |
| gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC, no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertake or not. | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.102 |  | YES | ignore |
| SCG Activation Status | O |  | 9.3.1.105 |  | YES | ignore |
| MT-SDT Information Request | O |  | ENUMERATED (true, …) | Indicates to request the report of MT-SDT Information. | YES | ignore |
| SDT Data Size Threshold | O |  | INTEGER (1.. 192000, ...) | Unit: byte. | YES | Ignore |
| Data Collection ID | O |  | 9.3.1.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.1.xxx Data Collection ID

This IE indicates the NG-RAN Node Measurement IDs which identify a Data Collection Reporting context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Together with gNB-CU-UP Measurement ID, identifies a Data Collection Reporting context |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Together with gNB-CU-CP Measurement ID, identifies a Data Collection Reporting context |

The presence of the additional IE triggers the gNB-CU-UP to collect gNB-CU-UP-based delay metrics and also to be prepared to receive the delay measurements from the gNB-DU. The overall RAN delay measurement is then aggregated and signalled to the gNB-CU-CP at the specified reporting periodicity over the E1 interface.

Proposal 3: Extend the E1 BEARER CONTEXT SETUP REQUEST message with the Data Collection ID

The final component of the solution is signalling the delay measurements aggregated at the gNB-CU-UP to the gNB-CU-CP. As part of the release 19 discussions on the split architecture, it has been agreed in R3#127bis that the Data Collection Reporting Initiation and Data Collection Reporting procedure shall be introduced on the E1 interface. As part of the agreed BLCR in R3-252503, new bits have been already introduced in the Report Characteristics for Data Collection IE included in the E1: DATA COLLECTION REQUEST message. However, the UE Performance Collection Configuration has not been included in this message.
Similarly, equivalent bits have not been introduced in the Node Measurement Failed Report Characteristics IE to be included in the E1: DATA COLLECTION RESPONSE. 
There is also a need to introduce the delay measurements in the E1: DATA COLLECTION UPDATE message.

Proposal 4: Extend the E1 Data Collection Reporting Initiation procedure with a new Node Measurement Failed Report Characteristics IE to be included in the E1: DATA COLLECTION RESPONSE, where a bit is included for the “Average Packet Delay”.

Proposal 5: Extend the E1 Data Collection Reporting Update message with IEs corresponding to the delay measurements aggregated at the gNB-CU-UP. This IE shall be a UE associated IE.

Proposal 6: Introduce the UE Performance Collection Configuration IE as part of the Data Collection Reporting Initiation procedure to configure parameters for UE performance collection.

Changes to E1 AP include changes to both Bearer Context Setup procedures and Data Collection procedures.  Changes to the Bearer Context related procedures have been shown above. Below we detail the changes to the Data Collection Reporting Initiation message.

#### 9.2.1.CC	DATA COLLECTION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average UE Throughput UL, Second bit = Average UE Throughput DL, Third Bit = Average Packet Delay, Other bits are ignored by the gNB-CU-UP. FFS on the coding | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | Ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.xxx |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### 9.3.1.xxx UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful UE context setup within which the UE performance measurements are collected. Unit: millisecond |

Below are the enhancements that are needed to the Data Collection Reporting Update message. Followed by the newly introduced IE group’s definition.

#### 9.2.1.DD	DATA COLLECTION RESPONSE

This message is sent by gNB-CU-UP to gNB-CU-CP to indicate that the requested information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Ignore |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFa iledMea sPerNo de> |  |  | - |  |
| >> Node Measurement Failed Report Characteristic | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average UE Throughput UL, Second bit = Average UE Throughput DL, Third Bit = Average Packet Delay, Other bits are ignored by the gNB-CU-UP. FFS on the coding | - |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. |  |  |

#### 9.2.1.FF	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the requested information.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| FFS other information |  |  |  |  |  |  |
| UE Associated Info Result list |  | 0..1 |  |  | YES | Ignore |
| >UE Associated info Result Item |  | 1 .. < maxnoofUEReports > |  |  | - |  |
| >> E1AP UE Assistant Identifier | M |  | RANUE ID |  | - |  |
| >> DRB Assistant Identifier | M |  | DRB ID 9.3.1.16 |  | - |  |
| >> Delay measurements for UE performance | O |  | 9.3.1.xxx |  | - |  |
| FFS other information |  |  |  |  |  |  |

#### 9.3.1.xxx Delay measurements for UE performance

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Average RAN delay in UL. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Average RAN delay in DL. Unit: 0.1 millisecond |

#### 2.2 Aggregation of delay components

The delay measurements defined in TS 28.558 Sec. 6.3.1.1, and the corresponding MDT measurements defined in TS 37.320 are defined per QoS level, or per S-NSSAI, or at a granularity of per-DRB- per UE.

UE performance may be collected for the whole UE (legacy case) or on a per network slice level (per slice UE performance), or on a per DRB level (UE Performance reporting from SN to MN for NR-DC). RAN3 in meeting #124 agreed that the UE performance per UE is derived by average over the per-DRB measurements of different metrics corresponding to the UE.

UE level performance is derived as the average value of per DRB level measurements of the UE.

Following a similar approach, the delay measurements included in the per slice UE Performance can be derived by averaging the UE performance for all DRBs associated to the same network slice.

Equivalently, the delay measurements included in the UE Performance at the SN (for NR-DC cases) can be derived by averaging the UE performance for all the SN terminated bearers.

Hence, the core of the proposed solution for delay averaging is to measure and signal per-DRB-level delay measurements from the gNB-CU-UP to the gNB-CU-CP. The advantage of signalling per-DRB-level metric is the following:

-	In the context of Network slicing use-case, per-slice UE performance metrics may need to be derived for slice-level performance observability. The gNB-CU-CP is aware of the relationship between the DRBs of a UE to the network slice that the DRB was mapped to. In order to derive a network slice level delay measurement, the gNB-CU-CP may aggregate the delay measurements per DRBs (received from the gNB-CU-UP) corresponding to the particular slice of interest.

-	In the context of the NR-DC, UE performance for the UE at the SN may be required. However, the same mechanism as described in this contribution may be used by the SN to collect UE performance metrics of all SN terminated bearers. The SN may then report the measured values at a per-DRB level granularity to the MN. The MN can, from this information, understand the delay part of the UE performance at the SN.

Observation 1: The proposed mechanism to collect delay measurements for UE performance applies equally well to the Network slicing and NR-DC use cases.

In conclusion, delay measurements included in the UE Performance metrics can be collected on a per DRB basis.

Proposal 7: RAN3 to agree to average on a pr DRB basis the delay measurements included in the UE Performance

# 3. Conclusion

# This paper makes the following observations and proposals:

Proposal 1: RAN3 to discuss solutions for UE performance according to the following principles:

- It should be possible for UE performance collection to be enabled independently of QoS monitoring

- It should be possible for UE performance collection and QoS monitoring to co-exist, with potentially different reporting periodicities

- The gNB-CU-UP receiving the measurements should be able to generate reports corresponding to QoS reporting or UE performance.

- The agreed principles and procedures should work for UE performance metrics collection in case of NR-DC, where feedback for SN terminated bearers is signalled by the SN to the MN

- The agreed principles and procedures shall facilitate obtaining/deriving either per DRB (to cover the NR-DC UE Performance case), per slice (to cover the per slice UE Performance case), or (legacy) per-UE UE performance metrics without additional complexity

Proposal 2: Extend the F1-C UE CONTEXT SETUP REQUEST message with two IEs (Delay monitoring for UE performance, Reporting frequency for UE performance and Delay Monitoring for UE Performance Duration)

Proposal 3: Extend the E1 BEARER CONTEXT SETUP REQUEST message with the Data Collection ID

Proposal 4: Extend the E1 Data Collection Reporting Initiation message with a new Node Measurement Failed Report Characteristics IE to be included in the E1: DATA COLLECTION RESPONSE, where a bit is included ofr the “Average Packet Delay”.

Proposal 5: Extend the E1 Data Collection Reporting Update message with IEs corresponding to the delay measurements aggregated at the gNB-CU-UP. This IE shall be a UE associated IE.

Proposal 6: Introduce the UE Performance Collection Configuration IE as part of the Data Collection Reporting Initiation procedure to configure parameters for UE performance collection.

Observation 1: The proposed mechanism to collect delay measurements for UE performance applies equally well to the Network slicing and NR-DC use cases.

Proposal 7: RAN3 to agree to average on a pr DRB basis the delay measurements included in the UE Performance

A TP reflecting the above proposals is available in R3-253434 (F1-C), R3-253435 (F1-U) and R3-253436 (E1).