3GPP TSG RAN WG3 Meeting #128	R3-253459

St Julian, Malta, 19 - 23 May 2025

Title: 	Discussion on RAN2 LSs on AI/ML Data collection

Source: 	Ericsson

Agenda item:	20.2

Document Type:	Discussion and decision

# 1. Introduction

RAN3 has received the LSs in [1] and [2] on “signalling feasibility of dataset and parameter sharing” and “OAM-centric solution for NW-side data collection”, respectively.

This paper discusses these LSs and proposes a way forward.

# 2. Discussion

## 2.1 Discussion on LS on signalling feasibility of dataset and parameter sharing

In their LS on signalling feasibility of dataset and parameter sharing, RAN2 asks RAN3 to evaluate the feasibility of non Over The Air solutions identified for dataset/model parameter transfer from NW to UE training entity.

This is clearly stated in the LS actions quoted below:

“Action: RAN2 respectfully asks RAN3/SA2/SA5 to confirm RAN2 assumption on non-OTA solutions.”

The dataset/model parameters refer to two-sided model use cases developed by RAN1, see [3].

In their LS, RAN2 presents the following model for non-OTA solutions:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

The LS also describes the following solution options:

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1). From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can further confirm the above RAN2 assumption. Furthermore, it does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below. Table 1. non-OTA candidate solutions Option; Impacted WG; Specification impact/Implementation impact. OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity; SA5, SA3; Up to SA5 (any intermediate node between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion). CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity; SA2, SA3; Up to SA2 (any intermediate node between CN and UE-side OTT server is up to SA2). gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity; RAN3, SA2, SA5, SA3; Up to RAN3, SA2, SA5 (any intermediate node between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5).

The first clarification to make is that, as shown in the Alternative 1 figure, the gNB is always the entity that starts transferring of the dataset/model parameters.

Therefore, for the candidate solution 1 the transferring follows the order: gNB -> OAM -> UE-side training entity, while for the candidate solution 2 the transferring follows the order: gNB -> CN -> UE-side training entity.

With this clarification, it appears that candidate solution 3 is the sum of Candidate Solution 1 and Candidate Solution 2. Therefore, it would be sufficient to analyse Candidate Solution 1 and Candidate Solution 2 to fulfil the actions of the LS.

Proposal 1: RAN3 to draft a reply LS to RAN2 including an analysis of candidate solution 1 and candidate solution 2, while not evaluating candidate solution 3, which is the sum of the two previous solutions. The analysis of candidate solution 1 and 2 provide feedback on RAN involvement in the data transfer process.

Analysis of Candidate Solution 1:

Candidate Solution 1 is based on transferring of dataset/model parameters from the gNB to the OAM and from the OAM to the UE-side training entity.

It should be pointed out that, as per TS38.300, the following principles have been agreed:

“AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.”

Therefore, transferring of model parameters shall not be enabled, as it would clearly disclose parts of the model implementation.

Proposal 2: for candidate solution 1, RAN3 to reply that, on the basis of normative specifications in TS38.300, spelling out that “AI/ML algorithms and models are out of 3GPP scope”, model parameters shall not be transferred from the NG-RAN to the OAM.

It shall be noted that the dataset/model parameters to be transferred by the gNB are non UE associated information, namely information associated to a two-sided AI/ML model and not to a specific UE, as explained in the RAN1 LS in [3].

With this in mind, it should be noted that there are mechanisms available for the transferring of non UE associated data from the NG-RAN to the OAM. Such mechanisms consist of the transferring of Performance Measurements (PM) defined in the SA5 specifications, for example TS28.532/TS28.662 (describing OAM procedures to configure the RAN to report PMs) and TS28.552 (describing measurements corresponding to PMs).

While configuration of PM reporting from the OAM to the RAN is under SA5 domain, it is plausible from a RAN3 point of view to assume that dataset parameters could be included in the data collected by the OAM from the RAN. This would have no impacts on the RAN.

Proposal 3: For candidate solution 1, RAN3 to reply that it is feasible to enable transferring of dataset parameters from the RAN to the OAM as part of current management level performance measurement collection procedures

Analysis of Candidate Solution 2:

Candidate Solution 2 is based on transferring of dataset/model parameters from the gNB to the CN and from the CN to the UE-side training entity.

Just like for candidate solution 1, it shall be noted that the dataset/model parameters to be transferred by the gNB are non UE associated information.

There are currently no procedures over the NGAP to transfer AI/ML associated information from the RAN to the CN and neither there are discussions in Rel19 to enable such procedures. Therefore, a solution based on transferring of dataset/model parameters for two sided AI/ML models, from the gNB to the 5GC, is not feasible for Rel19.

Proposal 4: For candidate solution 2, RAN3 to reply that there are currently no procedures over the NGAP to transfer AI/ML associated information from the RAN to the CN and neither there are discussions in Rel19 to enable such procedures. Therefore, the candidate solution is not feasible for Rel19.

A draft reply LS to RAN2 for the LS on signalling feasibility of dataset and parameter sharing can be found in R3-253460

## 2.2 Discussion on LS on OAM-centric solution for NW-side data collection

In their LS in [2], RAN2 makes the following statement:

For OAM-centric solution for NW-side data collection, RAN2 has made the following agreements: RAN2 assumes to only adopt management based immediate MDT for NW-side data collection. For NR-DC, the configuration and reporting of NW sided data collection can only be via MN.

RAN2 adds the following actions for RAN3:

“RAN2 respectfully asks SA5 and RAN3 to take these agreements into account for their work and notify RAN2 if there are any concerns to support them for Rel-19.”

From a RAN3 point of view it is feasible to adopt management based immediate MDT for NW side data collection in Rel19, assuming that the data that need to be collected are included in the Rel19 specifications describing Management Based Immediate MDT, e.g. in TS32.422.

It is therefore suggested to reply as follows:

Proposal 5: RAN3 to reply to RAN2 that the use of Management Based Immediate MDT for NW side data collection is feasible in Rel19 from a RAN3 point of view, assuming that the measurements that need to be collected are within the currently specified Management Based Immediate MDT measurements defined in SA5 specifications such as TS32.422

A draft reply LS to RAN2 for the LS on OAM-centric solution for NW-side data collection can be found in the Appendix section.

# 3. Conclusion

In this paper the LSs from RAN2 in [1] and [2] have been analysed. The following conclusions and proposals were derived:

For the LS on signalling feasibility of dataset and parameter sharing:

Proposal 1: RAN3 to draft a reply LS to RAN2 including an analysis of candidate solution 1 and candidate solution 2, while not evaluating candidate solution 3, which is the sum of the two previous solutions

Proposal 2: for candidate solution 1, RAN3 to reply that, on the basis of normative specifications in TS38.300, spelling out that “AI/ML algorithms and models are out of 3GPP scope”, model parameters shall not be transferred from the NG-RAN to the OAM.

Proposal 3: For candidate solution 1, RAN3 to reply that it is feasible to enable transferring of dataset parameters from the RAN to the OAM as part of current management level performance measurement collection procedures

Proposal 4: For candidate solution 2, RAN3 to reply that there are currently no procedures over the NGAP to transfer AI/ML associated information from the RAN to the CN and neither there are discussions in Rel19 to enable such procedures. Therefore, the candidate solution is not feasible for Rel19.

For the LS on OAM-centric solution for NW-side data collection:

Proposal 5: RAN3 to reply to RAN2 that the use of Management Based Immediate MDT for NW side data collection is feasible in Rel19 from a RAN3 point of view, assuming that the measurements that need to be collected are within the currently specified Management Based Immediate MDT measurements defined in SA5 specifications such as TS32.422

# 4. References

[1] R3-253010, Reply LS on signalling feasibility of dataset and parameter sharing, RAN2

[2] R3-253011. LS on OAM-centric solution for NW-side data collection, RAN2

[3] R1-2410922, LS on signalling feasibility of dataset and parameter sharing, RAN1

# 5. Annex

A reply to the LS on OAM-centric solution for NW-side data collection is provided below:

3GPP TSG RAN WG3 Meeting #128	R3-25xxxx

St Julian, Malta, 19 - 23 May 2025

Title:	Reply LS on OAM-centric solution for NW-side data collection

Response to:	R3-253011 (R2-2503170)

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	RAN3

To:	SA5, RAN2

Cc:

Contact person:	Angelo Centonza

Angelo.Centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks RAN2 for their LS on OAM-centric solution for NW-side data collection.

In their LS, RAN2 stated the following:

For OAM-centric solution for NW-side data collection, RAN2 has made the following agreements: RAN2 assumes to only adopt management based immediate MDT for NW-side data collection. For NR-DC, the configuration and reporting of NW sided data collection can only be via MN.

RAN3 would like to reply that the use of Management Based Immediate MDT for NW side data collection is feasible in Rel19 from a RAN3 point of view, assuming that the measurements that need to be collected are within the currently specified Management Based Immediate MDT measurements defined in SA5 specifications such as TS32.422.

# 2	Actions

To SA5 and RAN2:

ACTION: 	RAN3 respectfully asks SA5 and RAN2 to take the above into account and to provide feedback, if any.

# 3	Dates of next TSG RAN WG3 meetings

TSG-RAN3 Meeting #129		August 25-28, 2025			Bangalore, India

TSG-RAN3 Meeting #129-bis	October 13-17, 2025			Prague, Czech Republic