3GPP TSG-RAN WG3 Meeting #128															R3-253482

St. Julian - Malta, 19th – 23rd May 2025

Agenda Item:	13.2

Source:	Ericsson

Title:	Further discussion on inter-CU LTM

Document for:	Discussion, Agreement

# Introduction

At the last RAN3 meeting, good progress was achieved for inter-CU LTM. In this contribution, we provide further discussion on how to support inter-CU LTM for standalone and NR-DC.

The agreements and open points relevant for the discussion are reported in the relevant sections below. The TPs corresponding to our proposals can be found in R3-253483 for XnAP, F1AP, and stage-2 (standalone and NR-DC).

# Discussion

## Reference Configuration for inter-CU LTM

At the last RAN3 meeting, good progress was achieved for inter-CU LTM. 
The following open point: “FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.” remains unresolved, and the following approaches were discussed:

- Approach 1: Similar as F1AP, the source gNB can request a candidate-gNB to provide a reference configuration.

- Approach 2: For inter-CU LTM, only the source gNB can provide the reference configuration.

- Approach 3: If the source CU does not provide the reference configuration, it is up to the candidate DU to provide the reference configuration to the source CU and the source DU can decide which one to adopt as a reference configuration.

At RAN2#127 meeting, RAN2 agreed that inter-CU LTM re-uses the same principle as rel-18 LTM, and no multiple reference configurations are supported:

Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

Since multiple reference configurations are not supported, if a candidate gNB (or a candidate DU in a candidate gNB) is allowed to provide a reference configuration, this calls for a coordination that involves the source gNB and all the other candidate gNBs, with an increase in signaling, both in Xn and F1 (if the candidate gNBs are split gNBs).

If we allow Approach 1 or Approach 3, it can happen that more candidate gNBs (or a candidate DUs in one or more candidate gNBs) generate a reference configuration. Then the source gNB receiving such reference configuration(s) can decide whether to use one of them or ignore them and inform the candidate gNBs of its choice. In the following figure some examples of deployments are shown, with three vendors involved. With Approach 1 or Approach 3, the coordination effort and the complexity of the implementation is larger compared to Approach 2. To visualize this, we propose some examples of deployment where inter-CU LTM can be used in the figure below (different vendors are shown in different colours).

In scenario 1, gNB1 includes CU1 provided by vendor 1, DU11 by vendor 2 and DU12 by vendor 3. If we assume that CU1 is the source CU and DU11 is the source DU, the intra-CU LTM case is limited to gNB1. With the addition of gNB2 (CU2 + DU21 + DU22) and gNB3 (CU3 + DU31 + DU32) the coordination between all the network entities requires further signaling and a more complex job e.g., for CU1. Similar considerations can be made for the other proposed example scenarios.

We note that, according to the rel-18 solution, when the source DU provides to a candidate DU the reference configuration, and the candidate DU does not find it useful, the candidate DU can respond with a complete configuration. With this approach the solution works, and it is simple. The same approach can be used for inter-CU as well: when the source gNB provides to a candidate gNB the reference configuration, and the candidate gNB does not find it useful, the candidate gNB responds with a complete configuration.

- If the source DU sends to a candidate DU the reference configuration, and the candidate DU does not want to use it, it can reply with a complete configuration.

- If the source gNB sends to a candidate gNB the reference configuration, and the candidate gNB does not want to use it, it can reply with a complete configuration.

Back to the proposed example, if Approach 2 in used, in all the provided example scenario, only the source CU (e.g., CU1) provides the reference configuration, and if any of the candidate gNBs (or any candidate DUs) does not accept that, they can provide a complete configuration. The source CU just collects this input and it is straightforward for gNB1 to build the configuration for the UE.

As an additional consideration, we note that in NR-DC scenarios, if a candidate gNB (target MN or target SN) can provide a reference configuration (for MCG or for SCG), this will make the coordination even more complex, e.g., due the fact that source and target SN need to communicate via MN. The overall signalling and complexity of the solution will just increase. And we can observe, again, that this is not needed: if only the source MN (or SN) can send the reference configuration, a candidate MN (or SN) can reply with a complete configuration.

- If only the source gNB can provide the reference configuration, the solution is simpler.

Based on the provided analysis, we think that the simpler option (which still provides a working solution) is that only the source gNB prepares the reference configuration and we propose to resolve the FFS adopting approach 2, i.e., for inter-CU LTM, only the source gNB can provide the reference configuration.

- Only the source gNB can provide the reference configuration.

## TA Information Transfer

At the last RAN3 meeting the following was captured:

Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Include the following information in the TA information transfer message:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- FFS on the XnAP IDs.

The agreed new procedure is non-UE associated, therefore the candidate gNB may transfer multiple TA values related to multiple candidate cells of the same candidate gNB, where the TA values pertain to multiple UEs. The current baseline indicates that only one TA value is transferred, which would make the procedure UE-associated. Therefore, a first detail to fix in the XnAP BL CR is to allow the transfer of a list of TA values.

- In the TA INFORMATION TRANSFER XnAP message, include a list of TA values associated with the candidate cells.

Before the source DU triggers the LTM execution to a UE, it is preferrable that all the TA values associated with that UE have been received. Therefore, it is preferrable if the source DU can wait until all the candidate gNBs has transferred the respective TA values to the source CU. The delay to transfer all the TA values for a certain UE from all the candidate cells (potentially controlled by different gNBs) to the source DU can include:

- the delay to transfer the TA values for a certain UE from all the candidate gNBs to the source gNB, potentially in different messages (one per candidate cell). Some notes:

  - Note 1: the TA value associated to a certain candidate cell and a certain UE can be sent together with the TA values associated to the same candidate cell and other UEs. For example, if we’re interested in the TA values for UE1, the candidate gNB may use multiple TA Information Transfer -  one per candidate cell -  and transfer to the source gNB the TA values for UE1 together with the TA values for UE2, UE3, … UEn (this also requires the candidate gNB to wait until all the TA values are collected!).

  - Note 2: the UE1, UE2, UE3, … UEn may have different source cell, and the request to receive TA values for those UEs will likely arrive at different times at the candidate gNB.

- the delay at the source gNB, where the source CU can group the TA values received for multiple UEs (because they have the same source cell in common) before transferring the TA values to the source DU. Depending on how many TA values the CU receives, and from which candidate gNBs, the source CU may need to send multiple F1AP TA Information Transfer message, e.g., one for each TA Information Transfer received from a different candidate gNB.

Even if the delay to transfer the single TA value over Xn and F1 is small (in the order of 10’th of ms), the whole process may accumulate a non-negligible delay (for example, one order of magnitude larger) and even if the source DU receives TA values which are still valid, the overall delay might be enough to lead failures in mobility, and the source DU may realize that it is better to the LTM Cell Switch command before the UE is lost, rather than waiting to collect all the TA values. In this case the TA acquisition process becomes useless. One way to reduce the delay in the TA Information Transfer is to also consider the UE association to which a TA value refers to. Since a UE association is established for a UE between the source CU and the candidate CU during the preparation phase, when a candidate CU transfers the TA values to the source CU, it can consider sending TA values of certain UEs before others. The same is valid for the transfer from source CU to source DU. There was a comment in the last meeting regarding how the candidate cell gets to know the assigned XnAP IDs, which refer to UE associations. This can only be addressed if the preamble index is requested based on UE-associated signaling, in which case, it would be part of a complete solution.

- Include the UE XnAP IDs of the source and target gNB in the XnAP messages for TA information transfer.

- Include the UE F1AP IDs in the DU-CU/CU-DU TA Information Transfer messages.

## Single UE association for multiple LTM handover requests

At the last meeting the group agreed to turn a previous WA into agreement:

Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB.

It remains to be clarified how to identify the same UE from multiple Handover preparation procedure. The current baseline indicates the following for the Handover Preparation procedure:

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

Looking at the above text, one can realize that, while there are similarities between conditional handover and LTM (in both cases parallel transactions are possible), simply reusing the sentence “Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.” is not accurate for LTM. This is because what we need to ensure is that, for the LTM case, both source UE AP IDs and target UE AP IDs - and not just the source UE AP ID – are the same when there is a need to identify a specific UE. This is the case for subsequent LTM, where the new candidate gNBs (after LTM execution) needs to recognize the same UE for which they have established an UE association towards the previously serving gNB. A similar need does not exist for conditional handover.

- For subsequent LTM the new candidate gNBs (after LTM execution) needs to recognize the same UE for which they established a UE association towards the previously serving gNB. Similar need does not exist for conditional handover.

To reduce the specification impact and clarifies the differences between LTM and normal handover and conditional handover, we propose to modify the general section of the Handover Preparation procedure, creating a new sentence only for LTM, as follows:

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM when the candidate cells are configured within the same target NG-RAN node, parallel transactions are allowed using the same pair of source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID.

- In the general description for the Handover Preparation indicates that if this concerns LTM, and the configured candidate cells are in the same target NG-RAN node, parallel transactions are allowed using the same pair of source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID.

- Include the assigned Target NG-RAN Node UE XnAP ID in the HANDOVER REQUEST ACKNOWLEDGE message during the first round of preparation.

## Support for subsequent inter-CU LTM

In this section we provide some further discussion on support for subsequent inter-CU LTM. At RAN3#126 it was agreed that: “To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.”.

At RAN3#127-bis the following was agreed:

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

During the handover preparation, the very first serving gNB established a UE association towards each candidate gNB. After the LTM cell switch, the new serving gNB establishes UE associations between itself and the other candidate gNB. There will be then new pairs of UE XnAP IDs. To visualize the situation and understand what type of information is needed to “follow” the UE as it moves among the gNBs, we propose an example, where gNB1 is the initial source gNB, and gNB2, gNB3, gNB4 are candidate gNBs.

During the LTM preparation, the first serving gNB1 established a UE association towards each candidate gNB, i.e. a pair of values “source NG-RAN node UE XnAP ID, target NG-RAN node UE XnAP ID” for each UE association is used. In the proposed example:

- between gNB1 and gNB2: “A” = {(gNB1 to gNB2) UE XnAP ID, (gNB2 to gNB1) UE XnAP ID}

- between gNB1 and gNB3: “B” = {(gNB1 to gNB3) UE XnAP ID, (gNB3 to gNB1) UE XnAP ID}

- between gNB1 and gNB4: “C” = {(gNB1 to gNB4) UE XnAP ID, (gNB4 to gNB1) UE XnAP ID}.

Let’s assume the first LTM cell switch is executed from gNB1 to gNB2. In line with the agreement above, the gNB2 can keep the existing association towards gNB1 and setup new UE associations towards gNB3 and gNB4, identified by:

- “D” = {(gNB2 to gNB3) UE XnAP ID, (gNB3 to gNB2) UE XnAP ID}

- “E” = {(gNB2 to gNB3) UE XnAP ID, (gNB4 to gNB2) UE XnAP ID}

The gNB3 needs to understand that the UE for which the new association established with gNB2 is the same UE as the association established with gNB1. One possibility to achieve this, it that the old serving gNB (gNB1) sends the old UE association “A” to the new serving gNB (gNB2) in the CELL SWITCH NOTIFICATION message, and the gNB2 sends it to the gNB3 in the LTM CONFIGURATION UPDATE message. An alternative is that the old UE association is sent before the LTM cell switch execution, as part of the LTM Configuration Update procedure used for the consolidation.

A certain UE is uniquely referenced by a certain UE association only between a given pair of gNBs, since the same UE XnAP IDs may be used across multiple nodes. Therefore, after an LTM cell switch, what a candidate gNB needs to uniquely identify the UE is the old UE association between itself and the old serving gNB, together with the respective node identities. In our example, the gNB3 needs to receive from gNB2 the UE association “B”, but also “gNB1 ID” and “gNB3 ID”. The same logic applies for gNB4.

Using more generic terms, we can say that after one LTM cell switch the “source NG-RAN node UE XnAP ID” assigned by the gNB that served the UE before the LTM cell switch (gNB1) becomes the “old source NG-RAN node UE XnAP ID” and the “target NG-RAN node UE XnAP ID” assigned by one of the candidate gNBs not used as new serving gNB (gNB3 or gNB4) becomes the “old target NG-RAN node UE XnAP ID”. The new serving gNB (gNB2) establishes new UE associations, and to let one of new candidate gNB (e.g., gNB3) identify the same UE, it sends the following to that candidate gNB: “old source NG-RAN node UE XnAP ID”, “old target NG-RAN node UE XnAP ID”, “old source NG-RAN ID”, “old target NG-RAN node ID”.

- The LTM CONFIGURATION UPDATE message includes the following information to allow the new candidate gNB to identify the same UE for which LTM preparation: old source NG-RAN node UE XnAP ID, old target NG-RAN node UE XnAP ID, old source NG-RAN ID, old target NG-RAN node ID.

At RAN3#127 meeting the following was agreed:

- Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

- It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

We propose to capture the above as part of the signaling sequence defined in TS 38.300 for LTM, as shown in the companion contribution in R3-253483.

- Clarify the sync up among candidate gNBs for subsequent LTM in the procedure being defined for LTM in TS 38.300 baseline.

Regarding the sync up among candidates, at RAN3#127-bis it was further agreed that “Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).” The granularity of each item in the list remains to be decided, e.g., should this information be per-node, per-PDU session, per DRB?

If we consider the Handover Preparation procedure, the candidate gNB indicates to the source gNB a PDU Session Resources Admitted List IE, within which, for one admitted PDU Session, it sends data forwarding information (in the Data Forwarding Info from target NG-RAN node). The sync up among the candidate is performed with the LTM Configuration Update, we propose to keep a consistent approach between the preparation phase (Handover Preparation) and the sync up phase (LTM Configuration Update), and to reuse resolve the FFS indicating that Data Forwarding Info contains the same information as the Data Forwarding Info from target NG-RAN node IE.

- The encoding of the Data Forwarding Info IE in LTM CONFIGURATION UPDATE message is a list of elements encoded as the Data Forwarding Info from target NG-RAN node IE.

Another piece of information that one candidate gNB needs to know to support subsequent LTM is the setting of Rel-19 Set ID of other candidate gNBs. A simple solution is that the source gNB assigns the Rel-19 Set ID of the candidate gNBs (in addition to its own Rel-19 Set ID). To make sure that all the candidate gNBs are in sync, the source gNB sends the Rel-19 Set ID assigned to a certain candidate gNB in the HANDOVER REQUEST and in the LTM CONFIGURATION UPDATE signalled to a certain candidate gNB, the source gNB includes the list of the Rel-19 Set ID assigned to all other candidate gNBs.

- The source gNB sends the Rel-19 Set ID assigned to the candidate gNB in the HANDOVER REQUEST message.

- A list of Rel-19 Set IDs should be introduced to the LTM CONFIGURATION UPDATE message as well.

To ensure that after LTM cell switch execution is completed the security requirements are fulfilled, at the last meeting the following agreement was taken: “Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.”. This was captured in the XnAP baseline with a new IE whose name is FFS. We propose to resolve the FFS, and use the name Security Information for LTM.

- Confirm the IE name Security Information for LTM.

At RAN3#127-bis, further open points related to security for subsequent LTM were captured as shown below:

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy.

The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

Whether to define additional information from CU to DU to deliver NCC value.

In the initial preparation, one Handover Preparation procedure is used to prepare only one candidate cell. The source gNB sends to the target node the KgNB* and the NCC for the target cell, and this was already specified for normal handover. To support security in subsequent inter-CU LTM when multiple candidate cells belong to one candidate gNB, the serving gNB can either send to the candidate gNB separate messages, each message containing one KgNB* for one candidate cell, or it can send to the candidate gNB only one message containing a list of KgNB*, with one KgNB* per candidate cell. The agreed message for providing the new KgNB*(s) is the LTM Configuration Update, which offers more flexibility compared to the Handover Request, since it is only used for LTM (and not for legacy handover). If we consider the current message structure, we have one FFS on the need for a Target Cell Global ID IE. We propose to resolve both the FFS on the need for the IE and the FFS on the procedure design at the same time as follows. The LTM Configuration Update procedure is UE associated, and its messages already contains the UE XnAP IDs. Therefore, the is no need to point at a specific target cell, because this is implicitly known from the UE association.

If the Target Cell Global ID IE is removed from the current LTM CONFIGURATION UPDATE/ACKNOWLEDGE messages, the procedure is still per-UE, but no longer “per-cell”. This allows to transfer the new KgNB*(s) for all the candidate cells in a candidate gNB using only one message, reducing the amount of signaling, which we prefer compared to sending multiple LTM CONFIGURATION UPDATE messages, one message containing only one KgNB* for one candidate cell. Finally, we see no need to include in the message the gNB ID of the candidate gNB, since this is implicit in the message direction. This leads us to the following proposals:

- Remove the Target Cell Global ID (FFS) from the LTM CONFIGURATION UPDATE and LTM CONFIGURATION UPDATE ACKNOWLEDGE messages.

- In the LTM CONFIGURATION UPDATE, replace the Key NG-RAN Star IE with a list where each item includes one cell of NG-RAN node2 and one KgNB* and move the IE under the Security Information for LTM IE.

Regarding the other open point from the last meeting on “Whether to define additional information from CU to DU to deliver NCC value”, we should consider that we also agreed that “The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure.” and this was captured in the current F1AP baseline (R3-253149) where the NCC is transferred in a new LTM Security Information IE included in UE CONTEXT MODIFICATION REQUEST. We think no further action is needed and we can close the open point.

- No additional information is needed from CU to DU to deliver NCC.

We want now to raise an issue that can occur, which impacts how to achieve a proper sync up of the configurations among the candidate gNBs for subsequent LTM. We think that a deployment scenario as in the figure below is possible, where there is NOT a full mesh Xn connectivity among all the gNBs involved in the LTM. The source gNB is gNB A and gNBs B, C, D, E are candidate gNBs for the first LTM Cell Switch. There is no Xn connectivity between gNB B and gNB E, nor between gNB D and gNB C. An obvious reason for the absence of the Xn connectivity can be that there is no coverage continuity between gNB B and gNB E (nor between gNB D and gNB C).

Figure 1 - Subsequent LTM when no full mesh Xn connectivity is present among the gNBs.

If the UE moves from gNB A to gNB B, the gNB E will not be used for the very next LTM Cell Switch, i.e., the gNB B will never send an LTM Cell Switch command to move the UE from gNB B to gNB E, so, for example, what would be the purpose of sending the Data Forwarding Info of gNB E from gNB A to gNB B? Also, if gNB A sends to gNB B the Data Forwarding Info of gNB E, how can gNB B understand it?

As per RAN3#126 agreement, after an inter-CU LTM Cell Switch, the new source gNB can use the LTM Configuration Update to establish the UE association between itself and the other candidate gNBs. In the example, when the UE moves from gNB A to gNB B, the gNB B can establish a new UE association between itself and gNB C and gNB D (and keep the UE association towards gNB A). Even if the UE continues to keep the same LTM configuration provided by gNB A, at the network side, there is no need to keep the previously established UE associations between gNB A and gNB E, nor between gNB E and gNB C. In fact, keeping this UE association is a waste of resources in gNB E. We would like to discuss this issue, and hopefully find a solution for that.

- RAN3 to discuss how to sync up configuration among candidate gNBs in case of non-full Xn connectivity among the candidate gNBs.

## Inter-CU LTM with NR-DC

For inter-CU LTM with NR-DC, the following agreements and open points were captured at the last RAN3 meeting:

Include the following information in SN Change Required (Source SN to MN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for each candidate SN (FFS)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

Include the following information in SN Addition Request (MN to Candidate SN) message:

- LTM request indication (explicit way)

- Suggested PScell list (implicit way in INM, FFS on explicit way)

- Measurement results (implicit way in INM)

- Upper limit number of PScell(s) for the candidate SN (explicit way)

- CSI Resource configuration request (explicit way)

- Early sync information request (explicit way)

It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message.

For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN.

The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN.

Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

For the scenario: SN initiated inter-CU SCG LTM without MCG changes, the source SN initiates the procedure sending an S-NODE CHANGE REQUIRED message. It remains to be decided whether the source SN can explicitly indicate a list PSCells and an upper limit of PSCell(s) for each candidate SN. On the first point (list of PSCells) our understanding is that RAN2 did not take any agreement on sending this information via INM, because this is not a UE related configuration parameter. Therefore, the source SN needs to send this information explicitly to the MN. As a consequence, the MN needs to forward the same information explicitly to the target SN in the S-NODE ADDITION REQUEST message.

- For SN-initiated inter-CU SCG LTM, the S-NODE CHANGE REQUIRED message contains an explicit list of PSCells for a candidate SN, and the MN forwards the information to the candidate SN in the S-NODE ADDITION REQUEST message.

A candidate SN may accept or reject a cell suggested as PSCell, and to inform the source SN about it, the candidate SN transfers to the MN the list of accepted PSCells (in S-NODE ADDITION REQUEST ACKNOWLEDGE) and the information is forwarded to the source SN (in S-NODE MODIFICATION REQUEST).

- For SN-initiated inter-CU SCG LTM, the S-NODE ADDITION REQUEST ACKNOWLEDGE message contains an explicit list of PSCells accepted by the candidate SN, which the MN forwards to the source SN in the S-NODE MODIFICATION REQUEST message.

Regarding the second open point (upper limit of PSCell(s)) the current RRC running CR for inter-CU LTM indicates a maximum number of LTM candidate configurations that an SN is allowed to configure (maxNumberLTM-CandidatesSCG). At the same time, as agreed in RAN2, the maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations and therefore the MN needs to be aware of the maximum number of inter-CU LTM candidate that a certain target SN can configure, for example in case the MN wants itself to configure another intra-CU LTM candidate configuration. To achieve this type of coordination, we propose that the source SN sends to the MN the upper limit number of PScell(s) for each candidate SN (in the S-NODE CHANGE REQUIRED message) and MN forwards the information to the target SN.

- For SN-initiated inter-CU SCG LTM, the S-NODE CHANGE REQUIRED message contains an explicit upper limit number of PScell(s) for a candidate SN, and the MN forwards the information to the candidate SN in the S-NODE ADDITION REQUEST message.

The third open point from the last meeting is: FFS on whether MN can request SCG reference configuration from the source SN. According to the current agreements on SCG reference configuration, the source SN can provide it. It is up to the source SN decision to provide it, and in case the MN receives it, it will just use it to consolidate the RRC message to be sent to the UE. In fact, our preference for inter-CU LTM in general is that only the source gNB (which in this context is the source SN) provides the reference configuration. We propose to modify the previous agreement, by removing the part where the “MN can request SCG reference configuration from one of the candidate SN(s)” and to resolve the FFS with no request for SCG reference configuration from MN to source SN.

- Only the source SN can provide the SCG reference configuration.

- Modify the agreement “For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s),” as follows: “For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message.”.

Another aspect which was not discussed at the last RAN3 meeting is how to avoid that an inter-CU LTM configuration is present at the same time at MN and SN. According to the running CR for RRC, RAN2 agreed that MN can indicate to the SN, via Inter-Node RRC signaling, a allowedInterSN-LTM-r19 IE. If this information is sent the SN is not allowed to configure inter-CU LTM

allowedInterSN-LTM Used to indicate whether the SCG can configure inter-SN LTM candidate configuration. If the field is absent the SN is not allowed to configure inter-SN LTM candidate configurations. This field is only used in NR-DC.

We don’t see any benefit to signal the same indication also explicitly from MN to SN, so we propose to do nothing in RAN3 for that.

- The indication on whether SN is allowed to configure inter-CU LTM is already passed from MN to SN via inter-node RRC signaling, and no RAN3 impact is needed.

At RAN3#127, it was agreed to reuse the Cell Switch Notification message from source SN to target SN via MN. Here we further discuss the details left FFS.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

For Inter-CU MCG LTM with SN addition and for SN initiated inter-CU SCG LTM (without MCG change), the MN needs to send to the new SN the security configuration that SN can use (SN security key and the list of possible sk-counters the UE can apply).

- For SN initiated inter-CU SCG LTM, the MN sends to the target SN the SN security keys and a list of sk-counters in the CELL SWITCH NOTIFICATION message.

- For Inter-CU MCG LTM with SN addition, the MN sends to the target SN the SN security keys in the CELL SWITCH NOTIFICATION message.

At completion of the SN initiated inter-CU SCG LTM, the target SN (i.e. the new serving SN) informs the source SN (i.e., the old serving SN), via the MN, that UE has successfully accessed the target cell. We propose to reuse the HANDOVER SUCCESS for this purpose.

- At completion of SN initiated inter-CU SCG LTM, the target SN sends a HANDOVER SUCCESS to the MN, which forwards it to the source SN, to inform the source SN the UE successfully accessed the target cell.

- Agree on the TPs for 38.423, 38.473, 38.300, and 37.340 in R3-253483 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	If the source DU sends to a candidate DU the reference configuration, and the candidate DU does not want to use it, it can reply with a complete configuration.

Observation 2	If the source gNB sends to a candidate gNB the reference configuration, and the candidate gNB does not want to use it, it can reply with a complete configuration.

Observation 3	If only the source gNB can provide the reference configuration, the solution is simpler.

Observation 4	For subsequent LTM the new candidate gNBs (after LTM execution) needs to recognize the same UE for which they established a UE association towards the previously serving gNB. Similar need does not exist for conditional handover.

Proposal 1	Only the source gNB can provide the reference configuration.

Proposal 2	In the TA INFORMATION TRANSFER XnAP message, include a list of TA values associated with the candidate cells.

Proposal 3	Include the UE XnAP IDs of the source and target gNB in the XnAP messages for TA information transfer.

Proposal 4	Include the UE F1AP IDs in the DU-CU/CU-DU TA Information Transfer messages.

Proposal 5	In the general description for the Handover Preparation indicates that if this concerns LTM, and the configured candidate cells are in the same target NG-RAN node, parallel transactions are allowed using the same pair of source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID.

Proposal 6	Include the assigned Target NG-RAN Node UE XnAP ID in the HANDOVER REQUEST ACKNOWLEDGE message during the first round of preparation.

Proposal 7	The LTM CONFIGURATION UPDATE message includes the following information to allow the new candidate gNB to identify the same UE for which LTM preparation: old source NG-RAN node UE XnAP ID, old target NG-RAN node UE XnAP ID, old source NG-RAN ID, old target NG-RAN node ID.

Proposal 8	Clarify the sync up among candidate gNBs for subsequent LTM in the procedure being defined for LTM in TS 38.300 baseline.

Proposal 9	The encoding of the Data Forwarding Info IE in LTM CONFIGURATION UPDATE message is a list of elements encoded as the Data Forwarding Info from target NG-RAN node IE.

Proposal 10	The source gNB sends the Rel-19 Set ID assigned to the candidate gNB in the HANDOVER REQUEST message.

Proposal 11	A list of Rel-19 Set IDs should be introduced to the LTM CONFIGURATION UPDATE message as well.

Proposal 12	Confirm the IE name Security Information for LTM.

Proposal 13	Remove the Target Cell Global ID (FFS) from the LTM CONFIGURATION UPDATE and LTM CONFIGURATION UPDATE ACKNOWLEDGE messages.

Proposal 14	In the LTM CONFIGURATION UPDATE, replace the Key NG-RAN Star IE with a list where each item includes one cell of NG-RAN node2 and one KgNB* and move the IE under the Security Information for LTM IE.

Proposal 15	No additional information is needed from CU to DU to deliver NCC.

Proposal 16	RAN3 to discuss how to sync up configuration among candidate gNBs in case of non-full Xn connectivity among the candidate gNBs.

Proposal 17	For SN-initiated inter-CU SCG LTM, the S-NODE CHANGE REQUIRED message contains an explicit list of PSCells for a candidate SN, and the MN forwards the information to the candidate SN in the S-NODE ADDITION REQUEST message.

Proposal 18	For SN-initiated inter-CU SCG LTM, the S-NODE ADDITION REQUEST ACKNOWLEDGE message contains an explicit list of PSCells accepted by the candidate SN, which the MN forwards to the source SN in the S-NODE MODIFICATION REQUEST message.

Proposal 19	For SN-initiated inter-CU SCG LTM, the S-NODE CHANGE REQUIRED message contains an explicit upper limit number of PScell(s) for a candidate SN, and the MN forwards the information to the candidate SN in the S-NODE ADDITION REQUEST message.

Proposal 20	Only the source SN can provide the SCG reference configuration.

Proposal 21	Modify the agreement “For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s),” as follows: “For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message.”.

Proposal 22	The indication on whether SN is allowed to configure inter-CU LTM is already passed from MN to SN via inter-node RRC signaling, and no RAN3 impact is needed.

Proposal 23	For SN initiated inter-CU SCG LTM, the MN sends to the target SN the SN security keys and a list of sk-counters in the CELL SWITCH NOTIFICATION message.

Proposal 24	For Inter-CU MCG LTM with SN addition, the MN sends to the target SN the SN security keys in the CELL SWITCH NOTIFICATION message.

Proposal 25	At completion of SN initiated inter-CU SCG LTM, the target SN sends a HANDOVER SUCCESS to the MN, which forwards it to the source SN, to inform the source SN the UE successfully accessed the target cell.

Proposal 26	Agree on the TPs for 38.423, 38.473, 38.300, and 37.340 in R3-253483 that capture the above proposals.