3GPP TSG-RAN WG3 Meeting #128	R3-253488

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	Discussion on per slice UE performance in AI/ML-based Network Slicing

Source: 	Huawei, Jio Platforms, Deutsche Telekom

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing.

This contribution analyzes further the technical enablers for the AI/ML-based Network Slicing use case to be introduced over the Xn interface, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting the following agreements were achieved [4]:

Stage3 signalling design: Introduce the measured CCCV within the predicted slice available capacity. Slice UE performance: For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap. Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics. Add Slice UE performance IE in the DATA COLLECTION UPDATE message. Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

We will discuss further the granularity of the UE performance feedback for the AI/ML-based Network Slicing use case.

## 2.1 On per-slice per-UE UE performance

RAN3 agreed that the Data Collection Reporting Initiation and Data Collection Reporting procedures over Xn need to be enhanced to request and report the predicted radio resource status per slice and the predicted slice available capacity. Moreover, the Slice To Report List for Data Collection IE was agreed to be introduced in the DATA COLLECTION REQUEST message pertaining the Data Collection Reporting Initiation procedure to indicate the slice(s) for which the predicted radio resource status per slice and predicted slice available capacity need to be reported.

In RAN3#126 [5], instead, further agreements were reached concerning the encoding of the predicted radio resource status per slice (which reuses the Rel-18 Predicted Radio Resource Status IE) and the predicted slice available capacity (which is a new per cell IE in the DATA COLLECTION UPDATE message). Moreover, in RAN3#127 [6], RAN3 agreed to enhance the Rel-18 partial reporting mechanism to cover the case where the reporting node is requested to provide slice-specific predictions but it is unable to comply to such request. Therefore, RAN3 agreed to introduce in the DATA COLLECTION RESPOSE message the Slice Measurement Initiation Result IE (within the Cell Measurement Initiation Result List IE) to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated. In the last meeting, RAN3 agreed to enhance the newly introduced IE for the predicted slice available capacity to also include the measured Cell Capacity Class Value (CCCV), so that the predicted slice available capacity can be correctly scaled with respect to the total capacity of the cell for cases where some slices share resources from the same resource pool. The remaining agreements from the last meeting pertain the configuration and reporting of per-UE per-slice UE performance metrics which can be reported as part of a new Slice UE Performance IE within the DATA COLLECTION UPDATE message, while the reporting for such metrics can be requested by enabling the corresponding new bits introduced in the existing Report Characteristics for Data Collection IE bitmap – all the above agreements have already been reflected in the XnAP BLCR for enhancements on AI/ML for NG-RAN in [7].

By referring to the new Slice UE Performance IE as defined in the XnAP BLCR in [7], it can be noticed that there are a couple of open points (FFS) concerning how the slice based UE performance metrics are derived, as well as whether and how the Slice Average Packet Loss DL IE is defined:

#### 9.2.3.x2 Slice UE Performance This IE indicates per Slice UE performance measurements metrics. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Slice UE Performance; 1..< maxnoofBPLMNs >. >PLMN Identity; M; 9.2.2.4; Broadcast PLMN. >S-NSSAI UE Performance List; 1; Indicates the UE performance per network slice (details are FFS). >>S-NSSAI UE Performance Item; 1 .. < maxnoofSliceItems>. >>>S-NSSAI; M; 9.2.3.21. >>>Slice Based UE Performance; M; UE Performance 9.2.3.179; [It is FFS how the slice based UE performance metrics are derived] [It is FFS whether and how the Slice Average Packet Loss DL IE is defined]. a

In our view, the FFS pertaining the definition of the Slice Average Packet Loss DL IE depends on the feedback from SA5 related to whether it is feasible to measure the packet loss in DL (and in UL as well) at PDCP level for both split and non-split architecture, as requested by RAN3 via the LS in [8] and excerpted below:

*** skip non-relevant parts *** RAN3 would like to seek confirmation from SA5 on following: *** skip non-relevant parts *** - The feasibility of measuring Packet Loss Rate in UL and DL at PDCP level for both split and non-split architecture. And if it is feasible, RAN3 would like to ask SA5 to specify these measurements for both split and non-split architecture. *** skip non-relevant parts ***

Observation 1: The FFS pertaining the definition of the Slice Average Packet Loss DL IE depends on the feedback from SA5 related to whether it is feasible to measure the packet loss in DL (and in UL as well) at PDCP level for both split and non-split architecture.

Proposal 1: RAN3 to agree to postpone the discussion on the FFS pertaining the definition of the Slice Average Packet Loss DL IE once RAN3 receives the feedback from SA5 on the feasibility to measure the packet loss in DL (and in UL as well) at PDCP level for both split and non-split architecture.

For the FFS on how the slice based UE performance metrics are derived, instead, and based on Proposal 1 above, the discussion should be limited to Average UE Throughput DL/UL and Average Packet Delay DL/UL.

On UE Throughput DL/UL

Even though the LS sent to SA5 in [8] takes the throughput DL/UL into account as part of the feasibility check requested by RAN3, at least for the non-split architecture the information provided by SA5 in [9] may be considered, where SA5 confirmed that Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2 [10] are per-UE measurements, originating from corresponding per-DRB measurements being aggregated to reflect a per-UE throughput:

*** skip non-relevant text *** SA5 confirms that the formulas specified in fields (c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 calculate average UE throughput based on per DRB level measurement for a UE. The formulas aggregate data across DRBs for each UE, ensuring that the reported throughput measurement represents the overall throughput per UE rather than being limited to individual DRBs. The rationale for using the "DRB." prefix in the measurement names, as listed in field (e), is to indicate that the measurements originate from data transmissions occurring at the DRB level. However, these measurements are aggregated to provide a per-UE throughput metric. Specifically: - The formulas include a summation over DRBs, ensuring that the result represents the total throughput per UE rather than individual DRB-level values. - The "DRB." prefix in field (e) indicates that the contributing measurements originate from DRBs, but they are aggregated to reflect per-UE throughput. - The definitions in the tables in field (c) in clauses 6.3.1.4.1 and 6.3.1.4.2 are based on DRB level measurements, but the summation ensures that the final metric is at the UE level. *** skip non-relevant text ***

Observation 2: SA5 confirmed that Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2 are per-UE measurements, originating from corresponding per-DRB measurements being aggregated to reflect a per-UE throughput.

Based on the above Observation 2, and considering that TS 28.558 specifies the following, e.g., for the Average DL UE throughput in gNB measurement (similar statements are present for the Average UL UE throughput in gNB measurement):

#### 6.3.1.4 UE throughput ##### 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. *** skip non-relevant parts ***

it is our understanding that the UL/DL throughput measurements in TS 28.558 have already been defined with a per-slice per-UE granularity.

Observation 3: SA5 already specified in TS 28.558 that UL/DL throughput measurements have already been defined with a per-slice per-UE granularity.

As previously said, RAN3 is waiting for the feedback from SA5 related to whether it is feasible to measure the UE throughput in DL (and in UL as well) at PDCP level for both split and non-split architecture as requested via the LS in [8], therefore it should be discussed in RAN3 whether to consider what is currently specified in TS 28.558 (at least for the non-split architecture case) or postpone the discussion while waiting for the feasibility check from SA5.

Proposal 2: RAN3 to discuss whether, for the UE throughput DL/UL, either the current measurements definitions in TS 28.558 (already available with a per-slice per-UE granularity) can be referred for the Slice Average UE Throughput DL/UL or wait for the feasibility check from SA5 on PDCP-level throughput measurements.

On packet delay DL/UL

The Rel-18 per-UE Average Packet Delay DL/UL measurements refer to TS 38.314 [11], as specified in TS 38.423 [12]:

#### 9.2.3.187 Average Packet Delay This IE indicates the RAN part of the average packet delay in the UL and DL directions. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Average Packet Delay UL; M; INTEGER (0.. 10000); Defined in TS 38.314 [42]. Unit: 0.1 millisecond. Average Packet Delay DL; M; INTEGER (0.. 10000); Defined in TS 38.314 [42]. Unit: 0.1 millisecond. a

In TS 38.314, the packet delay measurements which constitute the overall packet delay DL/UL are measured with a per-DRB per-UE granularity – refer to the excerpt below from TS 38.413 for the packet delay [11]:

#### 4.2.1.2 Packet delay ##### 4.2.1.2.1 General Packet delay includes RAN part of delay and CN part of delay. The RAN part of DL packet delay measurement comprises: - D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1. - D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3. - D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2. - D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1. The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE. NOTE: The delay measurements D1, D2 and D4 are also applicable for EUTRA in case of EN-DC related DL delay measurements on the MN side. The RAN part (including UE) of UL packet delay measurement comprises: - D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1). - D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2). - D2.2 (average RLC packet delay, as defined in 4.2.1.2.3). - D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2). - D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4). The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms.

Observation 4: The packet delay DL/UL measurements are defined with a per-DRB per-UE measurements in TS 38.314.

TS 28.558 also specifies that packet delay DL/UL measurements can be calculated per supported S-NSSAI – as an example, refer to the Average delay DL air-interface measurement [10]:

### 6.3.1 UE level measurements definitions for gNB #### 6.3.1.1 Packet delay ##### 6.3.1.1.1 Average delay DL air-interface a) This measurement provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction. The measurement is calculated per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as D1 (see TS 38.314 [8]) as part of DL M6 measurement (see TS 37.320 [9]). b) DER (n=1) *** skip non-relevant parts ***

Observation 5: The packet delay DL/UL measurements can be provided also per-slice as specified in TS 28.558.

Based on Observation 4 and Observation 5 above, it is our understanding that the packet delay measurements which constitute the overall packet delay DL/UL can be measured with a per-DRB per-slice per-UE granularity.

Proposal 3: RAN3 to confirm the understanding that the packet delay measurements which constitute the overall packet delay DL/UL can be measured with a per-DRB per-slice per-UE granularity.

In order to derive per-slice per-UE Average Packet Delay DL/UL measurements, we think that the RAN3 common understanding as captured in RAN3#124 meeting may be considered [13]:

#### UE level performance is derived as the average value of per DRB level measurements of the UE. **** Skip non-relevant text ****

meaning that per-slice per-UE Average Packet Delay DL/UL measurements are derived by averaging per-DRB per-slice per-UE measurements across all configured DRBs of a slice in use by the concerned UE.

Proposal 4: RAN3 to agree that per-slice per-UE Average Packet Delay DL/UL measurements are derived by averaging per-DRB per-slice per-UE measurements across all configured DRBs of a slice in use by the concerned UE.

The TP for the XnAP BLCR in [14] addresses the above proposals.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Observation 1: The FFS pertaining the definition of the Slice Average Packet Loss DL IE depends on the feedback from SA5 related to whether it is feasible to measure the packet loss in DL (and in UL as well) at PDCP level for both split and non-split architecture.

Proposal 1: RAN3 to agree to postpone the discussion on the FFS pertaining the definition of the Slice Average Packet Loss DL IE once RAN3 receives the feedback from SA5 on the feasibility to measure the packet loss in DL (and in UL as well) at PDCP level for both split and non-split architecture.

Observation 2: SA5 confirmed that Average UE Throughput DL/UL in TS 28.558 clauses 6.3.1.4.1/6.3.1.4.2 are per-UE measurements, originating from corresponding per-DRB measurements being aggregated to reflect a per-UE throughput.

Observation 3: SA5 already specified in TS 28.558 that UL/DL throughput measurements have already been defined with a per-slice per-UE granularity.

Proposal 2: RAN3 to discuss whether, for the UE throughput DL/UL, either the current measurements definitions in TS 28.558 (already available with a per-slice per-UE granularity) can be referred for the Slice Average UE Throughput DL/UL or wait for the feasibility check from SA5 on PDCP-level throughput measurements.

Observation 4: The packet delay DL/UL measurements are defined with a per-DRB per-UE measurements in TS 38.314.

Observation 5: The packet delay DL/UL measurements can be provided also per-slice as specified in TS 28.558.

Proposal 3: RAN3 to confirm the understanding that the packet delay measurements which constitute the overall packet delay DL/UL can be measured with a per-DRB per-slice per-UE granularity.

Proposal 4: RAN3 to agree that per-slice per-UE Average Packet Delay DL/UL measurements are derived by averaging per-DRB per-slice per-UE measurements across all configured DRBs of a slice in use by the concerned UE.

The TP for the XnAP BLCR in [14] addresses the above proposals.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- R3-252504, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital

- R3-252436, LS on clarification on UE performance measurement at PDCP level, Rel-19, RAN3 (ZTE Corporation)

- R3-253023 (S5-252056), Reply LS on per-UE UE performance metrics (Response to R3-247775), Rel-19, SA5 (NEC, Intel, Ericsson)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-19

- 3GPP TS 38.314, NR; Layer 2 measurements, Rel-18

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), Rel-18

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1.doc)

- R3-253490, (TP for AI/ML BLCR to TS 38.423) XnAP enhancements for AI/ML-based Network Slicing, Huawei, Jio Platforms, Deutsche Telekom