3GPP TSG-RAN WG3 Meeting #128	R3-253490

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	(TP for AI/ML BLCR to TS 38.423) XnAP enhancements for AI/ML-based Network Slicing

Source: 	Huawei, Jio Platforms, Deutsche Telekom

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

This is a TP for the AI/ML BLCR to TS 38.423 [1] reflecting proposals in our paper [2].

# 2. Reference

- R3-252504, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital

- R3-253488, Discussion on per slice UE performance in AI/ML-based Network Slicing, Huawei, Jio Platforms, Deutsche Telekom

# 3. TP for AI/ML BLCR to TS 38.423 (based on R3-252504)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x2	Slice UE Performance

This IE indicates per Slice UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice (details are FFS) |
| >>S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 | Average UE Throughput DL and Average UE Throughput UL refer to measurements as defined in TS 28.558 [XX] clauses 6.3.1.4.1 and 6.3.1.4.2. Average Packet Delay DL and Average Packet Delay UL are derived by averaging per DRB measurements across all configured DRBs of the S-NSSAI in use by the concerned UE. [It is FFS whether and how the Slice Average Packet Loss DL IE is defined] |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>