3GPP TSG-RAN WG3 Meeting #128	R3-253491

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	Discussion on AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei, Orange, Deutsche Telekom, Rakuten

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based CCO (Coverage and Capacity Optimization).

This contribution analyzes further the technical enablers for the AI/ML-based CCO use case to be introduced over the Xn and F1 interfaces, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting the following agreements and open issues were achieved [4]:

Timing information: The maximum value of the Time interval for predicted CCO issue and future CCO state is 60s. Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn? Feedback information for AI/ML-based CCO: Legacy UE performance measurement metrics can be reused for CCO. Whether/what additional UE performance measurement metrics is needed? Assistance information (CUDU) for deriving the matching future CCO state: For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state? Option1: no need Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU. Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

We further discuss here the FFS aspects above from the previous RAN3 meeting.

## 2.1 Timing information for AI/ML-based CCO

In previous RAN3 meetings it was agreed to specify separate timing information for predicted CCO issue and future CCO state, and also definitions and encoding for these two pieces of information were agreed [4][6][7]:

Timing information: Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen. Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1. Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn. Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds. The maximum value of the Time interval for predicted CCO issue and future CCO state is 60s.

while, in the last RAN3 meeting, as part of the discussion on the timing aspects, there is still an open point as follows [4]:

Timing information: Timing information for predicted CCO issue is NOT needed to exchange over Xn? Whether the predicted CCO issue and /or future CCO state can be updated over Xn?

We will start our discussion on timing aspects by focusing on the first part of the FFS on timing information above from the last meeting first, i.e., Timing information for predicted CCO issue is NOT needed to exchange over Xn?.

The timing information associated to the predicted CCO issue (i.e., Time for Predicted CCO Issue IE in the F1AP BLCR [7]) is transferred from the gNB-CU to the gNB-DU so that the gNB-DU is made aware of the fact that the new CCO state to cope with the predicted CCO issue is to be applied in the future and not immediately because the CCO issue did not occur yet. Conversely, the timing information of the future CCO state (i.e., Time for Future Coverage Modification IE in the F1AP BLCR [8]) is sent from the gNB-DU to the gNB-CU so that the gNB-CU will know when the future CCO state will be activated so to prevent (or limit at an early stage) the occurrence of the CCO issue. It should be straightforward to state that the time information of the future CCO state is lower than (or equal to) the time for predicted CCO issue, as it is natural to assume that once a predicted CCO issue is known to occur at a certain future time instant, the gNB-DU should derive a future CCO state to be activated before the occurrence of the predicted CCO issue, in order to avoid the CCO issue itself.

Observation 1: A reasonable implementation of the AI/ML-based CCO assumes that the Time for Future Coverage Modification is lower than the Time for Predicted CCO Issue.

Concerning the XnAP signalling, RAN3 agreed to transfer the timing information of the future CCO state (i.e., Time for Future Coverage Modification IE in the XnAP BLCR [11]) over Xn, within the NG-RAN CONFIGURATION UPDATE message. However, it needs to be further discussed whether also the time for predicted CCO issue (also indicated as time information of the coverage modification cause in the FFS from RAN3#126 meeting) needs to be transferred over Xn as well.

Let us assume that we have a split gNB1 (gNB-CU1 and gNB-DU1) where the CCO issue is predicted (in gNB-CU1), and a neighbor split gNB2 (gNB-CU2 and gNB-DU2) that is also impacted by this predicted CCO issue.

In order for gNB(-DU)2 to take proper counter-measures following the future CCO state derived at gNB(-DU)1 side, and based on the F1 impacts captured in TR 38.743, gNB-CU2 shall send to gNB-DU2 a predicted CCO issue (including predicted affected cells and SSBs of gNB(-DU)1) and Time for Predicted CCO Issue.

In our view the predicted CCO issue signalled by gNB-CU2 to gNB-DU2 is not a new issue but it is the same issue predicted by gNB-CU1 and signalled to gNB-CU2 as the coverage modification cause over Xn, so that the gNB-DU2 can determine its own future CCO state which matches the future CCO state previously determined by gNB-DU1. This is as in the legacy CCO, where a CCO issue detected by gNB-CU1 is also signalled to gNB-CU2 as the coverage modification cause over Xn and, in turn, it is signalled to gNB-DU2 by gNB-CU2 over F1.

Proposal 1: RAN3 to agree that the predicted CCO issue signalled by gNB-CU2 to gNB-DU2 is the same issue predicted by gNB-CU1 and signalled to gNB-CU2 as the coverage modification cause over Xn.

Therefore, gNB-CU2 needs to receive over Xn the predicted coverage modification cause (reflecting the predicted CCO issue at gNB1), the future CCO state and the Time for Future Coverage Modification both at gNB1, and the Time for Predicted CCO Issue (indicating when the CCO issue is predicted to occur). The first three pieces of information have already been captured in the XnAP BLCR in [11]. The above Proposal 1 also addresses the following open point from the last RAN3 meeting [4], where in our view Option2 is the most straightforward solution to allow the neighbor gNB to determine a matching future CCO state.

Assistance information (CUDU) for deriving the matching future CCO state: For the receiving side, whether gNB-CU can also provide to gNB-DU a future CCO state as assistance information to derive a matching future CCO state? Option1: no need Opton2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU. Option3: gNB-CU generates a recommend further CCO state and send to gNB-DU.

Proposal 2: RAN3 to agree that the gNB-CU2 directly forwards to gNB-DU2 over F1 the future CCO state of the neighbor gNB-DU1 received over Xn.

Another reason for which we think gNB-CU2 needs to know the Time for Predicted CCO issue is because gNB-CU2 has  to inform gNB-DU2 about a predicted CCO issue type and the corresponding timing information, so that gNB-DU2 can determine its future CCO state (matching the future CCO state of gNB-DU1) within a proper time interval starting from the point in time where the future CCO state in gNB-DU1 is activated (i.e., Time for Future Coverage Modification in gNB-DU1) up to the point in time where the CCO issue in gNB-CU1 is predicted to occur, as depicted in Figure 1.

Figure 1 - Time relation between the Time for Future Coverage Modification (from gNB-DU1) and Time for Predicted CCO issue (from gNB-CU1).

Without the information on the point in time where the CCO issue in gNB-CU1 is predicted to occur (i.e., Time for Predicted CCO issue t0 in Figure 1) the case as shown in Figure 2 may happen.

Figure 2 – Scenario where the Time for Predicted CCO issue is not known at gNB-DU2.

Specifically, the gNB-DU2 may determine a matching future coverage state to be activated at a point in time (i.e., Time for Future Cell Coverage State t2 in Figure 2) that is after the point in time in the future where the CCO issue is predicted to occur (i.e., Time for Predicted CCO issue t0 in Figure 2), simply because the gNB-DU2 knows that there will be a CCO issue in the future but it does not know when. This means that the whole AI/ML optimization for CCO is basically useless, since likely the CCO issue actually occurred and it was not possible to timely adapt the coverage states of affected cells/SSBs in gNB-DU1 and gNB-DU2 to avoid the issue.

We acknowledge that, in principle, it would be better that the matching future coverage state in gNB-DU2 is activated at the same point in time where the future coverage state in gNB-DU1 is activated. But we also think that this poses a strict synchronization requirement (and hence it adds complexity to the whole mechanism) that provides negligible benefits. In fact, it should be discussed whether the negative impact on performance (network and UE) deriving from having the matching future coverage state in gNB-DU2 being activated after the activation time of the future coverage state in gNB-DU1 (but before the predicted CCO issue time, to avoid the previous scenario) is significant considering that the mismatch between the activation times would be really short in time, and very likely to be solved well before the occurrence of the predicted CCO issue, i.e., before the predicted CCO issue time. In our view, having the future coverage states activated at the same time should be more a “nice-to-have” rather than a strict design requirement, as what it matters is that, at the time the issue is predicted to occur, the involved DUs have rearranged their own coverage configurations properly (to avoid the actual occurrence of the CCO issue).

In our view, the above scenario in Figure 2 can be avoided if gNB-DU2 knows the Time for Predicted CCO issue in gNB-CU1 in addition to the Time for Future Coverage Modification in gNB-DU1. A reasonable implementation, meaning that the Time for Future Coverage Modification in gNB-DU1 is always lower than the Time for Predicted CCO Issue in gNB-CU1, properly allows gNB-DU2 to set the time boundaries within which it needs to adapt to coverage changes of affected cells in gNB-DU1.

Observation 2: The Time for Predicted CCO issue is needed to be known at gNB-CU2 in order for gNB-DU2 to set the time boundaries within which it needs to determine the matching coverage change and avoid the predicted CCO issue to actually occur.

Observation 3: Having “misaligned” activation of future coverage states at gNB-DU1 and at gNB-DU2, if occurs, is a temporary situation that will be solved in a short timing horizon, therefore the impact on UE and network performance is negligible.

In the last meeting, in contribution [14], it was stated that by delaying the application of matching coverage configuration at gNB-DU2, the gNB-CU1 will not know when gNB-DU2 has applied the matching coverage state. As a consequence, the mechanism of collecting and reporting of feedback information to gNB-CU1 that may be required to validate the outcome of the CCO issue prediction (selection of Future CCO state) will be negatively impacted as gNB-CU1 cannot determine the time of change of coverage configuration at gNB-DU2. In our view, this statement is based on a couple of assumptions that still need to be discussed in RAN3: firstly, it is yet unclear what we want to validate by means of feedback information, that is, whether it is the predicted CCO issue or the future coverage state, and contribution [14] assumes that it is the future CCO state that is validated by such feedback information. Secondly, it seems to us that gNB-DU2 is forced to apply the matching coverage state for its cells/SSBs at the same time instant at which the future coverage state is activated by the gNB-DU1. As previously stated, this represents an unnecessary implementation constraint for gNB-DU2 because what matters is that the future coverage states at gNB-DU1 and gNB-DU2 are already activated when reaching the point in time where the CCO issue is expected to occur.

Observation 4: Whether the mechanism for collecting feedback information at gNB-CU2 is impacted by a delayed application of the matching coverage configuration at gNB-DU2 is not clear, as it is based on assumptions – feedback information to validate the future CCO state and activation time of matching future coverage state in gNB-DU2 to be exactly the same as for the future coverage state in gNB-DU1 – that still need to be tackled in RAN3.

Moreover, the gNB-CU2 would benefit from knowing the Time for Predicted CCO Issue in order to understand the point in time after which it needs to start collecting metrics to be used as feedback information. It is our understanding that neighbor nodes need to collect and report feedback information – the exact metrics are yet to be discussed in RAN3 – to be transferred back to the gNB-CU1 immediately after the predicted CCO issue was expected to occur. We will discuss feedback information more in detail in Section 2.2.

Observation 5: The Time for Predicted CCO issue is needed to be known at gNB-CU2 in order for gNB-CU2 to understand the point in time after which it needs to start collecting feedback information.

In the last meeting, contribution [15] highlighted another reason for which the knowledge of the Time for Predicted CCO issue at the gNB-CU2 side is beneficial: if multiple predicted CCO issues from multiple gNB-CUs are provided to the same gNB-CU2 (acting as neighbour node), then receiving the time information related to each predicted CCO issue from each gNB-CU over Xn can be used by the gNB-CU2 to understand which predicted CCO issue has precedence over others, i.e., when it is more appropriate to apply a certain future CCO state (to cope with the predicted CCO issue in the “most near” future) or another.

Observation 6: The Time for Predicted CCO issue is needed to be known at gNB-CU2 in order for gNB-CU2 to cope with the most imminent predicted CCO issue (and gNB-DU2 to take corresponding actions) in case multiple predicted CCO issues from multiple gNB-CUs are received simultaneously.

Based on the above observations, we propose the following:

Proposal 2: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn also the Time for Predicted CCO Issue.

We now discuss the second part of the open point on timing information from the last meeting, i.e., Whether the predicted CCO issue and /or future CCO state can be updated over Xn?. There could be at least the following scenarios requiring a certain predicted CCO issue (and corresponding future CCO state) to be updated:

- a certain CCO issue is predicted by gNB-CU1 but, before its occurrence, an actual CCO issue is detected by gNB-CU1 itself;

- a certain CCO issue is predicted by gNB-CU1 but, before its occurrence, an updated CCO issue is predicted by gNB-CU1 itself.

For case a), we think that the detection of an actual CCO issue by gNB-CU1 should be prioritized over a predicted CCO issue, meaning that a ‘real’ issue should be addressed as soon as possible to avoid negative impacts on both UE and network performance. This implies that the new CCO state determined by gNB-DU1 to cope with the detected CCO issue in gNB-CU1 renders the network situation that led to the predicted CCO issue invalid, hence the predicted CCO should also be considered invalid as well. The reception by the gNB-DU1 over F1 (and by gNB-CU2 over Xn) of a detected CCO issue should be considered as an implicit indication that the previous predicted CCO issue needs be discarded.

For case b), instead, this could be dependent on the accuracy of the prediction, and hence model-dependent. Also in this case, we think that the reception by the gNB-DU1 over F1 (and by gNB-CU2 over Xn) of an updated predicted CCO issue should be considered as an implicit indication that the previous predicted CCO issue needs be discarded.

Proposal 3: RAN3 to agree that if either an actual CCO issue or an updated predicted CCO issue is received over F1 and Xn before the occurrence of a CCO issue previously predicted, this implicitly indicates that the previous predicted CCO issue needs to be discarded.

## 2.2 Feedback information AI/ML-based CCO

In RAN3#125bis meeting the following open point was in the meeting minutes [7]:

Information to validate the prediction is needed.

The above open issue is related to the discussion RAN3 had on the feedback information for the AI/ML-based CCO use case, in particular on UE performance feedback which is one of the items captured in TR 38.743 clause 4.2.2.4 along with measured radio resource status and SON reports [3]. Before discussing which kind of measurements (e.g., legacy per UE UE performance, radio-level UE measurements, network-level UE performance, etc.) should the neighbour node report to the node hosting the AI/ML model, it should be clarified in RAN3 what is intended to be validated by such report. As can be seen from the text captured as open point, such text is very generic as it refers to “prediction” without clarifying whether it is the “CCO prediction”, the “future CCO state” (if derived via AI/ML in gNB-DU, which is possible only by implementation) or a combination of the two.

In our view, we should base the discussion according to the definition of “feedback” as captured in TR 37.817 [12]:

## 4.2 Functional Framework Figure 4.2-1. Functional Framework for RAN Intelligence This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5. **** Skip non-relevant text **** - Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself. - Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

Based on the above it is clear that the feedback information is strictly related to the AI/ML model, and for the AI/ML-based CCO the only AI/ML model which is supported by the standard is the one hosted by the gNB(-CU) which predicts the CCO issue. As a consequence, the feedback information that should be discussed in RAN3 should be for evaluating the CCO issue prediction. There were comments from some companies in the last RAN3 meeting that it is not possible to evaluate the predicted CCO issue, and that only the future CCO state can be assessed. However, in our understanding, when deriving the future CCO states for both local and neighbour nodes, at the time the predicted CCO issue was expected to occur these future CCO state should already have been activated to prevent the issue. Therefore, if the issue does not occur – for example, by checking whether UE performance collected just after the Time for Predicted CCO issue is still “good” – this is already a feedback information to evaluate the CCO issue prediction. This suggests that the model correctly predicted that at a certain time the issue was expected but, due to the proactive CCO mechanism, it was avoided. If UE performance is “bad”, then it means that the model provided inaccurate predictions (in terms of CCO issue type and/or Time for Predicted CCO issue).

Proposal 4: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

Moreover, we think it would be beneficial for AI/ML model’s monitoring purposes that the local node could receive from the neighbour node information related to its current CCO state just after the Time for Predicted CCO Issue, that is, after the previously predicted CCO issue was expected to occur and the corresponding future CCO state was actually applied by the local node at the Time for Future Coverage Modification to avoid such predicted CCO issue. This would allow the model in the gNB(-CU) to have a clearer overall view of the AI/ML-based CCO mechanism and to fine-tune the model accordingly.

Proposal 5: RAN3 to agree to transfer the current CCO state of the neighbour gNB to the local node over the Xn interface as a feedback information. No specification impact if foreseen.

In RAN3#127, contribution [16] proposed to consider UE-measured radio-level metrics as part of feedback information. The reason is that, regardless of whether the predicted issue type is “coverage” or “cell edge capacity”, the action that is needed to cope with it translates into a changed in coverage, and when a cell/SSB shrinks and another cell/SSB expands their respective coverages, UEs in such cells/SSBs will experience a change in the signal strength and/or quality of the radio channel. Whether the CCO changes at radio level are beneficial or not in terms of coverage for the UE may not be reflected by UE performance metrics as specified in Rel-18 because, even though such metrics for UEs in cells/SSBs affected by the CCO issue differ, UE performance metrics may be influenced by other factors, e.g., user behavior or RRM policy of the UE.

By focusing on UEs that are handed over between cells affected by a CCO state change, a neighbor gNB, i.e., gNB(-CU)2 may be requested to collect and report to the local gNB which predicted the CCO issue, i.e., gNB(-CU)1 over Xn the radio-level measurements (RSRP, RSRQ, SINR) associated to those UEs which have been handed over from gNB(-CU)1 to gNB(-CU)2. In this way, gNB(-CU)1 may use such measurements to assess whether the predicted CCO issue was prevented by means of the CCO State changes previously performed by involved (split) gNBs.

We think that UE-measured radio-level metrics may be useful for the proper validation of the predicted CCO issue, so we propose to introduce them by introducing a new IE UE Measured Radio Performance in XnAP.

Proposal 6: RAN3 to agree to introduce a new IE UE Measured Radio Performance in XnAP in order to also exchange UE-measured radio-level metrics (RSRP, RSRQ, SINR) corresponding to those UEs handed over between cells affected by a CCO state change.

The existing Data Collection Report Initiation procedure can be extended to request the reporting of UE-measured radio-level metrics, as well as for partial reporting aspects.

TP to AI/ML BLCR to TS 38.423 and TP to AI/ML BLCR to TS 38.473 reflecting the above proposals can be found in [17] and [18], respectively.

# 3. Conclusion

Based on the above discussion, we have the following proposals:

Timing information for AI/ML-based CCO

Observation 1: A reasonable implementation of the AI/ML-based CCO assumes that the Time for Future Coverage Modification is lower than the Time for Predicted CCO Issue.

Proposal 1: RAN3 to agree that the predicted CCO issue signalled by gNB-CU2 to gNB-DU2 is the same issue predicted by gNB-CU1 and signalled to gNB-CU2 as the coverage modification cause over Xn.

Proposal 2: RAN3 to agree that the gNB-CU2 directly forwards to gNB-DU2 over F1 the future CCO state of the neighbor gNB-DU1 received over Xn.

Observation 2: The Time for Predicted CCO issue is needed to be known at gNB-CU2 in order for gNB-DU2 to set the time boundaries within which it needs to determine the matching coverage change and avoid the predicted CCO issue to actually occur.

Observation 3: Having “misaligned” activation of future coverage states at gNB-DU1 and at gNB-DU2, if occurs, is a temporary situation that will be solved in a short timing horizon, therefore the impact on UE and network performance is negligible.

Observation 4: Whether the mechanism for collecting feedback information at gNB-CU2 is impacted by a delayed application of the matching coverage configuration at gNB-DU2 is not clear, as it is based on assumptions – feedback information to validate the future CCO state and activation time of matching future coverage state in gNB-DU2 to be exactly the same as for the future coverage state in gNB-DU1 – that still need to be tackled in RAN3.

Observation 5: The Time for Predicted CCO issue is needed to be known at gNB-CU2 in order for gNB-CU2 to understand the point in time after which it needs to start collecting feedback information.

Observation 6: The Time for Predicted CCO issue is needed to be known at gNB-CU2 in order for gNB-CU2 to cope with the most imminent predicted CCO issue (and gNB-DU2 to take corresponding actions) in case multiple predicted CCO issues from multiple gNB-CUs are received simultaneously.

Proposal 2: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn also the Time for Predicted CCO Issue.

Proposal 3: RAN3 to agree that if either an actual CCO issue or an updated predicted CCO issue is received over F1 and Xn before the occurrence of a CCO issue previously predicted, this implicitly indicates that the previous predicted CCO issue needs to be discarded.

On feedback information AI/ML-based CCO

Proposal 4: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

Proposal 5: RAN3 to agree to transfer the current CCO state of the neighbour gNB to the local node over the Xn interface as a feedback information. No specification impact if foreseen.

Proposal 6: RAN3 to agree to introduce a new IE UE Measured Radio Performance in XnAP in order to also exchange UE-measured radio-level metrics (RSRP, RSRQ, SINR) corresponding to those UEs handed over between cells affected by a CCO state change.

TP to AI/ML BLCR to TS 38.423 and TP to AI/ML BLCR to TS 38.473 can be found in [17] and [18], respectively.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-252505, (BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN, Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei

- 3GPP TS 38.401 “NG-RAN; Architecture description”, Rel-19

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_EOM1.doc)

- R3-252504, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital

- 3GPP TR 37.817 “Study on enhancement for Data Collection for NR and EN-DC”, Rel-17

- R3-243467, (TP to TR 38.743) AI/ML-based Coverage and Capacity Optimization, Huawei

- R3-250533, Further discussion on signalling for AI/ML-based CCO, Nokia

- R3-250468, AI/ML enabled CCO across multiple nodes, Ericsson, Jio Platforms (JPL), InterDigital

- R3-250465, Further discussions on AI/ML enabled CCO, Ericsson, Jio Platforms (JPL), InterDigital

- R3-253492, (TP for AI/ML BLCR to TS 38.423) XnAP enhancements for AI/ML-based Coverage and Capacity Optimization, Huawei, Orange, Deutsche Telekom

- R3-253493, (TP for AI/ML BLCR to TS 38.473) F1AP enhancements for AI/ML-based Coverage and Capacity Optimization, Huawei, Orange, Deutsche Telekom