3GPP TSG-RAN WG3 Meeting #128	R3-253502

St. Julian’s, Malta, 19th – 23rd May, 2025

Title: 	Discussion on the Reply LS from SA5 on Continuous MDT for AI/ML

Source: 	Huawei, FiberCop, Jio Platforms

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 started discussing the objectives of the new Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to Continuous MDT collection targeting the same UE across RRC states.

This contribution discussed the topic of Continuous MDT in light of the LS exchange involving RAN3 and SA5, and provides proposals for discussion.

# 2. Background

The main motivation for introducing Continuous MDT arose during the Rel-18 WI “AI/ML for NG-RAN” [4], where the issue on how to collect measurements from UE(s) to be used as datasets for AI/ML models’ (re-)training was initially discussed. Back then it was argued that, for the AI/ML model (re-)training to be successful, the MDT data collected from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. It was claimed that, when the UE enters to connected state from idle/inactive or when the UE is subject to connected mode mobility (handover), consecutive (without time gaps) information would not be collected, with the consequence that the data gathered from the UE would not constitute a valid training dataset for AI/ML models’ (re-)training. By means of the Continuous MDT, that is, an enhancement of the current MDT mechanism suited for AI/ML models’ (re-)training, the continuous collection of MDT data from the same UE during the same RRC state as well as across RRC states can be enabled.

During the Rel-19 SI, for Continuous MDT, the use case description and problems to be solved have been acknowledged and captured in TR 38.743 clause 5.4 as follows [3]:

## 5.4 Continuous MDT collection targeting the same UE across RRC states ### 5.4.1 Use Case Description The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection. The Data Collection continuity in this scenario can be split into two tasks as below: - Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states. - Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE. ### 5.4.2 Potential Standard impacts Potential solutions addressing the problems above can be discussed during normative phase.

In RAN3#126 an initial attempt to down-select solutions for Continuous MDT was made and RAN3 agreed that a solution for Continuous MDT needs to be applicable not only to Rel-19 UEs but also to legacy UEs. Moreover, RAN3 agreed to send a LS to SA5 aiming at explaining which is the problem to be solved by this feature, which are the solutions’ requirements and a description of these solutions in order for RAN3 to then select a solution based on SA5 feedback. The outcome of RAN3#126 for Continuous MDT is report as follows [5]:

In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs. Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS. Identification of requirements and description of problem statement, followed by LS drafting to be continued.

As per the FFS above, the LS drafting was expected to occur in RAN3#127 meeting where the above agreements have been taken into account along with the following agreement from RAN3#125bis [6]:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Eventually RAN3 agreed to send the LS in [8] to SA5 to introduce the new concept of Continuous MDT, explain which problems needs to be addressed to enable Continuous MDT (i.e., measurement continuity across RRC state changes and during connected mode mobility, as well as trace correlation of MDT reports provided by the same UE) and which are the requirements for a Continuous MDT solution from RAN3 perspective. The ACTION to SA5 was to check whether it if feasible to fulfil the requirements for Continuous MDT and solve the problem statements outlined, along with providing some guidance from SA5 on possible solutions for Continuous MDT for further RAN3 discussions.

SA5 replied to the RAN3 LS as in [8] with a set of questions for clarification related to the Measurement Continuity Problem and corresponding OAM requirements, as well as to the Trace Correlation Problem.

In the next section we provide our views on the content of the reply LS from SA5 in on Continuous MDT.

# 3. Discussion

## 3.1 On the Measurement Continuity Problem

In their reply LS in [8], for the Measurement Continuity Problem SA5 commented on three main aspects, i.e.:

- means to be provided by OAM to NG-RAN so that NG-RAN can identify that the management based MDT is for Continuous MDT;

- the formulation of the Measurement Continuity Problem across different UE’s RRC states;

- the requirement related to the fact that the Continuous MDT solution should minimize the impact with respect to the current management based MDT framework.

Among those aspects, only a. and b. require more input from RAN3, since for aspect c. SA5 replied by agreeing such requirement.

Observation 1: SA5 indicated in the reply LS that they agree with the requirement that the Continuous MDT solution should minimize the impact with respect to the current management based MDT framework. No further RAN3 input is needed.

For aspect a., below an excerpt from the SA5 reply LS is reported below [8]:

RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.” Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

SA5 basically states that the OAM can provide a management based MDT configuration which is specific for Continuous MDT, however the configuration details are unclear and not easy to be derived by SA5. In our view this is because the configuration details, e.g., Trace Reference (TR), Job Type, etc., highly depends on the solutions so far proposed in RAN3 for Continuous MDT. In the LS that RAN3 sent to SA5 in [7] we did not mention any solutions description which, in our view, would have helped SA5 a lot in discussing how to derive the configuration details specific for the management based MDT to be used for Continuous MDT, assuming that SA5 had already performed some kind of down-selection of solutions which, by the way, should have been done as part of the RAN3 ACTION to provide guidance on possible solutions on Continuous MDT (which is missing the reply LS from SA5).

Observation 2: The details of the management based MDT configuration which is specific for Continuous MDT cannot be derived by SA5 without any discussion in SA5 on the solutions currently proposed in RAN3.

Observation 3: SA5 was not able to provide any guidance on possible solutions for Continuous MDT, as they had no idea about solutions RAN3 came up with so far.

During the discussion on Continuous MDT in past RAN3 meetings it was argued by some companies that the details of the management based MDT configuration specific for Continuous MDT cannot be discussed in RAN3, as they completely fall in the realm of SA5. But again, such details depend on the proposed solutions for Continuous MDT, however it could be stated that the OAM needs to provide the gNB with a configuration encompassing both Immediate MDT and Logged MDT measurements and indicates that this configuration is used for Continuous MDT.

Observation 4: OAM requirements to be provided to SA5 should include at least the one based on which the OAM needs to provide the gNB with a configuration encompassing both Immediate MDT and Logged MDT measurements and indicates that this configuration is used for Continuous MDT.

Given the strong dependency on SA5 for the technical solution, we think that it is SA5 the proper place where the solution down-selection should occur. Hence, in light of the above observations, we propose the following:

Proposal 1: RAN3 to agree to send a LS to SA5 (reply to R3-253024) including a summary of solutions currently proposed in RAN3 and fulfilling the agreements already taken in past RAN3 meetings.

A summary of solutions is provided in the following Table 1, which we propose to include in the reply LS to SA5:

Table 1 - Network-based solutions for Continuous MDT.

| Solution ID | Description | Measurement Continuity | Trace correlation | Pros. & Cons. |
| --- | --- | --- | --- | --- |
| Solution 1 (R3-247537) | - OAM configures Management Based MDT to the gNBs taking part in Continuous MDT, including TRs dedicated to Continuous MDT; - Serving gNB selects a UE and configures TRSR which is reassigned to the UE across RRC states and during connected mobility. Involved domains: RAN and OAM | - explicit indication in handover signalling that UE is selected for Continuous MDT (e.g., assigned TR/TRSR); - presence of TR/TRSR in the Logged MDT report sent to the serving gNB after idle-to-connected transition. | Reusing part/all of the identifiers in the Trace Record header, e.g. the TRSR, for the whole duration of the Continuous MDT trace. | Pros: only RAN and OAM involved. Cons: - Dedicated TRs for Continuous MDT; - gNB coordination in TRSR assignment to enable trace correlation. |
| Solution 2 (R3-247623) | - OAM configures Management Based MDT at a gNB; - At connected-to-idle transition the MDT configuration is stored by the serving gNB in 5GC/AMF; - Upon idle-to-connected transition the 5GC/AMF indicates to the (new) serving gNB the MDT configuration of the UE along with an identifier that the UE needs to be selected again for management-based immediate MDT; - During handover, the source gNB sends the Immediate MDT configuration to the target gNB as part of the handover preparation. Involved domains: RAN, OAM and 5GC/AMF | - target gNB receives at handover the UE’s MDT configuration used in the source gNB; - (new) serving gNB receives the UE’s MDT configuration previously stored in the 5GC/AMF. | Ensured by the identifier assigned and maintained by the 5GC/AMF; the identifier is assigned to the UE as part of the MDT configuration, and the serving gNB includes this identifier in the trace files sent to the TCE. | Pros: no need for gNB coordination in TRSR assignment. Cons: - AMF is impacted as it needs to define, assign and maintain an ID for the UE; - security issues due to the ID for the UE. |
| Solution 3 (R3-247419) | - OAM configures Management Based MDT with a new Job Type “Immediate MDT and Logged MDT” and assigns a unique TR to this MDT session (valid for all UEs in the area of gNBs where this configuration is provided); - The gNB selects a UE and assigns a TRSR which is UE-specific; - At connected-to-idle transition the MDT configuration (at least Job Type and TRSR) is sent by the serving gNB to the 5GC/AMF which stores this information; when the UE resumes the (new) serving gNB receives from 5GC/AMF the stored MDT configuration; the presence of the MDT configuration is an indication for the (new) serving gNB that the UE needs to be reselected for Continuous MDT - At handover, the source gNB sends the MDT configuration (at least Job Type and TRSR) to the target gNB; the presence of the MDT configuration is an indication for the target gNB that the UE needs to be reselected for Continuous MDT. Involved domains: RAN, OAM and 5GC/AMF | - source gNB provides the target gNB with MDT configuration which is an indication for the target gNB to re-select the UE for Continuous MDT; - serving gNB receiving from 5GC/AMF the MDT configuration previously stored in the 5GC/AMF by the old serving gNB, so that the same UE can be re-selected. | Ensured by correlating at TCE the MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE. | Pros: no need for gNB coordination in TRSR assignment. TRSR conflicts at the target gNB (or new serving gNB) are solved by keeping the mapping relationship of TRSRs allocated to the UE (and sent to the TCE). Cons: OAM impacted due to the need to define a new Job Type. |
| Solution 4 (R3-247393) | - OAM configures gNBs in an area with same immediate MDT configuration (TR1) and logged MDT configuration (TR2); - A gNB chooses UE and configures the UE for Immediate MDT and Logged MDT separately (with separate TRSRs, i.e. TRSR1 and TRSR2); - At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT; - At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the Immediate MDT. Involved domains: RAN and OAM | Same as in Solution 2 | Ensured by correlating at TCE the Immediate MDT report (TR1/TRSR1) which includes also TR2/TRSR2 of the Logged MDT session with the Logged MDT report (TR2/TRSR2). | Pros: no need for gNB coordination in TRSR assignment. Cons: Multiple TRSRs are assigned to the same UE even if there are no TRSR conflicts. |

For aspect b., that is, clarification on the “Measurement Continuity Problem across different UE’s RRC states” formulation, the following is excerpted from the SA5 reply LS in [8]:

RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.” Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

We think that, in the reply LS to SA5 as proposed in Proposal 1, the description as in clause 5.4 of TR 38.743 [3] can be referenced and complemented by a minimal call flow showing the interaction among the involved network entities, as proposed in Figure 1, which has no relation with a particular solution for Continuous MDT proposed in RAN3 so far.

Figure 1 - Call flow showing the Measurement Continuity Problem across different UE’s RRC states.

- The OAM provides gNBs in a certain area with a Continuous MDT configuration identified by parameters including Trace Reference(s), Job Type(s), etc.;

- gNB1 selects a connected mode UE (UE1) for Continuous MDT and assigns Trace Recording Session Reference(s) to UE1;

- gNB1 configures UE1 with MDT measurements (both immediate and logged measurements) according to the Continuous MDT configuration as received in step 1.;

- UE1 performs Immediate MDT measurements according to the configuration received from gNB1 in step 3. and reports such measurements to gNB1;

- gNB1 sends to TCE a Trace Record file containing Immediate MDT measurements of UE1 including mean(s) to enable trace correlation at TCE (which “mean(s)” is FFS);

- UE1 moves to RRC idle/inactive and starts Logged MDT measurements according to the configuration received from gNB1 in step 3.;

- After some time, UE1 goes back to RRC connected under the coverage of a different gNB (gNB2);

NOTE: The call flow works also if UE1 goes back to RRC connected under the coverage of the same gNB (gNB1).

- Based on the definition of Continuous MDT, UE1 needs to be reselected by the new serving gNB2 (FFS how gNB2 understands that UE1 was previously selected by gNB1 for Continuous MDT) and it is assigned with Trace Recording Session Reference(s) (FFS how to solve the issue of TRSR(s) allocation conflict);

- gNB2 configures UE1 with MDT measurements (both immediate and logged measurements) according to the Continuous MDT configuration as received in step 1.;

- gNB2 retrieves the Logged MDT measurements from UE1;

- gNB1 sends to TCE a Trace Record file containing Logged MDT measurements of UE1 including mean(s) to enable trace correlation at TCE (which “mean(s)” is FFS);

- TCE correlates Trace Record files relying on “mean(s)” to enable trace correlation (which “mean(s)” is FFS).

Proposal 2: RAN3 to agree to include in the reply LS to SA5 a reference to clause 5.4 of TR 38.743 and the call flow in Figure 1 to clarify the scenario of “Measurement Continuity Problem across different UE’s RRC states”.

## 3.2 On the Trace Correlation Problem

In their reply LS in [8], for the Trace Correlation Problem SA5 commented as follows:

RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.” Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

In our understanding SA5 sees benefits in re-using at least the existing MDT configuration parameters, i.e., TR and TRSR, to enable trace correlation at TCE of the MDT measurements from the same UE. Based on this, and considering solutions as summarized in Table 1, we think that Solution 2 (as proposed in R3-247623) in Table 1 may already be down-selected by RAN3 since it relies on an identifier assigned and maintained by the 5GC/AMF for the trace correlation at TCE, hence not in line with the SA5 indications above.

Observation 5: Not all solutions for Continuous MDT proposed so far in RAN3 rely on at least TR and TRSR for the trace correlation of MDT measurements at TCE as indicated by SA5. Hence, it makes sense to provide only those solutions in line with the SA5 indication.

Proposal 3: RAN3 to agree to provide in the reply LS to SA5 a summary of Continuous MDT solutions proposed in RAN3 which rely on at least TR and TRSR for the trace correlation of MDT measurements at TCE.

On the specific request from SA5 to RAN3 to elaborate the details of how to avoid overlapping, it is our understanding that this refers to the possible conflict of TRSRs allocation where a UE, previously selected for Continuous MDT by the serving gNB (say gNB1) and hence been assigned with a certain TRSR, such UE is then subject to a mobility event resulting in this UE to be served by another gNB which, in turn, reselects the UE but cannot assign the same TRSR to this UE (as done by the old serving gNB) due to e.g. the TRSR been already assigned to another UE. If this is the case, then we proposed a solution where the new serving gNB assigns a new TRSR to the UE, keeps the mapping relationship of TRSRs allocated to the UE and provides it to TCE for the purpose of trace correlation at the TCE. However, we think it would be better to ask further clarifications to SA5 on what they mean with “overlapping”.

Observation 6: It may be unclear in RAN3 what SA5 intends with “overlapping”, but one possible interpretation could be related to a conflict in the allocation of the TRSR to the UE by the serving gNB after a mobility event.

Proposal 4: RAN3 to agree to discuss whether to include an interpretation of the “overlapping” aspect as raised by SA5 and ask SA5 confirmation, or ask clarifications to SA5 directly.

A draft reply LS to SA5 is provided in [9].

# 4. Conclusion

Based on the discussion in this paper, we provide the following proposals:

On the Measurement Continuity Problem across UE’s RRC state changes

Observation 1: SA5 indicated in the reply LS that they agree with the requirement that the Continuous MDT solution should minimize the impact with respect to the current management based MDT framework. No further RAN3 input is needed.

Observation 2: The details of the management based MDT configuration which is specific for Continuous MDT cannot be derived by SA5 without any discussion in SA5 on the solutions currently proposed in RAN3.

Observation 3: SA5 was not able to provide any guidance on possible solutions for Continuous MDT, as they had no idea about solutions RAN3 came up with so far.

Observation 4: OAM requirements to be provided to SA5 should include at least the one based on which the OAM needs to provide the gNB with a configuration encompassing both Immediate MDT and Logged MDT measurements and indicates that this configuration is used for Continuous MDT.

Proposal 1: RAN3 to agree to send a LS to SA5 (reply to R3-253024) including a summary of solutions currently proposed in RAN3 and fulfilling the agreements already taken in past RAN3 meetings.

Proposal 2: RAN3 to agree to include in the reply LS to SA5 a reference to clause 5.4 of TR 38.743 and the call flow in Figure 1 to clarify the scenario of “Measurement Continuity Problem across different UE’s RRC states”.

On the Trace Correlation Problem

Observation 5: Not all solutions for Continuous MDT proposed so far in RAN3 rely on at least TR and TRSR for the trace correlation of MDT measurements at TCE as indicated by SA5. Hence, it makes sense to provide only those solutions in line with the SA5 indication.

Proposal 3: RAN3 to agree to provide in the reply LS to SA5 a summary of Continuous MDT solutions proposed in RAN3 which rely on at least TR and TRSR for the trace correlation of MDT measurements at TCE.

A draft reply LS to SA5 is provided in [9].

# 5. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-250891, LS on Continuous MDT, RAN3 (Ericsson)

- R3-253024 (S5-252114), Reply LS to LS on Continuous MDT, Response to R3-250891, SA5 (Huawei, Nokia)

- R3-253503, [DRAFT] Reply LS from SA5 on Continuous MDT for AI/ML, Response to R3-253024 (S5-252114), Rel-19, Huawei, FiberCop, Jio Platforms