3GPP TSG-RAN WG3 Meeting #128									       R3-253505

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	20.2

Source:	Huawei

Title:	(TPs for AI/ML BLCR to TS 38.305, TS 38.455 and TS 38.413) Discussion on RAN3 impacts for NG-RAN node-assisted AI/ML positioning (Case 3a)

Document for:	Discussion and decision

# Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” – latest WID in [1]. To be specific, the discussion was on the specification support for AI/ML-based positioning accuracy enhancements.

This contribution focuses on AI/ML assisted positioning Case 3a – being the only case of AI/ML positioning that have RAN3 impacts, together with Direct AI/ML positioning Case 3b, discussed in a separate contribution [2] – and provides proposals based on the latest progress in RAN3.

# Discussion

In the following we further discuss aspects related to model training, model inference and model monitoring for AI/ML positioning Case 3a (i.e., NG-RAN node-assisted positioning with a gNB-side model).

In the last meeting RAN3 discussed technical aspects for Case 3a, and the following agreements and open issues were captured in the meeting minutes [3]:

An NRPPA based solution could be used to collect UE location information in an opportunistic way, i.e. from UEs that are being positioned. FFS on Whether to introduce an NGAP based procedure to collect UE location information in a proactive way, i.e. from UEs that are being served by the NG-RAN. (Updated Option 3 as in the LS)

Moreover, RAN3 agreed a TP for the TS 38.305 BLCR in [4] which includes a high-level description of the solution worked out in RAN3 based on the “opportunistic” data collection approach at the gNB, where UEs being already positioned are exploited to gather data for training purposes. RAN3 also discussed a possible solution for the “proactive” data collection at the gNB, where a positioning session may be triggered to gather training data. Due to the potential impacts on SA2, RAN3 agreed to send the LS in [5] to SA2 asking for feedback on both solutions (“opportunistic” and “proactive” data collection) for data collection for Case 3a, and whether the solution for the “proactive” data collection is feasible within the Rel-19 timeframe.

In order to assess the RAN3 impacts for supporting AI/ML positioning we will discuss model training, model inference and model monitoring aspects based on the latest progress in RAN1 (i.e., up to RAN1#120bis meeting).

## 2.1 Model training considerations

While waiting for the SA2 feedback on the proposed solutions for training data collection for Case 3a, it would be worthwhile to report the latest RAN1 status related to the AI/ML model training in the gNB. As per RAN1 agreements below, a training data sample is composed of two parts, a “measurement” (Part A in RAN1 discussions) and a “ground-truth label” (Part B in RAN1 discussions).

******** RAN1#116bis ******** Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: "Part A" and "Part B" terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. ******** RAN1#118bis ******** Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; FFS: details on how to associate quality indicator to timing information ******** RAN1#120 ******** Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. ******** RAN1#120bis ******** Agreement For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

In the agreed TP for the TS 38.305 BLCR in [4] it is mentioned that, to correlate Part A and Part B, some correlation information needs to be provided by the LMF when it provides information related to UE location (i.e., Part B) to the gNB in step 4. of the “opportunistic” data collection. The correlation information mentioned in step 4. includes the LMF Measurement ID and RAN Measurement ID. While checking RAN1 agreements, in RAN1#120 it was agreed to use time stamp of Part B (which, for Case 3a, is the transmitted information in RAN1#120 agreement).

Proposal 1: RAN3 to agree that, for Case 3a training data collection, the correlation of Part A and Part B relies also on the time stamp of Part B (provided by the LMF to the gNB).

A TP for the AI/ML BLCR to TS 38.305 in [8] reflecting Proposal 1 is in Annex 1.

RAN1 agreed in RAN1#120 meeting that the purpose for which a certain information is provided does not need to be indicated in the specifications. In our view, this is in line with the principle which drove the specification of “AI/ML for NG-RAN” since Rel-18 (refer to TS 38.300 clause 16.20.2). Therefore, it is suggested that the same principle is also considered during the discussions in RAN3 about the impacts for supporting Case 3a. Based on the above, we have the following proposal:

Proposal 2: For Case 3a, the signalling procedure in RAN3 specifications from LMF to gNB for transferring Part B information should not indicate the purpose such as “training data collection”.

In our understanding, the AI/ML model training function can also be located in the OAM. This is based on what has been reported in TR 38.843 clause 7.2.4, where the training data generation and termination points are identified for the gNB-side model as follows [4]:

### 7.2.4 Positioning accuracy enhancements **** Skip non-relevant text **** For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: **** Skip non-relevant text **** - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM.  Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. **** Skip non-relevant text ****

Both possibilities concerning the termination point for training data – gNB or OAM – are aligned with the deployment scenarios also specified by RAN3 as part of the “AI/ML for NG-RAN” WI in Rel-18 (refer to TS 38.300 clause 16.20.2). Therefore, we have the following proposal:

Proposal 3: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

## 2.2 Model inference considerations

For Case 3a, to enable the AI/ML model in the gNB to generate measurements for successive determination at LMF side of the UE location, some input data needs to be provided to the AI/ML model itself which will then generate a proper output. In previous RAN1 meetings the following agreements were achieved concerning the AI/ML model input information:

******** RAN1#116 ******** Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. Note: The use of SRS for MIMO resource is transparent to UE. ******** RAN1#118 ******** Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Based on the above agreements, our understanding is that the AI/ML model in the gNB uses UL SRS-based measurements as model input for inference purposes (other than for other procedures such as training and monitoring). And, as per Proposal 2 above and based on RAN1 agreements, the purpose for which information are provided and used shall not be indicated in RAN3 specifications.  RAN1 made a conclusion in RAN1#118 meeting that the existing procedures can be reused for the SRS configuration, even though the details of the existing parameters (e.g., values) can be further discussed.

Proposal 4: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

With regards to the AI/ML model output for Case 3a, i.e., information that could be reported from the NG-RAN to the LMF, RAN1 agreed that at least LOS/NLOS indicator and/or timing information are supported for reporting.

******** RAN1#116 ******** Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

For both pieces of information RAN1 also considered the existing methods already specified, that is, soft/hard indicator for the LOS/NLOS indicator, and UL RTOA (Uplink Relative Time of Arrival) or gNB Rx-Tx time difference for the timing information.

RAN1 progressed further and agreed aspects related to the content of the measurement report from gNB to LMF for Case 3a, as follows:

******** RAN1#118bis ******** Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. ******** RAN1#119 ******** Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements ******** RAN1#120 ******** Conclusion For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF, - Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective.

The first agreement above from RAN1 is related to the presence (mandatory/optional) of components (Information Elements, IEs) that constitute the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported as part of the AI/ML model output for Case 3a. In the RAN1 agreement there is also a Note stating that the final decision of “mandatory” or “optional” presence of each field is up to RAN3.

By checking the latest NRPPa specification [6], the following can be excerpted:

#### 9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |

#### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| *** skip non-relevant parts *** |  |  |  |  |  |  |

Based on the current NRPPa specification, in our understanding the presence of IEs (timing information, quality of the timing information, time stamp) in the measurement report from gNB to LMF in case timing information (either UL RTOA or gNB Rx-Tx difference) is reported is already aligned with the presence as suggested in the RAN1 agreement. Therefore, we think RAN3 should confirm what RAN1 already indicated for the presence of IEs in the measurement report for Case 3a, which has no additional standard impact on NRPPa. This is also motivated by the following agreement and common understanding from the RAN3#126 meeting [6]:

Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

Proposal 5: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

For the LOS/NLOS indicator, which is an optional IE in NRPPa and which is still FFS in RAN1, RAN1 only agreed that if it is reported, it cannot be reported independently from other measurements. In any case, we think that we can reuse the existing IE already defined in NRPPa.

Proposal 6: RAN3 to agree that the LOS/NLOS indicator (pending RAN1 decision on the related FFS) is an optional IE and reuses the existing LoS/NLoS Information IE in NRPPa.

An FFS from RAN3#126 meeting is related on whether the LMF should be made aware of the fact that measurements reported by the gNB are generated by an AI/ML model or they are generated as in legacy positioning [7]:

FFS if LMF should be aware that the reported measurements are AI/ML generated.

In light of the approach used so far by RAN1 and RAN3 to strive to reuse the existing procedures and IEs for positioning measurements, we believe that it would be useful for the LMF to be able to distinguish whether the reporting is related to predicted information. We think that a simple flag “predicted” introduced in the existing TRP Measurement Result IE could serve the purpose, as it is not only a simple solution but also more flexible in case RAN1 decides in the future to include additional predicted information in the reporting to the LMF.

Proposal 7: RAN3 to agree to introduce a flag in the existing TRP Measurement Result IE to allow the LMF to distinguish whether the measurement report from the gNB is related to predicted information. The flag can be an ENUMERATED IE type with codepoint “true”.

## 2.3 Model monitoring considerations

Concerning the AI/ML model monitoring, in RAN3#126 meeting it was agreed that the NG-RAN performs monitoring metric calculation for its own model [7]:

Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

On top of that, we think RAN3 should agree on a general principle concerning the location of the model monitoring function with respect to the entity which is in charge of the AI/ML model inference or, in general, of taking a proper “monitoring decision” for the model based on inference results (e.g., (de-)activation, fallback to legacy positioning methods, etc.). In our view, it would be better that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation, which is also the entity which has knowledge of the model being used. This because in our understanding the “monitoring decision” should be based on the model-specific monitoring metric calculated by considering two main components, that is, the inference results (i.e., model output) and the ground-truth labels.

Proposal 8: RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

A TP for the AI/ML BLCR to TS 38.455 is in Annex 2.

The TP in Annex 2 also includes an initial description of the new function introduced to deliver Part B information from the LMF to the gNB (whose corresponding POSITIONING DATA COLLECTION REPORT message was agreed as in the BLCR to TS 38.305 in [8]), and a TP to the BLCR to TS 38.413 is also provided in Annex 3 accordingly.

# Conclusions

Model training considerations

Proposal 1: RAN3 to agree that, for Case 3a training data collection, the correlation of Part A and Part B relies also on the time stamp of Part B (provided by the LMF to the gNB).

A TP for the AI/ML BLCR to TS 38.305 in [8] reflecting Proposal 1 is in Annex 1.

Proposal 2: For Case 3a, the signalling procedure in RAN3 specifications from LMF to gNB for transferring Part B information should not indicate the purpose such as “training data collection”.

Proposal 3: For AI/ML assisted positioning Case 3a the AI/ML model training can also be located in the OAM other than in the gNB.

Model inference considerations

Proposal 4: The AI/ML model in the gNB for Case 3a uses SRS-based measurements as model input also for inference purposes, by reusing the existing procedures for the SRS configuration.

Proposal 5: RAN3 to confirm the RAN1 agreement that, when timing information is reported for Case 3a, the presence of IEs in the measurement report from gNB to LMF is as in legacy NRPPa, that is:

- Timing information: mandatory IE

- Quality of the timing information: optional IE

- Time stamp: mandatory IE.

Proposal 6: RAN3 to agree that the LOS/NLOS indicator (pending RAN1 decision on the related FFS) is an optional IE and reuses the existing LoS/NLoS Information IE in NRPPa.

Proposal 7: RAN3 to agree to introduce a flag in the existing TRP Measurement Result IE to allow the LMF to distinguish whether the measurement report from the gNB is related to predicted information. The flag can be an ENUMERATED IE type with codepoint “true”.

Model monitoring considerations

Proposal 8: RAN3 to agree the general principle that the entity hosting the AI/ML model is also the entity that performs the model monitoring metric calculation and takes the corresponding monitoring decision.

A TP for the AI/ML BLCR to TS 38.455 is in the Annex 2.

A TP for the AI/ML BLCR to TS 38.413 is in the Annex 3.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107

- R3-253506, (Draft Reply LS to RAN2’s R3-253010 & TP for AI/ML BLCR to TS 38.455) Discussion on RAN2 Reply LS on signalling feasibility of dataset and parameter sharing & RAN3 impacts for Direct AI/ML positioning (Case 3b), Huawei

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- R3-252486, (TP to TS 38.305) Model training at gNB for Case 3a, CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT

- R3-252487, LS on AI/ML Positioning Case 3b, RAN3 (Ericsson)

- 3GPP TS 38.455, NG-RAN; NR Positioning Protocol A (NRPPa), Rel-18

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- R3-252500, Introduction of AIML air, CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT

# Annex 1 – TP for the AI/ML BLCR to TS 38.305 (based on R3-252500)

START OF CHANGES

## 7.x	NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

### 7.x.1	General

Support for NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning requires inputs of ground truth and related data from LMF and measurements by the gNB.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve timing information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

4.	The LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes UE’s timing information and correlation information that enables the gNB to correlate the UE’s timing information with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID, time stamp of the UE’s timing information).

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

END OF CHANGES

# Annex 2 – TP for the AI/ML BLCR to TS 38.455

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

# 7		Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

*** skip unmodified text ***

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Positioning Data Collection Information Transfer. This function allows the LMF to provide the NG-RAN with information for the purpose of NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning data collection. FFS if further text refinements are neeeded.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| *** skip unmodified parts *** |  |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Positioning Data Collection Information Transfer | Positioning Data Collection Reporting |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

*** skip unmodified parts ***

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| *** skip unmodified parts *** |  |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Positioning Data Collection Reporting | POSITIONING DATA COLLECTION REPORT |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 8.2.XX	Positioning Data Collection Reporting

#### 8.2.XX.1		General

The purpose of Positioning Data Collection Reporting procedure is to provide information to be used for, e.g., NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning.

#### 8.2.XX.2		Successful Operation

Figure 8.2.4.2-1: E-CID Measurement Termination procedure, successful operation

The LMF initiates the procedure by sending the POSITIONING DATA COLLECTION REPORT message to the NG-RAN node.

FFS the actual content of the message.

#### 8.2.XX.3		Unsuccessful Operation

FFS.

#### 8.2.XX.4		Abnormal Conditions

FFS.

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.1.X	Messages for Positioning Data Collection Information Transfer Procedures

#### 9.1.X.1		POSITIONING DATA COLLECTION REPORT

This message is sent by the LMF to provide the NG-RAN node with information to be used for, e.g., NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | Reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | Reject |
| FFS other IEs |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | Ignore |
| >Predicted Indication (NAME FFS) | C-ifMeasurements |  | ENUMERATED(true,…) | If present, it indicates predicted TRP Measurement Result | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

| Condition | Explanation |
| --- | --- |
| ifMeasurements | This IE shall be present if the Measured Results Value Time of the Measured Result Item IE is set to either “UL RTOA Measurement” or “gNB Rx-Tx Time Difference”. |

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

*** skip unmodified parts ***

id-SRSPosPeriodicConfigHyperSFNIndex

id-PredictedIndication(NAME-FFS)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- P

PathlossReferenceInformation ::= SEQUENCE {

pathlossReferenceSignal					PathlossReferenceSignal,

iE-Extensions					ProtocolExtensionContainer { { PathlossReferenceInformation-ExtIEs } } OPTIONAL,

...

}

*** skip unmodified parts ***

PointA  ::= INTEGER (0..3279165)

PredictedIndication(NAME-FFS) ::= ENUMERATED (true,...)

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

#### -- T

TAC ::= OCTET STRING (SIZE(3))

*** skip unmodified parts ***

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional},

...

}

*** skip unmodified parts ***

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSResourcetype	CRITICALITY ignore EXTENSION SRSResourcetype PRESENCE optional}|

{ ID id-ARP-ID		CRITICALITY ignore EXTENSION ARP-ID 	PRESENCE optional}|

{ ID id-LoS-NLoSInformation		CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional }|

{ ID id-Mobile-TRP-LocationInformation	CRITICALITY ignore EXTENSION Mobile-TRP-LocationInformation	PRESENCE optional }|

{ ID id-MeasuredFrequencyHops			CRITICALITY ignore EXTENSION MeasuredFrequencyHops					PRESENCE optional }|

{ ID id-AggregatedPosSRSResourceID-List	CRITICALITY ignore EXTENSION AggregatedPosSRSResourceID-List	PRESENCE optional }|

{ ID id-MeasBasedOnAggregatedResources			CRITICALITY ignore EXTENSION MeasBasedOnAggregatedResources PRESENCE optional }|

{ ID id-PredictedIndication(NAME-FFS)	CRITICALITY ignore EXTENSION PredictedIndication(NAME-FFS)	PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

*** skip unmodified parts ***

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-SRSPosPeriodicConfigHyperSFNIndex							ProtocolIE-ID ::= 165

id-PredictedIndication(NAME-FFS)								ProtocolIE-ID ::= aaa

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>

# Annex 3 – TP for the AI/ML BLCR to TS 38.413

<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>

## 8.10	NRPPa Transport Procedures

### 8.10.1	General

The purpose of the NRPPa Transport procedures is to carry NRPPa signalling (defined in TS 38.455 [19]) between the NG-RAN node and the LMF over the NG interface.

The Downlink UE Associated NRPPa Transport procedure and the Uplink UE Associated NRPPa Transport procedure use UE-associated signalling. The UE-associated signalling is used to support E-CID Location Information Transfer, Positioning Information Transfer, Measurement Preconfiguration Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized UE-associated signalling.

The Downlink Non UE Associated NRPPa Transport procedure and the Uplink Non UE Associated NRPPa Transport procedure use non-UE associated signalling. The non-UE associated signalling is used to support OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer, PRS Information Transfer, Area-specific Information Transfer, Positioning Data Collection Information Transfer and Reporting of General Error Situations due to reception of an NRPPa message that utilized non-UE associated signalling.

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>